3GPP TSG-RAN WG4 Meeting # 117	R4- 2520717

# Dallas, USA, 17th – 21st November, 2025

Source:	LG Electronics

Title:	Discussion of general aspects and workplan for AI/ML mobility

Agenda item:	7.12.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s moderator summary [2] and WF [1].

Discussion

- Sub-topic 1-2: General issues for AI mobility

### Issue 1-2-2: Scenarios and sub-use cases

Agreement: RAN4 to discuss the potential RRM requirements for cell-level based prediction for the following scenarios: Evaluation scenario; Methodology. FR1 to FR1 intra-frequency temporal domain case A; intra-cell. FR1 to FR1 intra-frequency temporal domain case B; Intra-cell. FR1 to FR1 inter-frequency (frequency domain) (co-located case); Inter-cell. FR2 to FR2 intra-frequency temporal domain case A; Intra-cell. FR2 to FR2 intra-frequency temporal domain case B; Intra-cell. FFS whether to discuss the requirements for the following scenario: Evaluation scenario; Methodology. FR2 to FR2 inter-frequency (frequency domain) (co-located case); Inter-cell. Agreement: - As starting point, evaluate sub-case 2, L3-L3 measurement prediction as baseline for both FR1 and FR2. - Interested companies can also provide results for other sub-cases. - It is not precluded to add additional sub-case for FR2 in future meetings. - Further discuss how to define requirements based on the available simulation results. Ad-hoc agreements on Thursday: Agreement: As starting point, evaluate non-cluster (single-cell input and single-cell output) approach for frequency domain prediction as baseline. - Interested companies can also provide results for cluster approach. Further discuss the detailed simulation assumptions. - Further discuss how to define requirements based on the available simulation results. Way forward: - Consider the following frequency pairs as baseline for frequency domain prediction evaluation. - Candidate option 1: UE use the measurement on 2GHz to predict 4GHz. - Candidate option 2: UE use the measurement on 1.8GHz to predict 3.5GHz. - Candidate option 3: UE use the measurement on 1.8GHz to predict 2.1GHz. - Other options are not precluded. - Further discuss how to define requirements based on the available simulation results.

For frequency domain prediction evalution, the co-located case only considered. So, the specific frequency value is not important. The specific frequency value impact only to the offset between two carrier frequencies by pathloss. Considering the free space path loss definition, the path loss depends on distance between Tx and Rx and carrier frequency.

FSPLdB=20log10d [km]+20log10f [MHz]+32.45

In a co-located scenario, the distance between the UE and Cell 1 (d1) and the distance between the UE and Cell 2 (d1) are identical. So, the path loss depends only on carrier frequency.

Considering the candidate option 1, the pathloss offset is roughly 6 dB. Therefore, if the transmit power of the two cells is identical, it implies that, under ideal conditions, the received power at the UE from the two cells will consistently differ by approximately 6 dB.

We think that for frequency domain prediction evaluation scenario perspective, the offset value is not important. Accordingly, the specific frequency value pair is also deemed less critical. Instead, determining the correlation coefficient between the two frequencies is considered to be of greater significance. So, rather than spending significant time selecting specific frequency pairs, it is recommended to focus on identifying the correlation coefficient between the two frequencies.

Proposal 1: We would like to focus on identifying the correlation coefficient between the two frequencies, rather than spending significant time selecting specific frequency pairs. We are fine any frequency pair.

### Issue 1-2-3: Time point for UE prediction in temporal domain

- Proposal 1: (MTK) - For temporal prediction Case A and Case B, when and how to do the measurement and prediction is totally up to UE.

At the ultimate destination of AI, we believe MTK’s proposal represents a reasonable direction. However, in Rel-20, the rules are already under discussion, and following those rules is a necessary step before reaching AI’s ultimate goal. Therefore, we think the time point for UE prediction in temporal domain should follow the network configuration.

Proposal 2: RAN4 to consider that the time point for UE prediction in temporal domain should follow the network configuration.

### Issue 1-2-4: SSB transmission for temporal domain prediction

- Proposal 1: (MTK) - For temporal prediction Case B, only consider the case that NW transmit SSB periodically.

We believe that the UE’s temporal prediction pattern is independent of the network’s SSB transmission schedule. Since the network does not communicate with the UE on a one-to-one basis, SSB scheduling should remain consistent regardless of individual UE configurations. If there is any different agreement in RAN2, we should follow that; otherwise, it is reasonable to assume that SSBs are transmitted periodically by the network.

Proposal 3: If there is any different agreement in RAN2, we should follow that; otherwise, it is reasonable to assume that SSBs are transmitted periodically by the network.

### Issue 1-2-5: Impact of cell identification for frequency domain prediction

- Proposal 1: (MTK) - For scenario 3 (inter-frequency measurement prediction), discuss how UE gets cell ID of the to-be-predict cell(s) for reporting the prediction results to network.

We would like to focus on analyzing whether the AI/ML prediction performance is good or not for scenario 3. How UE gets cell ID of the to-be-predict cell for reporting the prediction results to network is not important for this WI.

Proposal 4: RAN4 doesn’t need to discuss how UE gets cell ID of the to-be-predict cell(s) for reporting the prediction results to network for scenario 3.

### Issue 1-2-6: Ground truth for predicted L3-RSRP

- Option 1: (OPPO, HW, ZTE, Nokia) - Use Alt2 (i.e., reported measurement value under certain conditions) to define the ground truth in FR1. - How to avoid UE cheating can wait the progress in R19 AI interface beam management test (HW) - If using Alt2: - Proposal 4: (MTK) - If to use reported RSRP as ground truth, the ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. Y is the defined measurement delay considering all kinds of UE implementation. - Proposal 5: (QC) - Rephrase Alt 2 as “Ground truth is defined as the measurement report from the UE” - Add the following clarifications: - The UE measurement reports used as ground truth are collected under the same channel fading conditions as those used for the L3-RSRP prediction reports. - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The channel power of the target prediction reference signal remains stable within ±Z dB during the time window from X - Y ms to X + Y ms. - The S(I)NR of the target cell during the ground truth measurement may or may not be the same as that used during AI/ML based prediction result measurement.

We support option 1 which use Alt2 to define the ground truth in FR1.

The predicted L3-RSRP for time instance X can be used as the ground truth for the L3-RSRP measured between X-Y and X+Y. The measurement time, not the reporting time, must be within a certain interval compared to the prediction time.

Proposal 5: Support option 1 which use Alt2 to define the ground truth in FR1. And the L3-RSRP measurement time, not the reporting time, must be within a certain interval compared to the prediction time to use the predicted L3-RSRP as ground truth.

- Sub-topic 1-3: Simulation assumption for AI mobility

### Issue 1-3-3: Simulation aspects for frequency domain prediction

Further discuss based on the open issues in [1] and the following way forward: - Discuss the proposals considering: - How to model the frequency-correlation for frequency domain prediction. - Option 1: RAN4 to consider the RAN2 definition of correlation coefficient as a starting point. - Option 2: Define correlation coefficient for inter-frequency predictions as the correlation between PDPs of different frequency layers channel profiles. - Whether and how to set varied BS configurations for different frequencies.

Although there is no strong view, it would be good to set the correlation coefficient defined in RAN2 as the starting point.

Proposal 6: Support option 1. It would be good to set the correlation coefficient defined in RAN2 as the starting point.

### Issue 1-3-4: UE trajectory assumption in the simulation

- Proposal 1 (Ericsson): - RAN4 to discuss whether to define a reference UE trajectory to be considered for AI/ML aided mobility simulations.

In AI/ML-aided mobility, the prediction results of the AI/ML model must follow slow fading, and since slow fading is affected by the UE trajectory, UE trajectory modeling is essential for AI/ML-aided mobility simulations.

Proposal 7: UE trajectory should be considered for AI/ML-aided mobility simulations.

### Issue 1-3-5: Data filtering during the simulation

- Proposal 1 (MTK): - When do the simulations, the data containing cells with too low SNR (e.g., <-6dB) should be excluded from the dataset.

We don’t have strong view in this issue.

### Issue 1-3-6: Impact of measurement error

Further discuss based on the open issues in [1] and the following way forward: - Discuss the error model assumption for RF error taking the following as baseline: - Use the following assumption for RF error model in simulation: - Truncated Gaussian distribution under [±4] dB RF error (μ=0, σ2=[4]) for FR2 - Truncated Gaussian distribution under [±2.5] dB RF error (μ=0, σ2=[2.5]) for FR1 - RAN4 to not consider different RF error values for different RF chains for RRM measurement prediction simulations. - Discuss how to add the measurement error including BB and RF in the simulation based on proposals 5-8.

We think that an independent RF error value per RF chain is more realistic.

Proposal 8: RAN4 to consider independent RF error values per RF chains for RRM measurement prediction simulations.

- Conclusion

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s moderator summary [2] and WF [1], and we propose

Proposal 1: We would like to focus on identifying the correlation coefficient between the two frequencies, rather than spending significant time selecting specific frequency pairs. We are fine any frequency pair.

Proposal 2: RAN4 to consider that the time point for UE prediction in temporal domain should follow the network configuration.

Proposal 3: If there is any different agreement in RAN2, we should follow that; otherwise, it is reasonable to assume that SSBs are transmitted periodically by the network.

Proposal 4: RAN4 doesn’t need to discuss how UE gets cell ID of the to-be-predict cell(s) for reporting the prediction results to network for scenario 3.

Proposal 5: Support option 1 which use Alt2 to define the ground truth in FR1. And the L3-RSRP measurement time, not the reporting time, must be within a certain interval compared to the prediction time to use the predicted L3-RSRP as ground truth.

Proposal 6: Support option 1. It would be good to set the correlation coefficient defined in RAN2 as the starting point.

Proposal 7: UE trajectory should be considered for AI/ML-aided mobility simulations.

Proposal 8: RAN4 to consider independent RF error values per RF chains for RRM measurement prediction simulations.

.

Reference

[1] R4-2514827, “WF on NR_AIML_Mob_RRM”, Moderator(OPPO)

[2] R4-2513532, “Topic summary for [116bis][226] NR_AIML_Mob_RRM”, Moderator(OPPO)