# 3GPP TSG-RAN WG4 Meeting #117	R4-2520732

# Dallas, US, 17th – 21st November, 2025

Source: 	vivo

Title: 	Discussion on HPUE for NR single carrier operation

Agenda Item:	7.1.4

Document for:	Approval

Introduction

In RAN#109, a new WID for UE RF enhancements for NR FR1 for Phase 5 was approved [1]. One objective is to introduce HPUE for single carrier operation, i.e. single carrier TDD to support 4Tx for CPE/FWA UE, PC1.5 with 2Tx for FDD bands for handheld and FWA UE. In the last meeting, RAN4 discussed these HPUE topics and the WF was captured in [2]. In this contribution, we further discuss on these two HPUE topics.

Discussion

Single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

The objective for single carrier TDD to support 4Tx for CPE/FWA is captured as follows:

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class

The 3GPP standardization effort is considering defining a maximum transmitter output power level of 32 dBm. Two primary technical approaches have been identified: one way is to reuse PC1 and another is to define a new power class.

In the last meeting, we had the following issue on power class:

Issue 2-1-2: Power class - Proposals - Option 1: Use PC1 (ZTE, Samsung, Skyworks, Vivo, LGE) - Option 2: PC1 with 1dB power boosting (Ericsson, Skyworks, Vivo) - Option 3: PC1Bis (Nokia, Ericsson, Huawei, NTT Docomo, OPPO, Spreadtrum, MediaTek) - Recommended WF - TBA

From our perspective, we prefer to reuse PC1 instead of introducing a new power class in Rel-20. Usually, PC1 is often associated with a nominal max of 31 dBm with +2/-3 dB tolerance. Actually, it is 33 dBm with +2dB tolerance, which can support up to 32 dBm maximum output power. However, strictly speaking, power tolerance cannot always guarantee the maximum output power 32dBm, e.g. when the tolerance is -3dB. It is OK for us just reusing current PC1 definition.

If we define the maximum output power up to 32dB without considering tolerance, the general power booting scheme with 1dB can be considered with PC1 definition. Power boosting scheme was widely specified in current TN and NTN specifications. It allows UE to transmit at a slightly higher output power than its baseline power class (e.g., +1dB above PC1’s nominal maximum) under specific conditions.

From our perspective, reusing PC1 has the minimal standard impact compared to the way of introducing new power class. Changes would focus primarily on specifying the boost conditions and tolerances.

Proposal 1: It is suggested to reuse PC1 with or without general power boosting scheme to achieve up to 32 dBm in Rel-20.

For 4Tx ULFPTx, we had the following issues need to be discussed:

- 4Tx ULFPTx Issue 2-3-1: Mode-1 and Mode-2 feasibility - Proposals - Option 1: It should be confirmed whether both Mode-1 and Mode-2 are feasible for both single layer and two layers. If not, discuss how to capture it in the specifications. (Ericsson) - Option 2: Only Mode-1 is feasible for both single and two layers (ZTE) - Recommended WF - TBA Issue 2-4-1: Mode-full power feasibility for PC2 - Proposals - Option 1: Mode-full power is feasible for PC2. Discuss whether it can be introduced in this WI. (Ericsson) - Recommended WF - TBD

For a 4Tx configuration with a 4x26dBm PC1 power constraint, under Mode-1 operation, none of the PAs operate at full PC1 power. In contrast, Mode-2 requires at least one PA to transmit at full PC1 power. However, this is unachievable within the specified 4x26dBm PC1 configuration. Therefore, we believe only Mode-1 is a viable solution for this configuration.

Proposal 2：It is suggested to only consider Mode-1 for 4Tx with 4*26dBm PA configuration for uplink full power transmission.

For 4Tx with 4*26dBm PA configurations, it is meaningless to discuss mode-full power feasibility for PC2. While technically feasible within standards (single-Tx operation at 26 dBm aligns with PC2’s total power limit), introducing this for 4Tx (4×26 dBm) devices is unreasonable. No rational manufacturer would design hardware capable of PC1 certification (4*26 dBm total power), deliberately downgrade it to PC2, then implement ULFPTx to re-enable 26 dBm on a single chain—when achieving native PC1 offers identical performance at lower cost/complexity. The use case for ULFPTx resides in constrained UE designs (e.g., 2Tx PC2), not high-tier 4Tx devices.

Observation 1：No rational manufacturer would design hardware capable of PC1 certification, deliberately downgrade it to PC2.

Proposal 3: Do not consider mode-full power mode for PC2 with 4x26dBm PA configurations.

Another issue about 4Tx PC1 is the applicable bands:

Issue 2-4-2: 4Tx PC1/PC1bis requirements to be band agnostic - Proposals - Option 1: FDD and TDD bands are supported (Huawei) - Option 2: Requirements only for the exemplary TDD bands (Skyworks) - Recommended WF - TBD

According to the WID, we think it is clear to say ‘specify the band specific requirements for exemplary bands’. The example bands are n41, n77/n78, n79, n104, all TDD bands. From feasibility point of view, we think it is also feasible for FDD bands. However, it needs conformation in RAN plenary. We suggest the proponent of Option 1 can bring this issue in RAN plenary. Before that, we can follow the WID and only consider exemplary TDD bands.

Proposal 4: Follow the WID and only consider the exemplary TDD bands for 4Tx PC1.

PC1.5 with 2Tx for FDD bands for handheld and FWA UE

Another aspect of HPUE for NR single carrier operation is to specify UE RF requirements for PC1.5 with 2Tx for FDD bands for handheld and FWA UE. RAN4 already specified PC2 for FDD band in Rel-18. Besides adding higher power class for FDD band, other RF requirements can refer to TDD HPUE requirements. For above PC1.5 with 2Tx for FDD bands for handheld and FWA UE, we didn’t expect many technical issues for supporting this HPUE for FDD bands. Requirements for PC1.5 with 2Tx for TDD bands can be used as a starting point for PC1.5 with 2Tx for FDD bands for handheld and FWA UE.

One remaining issue for PC1.5 with 2Tx for FDD bands was REFSENS degradation.

REFSENS degradation Agreement: Encourage companies to provide the general methodology for deriving the REFSENS degradation (RSD) in next meeting.

RSD (REFSENS Degradation) occurs when an increase in the UE's transmit power potentially raises the interference level at its own receiver, leading to degraded sensitivity performance. Analyzing RSD critically depends on the performance of the duplexer within the specific FDD band. The duplexer's isolation directly determines the level of interference coupled into the receiver band. Additionally, whether the transmitter supports Tx diversity can also potentially impact RSD.

For RSD analysis methodology, we give our suggestions:

Step 1: Set UL frequency, BW, Tx Power, calculate the interference IRX falling into Rx BW, considering interference coupling pathes, e,g. the isolation of the duplexer, spatial coupling gain, Tx non-linear components.

Step 2: RSD calculation according to IRX

RSD(dB)=10lg(1+IRxN0)

Anyway, RSD analysis should be band specific. More directly, measurement provides superior fidelity for RSD characterization compared to analytical modeling.

Observation 2：The general methodology for deriving RSD needs to consider interference coupling paths, e.g. the isolation of the duplexer, spatial coupling gain, Tx non-linear components.

Conclusion

This contribution further discusses on HPUE requirements for NR single carrier operation. The following observations and proposals are made:

Proposal 1: It is suggested to reuse PC1 with or without general power boosting scheme to achieve up to 32 dBm in Rel-20.

Proposal 2：It is suggested to only consider Mode-1 for 4Tx with 4*26dBm PA configuration for uplink full power transmission.

Observation 1：No rational manufacturer would design hardware capable of PC1 certification, deliberately downgrade it to PC2.

Proposal 3: Do not consider mode-full power mode for PC2 with 4x26dBm PA configurations.

Proposal 4: Follow the WID and only consider the exemplary TDD bands for 4Tx PC1.

Observation 2：The general methodology for deriving RSD needs to consider interference coupling paths, e.g. the isolation of the duplexer, spatial coupling gain, Tx non-linear components.

- References

[1] RP-252952, New WID: UE RF enhancements for NR FR1, Phase 5, RAN#109.

[2] R4-2515087, Way Forward for [116bis][316] NR_UE_RF_Ph5, Nokia, Murata, RAN4#116bis.