3GPP TSG-RAN WG4 Meeting #117	R4-2520747

# Dallas, USA, 17th Nov 2025 – 21st Nov 2025

Agenda item:	8.4.2

Source: 	Viasat, Thales, Airbus,Terrestar

Title: 	Views on NTN related coexistence for 6G

Document for:	Approval

## 1.	Introduction

During the RAN4#116bis meeting, as part of the WF [1], it was requested that interested NTN companies to provide additional input on aspects related to NTN SAN RF requirements and coexistence study list.

The WF also provided a template for requirements analysis and guidance on filling the table. This paper provides our assessment on the study of SAN requirements as part of the 6G study.

## 2.	Discussion

There are several aspects that impact on 6GR SAN RF and likely have to be studied.

- NR BS RF specifications were used as the starting point for specifying SAN RF requirements for NR NTN. For many 6GR SAN requirements, that will also be the case. However, for certain requirements, based on unique aspects of satellite architecture, certain allowances unique may have to be considered in the specification for 6GR SAN

- In 6G, newer waveforms (e.g. DFT-s-OFDM) and SAN specific PA models may be considered which may lead to updates to the currently specified ACLR, dynamic range requirements.

- New modulation schemes may be introduced in 6GR which will require study from SAN perspective.

- In NR NTN, feeder link compensation was assumed to ideal; real-world deployment are non-ideal. A study is required how to capture impacts from non-ideal feeder links on transmit signal quality.

- Introduction of new requirements such as TAE will be required since carrier aggregation is expected to be specified from the 1st release of 6GR.

      - It is expected that new satellite orbits such as MEO, VLEO and polar orbits will be specified as part of 6G.

      - In addition to orbits, it is expected that new spectrum bands (e.g., Q/V band) may be introduced as part of 6G and may have its own unique RF constraints.

- Satellite operators have started to leverage their GSO systems to offer 3GPP based NTN services. As next generation NGSO systems are considered, there might regulatory requirements that may have to be met as alluded to in Annex C of TR 38.821 to minimize performance of GSO systems.

- As more operators receive market authorization in each band due to the increasing desire of administrations to offer satellite-based connectivity, it becomes important to ensure that adjacent channel operations remain unaffected.

- ITU-R is evaluating use of TN spectrum for NTN use; several regulators have already started to authorize use of TN spectrum for NTN [4,5]. Regulations in terms of EPFD limits are specified to ensure adjacent TN system performance is not degraded. This and above two bullets may require addition of new spurious co-existence clause that is currently not included.

Based on above considerations, a preliminary assessment of potential SAN RF requirements impacts is listed below.

## 3.	SAN RF requirements assessment

| 5G SAN RF requirements | No impact expected, might be reused as is for 6G | To be re-evaluated based on system parameters decision for 6G (SU, channel BW, …), following TR 38.817-02 principles/formula | Would need to be studied in the 6G SI scope |
| --- | --- | --- | --- |
| Conducted requirements |  |  |  |
| SAN output power |  | X (based on additional orbits, new SAN classes) |  |
| Output power dynamics |  |  |  |
| RE power control dynamic range |  | X (based on waveform, PA model) |  |
| Total power dynamic range |  | X (based on evolved SAN architecture, PA model, waveform) |  |
| NB-IoT RB power dynamic range for in-band operation | X |  |  |
| Transmit ON/OFF | X |  |  |
| Transmitted signal quality |  |  |  |
| Frequency error |  |  | X (VLEO, Q/V bands, non-ideal feeder link, etc.) |
| Modulation quality |  | X (potential new mod schemes) | X (real world experience from SAN development, SAN PA models) |
| Time alignment error |  |  | X (CA is expected to be part of initial release of 6G) |
| Unwanted emissions |  |  |  |
| Occupied bandwidth | X (per ITU-R) |  |  |
| ACLR |  |  | X (re-evaluate based on coexistence studies and other classes of NTN SAN, e.g., VLEO, MEO; integration of waveform models) |
| Operating Band Unwanted Emissions | X (per ITU-R) |  |  |
| Transmitter spurious emission |  |  |  |
| General requirement | X (per ITU-R) |  |  |
| Protection of BS receiver of own or different BS | X |  |  |
| Additional spurious (coex) |  |  | X (Based on NTN (SAN) to NTN (SAN) coexistence |
| Co-location | X |  |  |
| Transmitter intermodulation | X |  |  |
| Receiver sensitivity level | X (reuse for GEO/LEO) | X (new SAN class, e.g. VLEO, MEO, new RMCs/FRCs) | X (new channel models) |
| Dynamic range | X (reuse for GEO/LEO) | X (new SAN class, e.g. VLEO, MEO, new RMCs/FRCs) | X (new channel models) |
| In-band selectivity and blocking |  |  |  |
| ACS | X (reuse for GEO/LEO) | X (new values may be required for other orbits if introduced) |  |
| In-band blocking | X |  | X (may have to be evaluated for corner SAN – SAN cases) |
| Out of band blocking | X |  | X (may have to be evaluated for corner SAN – SAN cases) |
| Receiver spurious emissions | X |  |  |
| Receiver intermodulation | X |  |  |
| In channel selectivity | X (reuse for GEO/LEO) | X (new values may be required for other orbits if introduced, new RMCs/FRCs) |  |
| Radiated requirements |  |  |  |
| Radiated transmit power |  |  |  |
| OTA base station output power |  | X (based on additional orbits, new SAN classes) |  |
| OTA output power dynamics |  |  |  |
| OTA RE power control dynamic range |  | X (based on waveform, PA model) |  |
| OTA Total power dynamic range |  | X (based on evolved SAN architecture, PA model, waveform) |  |
| NB-IoT RB power dynamic rage for in-band operation | X |  |  |
| OTA transmit ON/OFF | X |  |  |
| OTA transmitted signal quality |  |  |  |
| OTA Frequency error |  |  | X (VLEO, Q/V bands, non-ideal feeder link, etc.) |
| OTA Modulation quality |  | X (potential new mod schemes) | X (real world experience from SAN development, SAN PA models) |
| OTA Time alignment error |  |  | X (CA is expected to be part of initial release of 6G) |
| OTA Unwanted emissions |  |  |  |
| OTA Occupied bandwidth | X (per ITU-R) |  |  |
| OTA ACLR |  |  | X (re-evaluate based on coexistence studies and other classes of NTN SAN, e.g., VLEO, MEO, integration of waveform models) |
| OTA Operating Band Unwanted Emissions | X (per ITU-R) |  |  |
| OTA Transmitter spurious emission |  |  |  |
| OTA General requirement | X (per ITU-R) |  |  |
| OTA Protection of BS receiver of own or different BS | X |  |  |
| OTA Additional spurious (coex) |  |  | X (Based on NTN (SAN) to NTN (SAN) coexistence, may have to consider EPFD like specifications |
| OTA Co-location | X |  |  |
| OTA transmitter intermodulation | X |  |  |
| OTA sensitivity |  | X (new SAN class, e.g. VLEO, MEO, new RMCs/FRCs) | X (new channel models) |
| OTA receiver sensitivity level | X (reuse for GEO/LEO) | X (new SAN class, e.g. VLEO, MEO, new RMCs/FRCs) | X (new channel models) |
| OTA dynamic range | X (reuse for GEO/LEO) | X (new SAN class, e.g. VLEO, MEO, new RMCs/FRCs) | X (new channel models) |
| OTA in-band selectivity and blocking |  |  |  |
| OTA ACS | X (reuse for GEO/LEO) | X (new values may be required for other orbits if introduced) |  |
| OTA In-band blocking | X |  | X (may have to be evaluated for corner SAN – SAN cases) |
| OTA out of band blocking | X |  | X (may have to be evaluated for corner SAN – SAN cases) |
| OTA receiver spurious emissions | X |  |  |
| OTA receiver intermodulation | X |  |  |
| OTA in channel selectivity | X (reuse for GEO/LEO) | X (new values may be required for other orbits if introduced, new RMCs/FRCs |  |

Proposal 1: Adopt Table 1 as a starting point for further studies and determination of the priorities amongst SAN RF requirements.

## 3.	MSR Specifications for SAN

In 5G, MSR specifications for SAN were not developed. It is quite likely that the same satellite node will support multiple radio interfaces via multi-RAT spectrum sharing (5G NR and 6GR).

Observation 1: A satellite node is expected to support multiple radio standards such as 5G NR, 6GR.

As part of the 6G effort, a new specification that encompasses, multiple standards such as LTE, NB-IoT, 5G NR and 6G radio along the lines of 37 series BS specifications need to be developed for SAN. As part of the 6GR study, an outline of the specification needs to be developed to guide the normative work in Release 21.

Proposal 2:  Assess and develop an outline/scope for the MSR specification for SAN.

## 3.	Conclusion

Proposal 1: Adopt Table 1 as a starting point for further studies and determination of the priorities amongst SAN RF requirements.

Proposal 2:  Assess and develop an outline/scope for the MSR specification for SAN.

## 4.		References

[1]	R4-2514638, WF on 6G BS RF and co-existence, feature lead (Ericsson)

[2]	TS 38.108, version 19.2.0