3GPP TSG-RAN WG4 Meeting#117		R4-2520761

Dallas, USA, Nov 17th - 21th, 2025

Agenda item:	8.2.1

Source: 	Spreadtrum, UNISOC

Title: 	Views on 6G waveform

Document for:	Discussion

Introduction

In RAN4#116bis meeting, a WF [1] about RAN4 6G system parameters was approved. In this contribution, we want to continue to provide some views on 6G waveform.

Discussion

Waveform

- Agreement - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1 - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects - Tx assumption including PA model - Related RF requirements which should be taken into consideration - Both UL and DL are considered - Take the candidate waveforms identified/discussed in RAN1 as the baseline - Identify the evaluation metric, e.g. net gain - Implementation constraints - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs - Evaluation cases could be selected step-by-step, based on RAN1 progress - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement

From RAN4 perspective, waveform determines the time domain and frequency domain characteristics of RF signals and RF requirements will be impacted by waveform. For uplink, from the perspective of frequency domain, in our understanding, ALCR, SEM and RF impairments including phased noise need to be taken into consideration; from the perspective of time domain, in our understanding, MPR/PAPR needs to be taken into consideration; from the point of modulation quality, EVM, frequency error and IQ mismatch need to be taken into consideration. For downlink, REFSENS needs to be taken into consideration. In summary, for uplink, RF requirements including EVM, ACLR, SEM, MPR/PAPR and RF impairments (e.g., IQ mismatch, phase noise) needs to be considered. For downlink, RF requirements including REFSENS and RF impairments (e.g., IQ mismatch, phase noise) needs to consider.

Proposal 1: For uplink, RF requirements including EVM, ACLR, SEM, MPR/PAPR and RF impairments (e.g., IQ mismatch, phase noise) needs to consider for waveform evaluation.

Proposal 2: For downlink, RF requirements including REFSENS, max input level, ACS, blocking and RF impairments (e.g., IQ mismatch, phase noise) needs to be considered for waveform evaluation.

Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding Agreement - Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

According to the RAN1’s 122bis meeting agreements, uplink low-PAPR waveform and DFT-s-OFDM for UL with number of layers ＞1 are under studying. For uplink low-PAPR waveform, net gain was agreed to be evaluation metric. For DFT-s-OFDM UL with number of layers, from RAN4 perspective, MPR for UL MIMO includes DFT-s-OFDM and CP-OFDM, for PUSCH configuration for uplink full power transmission doesn’t include >1 layer of DFT-s-OFDM.

Observation 1: Net gain is assumed metric for waveform evaluation in RAN1.

Observation 2: For DFT-s-OFDM UL with number of layers, from RAN4 perspective, PUSCH configuration for uplink full power transmission doesn’t include > 1 layers of DFT-s-OFDM.

PA model

RAN4 has received a LS [2] from RAN1 relating to PA models. And RAN4 has discussed about PA models and reached an agreement based on the LS as follow.

- Agreement - Consider PA modelling at least for the following aspects for RAN4 discussion - Memory effects for UE supporting larger CBW - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3) - Calibration conditions - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc. - PA models for different device types, handheld UE is prioritized - Applicable requirements - 5G-A requirements as starting point - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

The purpose of PA model is to modulate the input-to-output relationship as accurately as possible to assess the performance and inherent characteristics of PA. Then PA model can be applied to system performance simulation and evaluation perfectly. Therefore, it is better to simulate the realistic PA behaviour more accurately.

In 5G initial stage, RAN4 has surveyed some PA models in TR 38.803, some simple PA modes with memoryless, including Rapp Model, Saleh model and the Ghorbani model; and some rigorous models with memory effect, including Volterra series expansion, Hammerstein model, Winner model and Memory Polynomials. In 5G NR, due to the urgency to respond, RAN4 provided two possible PA models for RAN1 work, which are “modified Rapp model” including both AM/AM and AM/PM distortions (quasi-memoryless) and clipping model (memoryless) with different thresholds. In fact, these PA models with memoryless are not very realistic to modulate PA models especially for higher frequency (e.g., around 7GHz and large bandwidth (e.g., 200 MHz). Therefore, RAN4 needs to provide more realistic PA models for RAN1 work and RAN4 evaluation in 6GR. Based on the agreement as above, we need to consider memory effects. We have listed some memory-based models based on TR 38.803 and survey.

Proposal 3: Memory effects need to be considered for PA models.

Volterra series expansion model

The Volterra series is in common words described as a kind of “Taylor series with memory” and is able to model all weak non-linearity with fading memory. Volterra series can be described with the following equation,

yn=m=1Mk=1Kamkxn-m|x[n-m]| k-1   (1)

where K is the highest nonlinear order number, M is the memory length, and amk is the polynomial coefficient. The disadvantage of Volterra series is its high complexity due to the elevated number of parameters, which makes it highly impractical for a model implementation, especially for wide bandwidth applications.

Observation3: Volterra series is highly impractical for a model implementation, especially for large bandwidth.

Memory polynomial model

Common models like for example the memory polynomial can also be seen as a subset of the full Volterra series and can be very flexible in designing the model by simply adding or subtracting kernels from the full series. The MP model is a special case of Volterra series, MP decreases the overall system complexity while still maintaining the accuracy of the memory effects description. The disadvantage is that the parameters have to be empirically solved for the specific amplifier in use.

(2)

Generalized Memory polynomial model

Compared to the MP model, the performance of the MP model is further improved by including some leading and lagging cross-terms such as in the generalized memory polynomial (GMP), which can describe the different latency under different time for large bandwidth. GMP model is mathematically expressed as:

yGMPn=k=0Ka-1l=0La-1aklxn-lx(n-l)k+k=0Kb-1l=0Lb-1m=1Mbbklmxn-lx(n-l-m)k+                                              k=0Kc-1l=0Lc-1m=1Mccklmxn-lx(n-l+m)k     (3)

where x and  yGMP are the model’s input and output waveforms, respectively. Ka and La are the nonlinearity order and the memory depth of the MP sub-model, respectively; KbLband KcLc are the nonlinearity order and the memory depth of the lagging and leading cross-terms, respectively. Mb and Mc are the maximum orders of the lagging and leading cross-terms used in the GMP model. akl, bklm and cklm are the coefficients of the GMP model.

However, with the increasing of the bandwidth, the memory effects are more obvious, the non-linear order and memory depth increase, the MP models including GMP model require a large number of coefficients. It is necessary to consider the trade-off between complexity and accuracy, especially for handheld UE. So these models also require more advanced pruning methods for optimization. A pruning method based on the least absolute shrinkage and selection operator (LASSO) regression for pruning the redundant terms of the GMP model in paper [3].The pruning method takes advantage of the fact that the LASSO regression can prevent overfitting and yield sparse solutions by introducing the L1-norm as a penalty factor into the coefficient extraction process of the GMP model, which can directly reduce the number of coefficients in the model.

The squared error of the GMP model is defined as the loss function:

R=n=1Nyn-i=1Mθixi(n)2      (4)

The solution of the LASSO regression is to introduce the L1-norm in (4) to obtain a new loss function and obtain the coefficient vector θLASSO of the model by minimizing the new loss function. The new loss function is defined as:

R=n=1Nyn-i=1Mθixi(n)2+λi=1Mθi  (5)

Table 1 compares the NMSE, the number of complex coefficients, and the ACPR of the proposed model with the original GMP model. It can be found that when the GMP of coefficients is reduced by about 40%.

Table 1: performance comparison between proposed model and GMP model

| Behavioral Model | NMSE (dB) | Number of Coefficients | ACPR in DPD |
| --- | --- | --- | --- |
| GMP model | -34.76 | 93 | -48.57 |
| GMP model with LASSO | -34.34 | 57 | -47.49 |

Observation4: GMP model with LASSO can be considered to be a trade-off between performance and complexity.

Weighted Memory polynomial model

A weighted memory polynomial (WMP)-based model is proposed in the paper [4] to introduce an instantaneous-power-dependent weight function on the static and dynamic terms of the conventional MP model which gives more significant impact to the strongly dynamic mildly non-linear distortions at low power levels and mildly dynamic strongly nonlinear distortions at high power level. The WMP model equation given by:

yWMPn=i=0NSaiwsxnxnxni+i=0NDj=1MβijwDxnx(n-j)x(n-j)i  (6)

Based on the above analysis and the considering factors which described in the agreements, we need to consider the memory effects, the performance and complexity. In our understanding, we can consider these four options: MP model, GMP model, GMP mode with LASSO and WMP model.

Proposal 4: Regarding around 7GHz PA model, we can consider these four options: MP model, GMP model, GMP model with LASSO and WMP model. The trade-off between performance and complexity is necessary to consider.

In order to modulate the real practical PA in 6GR, some parameters including power supply, PA architecture, with and without DPD need to be clarified. For power supply, the mainstream choices are APT and ET. APT adjusts the voltage of PA by the average power of the signal. ET adjusts the voltage of PA by the envelope of signal. ET can boost the PA efficiency and improve the PA non-linearity to some extent. However, around 7GHz supports at least 200MHz bandwidth. Regarding 200MHz bandwidth, APT is not sensitive to it. ET will not achieve complete tracking, which cause the PA efficiency degradation and performance degradation. Therefore, APT is a more compatible option for large bandwidth. For PA architecture, Class A/AB is mainly used in UE side, Doherty PA is mainly used in NW side. Doherty PA with two power amplifiers, which can improve the PA efficiency obviously. However, Doherty PA has a large size and a higher cost. And it is difficult to integrate with multi PAs in one chip in current stage. The industry is exploring the possibility of using Doherty PA in UE side. Therefore, Class A/AB can be used in 6G day1. For DPD, which is an essential way to improve the PA non-linearity.

Proposal 5: Some parameters for PA model including APT, DPD can be considered in 6GR day1, Doherty PA can be studied in 6GR day2.

Conclusions

In this paper, we provide our initial views on 6G waveform, we have made the following proposals and observations.

Proposal 1: For uplink, RF requirements including EVM, ACLR, SEM, MPR/PAPR and RF impairments (e.g., IQ mismatch, phase noise) needs to consider for waveform evaluation.

Proposal 2: For downlink, RF requirements including REFSENS, max input level, ACS, blocking and RF impairments (e.g., IQ mismatch, phase noise) needs to be considered for waveform evaluation.

Observation 1: Net gain is assumed metric for waveform evaluation in RAN1.

Observation 2: For DFT-s-OFDM UL with number of layers, from RAN4 perspective, PUSCH configuration for uplink full power transmission doesn’t include > 1 layers of DFT-s-OFDM.

Observation3: Volterra series is highly impractical for a model implementation, especially for large bandwidth.

Observation4: GMP model with LASSO can be considered to be a trade-off between performance and complexity.

Proposal 3: Memory effects need to be considered for PA models.

Proposal 4: Regarding around 7GHz PA model, we can consider these four options: MP model, GMP model, GMP model with LASSO and WMP model. The trade-off between performance and complexity is necessary to consider.

Proposal 5: Some parameters for PA model including APT, DPD can be considered in 6GR day1, Doherty PA can be

studied in 6GR day2.

- Reference

- R4-2514650, WF on 6GR system parameter, Huawei, HiSilicon. TSG-RAN4#116bis meeting

- R1-2508069, LS on PA models in 6GR waveform evaluations. Huawei. TSG-RAN1#122bis meeting

- Di Wang,Yiming Lei, Liaoyuan Zeng, “A Pruning Method of the Generalized Memory Polynomial Model for Power Amplifiers Based on the LASSO Regression” in 2023 IEEE 6th International Conference on Electronic Information and Communication Technology

- Abdalla E. Abdelrahman and Andrew K. Kwan, “A Novel Weighted Memory Polynomial for Behavioral Modeling and Digital Predistortion of Nonlinear Wireless Transmitters” in IEEE TRANSACTIONS ON INDUSTRIAL ELECTRONICS, VOL. 63, NO. 3, MARCH 2016