# 3GPP TSG-RAN WG4 Meeting #117	R4-2520800

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.10.2.1

Source:	China Telecom

Title:	Discussion on RRM performance requirements for FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN4 #116bis meeting, RRM performance requirements for FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements for FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1] and summary [2]:

Issue 1-1-1: Whether to test FBS quitting Further discuss this issue based on the options in the topic summary R4-2513511. - Proposals - Option 1: (CT, LGE, MTK) - Not to test FBS quitting. - Option 2: (Huawei) - Define TC to test whether the UE will exit fast BSF measurement after duration of T1 when the BSF conditions are met. - Option 3: (CMCC) - It is proposed to discuss whether to test FBS quitting when FBS conditions are met, e.g. whether to test that UE fallback to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition. - Recommended WF - Check if option 1 is agreeable? - Option 1: Not to test FBS quitting.

Refer to the definition in the core part, L3 fast Rx beam sweeping operation is activated provided that the time since L3 fast Rx beam sweeping operation is activated is not larger than 90 s, and for the quitting condition of FBS, UE is allowed to fall back to normal measurement when it has performed fast measurement for at least 90 s from the time point when UE enters the FBS mode after meeting the FBS entry condition, in our understanding, UE is also allowed to do FBS when FBS condition is not met or when FBS condition is met. Therefore, it’s proposed not to test FBS quitting.

Proposal 1: Not to test FBS quitting.

- Conclusion

The contribution presents our views on RRM performance requirements for FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, with the following proposals:

Proposal 1: Not to test FBS quitting.

- References

- R4-2514801, WF on NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.

- R4-2513511, Topic summary for [116bis][205] NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.