# 3GPP TSG-RAN WG4 Meeting #117	R4-2520804

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.16.2

Source:	China Telecom

Title:	Discussion on RRM performance requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

In RAN4 #116bis meeting, RRM performance requirements for LP-WUS/WUR were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements for LP-WUS/WUR captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

Issue 3-2-3: Test case design for entry/exit conditions for case 1/case 3 Ad hoc meeting agreement: RAN4 to define at least test cases for exit conditions. FFS on whether to define test cases for evaluation of entry conditions. Continue discussion on whether to introduce a test case to verify when conditions for a particular state is not met, the UE should not enter into this particular state. Note: Candidate states includes LP-WUR monitoring, serving cell measurement offloading and RRM relaxation.

For entry/exit conditions for case 1/case 3, when the entry conditions are met, it’s up to UE whether to enter the state, therefore, entry conditions for case 1/case 3 are not testable. It’s proposed to consider only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions. However, if there is no consensus on whether to define test cases for evaluation of entry conditions, in order to make progress, we can compromise to introduce a test case to verify when conditions for a particular state is not met, the UE should not enter into this particular state.

Proposal 1: RAN4 to consider only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions.

Status in the WF [1] and summary [2]:

Issue 3-2-6: Test case design for LP-WUS monitoring Continue discussion - Proposals - P1: RAN4 to define one test case to verify the MR wake up delay when UE receives LP-WUS (CATT vivo ZTE Huawei Nokia) - P1-2: No need to define test case for LP-WUR monitoring mode if the MR wake up delay verification can be covered by other test cases (vivo) - P2: Discuss whether to introduce a test-case to verify the MR wake-up delay after receiving the LP-WUS (QC) - P3: No need to discuss LP-WUS missed detection or false-alarm rate performance for RRM. (QC) Recommendations: Check whether P1 can be used as base.

For LP-WUS monitoring, we support to define one test case to verify the MR wake up delay when UE receives LP-WUS, even though the wake up delay is defined in RAN1, the MR wake up delay when UE receives LP-WUS need to be verified in RAN4 from test and deployment perspective.

Proposal 2: RAN4 to define one test case to verify the MR wake up delay when UE receives LP-WUS.

Status in the WF [1] and summary [2]:

Issue 3-2-7: Test case design for higher priority frequency layer search Continue discussion - Proposals - Option 1: Introduce test case for high priority frequency search in case 1. (oppo Apple ZTE Nokia Huawei) - Option 1-1: The legacy test procedure on high priority frequency measurement shall be reused as starting point(ZTE) - Option 2: Suggest do not define test case for higher priority frequency layer cell reselection when UE is in offloading mode. (vivo) Recommendations:

For higher priority frequency layer search, we prefer to introduce test case for high priority frequency search in case 1 as the requirement on higher priority frequency layer search is defined in the core part.

Proposal 3: Introduce test case for high priority frequency search in case 1.

- Conclusion

The contribution presents our views on RRM performance requirements for LP-WUS/WUR, with the following proposals:

Proposal 1: RAN4 to consider only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions.

Proposal 2: RAN4 to define one test case to verify the MR wake up delay when UE receives LP-WUS.

Proposal 3: Introduce test case for high priority frequency search in case 1.

- References

- R4-2514811, WF on NR_LPWUS_RRM, vivo, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.

- R4-2513516, Topic summary for [116bis][210] NR_LPWUS_RRM, vivo, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.