3GPP TSG-RAN4 Meeting #117	R4-2520817

# Dallas, USA, November 17th – 21st, 2025

Source:	LG Electronics

Title: 	(6G system parameters) Modulation

Agenda Item:	8.2.2

Document for:	Discussion

Introduction

RAN4 made WF [1] for following 6G system parameters in RAN4#116bis.

- Waveform

  - Framework study for waveform

  - PA model

- Modulation

- Channel bandwidth (CBW)

  - Max CBW

  - Min CBW

  - FFT size

  - Numerology

  - Spectrum utilization

  - Asymmetric CBWs

- Channel arrangement

  - Channel raster

  - Sync raster

  - Channel spacing

- Irregular channel bandwidth

- Number of Tx and Rx

- Device types

We would like to provide our views on 6G modulation.

Discussion

RAN4 Agreement

RAN4#116bis - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

According to the RAN4 agreement, RAN4 needs to focus on UL 1024QAM with uniform constellation before RAN1 input on new non-uniform constellation.

To support 1024QAM, very high SNR and very low EVM are required, e.g, SNR of 35dB, EVM of 1.78%, and it requires high power amplifier. However, high power amplifier introduces nonlinear distortion when amplifying signal amplitude. These nonlinearities distort the positions of constellations. It worsens EVM and increases BER.

Pre-distortion techniques, e.g. DPD (Digital Pre-distortion) can be used to solve these nonlinearity issues, however, very high accuracy is required, increasing implementation difficulty and cost for 1024QAM.

So far, RAN4 was not necessary to consider Pre-distortion techniques, because UL highest modulation order is 256QAM.

The existing 5G NR UE RF requirements related to modulation order is as follows.

- 6.2.3 MPR

- 6.4.2 Transmit modulation quality (EVM, carrier leakage, in-band emission)

  - EVM

  - Carrier leakage

  - In-band emission (General, IQ image, carrier leakage)

EVM is main factor to MPR for 1024QAM.

General in-band emission for 1024QAM is as Figure 2-1 with below assumption, if the existing in-band emission is applied,

- Assumption

  - CBW = 100MHz, SCS = 30kHz, NRB = 273

  - LCRB = 50 RB

  - PC2 (26dBm)

Figure 2-1 : In-band emission for 1024QAM (including other modulation orders)

As seen in Figur2-1, there is no difference between 256QAM and 1024QAM. It means the existing in-band emission needs to be updated to accommodate 1024QAM.

Proposal 1: Study impact on the existing in-band emission to accommodate 1024QAM.

5G NR, IQ image (dB) is specified with -28dB when output power > 10dBm, and with -25dB when output power ≤ 10dBm.

To meet EVM of 1.78% (SNR of 35dB), IQ image needs to be updated.

Proposal 2: Study impact on the existing IQ image to accommodate 1024QAM.

5G NR, Carrier leakage (dB) is specified as follows.

Table 6.4.2.2-1: Requirements for Carrier Leakage

| Parameter | Relative Limit (dBc) |
| --- | --- |
| Output power > 10 dBm | -28 |
| 0 dBm ≤ Output power ≤ 10 dBm | -25 |
| -30 dBm ≤ Output power < 0 dBm | -20 |
| -40 dBm ≤ Output power < -30 dBm | -10 |

The carrier leakage is a significant hardware impairment that directly degrades performance for high order modulation. Even a small amount of carrier leakage can cause the constellation to shift. In 5G NR, -28dBc is specified when output power > 10dBm considering 256QAM. However, more stringent carrier leakage needs to be required to accommodate 1024QAM.

Proposal 3: Study impact on the existing carrier leakage to accommodate 1024QAM.

Conclusions

This document discusses 6G modulation, 1024QAM and provides our view.

Proposal 1: Study impact on the existing in-band emission to accommodate 1024QAM.

Proposal 2: Study impact on the existing IQ image to accommodate 1024QAM.

Proposal 3: Study impact on the existing carrier leakage to accommodate 1024QAM.

References

[1] R4-2514650, WF on 6G system parameter, TSG-RAN4 Meeting #116bis, October 2025