3GPP TSG-RAN WG4 Meeting #117	R4-2520866

# Dallas, USA, Nov. 17-21, 2025

Source: 	China Telecom

Title: 	Discussion on 6G radio spectrum

Agenda Item:	8.5

Document for:	Approval

Introduction

In the RAN#108 meeting, the new SID [1]: Study on 6G Radio was agreed, which aims to develop one non-backward compatible radio access technology. Then, in the later RAN4#116 meeting, the 6G Plan and Meeting Arrangement was proposed by RAN4 chair [2] to give the 6G RAN4 topics. In RAN4#116bis, the spectrum topic has been discussed and reached some agreements as captured in WF [3]. This contribution provides our further views on 6G spectrum.

Discussion

2.1 Definition of frequency ranges

In the adhoc discussion of RAN4#116bis meeting, 8 options are summarized by feature lead, and companies are encouraged to provide further views on down selecting these options. In our view, the frequency range extension especially for 7125MHz shall consider if impact to the antenna of existing frequency band of n104, or individual antenna or RF component are dedicated for up 7125MHz. In this case, we think it is not necessary to extend 5G FR1 range above 7125MHz. So option 2 that define new name for 7125MHz- 24250MHz is our preference, unless technical analysis shows extension of FR1 or FR2 is beneficial to implementation.

Proposal 1: option 2 that define new name for 7125MHz- 24250MHz is our preference, unless technical analysis shows extension of FR1 or FR2 is beneficial to implementation.

2.2 Band combination simplification

Up to ten thousands of band combinations are proposed in RAN4 to work on the requirements specification. MSD requirement for each combination needs to be analysis case by case. Now the MSD simplification work is under discussion for HPUE combinations targeting at introduction of unified Equations/LUT/Simplified guidelines to replace the existing method using HPUE MSD tables at the end of Rel-19 or earlier if agreed. The main idea is using a delta MSD relative to the default power class 3 MSD, while keeping the current procedure of PC3 band combination unchanged.

In our view, the work of band combinations for all the power classes including PC3 need to be improved for 6G. Classify the bands to be several of groups based on frequency ranges such as low/media/high etc., and then specify the MSD requirements based on band groups, which could simplify the CA requirements dramatically.

E.g., Define the group1 as combination of low+media+high+superhigh without MSD, in which to study the specific range or band lists of low, media, high and superhigh.

Another example is to define the group 2 as combination of low+media+high+superhigh with same MSD requirements, in which to study the specific range or band lists of e.g. 2nd harmonic of low fall into media.

Based on the CA groups, when introducing new band, just first checking if can be captured to the existing band list. If not, then the specific combination may be introduced except of CA groups.

Proposal 2: Define the band combinations as several groups based on different type of MSD requirements, each group includes band lists and share the same requirements.

2.3 Regulatory status survey: candidate 6G spectrum: 6GHz

In China, On May 23, 2023, the Ministry of Industry and Information Technology issued the new version of the "Radio Frequency Allocation Regulations of the People's Republic of China." In this revision of the regulations, all or part of the 6425-7125 MHz frequency band is allocated for IMT systems.

5G has specified the 6GHz (6425 MHz - 7125 MHz) as Band n104, but it is not deployed in commercial 5G networks.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n10417,18 | 6425 MHz – 7125 MHz | 6425 MHz – 7125 MHz | TDD |

Thus, it is proposed to introduce frequency range of (6425 MHz - 7125 MHz) as 6G spectrum.

Proposal 3: Introduce frequency range of (6425 MHz - 7125 MHz) as 6G spectrum.

Conclusion

In this contribution, we give our proposals on 6G frequency range, band combination simplification and our interested candidate 6G spectrum.

Proposal 1: option 2 that define new name for 7125MHz- 24250MHz is our preference, unless technical analysis shows extension of FR1 or FR2 is beneficial to implementation.

Proposal 2: Define the band combinations as several groups based on different type of MSD requirements, each group includes band lists and share the same requirements.

Proposal 3: Introduce frequency range of (6425 MHz - 7125 MHz) as 6G spectrum.

- References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (moderator)

- R4-2511652, Plan on RAN4 6G discussion and meeting arrangement in Rel-20, RAN4 Chair (Apple)

- R4-2514626, WF on [116bis][104] 6G spectrum, Nokia