3GPP TSG-RAN WG4 Meeting # 117	R4-2520936

Dallas, USA, Nov. 17-21, 2025

Title: 	Discussion on performance requirements for low band carrier aggregation via switching

Source: 	Huawei, HiSilicon

Agenda item:	6.3.2

Document for:	Discussion

Introduction

WI on low band carrier aggregation via switching is approved in [1].

- 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

The contribution provides further analysis on performance requirements for low band carrier aggregation via switching.

Discussion

- Remaining TC list

As per the latest conclusion, most TCs are agreed to be specified [2]. The open issue is whether to define TC#4 and #6 in below table.

| TC Index | Requirement | RS used for test | Target Cell for testing | Support this TC |
| --- | --- | --- | --- | --- |
| 1 | Intra-frequency L3 measurement for activated SDL SCell or PCell | SSB | Both PCell and SCell in the same TC PCell and SCell can be configured with different events, details can be FFS. PCell is in partially overlapped case. SCell is in fully overlapped case. |  |
| 2 | Deactivated SDL SCell measurement and interruption requirement | SSB | Verify measurement requirement on SDL SCell. And in the same TC verify interruption requirement to the PCell. |  |
| 3 | SDL SCell activation/deactivation | SSB(for sync and L1-RSRP) and CSI-RS for CQI | SDL SCell. Only MAC CE based SCell activation is verified. |  |
| 4 | BFD/CBD for SDL SCell | Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both | Option 1: SCell only (Apple) Option 2: Both PCell and SCell (OPPO, HW, ZTE, Ericsson) [Ericsson]: If option 2 is adopted, test PCell/SCell in the different test. | FFS whether this TC shall be introduced. |
| 6 | PL-RS switch delay for PCell | Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both | FDD PCell (Apple, OPPO, ZTE, Ericsson) | FFS whether this TC shall be introduced. |
| 7 | L1-RSRP for SDL SCell | SSB | SCell only |  |
| 8 | RLM requirement for PCell | SSB | FDD PCell |  |

As the core requirements for BFD/CBD and PL-RS switch for SDL SCell in LB CA via switching were specified, it is fine to define corresponding TC to verify the functionality. To simplify the TC, it is suggested that only SSB based to be defined, and BFD/CBD on SCell and PL-RS switching delay on PCell are to be tested, where SSBs are partial overlapping with switching pattern.

Proposal 1: Define TC on SSB based BFD/CBD for SDL SCell, where SSBs are partial overlapping with switching pattern.

Proposal 2: Define TC on SSB based PL-RS switch delay for PCell, where SSBs are partial overlapping with switching pattern.

- Test configuration

In legacy test case for SSB based L1-RSRP measurement, TReport is set 80ms. In R19 LB-CA test, if SSB is set as 20ms and switching pattern is set as 40ms, scaling factor P=2. To verify the impact of switching pattern on L1-RSRP measurement period, it is not proper to set TReport =80ms anymore. We suggest TReport is set to 40ms, then UE shall report L1-RSRP with 40ms periodicity.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |

Proposal 3: In L1-RSRP test for UE supporting LB CA via switching, TReport is set to 40ms.

Conclusions

This contribution provides further analysis on performance requirements for low band carrier aggregation via switching. The following proposals are provided:

Proposal 1: Define TC on SSB based BFD/CBD for SDL SCell, where SSBs are partial overlapping with switching pattern.

Proposal 2: Define TC on SSB based PL-RS switch delay for PCell, where SSBs are partial overlapping with switching pattern.

Proposal 3: In L1-RSRP test for UE supporting LB CA via switching, TReport is set to 40ms.

Reference

- RP-243317, New WID on low band carrier aggregation via switching

- R4-2514794, WF for [116bis][202] NR_LBCA_Sw_RRM, Apple