3GPP TSG-RAN WG4 Meeting #117	R4-2520944

Dallas, US, 17th November – 21st November, 2025

Agenda item:	6.10.2.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part of optimising CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

According to the last meeting’s way forward, most issues have been allocated to be treated in the CR discussion. In this contribution, we provide our further views on the 3-seacher test cases.

- Discussion

Test design of solution 3 test cases with inter-RAT MOs

In the previous agreements, RAN4 agreed that no need to enhance the inter-RAT without a measurement gap in solution 3. And the CSSF value of inter-RAT in the current solution is the same as the legacy value. But the UE will only follow one table based on the UE capability to schedule the MOs’ measurement. And the solution CSSF table included the inter-RAT. Therefore, the test case is needed to cover the case where the UE processes the SCell MOs with inter-RAT MO(s) simultaneously.

Here we would like to provide further explanation on it. It is needed to start from the test design of solution 3. In the current test case in RRM can only test the measurement of one target cell, for instance, test the measurement report delay of the target cell. For the 3-searcher solution, we designed the test case to ensure that the UE can report measurement reports of two cells on time, except for the PCell. There is no doubt that we are testing the UE’s performance of processing 3 CCs simultaneously, or not, instead of the measurement period of MOs. So, there are several possible scenarios for testing:

| Scenario | 3CC may need to be measured simultaneously |
| --- | --- |
| Scenario 1 | MO of PCC, MO of SCC1 and MO of SCC2 |
| Scenario 2 | MO of PCC, MO of PCC’s neighbour Cell (inter-frequency) and MO of SCC1 |
| Scenario 3 | MO of PCC, MO of PCC’s neighbour Cell (inter-RAT) Cell and MO of SCC1 |
| Scenario 4 | MO of PCC, MO of SCC1’s neighbour Cell (inter-frequency) and MO of SCC2 |

What we propose to test is that the UE supports 3-searcher capability, and the configured MOs include inter-RAT MO(s), the UE can measure the MO of inter-RAT without measurement gap and the two MOs of SCC simultaneously and send the measurement reports. Therefore, this test is not the general test for the inter-RAT measurement. It is to verify that UE can maintain 3 searchers on any MO configurations.

Observation 1: This test is not the general test for the inter-RAT measurement. It is to verify that UE can maintain 3 searchers on any MO configurations.

The test purpose should be updated as: this test is to verify that the UE makes correct reporting of an event on an intra-frequency layer based on measurement performed without measurement gaps when operating in standalone (SA) operation and one inter-RAT E-UTRAN measurements object is configured with [FR1 only CA and FR1 only NR-DC 3-searcher capability] in FR1.

Proposal 1: Define a test case for UE support FR1 only CA and FR1 only NR-DC 3-searcher capability, and inter-RAT MO(s) is(are) configured. Details can be discussed in the CR: R4-2520953 [2].

- Conclusions

Observation 1: This test is not the general test for the inter-RAT measurement. It is to verify that UE can maintain 3 searchers on any MO configurations.

Proposal 1: Define a test case for UE support FR1 only CA and FR1 only NR-DC 3-searcher capability, and inter-RAT MO(s) is(are) configured. Details can be discussed in the CR: R4-2520953 [2].

4. References

[1] R4-2514800 WF on RRM requirements for NR_RRM_Ph5_Part1

[2] R4-2520953 Draft CR on Inter-RAT SSB measurement without MG test of 3-searcher solution for Rel-19