# 3GPP TSG RAN WG4 Meeting#117	R4-2521006

# Dalls, USA, Nov 17 – 21, 2025

Source: 	Qualcomm Incorporated

Title: 	Simulation results for demodulation performance requirements of LP-WUS/WUR for NR

Agenda item:	6.16.3

Document for:	Discussion

# 1. Introduction

In the previous meeting, simulation assumptions were discussed and finalized [1]. In this paper, we present our simulation results for MDR requirements related to FR1 and FR2 scenarios for the OFDM receiver.

2. Simulation assumptions

Based on the agreed simulation assumptions [1], we consider the following parameters for OFDM receiver related to FR1 and FR2 scenarios.

Table 1: Simulation assumptions for FR1

| Parameters | Test 1-1 | Test 1-2 | Test 1-3 | Test 1-4 |
| --- | --- | --- | --- | --- |
| M | 4 | 4 | 4 | 4 |
| Wake-up information length (K) | 2 bits | 2 bits | 5 bits | 5 bits |
| LP-WUS waveform | OOK-4 | OOK-4 | OOK-4 | OOK-4 |
| Number of overlaid sequence (L1) | 4 | 4 | 4 | 4 |
| Number of coded bits after rate matching (LRM) | 12 | 12 | 32 | 32 |
| Reception time offset | 1us | 1us | 1us | 1us |
| Reception frequency offset | 5ppm | 5ppm | 5ppm | 5ppm |
| Channel model | TDLA30-10 | TDLA30-10 | TDLA30-10 | TDLA30-10 |
| Frequency range and SCS | 15kHz(10MHz) | 30kHz (40MHz) | 15kHz(10MHz) | 30kHz (40MHz) |
| Carrier freq | 900MHz | 900MHz | 2.6GHz | 2.6GHz |
| Antenna configuration | 1x1 | 1x1 | 1x1 | 1x1 |
| FAR | 1% | 1% | 1% | 1% |
| MDR target | 1% | 1% | 1% | 1% |

Table 2: Simulation assumptions for FR2

| Parameters | Test 2-1 | Test 2-2 |
| --- | --- | --- |
| M | 1 | 1 |
| Wake-up information length (K) | 2 bits | 5 bits |
| LP-WUS waveform | OOK-1 | OOK-1 |
| Number of overlaid sequence (L1) | 2 | 2 |
| Number of coded bits after rate matching (LRM) | 12 | 32 |
| Reception time offset | 1us | 1us |
| Reception frequency offset | 5ppm | 5ppm |
| Channel model | TDLA30-75 | TDLA30-75 |
| Frequency range and SCS | 120kHz(100MHz) | 120kHz(100MHz) |
| Carrier freq | 28GHz | 28GHz |
| Antenna configuration | 1x1 | 1x1 |
| FAR | 1% | 1% |
| MDR target | 1% | 1% |

3. Simulation results

In this section, firstly we provide our simulation results for MDR requirements based on the assumptions, as presented in Tables 1 and 2, for OFDM receiver considering repetition transmission.

Table 3: Simulation results for MDR requirements for OFDM receiver (FR1) with repetition processing

| Test cases | SNR@1% MDR |
| --- | --- |
| Test 1-1 | 6.5 |
| Test 1-1 | 5.9 |
| Test 1-3 | 5.3 |
| Test 1-4 | 4.5 |

Table 4: Simulation results for MDR requirements for OFDM receiver (FR2) with repetition processing

| Test cases | SNR@1% MDR |
| --- | --- |
| Test 2-1 | -3.7 |
| Test 2-2 | -5.7 |

The results presented in Table 3 indicate that the FR1 test case requires a comparatively higher SNR to achieve the same MDR target of 1% as FR2. In addition to the more challenging channel conditions, FR1 also utilizes a larger M value, which shortens the ZC sequence length. This reduction in complexity comes at the cost of inferior synchronization performance. Consequently, the selection of a lower timing offset for larger M values is well justified here.

Furthermore, as observed from Table 4, for FR2 test cases that an increase in the number of coded bits (post rate matching) correlates with a reduction in the required SNR to achieve the same MDR target of 1%. This improvement can be attributed to the higher number of repetitions associated with a larger coded bit count (post rate matching).

It also appears that 30 kHz SCS yields slightly better results compared to 15 kHz, likely due to its reduced sensitivity to frequency offset, since it is assumed to have the same frequency offset assumption for both cases.

Next, we provide our simulation results considering one-shot/early-termination for the same set of tests as presented above in Tables 1 and 2.

Table 5: Simulation results for MDR requirements for OFDM receiver (FR1) with one-shot (early termination) processing

| Test cases | SNR@1% MDR |
| --- | --- |
| Test 1-1 | 12.5 |
| Test 1-1 | 12.0 |
| Test 1-3 | 9.5 |
| Test 1-4 | 8.5 |

Table 6: Simulation results for MDR requirements for OFDM receiver (FR2) with one-shot (early termination) processing

| Test cases | SNR@1% MDR |
| --- | --- |
| Test 2-1 | 0 |
| Test 2-2 | -2.2 |

As expected, early termination leads to a notable increase in the required SNR to meet the same 1% MDR target, due to the absence of repetition combining and its associated processing gain.

4. Conclusions

In this paper, we share our simulation results related to the MDR requirement for the OFDM receiver.

5. References

[1] R4-2514813, “WF for [116bis][330] NR_LPWUS_Demod,” 3GPP TSG RAN WG4 Meeting #116bis, Prague, Czech Republic, Oct 13 – 17, 2025.