3GPP TSG-RAN WG4 Meeting #117                                                                             R4-2521082

Dallas, USA, Nov 17th - 21st, 2025

Title:	General Aspects - Post-Deployment Validation based on performance monitoring

Source:	Korea Testing Laboratory

Agenda item:	6.11.2

Document for:	Discussion

- Introduction

The current Option 2 performance monitoring framework cannot ensure consistency and accuracy around model updates. It lacks a common effective from time on the decision time axis. The PDV aggregation window can then include reports from before and after the change, and LCM cannot reliably distinguish them.

We add one element to the existing framework. The UE declares an effective from boundary at a slot on the decision time axis. The network classifies each inference and monitoring report by decision time relative to this boundary, and all linkage, timing anchors, metrics, and report formats remain unchanged.

This single addition restores consistency and accuracy to a practical degree for Post Deployment Validation. When cells provide similar environments, PDV results can stay within a credible range across networks. Residual differences need further study in RAN4, and we apply the same remedy used for AI/ML Beam Management [3] to CSI prediction here.

In Bengaluru meeting #116 Agenda Item: 7.18.1 Source: CAICT, NTU, Ericsson, Qualcomm, APPLE, Huawei, Hisilicon, OPPO, CATT, CMCC, NTT DOCOMO, INC., Vivo, Nokia, Xiaomi, Mediatek, Rohde & Schwarz, Samsung, Intel, ZTE Corporation, Sanechips, Korea Testing Laboratory Title: Proposed update for TR 38.843 with RAN4 part Document for: Discussion 7.3.2.9 Post deployment handling [1] When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed. - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable For dealing with potential changes in the performance of functionalities, two options may be available. The options are not mutually exclusive: - Option 1 (Post-deployment pre-activation functionality/configuration update testing): Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - Other aspects not precluded. - Option 2 (Post-deployment post-activation functionality testing based on performance monitoring): Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Option2 is prioritized. Reliable and testable monitoring procedures can be established for all cases.

- Consistency and accuracy issues

2.1 RAN1 #120

UE-side CSI prediction remains anchored to the decision-time domain and the inference path reuses the Rel-18 framework. For channel measurement and reporting, periodic, semi-persistent, and aperiodic CSI-RS are supported. When N4 ≥ 1, the Rel-18 ‘typeII-Doppler-r18’ codebook applies. For UE-assisted performance monitoring, the trigger must arrive no later than the first symbol of the earliest eligible measurement occasion. For aperiodic measurement this means the earliest of K aperiodic occasions that are no later than the CSI reference resource of the associated inference report. For periodic or semi-persistent measurement this means the earliest of the most recent KP occasions that satisfy the same “no later than the CSI reference resource” condition. Active resource and port counting is also defined on the decision-time axis and runs from the end of the inference report to the end of the triggered monitoring report. Taken together, these rules specify which occasions qualify for PAI and how monitoring links to inference while leaving the Rel-18 mechanics unchanged.

What is not specified is the exact moment when the updated UE behaviour becomes valid on that axis. Without a shared boundary, the first monitoring built around a model change or a fine-tuning can be classified differently even when all #120 rules are observed. The scheduler phase and the selection order of the earliest K or KP occasions relative to the change can place the initial report on opposite sides of the transition. The resulting windows may then contain different mixes of pre- and post-change reports, and the statistics are not comparable. A minimal and fully compatible remedy is a UE-originated “effective-from” indication that marks the decision-time slot from which the updated behaviour applies.

With this boundary in place, classification becomes deterministic under the existing #120 agreements. Any monitoring window whose CSI reference resource is strictly earlier than the boundary is treated as pre-change. Any window whose reference resource is at or after the boundary is treated as post-change. No modification is required to the Rel-18 reuse, to the supported CSI-RS resource types, to the N4/codebook choice, to the “no later than the first symbol of the earliest K/KP occasions” trigger rules, or to the definition of active resource and port counting. The only addition is the boundary itself, so post-deployment validation remains consistent and comparable when evaluated under similar radio-channel conditions and materially equivalent configuration across networks.

Observation 1:  Because #120 anchors CSI prediction and monitoring to the decision-time axis and admits occasions by the “no later than the first symbol of the earliest K/KP occasions” rule, the lack of a defined effective-from moment for updated UE behaviour lets the first monitoring after a model change or fine-tuning fall on either side of the transition depending on scheduler phase and selection order relative to the CSI reference resource, yielding the LCM that mix pre-and post-change reports and making statistics not comparable.

Table 1. RAN1 #120

Agreement in RAN1 #120 Agreement For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused. - For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported. - For inference report, for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18') Agreement - For CSI prediction using UE-side model, for UE assisted performance monitoring, - If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report; - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report - If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of the most recent KP inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report

2.2 RAN1 #120bis

The performance monitoring for CSI prediction with a UE side model adopts the UE assisted approach and is based on Type 3. In Type 3 the UE computes the performance metric and reports the result to the network, and SGCS is the agreed metric for this purpose. These choices keep evaluation on the decision time axis and allow post deployment validation to reuse the existing framework without changing report formats or resource types.

What remains unspecified is the precise moment when an updated UE model becomes valid on that axis. Because the UE generates the metric, any reports taken around a model change can mix pre-change and post-change behaviour if the network does not know the boundary. The first monitoring after the change can therefore be classified differently depending on scheduler phase and on which occasions are admitted by the decision time rules, which makes statistics hard to compare. In practice operators intend to use monitoring to trigger timely switching or fallback when performance degrades, yet a missing boundary can delay those actions. Monitoring is not always driven by continuous aperiodic requests, so the windows after a change may include older periodic or semi persistent occasions and the mixed content can mask the true drop in SGCS. As a result the network may observe an average that does not cross a threshold even though the updated model has degraded, and switching or fallback may occur later than intended, with risk of performance impact.

A minimal and compatible remedy is to add a UE originated effective from indication that marks the decision time slot from which the updated behaviour applies. The network then classifies monitoring windows only by the CSI reference resource measured against that boundary. Any window whose reference resource is earlier than the boundary is treated as pre change, and any window whose reference resource is at or after the boundary is treated as post change. This addition leaves Type 3 intact, keeps SGCS as the metric, and requires no change to formats or resource definitions. It supplies the missing boundary so that post deployment validation remains consistent and comparable when it is evaluated under similar radio channel conditions and materially equivalent configuration.

Observation 2: When updating the UE model, the lack of a defined effective time-from point causes monitoring reports to mix pre-change and post-change behaviors, making statistical comparisons unreliable and masking SGCS degradation, which can delay the LCM's switching or fallback action.

Table 2. RAN1 #120bis

Agreement in RAN1 #120bis Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring. Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

Table 3. RAN1 #114

Agreement in RAN1#114 Summary In RAN1#114, following agreements on performance monitoring for functionality-based LCM were made. Agreement For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 3: - UE calculate the performance metric(s) - UE report performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Functionality selection/activation/ deactivation/switching what is defined for other UE side use cases can be reused, if applicable. - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report. - Note: down selection is not precluded. Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW.

RAN1 #120bis confirms that training data collection for UE sided CSI prediction can reuse the CSI framework while remaining separate from inference and performance monitoring. In particular, a CSI-ReportConfig may be used to configure resources for data collection without producing a CSI report, and the specification left open how the configuration indicates the absence of a report. This open point was finalized at RAN1 #121, where the agreement states that for training data collection the configuration shall set reportQuantity to “none‑CSI‑r19”, which provides the explicit indication for operation without a CSI report. For channel measurement resources used for training collection, periodic and semi-persistent CSI-RS are supported, while aperiodic CSI-RS for CMR is not supported. At the same time, RAN1 defined how inference and monitoring select inputs on the decision time axis. The predicted PMI is taken at the S-th time instance configured by the monitoring parameter, and the non-predicted PMI is taken from the latest measurement occasion that is no later than the CSI reference resource of the associated report. For linkage under periodic or semi persistent resources, the UE is expected to receive the trigger for the monitoring report no later than the first symbol of the earliest occasion among the most recent KP occasions that are not later than the reference resource. These rules make training collection configurable without reports and keep inference and monitoring strictly anchored to time.

Although training data collection is separate from inference and performance monitoring, samples drawn across a model change can contaminate training and validation labels. A UE declared effective from boundary fixes inclusion and exclusion on the decision time axis and guarantees a clean separation of pre-change and post-change datasets. Since the PAI report uses a fixed format and a fixed length by padding, classification cannot rely on payload structure and must rely only on time. Without an explicit boundary on the decision time axis, the S based selector and the no later than selector can include reports that cross the transition, which mixes pre-change and post-change observations in one window and undermines validation and retraining.

To remove this ambiguity while keeping every agreement intact, PDV and training collection introduce an explicit effective from boundary on the slot timeline. At the first mention the document states that an S instance is counted in slots and that counting starts from the boundary slot time of effective. From that moment both selectors operate on the same era, because the S-th instance is taken on or after the boundary and the latest occasion that is no later than the reference resource is also taken on the same side of the boundary. The network then includes or excludes training samples using the same boundary and the same reference resource, which yields clean pre-change and post-change datasets under identical formats. The role of KP is preserved and clarified. It does not delay discovery. It gives a strict deadline for the monitoring trigger so that the association remains robust under scheduler jitter and processor load. This approach does not change anchors, metrics, or formats, and it respects the training collection agreements that allow configuration without reports.

Observation 3: Training reuses the CSI framework with reportQuantity = none‑CSI‑r19 and periodic or semi-persistent CSI‑RS. Inference and monitoring select inputs on the decision‑time axis by the S instance rule and the no later than rule, with KP setting the trigger deadline. Without a UE declared effective from boundary, these selectors can span a model change and mix pre- and post-reports.

RAN1 #120bis Agreement For CSI prediction using UE-side model, for training data collection, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - FFS on how to indicate without CSI report in CSI-ReportConfig . Agreement For CSI prediction using UE-side model, for data collection for training, reportQuantity is set to ‘none-CSI-r19’. TP 2.6-1 < Unchanged parts are omitted > 5.2.1.4.6 CSI-PAI reporting When the UE is configured with a first CSI Reporting Setting for reporting CSI prediction, based on a CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] and a second CSI Reporting Setting for reporting CSI-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of CSI-PAI corresponding to the second CSI Reporting Setting shall consider the following: - when aperiodic Reporting Setting is configured for both first and second Reporting Settings: - if aperiodic CSI-RS resources are configured for channel measurement for the first Reporting Setting, the UE is expected to receive the PDCCH triggering a CSI report of the second Reporting Setting, if any, no later than the first symbol of the earliest occasion of, K∈{4,8,12} , aperiodic CSI-RS resources in the resource set for channel measurement for the first Reporting Setting. - if a single periodic or semi-persistent CSI-RS resource is configured for channel measurement for the first Reporting Setting, the UE is expected to receive the PDCCH triggering a CSI report of second Reporting Setting, if any, no later than the first symbol of the earliest transmission occasion of the most recent, Kp∈{1,2,4}, periodic or semi-persistent CSI-RS resource, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting. - to report CSI-PAI for the second Reporting Setting, the UE shall: - determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting (S≤N4 , with N4>1 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting, - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting. - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand, based on the latest CSI-RS resource transmission occasion, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

2.3 RAN1 #121

The agreement from RAN1 #121 defines a linked, dual‑configuration framework for UE‑assisted performance monitoring in CSI prediction. The first Reporting Setting is the inference configuration. The second Reporting Setting is the CSI‑PAI configuration, and it shall be linked to the first by inferenceReportConfigId‑r19. Monitoring reports may be semi‑persistent or aperiodic. For the measurement resources that support monitoring, periodic, semi‑persistent, and aperiodic CSI‑RS are allowed. The same text fixes the valid pairs between the inference‑report type and the monitoring‑report type so that network scheduling forms predictable pairs. AP with AP is supported, SP with SP is supported, AP with SP is not supported, and SP with AP is supported. In this structure, a CSI‑PAI report is valid only when it is linked to a specific inference Reporting Setting and interpreted against that inference instance.

In post‑deployment validation, the goal is to assign every monitoring outcome to a single UE model state so that the statistics are auditable and repeatable. The agreed framework ensures that monitoring is defined with respect to an inference instance, but it does not expose to the network the exact effective‑from time at which the UE model changes. When a model update occurs near an AP trigger or around an SP occasion, the linked pair can still straddle the model‑change boundary at the level of evidence. For example, the first inference instance after the change may link to monitoring evidence whose measurement occasions are “no later than” the inference reference resource and can therefore include reports that reflect the pre‑update behavior. The reverse situation is also possible around the boundary. In such cases, the first post‑update monitoring window may contain pre‑update evidence, and the PDV decision may depend on scheduler phase rather than on the model’s true post‑update performance. The agreement, taken alone, does not give the network a way to prevent this window contamination at the boundary.

Under the RAN1‑agreed framework, the first Reporting Setting is the inference configuration and the second Reporting Setting is the CSI‑PAI configuration, and the second is configured to be linked to the first through inferenceReportConfigId‑r19. Monitoring reports may be semi‑persistent or aperiodic, and the measurement resources that feed monitoring may be periodic, semi‑persistent, or aperiodic. The supported pairings are APAP, SPSP, and SPAP, while APSP is not supported. The timing and evidence used for monitoring are anchored no later than the CSI reference resource of the associated inference instance, which means monitoring is interpreted with respect to the linked inference on the decision‑time axis rather than by the physical arrival time of UCI or PUSCH.

Building on these facts, we propose the following PDV classification rule. Classification uses decision time only, and the network does not reclassify based on delivery or transport latency. For APAP, the first post‑update pair is the first AP inference whose decision time is at or after the boundary together with the first AP monitoring instance that is linked to that inference and is also at or after the boundary, and any pair that straddles the boundary is excluded from the post‑update window. For SPSP, the post‑update window starts at the first configured SP occasion that is at or after the boundary, so both the inference report and the monitoring report use only post‑update evidence even if their SP phases were set long before the change. For SPAP, an AP monitoring instance is counted in the post‑update window only when both the AP trigger time and the decision time of the linked SP inference are at or after the boundary; if either time is before the boundary, the linked pair remains only in the pre‑update window. With a single UE‑originated effective‑from indication to mark the boundary, the agreed linkage is preserved, the three pairings remain valid, and PDV statistics become model‑epoch pure regardless of scheduler phase, buffering, or CSI‑RS resource type.

Observation 4:  Because the framework does not expose the exact effective‑from time of UE model changes, linked pairs near an AP trigger or an SP occasion can straddle the boundary, leaving post-update monitoring windows with pre‑update evidence and the Post Deployment Validation decision dependent on scheduler phase.

Table 4. RAN1 #121

Agreement in RAN1 #121 Agreement For CSI prediction using UE-side model, for UE assisted performance monitoring, - Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - For monitoring report type, semi-persistent and aperiodic are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported.

It fixes the reporting structure for UE side CSI prediction by using a single UCI part that carries a set of SGCS values. Within that structure SGCS1 is computed from the predicted CSI and the ground truth for one inference report, while SGCS2 is computed from the ground truth and the non‑predicted CSI taken from the latest CSI‑RS transmission occasion that is no later than the CSI reference resource of the associated inference instance. The entries are reported per layer up to the rank v that the inference report indicates. In the same set of agreements RAN1 defines wideband reporting and adopts a 4‑bit quantization table for each SGCS value, and for N4 greater than one only one time instance is reported per layer. These decisions maximize reuse of the CSI framework and place monitoring firmly on the decision‑time axis defined by the reference resource and the “no later than” rule.

The two SGCS values in one UCI part are anchored to different inputs. SGCS1 depends on the predicted CSI that reflects the current model, while SGCS2 depends on a non‑predicted CSI that is explicitly tied to the latest eligible measurement occasion by the “no later than” rule. If a model update or fine tuning occurs and the network does not know the exact decision‑time moment when the new behaviour becomes valid, the first reports after the change can admit a mixture of pre‑change and post‑change evidence even within a single UCI message. The mixture arises from how occasions are selected and linked on the decision‑time axis rather than from delivery timing, and it is amplified because each SGCS is quantized to four bits and only one time instance is allowed when N4 is greater than one. Under these conditions the same configuration can yield different windows and different pass or fail outcomes, which is the inconsistency highlighted in the post‑deployment validation analysis.

Introduce a UE originated effective from indication that marks the decision‑time slot from which the updated behaviour applies. The network then classifies every monitoring window and every SGCS pair in the existing single‑UCI format only by the CSI reference resource relative to that boundary. Any window whose reference resource is earlier than the boundary is treated as pre‑change and any window whose reference resource is at or after the boundary is treated as post‑change. This addition leaves the #121 agreements untouched, including the single UCI part with SGCS1 and SGCS2, the wideband and per‑layer granularity, the four‑bit quantization table, and the one time instance rule for N4 greater than one. It supplies only the missing boundary so that post‑deployment validation remains consistent and comparable when it is evaluated under similar radio‑channel conditions and materially equivalent configuration.

Observation 5:  Without the exact decision‑time effective‑from moment, the first single‑UCI report carrying SGCS1  (predicted) and SGCS2  (non‑predicted tied by the “no later than” rule) can mix pre‑ and post‑change evidence; 4‑bit quantization and only one time instance when N4 is greater than one amplify this, yielding inconsistent windows and different pass/fail outcomes in post‑deployment validation.

RAN1 #121 Agreement For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs. SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. - v is the value of the reported RI in the associated inference report.

The agreement for data collection states that the UE uses one CPU unit during training. The occupancy begins at the first symbol of each periodic or semi-persistent CSI‑RS occasion and it ends Z3’ symbols after that occasion. This occupancy model aligns with the CPU processing model in Clause 5.2.1.6 and with the CSI computation time model in Clause 5.4 [TS 38.214]. The monitoring configuration remains linked to the inference configuration through inferenceReportConfigId‑r19, and the association is interpreted on the decision time axis. Under this structure a practical issue appears at a model change. Several CSI reports can start on the same OFDM symbol, and the per symbol CPU budget may be exceeded. Clause 5.2.1.6 [TS 38.214] then allows the UE to skip updating the lowest priority reports on that symbol. The skipped set can contain the first post-update inference or the monitoring that is linked to it. The window that computes RS‑PAI or SGCS can then mix pre-update and post-update evidence even though the linkage and the timing anchors are respected. The statistics become sensitive to incidental load and scheduler phase rather than to the behaviour after the change.

A minimal remedy preserves the agreed structure. The UE provides an effective from indication that identifies the affected inference configuration and conveys the boundary on the decision time axis. For slot-based instances the boundary is interpreted as the first symbol of the boundary slot unless a finer decision time is configured. The network classifies every inference instance and every monitoring instance by comparing its decision time with the boundary. Classification does not depend on delivery latency and it does not change due to CPU induced drops. The linkage, the timing anchors, the metrics, and the report formats are reused without modification.

If CPU resources are scarce, omission on that symbol follows Clause 5.2.1.6 [TS 38.214], yet the classification axis remains stable. If CPU resources are available, the base station can suppress aperiodic triggers before the boundary or can restart semi-persistent reporting after the boundary by configuration. If the inference measurement resource is periodic or semi persistent, the gNB configures the smallest supported value of KP so that the earliest among the most recent KP occasions sets a strict no later than deadline for the monitoring trigger. If the inference measurement resource is aperiodic, the gNB selects a small value of K and schedules timely aperiodic triggers. These choices reduce boundary mixing and shorten the time to the first pure post update window while the agreed framework remains unchanged.

Observation 6:  At a model change, if the per‑symbol CPU budget is exceeded (Clause 5.2.1.6 [TS 38.214]), the UE may skip the lowest‑priority updates, so the first post‑update inference or its linked monitoring can be dropped, causing RS‑PAI/SGCS windows to mix pre‑ and post‑update evidence and making statistics depend on incidental load and scheduler phase.

RAN1 #121 Agreement For CSI prediction using UE-side model, for data collection for training, - OCPU=1 - the CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion. Agreement For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook, - For PU occupancy, support - Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or N is reported by UE - OCPU=0 or M is reported by UE - Note: Detailed values of N and M can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied - For occupancy duration of CPU and APU, - the same occupancy duration is used if both CPU and APU are reported non-zero value - if associated monitoring report is not configured, reuse following legacy occupancy duration - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP is indicated by UE capability. - Note: Detailed values of KP can be further discussed in UE feature. - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

2.5 RAN1 #122

RAN1 #122 completed the definition of SGCS reporting by adding a quantization mapping table to TS 38.214 Section 5.2.1.4.6. Each calculated SGCS value is now represented by a 4-bit field. The first code point, 0000, covers the range from zero to 0.3, and the remaining range from 0.3 to 1.0 is divided into linear steps of 0.05. This agreement provides a uniform numerical format for the UE-reported SGCS values and ensures that both prediction and monitoring reports are encoded in a consistent way. The change finalizes what had been an incomplete specification and allows performance monitoring and post-deployment validation to use SGCS as a normalized, quantized metric.

While this quantization table brings consistency to reporting, it also increases the sensitivity of the averaged SGCS values to small variations in sample composition. Because SGCS is quantized in coarse steps and averaged across subbands or layers, a small number of samples that belong to a different context can shift the mean noticeably. When a model update or replacement occurs, the first window that overlaps the transition may include only a few post-change samples among mostly pre-change ones. In that case the reported average SGCS can be biased toward the older behaviour, which may hide or delay the detection of a real performance drop. The effect is structural and does not arise from delivery timing but from the lack of a precise decision-time boundary that defines which samples are included in each window.

To avoid biased aggregation, a UE-originated effective-from indication should mark the decision-time slot from which the updated model behaviour applies. The network can then include or exclude SGCS samples strictly according to the reference resource relative to that boundary. Windows whose reference resources are earlier than the boundary are treated as pre-change, and those at or after the boundary are treated as post-change.

This approach leaves the RAN1 #122 quantization table and reporting format intact while adding a simple timing boundary that fixes the aggregation domain on the decision-time axis. With this boundary in place, the averaged and quantized SGCS values reflect each model version accurately, and post-deployment validation remains consistent and comparable when evaluated under similar radio-channel conditions and materially equivalent configuration. Since quantization and payload length are fixed by agreement, only the time boundary that defines the sample set can change the reported outcome. Without an explicit effective boundary slot, SGCS taken before the update and SGCS taken after the update can fall into the same window and a like for like comparison is not possible.

Observation 7:  With SGCS quantized to 4 bits and averaged across subbands or layers, a window that overlaps a model update without a precise decision time boundary can mix reports from before and after the change, biasing the averaged SGCS toward older behaviour and delaying detection of a performance drop.

RAN1 #122 Agreement Adopt following TPs for TS38.214 Section 5.2.1.4.6. Reason for change; In current specification, there is no SGCS quantization mapping table.. Summary of change; SGCS quantization mapping table is added.. Consequences if not approved; Incomplete capture of the agreement.. TS38.214 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> - each calculated SGCS value is quantized to a 4-bit field, where the first codepoint ‘0000’ is used to indicate (0 0.3] value range, and the value range [0.3, 1] is quantized in a linear scale with 0.05 step size. Detailed mapping is specified in Table 5.2.1.4.6-1. Table 5.2.1.4.6-1: SGCS Quantization Mapping Code Point; Range. 0; [0.0000, 0.3000]. 1; (0.3000, 0.3500]. 2; (0.3500, 0.4000]. 3; (0.4000, 0.4500]. 4; (0.4500, 0.5000]. 5; (0.5000, 0.5500]. 6; (0.5500, 0.6000]. 7; (0.6000, 0.6500]. 8; (0.6500, 0.7000]. 9; (0.7000, 0.7500]. 10; (0.7500, 0.8000]. 11; (0.8000, 0.8500]. 12; (0.8500, 0.9000]. 13; (0.9000, 0.9500]. 14; (0.9500, 1.0000]. 15; Reserved. << Unchanged parts are omitted >>.

It clarifies the premise for SGCS calculation by fixing the PMI basis to the Rel‑16 eType II CSI codebook for the precoders wk and  wk . In the CSI‑PAI configuration, the second reporting setting is configured with codebookType = ‘etypeII‑r16’, and the SGCS expressions use the predicted precoder wk together with the ground truth, while the non‑predicted precoder wk is taken from the latest CSI‑RS transmission occasion that is no later than the CSI reference resource of the associated inference report. The resulting SGCS values are reported per layer in the single UCI structure defined by RAN1, which means that both the inputs and the linkage remain anchored to decision time. This description reflects the #122 agreement on the PMI assumption and the related requirement to use codebookType = ‘etypeII‑r16’ in the second reporting setting, and it is consistent with the #121 text that places SGCS in a single UCI part with decision‑time linkage.

Against this background, the question is how these reports behave around a model change when the network does not know exactly when the new behaviour takes effect. Because the ground‑truth and the non‑predicted PMI follow the fixed eType II basis, they remain constant across the change, while the predicted side reflects the new behaviour as soon as the update is active. If the boundary is unknown, the first monitoring windows can admit a mixture of pre‑change and post‑change evidence even within a single UCI message. In that situation SGCS1 may already represent the updated model, whereas SGCS2 can still be paired with a non‑predicted PMI selected by the rule that admits only occasions no later than the reference resource. The composition of the window then depends on which occasions are admitted and linked rather than on a clear boundary, and the resulting averages can be biased even when configuration is otherwise identical.

To remove this ambiguity without changing the agreed framework, a minimal addition is a UE‑originated effective‑from indication that marks the decision‑time slot from which the updated behaviour applies. With that boundary in place, the gNB classifies every SGCS pair and every monitoring window only by the reference resource relative to the boundary. Any window whose reference resource precedes the boundary is treated as pre‑change, and any window whose reference resource is at or after the boundary is treated as post‑change. This preserves all choices made in RAN1 #122, including the Rel‑16 eType II PMI assumption and the single UCI reporting, while supplying the missing boundary that keeps validation consistent when the evaluation is performed under similar radio‑channel conditions and materially equivalent configuration.

Observation 8:  With the PMI basis fixed to the Rel‑16 eType II codebook and SGCS reported per layer in a single UCI anchored to decision time, an unknown model-update boundary can make SGCS1 reflect the updated model while SGCS2 still pairs with non‑predicted wk from occasions no later than the reference resource, mixing pre‑ and post‑change evidence and biasing the averages.

RAN1 #122 Agreement For PMI assumption on SGCS calculation of wk and wk ,support Rel-16 eType II CSI.

The linkage between the inference report and the monitoring report was finalized by endorsing TP 2.6-2 to TS 38.214 § 5.2.1.4.2. According to this agreement, when the UE is configured with a CSI-ReportConfig whose reportQuantity-r19 is set to csi-pai-r19, it must also be configured with an inferenceReportConfigId-r19 that links it to another CSI-ReportConfig containing the higher-layer parameter csi-InferencePrediction-r19. The same clause clarifies that if a semi-persistent reporting setting is applied to the inference report, a periodic setting for the associated csi-pai-r19 report shall not be configured, and if an aperiodic reporting setting is applied to the inference report, a periodic or semi-persistent setting for the linked csi-pai-r19 report shall not be used. Through these constraints, RAN1 ensured that the prediction report and the monitoring report are explicitly paired and that their timing relationships are consistent on the decision-time axis.

Although the linkage between the two reporting settings is now mandatory, the specification does not define how the network can determine when the updated UE model becomes valid. If a model change or fine-tuning occurs and the network is unaware of the precise transition time, the linked pair can unintentionally span two different model behaviours. In such a case the inferenceReportConfigId-r19 connects a monitoring report that still reflects the pre-change model with an inference report already produced by the post-change model. The result is a mismatched PAI–inference pair, which undermines the assumption of a one-to-one correspondence between the two reports and can distort the statistics used for validation. This inconsistency appears even when all RAN1 #122 configuration rules are followed, because the missing boundary leaves the network unable to separate pre-change and post-change linkages on the decision-time axis.

To maintain proper temporal alignment between the linked reporting settings, a UE-originated effective-from indication should mark the decision-time slot from which the updated model behaviour applies. With this boundary in place, the network can verify whether each linked PAI and inference report pair belongs entirely to the same model context. A pair whose reference resource is earlier than the boundary is treated as pre-change, while a pair whose reference resource is at or after the boundary is treated as post-change. This simple addition preserves all agreements of RAN1 #122, including the required linkage rules and the mutual exclusivity of reporting settings, while ensuring that validation windows maintain a one-to-one correspondence between inference and monitoring reports when evaluated under similar radio-channel conditions and materially equivalent configuration.

Observation 9:  Despite mandatory linkage via inferenceReportConfigId‑r19, the spec does not define when the updated UE model becomes valid on the decision‑time axis, so a model change can cause a mismatched PAI–inference pair (pre‑change monitoring linked to post‑change inference), undermining the assumed one-to-one correspondence and distorting validation statistics.

Agreement in RAN1 #122 Agreement The following TP2.6-1 and TP2.6-2 are endorsed as editorial corrections for TS 38.214. TP 2.6-2 < Unchanged parts are omitted > 5.2.1.4.2 Report quantity configurations If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19', - the UE shall be configured with inferenceReportConfigId-r19 to link another CSI-ReportConfig configured with a higher layer parameter [RRC_name-r19], - when semi-persistent Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19], the UE is not expected to be configured with a periodic Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19'. - when aperiodic Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19], the UE is not expected to be configured with a periodic or semi-persistent Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19'. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

In RAN1 #122, the UE computes SGCS per configured subband and reports a wideband SGCS that is the average across subbands. Each reported SGCS is 4‑bit quantized with 0.05 steps over [0.3,1]. Inputs are taken for the S‑th time instance and from occasions no later than the CSI reference resource. With averaging and 4‑bit quantization, small or narrowband gains may not move the wideband codepoint. If only a few subbands improve, the mean can stay within the same 0.05 step. The reported value may not change until enough subbands move or the mean crosses the next step.

PDV adds a decision‑time effective‑from boundary without changing the agreed structure. The network classifies only S‑indexed instances whose decision times are at or after the boundary. This avoids mixing pre‑ and post‑update evidence. It also acknowledges that the network observes wideband SGCS only and must infer improvement over time.

When the inference measurement resource is periodic or semi‑persistent, choose the lowest allowed KP to minimize the anchor window. Use aperiodic triggers to reduce time to the next measurement. KP sets a strict “no‑later‑than” deadline at the first symbol of the earliest among the most recent KP occasions. This guards the linkage between inference and monitoring under scheduler jitter and contention. The anchor also gives a known start symbol, which simplifies processor‑occupancy accounting. With these rules, the procedure stays consistent with the agreement, reduces boundary mixing, and exposes post‑update behavior as early as the framework allows.

Observation 10:  Without a decision‑time effective‑from boundary at a model update, S-indexed instances admitted by the “no‑later‑than the CSI reference resource” rule can mix pre‑update and post‑update evidence in the wideband SGCS.

Agreement in RAN1 #122 Agreement: Adopt following TPs for TS38.214 Section 5.2.1.4.6. Reason for change; Based on the agreement, the reported wideband SGCS is the average of SGCS values across all subbands, not directly derived from the average precoder.. Summary of change; The definition of SGCS needs to be changed to the average of SGCS values across all subbands.. Consequences if not approved; Incorrect capture of the agreement.. TS38.214 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> to report CSI-PAI for the second Reporting Setting, the UE shall: - determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting (S≤N4 , with N4 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting, - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting. - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting - for k ∈1,2,…,v, v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as, SGCSk1=n=1NwknHwknwkn.wkn2N, SGCSk2=n=1NwknHwknwkn.wkn2N, where wkn is the predicted precoder represented by predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, n-th subband, and for S-th time instance corresponding to the report of the first Reporting Setting, wkn is the precoder represented by non-predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, n-th subband, based on the channel measurement corresponding to the second Reporting Setting, and wkn is the precoder represented by PMI for k-th layer and averaged over all the subbands configured by csi-ReportingBand n-th subband, n∈1,2,…,N corresponds to the subbands configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. << Unchanged parts are omitted >>.

The adopted text for TS 38.214 Section 5.2.1.4.6 defines how the UE forms CSI PAI and how the inputs are chosen. The predicted PMI is taken at the S-th time instance configured by the monitoring parameter, and the non-predicted PMI is taken from the latest measurement occasion that is no later than the CSI reference resource of the associated report. The agreement also fixes the size of the CSI-PAI report by adopting UCI padding so that the report length equals two times the maximum rank times four regardless of the reported rank, and it specifies that when the linked inference report indicates a lower rank the remaining SGCS fields are padded with ones in the least significant bits. For linkage under periodic and semi persistent resources the UE is expected to receive the trigger for the monitoring report no later than the first symbol of the earliest occasion among the most recent KP occasions that are not later than the reference resource. These rules define a reproducible mapping between inference, monitoring, and report size.

Consistency at deployment time requires that samples taken before a change and samples taken after a change are not mixed inside the same decision window. The format and length of the report are fixed by the padding rule, therefore time is the only valid classifier for separating windows. In other words, since format and length are fixed, the only classification axis is time, and a boundary slot identified as teff must be declared to prevent any mixing of pre change and post change samples under the same format. Without an explicit boundary the combination of the S-th index and the no later than selector may still pick inputs that lie on different sides of a transition, which leaves the first reports after a model update hard to classify and makes results depend on scheduler micro timing rather than on the updated behaviour.

Therefore, PDV can introduce an explicit effective from boundary on the slot timeline while keeping every RAN1 rule intact. At first mention the document states that an S instance is counted in slots and that counting starts from the boundary slot teff. From that point both selectors operate on a single era, because the S-th instance is taken on or after the boundary and the latest no later than occasion is also taken on the same side of the boundary. This keeps the adopted linkage intact and removes mixed windows. The role of KP is preserved and clarified. It does not delay discovery. It provides robust association under real schedulers, since the monitoring trigger must arrive no later than the first symbol of the earliest among the most recent KP occasions that are not later than the reference resource. The fixed size report continues to flow unchanged, and the classification of windows becomes strictly time aligned and audit friendly.

Observation 11:  Without an explicit effective from boundary, the S‑th time instance and the “no later than the CSI reference resource” selector can pick inputs from opposite sides of a model update, mixing pre‑ and post‑change reports in LCM.

Agreement in RAN1 #122 Agreement For CSI-PAI report, when the UE is configured with two linked CSI report configurations, adopt UCI padding to fixed size - based on the maximum rank determined from configuration of RI restriction (for simplicity denoted by max_rank here), the size of the CSI-PAI report is 2×max_rank×4 irrespective of the reported rank ‘v’ in the inference report. - When the linked inference report indicates rank ‘v’, all ones is padded to LSB of the UCI sequence for each of max_rank-v fields of SGCS#1 or SGCS#2

Support for training data collection for CSI prediction using a UE side model is established. The agreement allows CSI‑ReportConfig to be used to configure resources for data collection without a CSI report and leaves the indication method for further study. RAN1 #122 defines the CPU processing model for CSI‑PAI monitoring. The UE reports OCPU as a capability. For an aperiodic monitoring report, the CPU occupancy starts at the first symbol after the PDCCH that triggers the CSI report and ends at the last symbol of the scheduled PUSCH that carries the CSI‑PAI report. For a semi-persistent monitoring report, the CPU occupancy starts at the first symbol of the latest periodic or semi- persistent CSI‑RS occasion of the inference report that is not later than the CSI reference resource of the inference report and it ends at the last symbol of the configured PUSCH that carries the CSI‑PAI report. These agreements place the construction of monitoring evidence on a symbol timeline that is tied to the reference resource and they complete the basic processing model for data collection and monitoring.

However, the agreements do not define how the network can know the exact effective from time when a new UE model becomes valid. When a model update or fine-tuning occurs under this condition the first window after the change can include inputs that fall on different sides of the transition because the aperiodic case is anchored by the trigger and the semi-persistent case is anchored by the latest measurement occasion that is not later than the reference resource. As a result, the reports can mix pre-change and post-change evidence even though the CPU occupancy rules are respected and the observed statistics can depend on scheduler timing rather than on the updated behaviour.

To address this within the agreed framework the UE should provide an effective from indication that identifies the boundary slot on the same symbol timeline. The gNB then classifies each inference and monitoring instance by comparing its decision time with the boundary and it includes in the post update window only those instances at or after the boundary. This approach keeps the use of CSI‑ReportConfig for data collection unchanged and it keeps the OCPU and CPU occupancy rules for aperiodic and semi persistent monitoring unchanged while it removes mixing at the transition and stabilizes post deployment validation.

Observation 12:  Because of CPU scheduling, the first report after a model update may not reflect the new model but older behavior, since the UE can skip or delay reports when the per-symbol CPU budget is exceeded.

RAN1 #122 Agreement For CSI-PAI report, support - OCPU =X, X is reported by UE capability - For AP monitoring report, CPU occupancy starts from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the CSI-PAI report - For SP monitoring report, CPU occupancy starts from the first symbol of the latest periodic/semi-persistent CSI-RS occasion of the inference report not later than CSI reference resource of the inference report, until the last symbol of the configured PUSCH carrying the CSI-PAI report

2.6 RAN1 #122bis

Based on RAN1 #122bis and TS 38.214 §5.1.6.1, performance monitoring is anchored to decision time: windows are formed from the N latest occasions no later than the CSI reference resource, and each occasion is linked to its associated inference on the decision-time axis.

When the DTX state toggles around a fine tuning or a model change, this anchoring can cause the set of occasions that qualify for PAI to drift. Periods of DTX suppress periodic or semi persistent occasions and the window back fills with older reports. When DTX clears, the first anchor after the change can depend on scheduler micro timing rather than on the behaviour that is being validated. As a result, two deployments with identical Option 2 settings can admit different mixtures of pre change and post change occasions due only to timing and the resulting statistics are not comparable. This sensitivity is not a delivery time issue that can be solved by reordering reports. RAN1 reuses legacy priority with k = 0 and keeps validation on the decision time axis, so delivery jitter, CPU load or DTX and DRX patterns must not trigger reclassification.

In practice, if a fine tuning or a model update occurs and DTX is activated immediately, the N latest rule reaches back into pre change samples to fill the window. When DTX deactivates, the earliest usable anchor may fall on different sides of the transition across networks, which makes the first post change report ambiguous. Without a shared boundary, the first report under the new inference outcome can be treated as pre change in one cell and as post change in another and the resulting PAI windows are not comparable. A minimal and compatible remedy is to provide a UE originated effective from indication that marks the decision time slot from which the updated behaviour applies while leaving all RAN1 anchors, metrics and formats unchanged. Classification then becomes deterministic. Any monitoring window whose reference resource is earlier than the effective from time is pre change. Any window whose reference resource is at or after the effective from time is post change. With this single boundary, mixed windows disappear and Option 2 validation remains anchored on the decision time axis without any change to PAI, to the N latest rule or to the k ≤ 64 linkage.

At t = 100 the UE applies the model update or fine tuning and establishes the time of effective from and it transmits the effective from indication in the same slot or in the slot immediately before so that the decision time boundary is unambiguous. From t = 100 to t = 160 the cell operates under DTX and periodic and semi persistent CSI‑RS are absent and only aperiodic resources may occur. Under the N-latest occasions no later than the CSI reference resource rule the window may therefore back fill with older pre change occasions, but classification remains anchored to the time of effective from so any occasion that is earlier than the time of effective from is treated as pre change. At t = 160 DTX is lifted and the first resumed monitoring configuration is evaluated against the reference resource. If that reference resource is at or after the time of effective from the resulting window is post change. Throughout, the minimal slot offset linkage with k ≤ 64 continues to apply and it binds each report to the earliest eligible occasion within the offset bound and it preserves the decision time anchoring.

Observation 13:  With decision‑time anchoring (N latest occasions no later than the CSI reference resource) and minimal slot‑offset linkage (k ≤ 64), DTX toggling around a fine‑tuning/model change mixes pre‑ and post‑change occasions (drift, back‑fill, micro‑timing), yielding incomparable statistics across deployments.

RAN1 #122bis Agreement: The following TP for TS 38.214 is endorsed. Reason for change; During non-active periods of cell DTX, UE is not expected to receive P/SP CSI-RS for CSI report associated with report quantity comprising at least RI which means this can be applied to SP inference report. However, whether it is applied for monitoring report is not clear.. Summary of change; Report quantity ‘csi-pai-r19’ is added.. Consequences if not approved; Inconsistent UE behaviour during non-active periods of cell DTX.. TS38.214 5.1.6.1 CSI-RS reception procedure << Unchanged parts are omitted >> During non-active periods of cell DTX if cell DTX is activated for a serving cell, the UE is not expected to receive the periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. If the cell DTX is activated for a serving cell [10, TS 38.321], the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in active periods of cell DTX for CSI report configured by CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. << Unchanged parts are omitted >>.

The second Reporting Setting for CSI-PAI is linked to the first Reporting Setting for inference by inferenceReportConfigId-r19. The association is interpreted on the decision time axis and it is independent of delivery. The agreed RAN1 timing anchors remain unchanged. Clause 5.2.1.6 defines CPU processing on a per symbol basis. The UE reports NCPU as the number of supported simultaneous CSI calculations and each CSI report has a CPU occupancy OCPU. When several CSI reports start on the same OFDM symbol and the available budget is not sufficient, the UE is not required to update a subset with the lowest priority until the sum of selected occupancies is less than or equal to NCPU - L. Inference and monitoring reuse the legacy CSI priority rule with k = 0. Clause 5.4 provides the computation and occupancy references for csi-InferencePrediction-r19 on the symbol timeline. These rules keep decision time separate from delivery time. If there is no effective from boundary, the first instance after the model change can be dropped when the per symbol CPU budget is exceeded. Multiple reports may start on the transition symbol and only a subset is updated. The monitoring window can then mix evidence from before and after the change even when the linkage and the timing anchors are respected. This creates unstable PDV statistics that depend on load and scheduler phase rather than on the updated behaviour.

The remedy is a UE originated effective from indication. The indication identifies the affected inference configuration and carries the boundary on the decision time axis. For slot-based instances the boundary is interpreted as the first symbol of the boundary slot unless a finer decision time is configured. The network classifies each inference instance and each monitoring instance by comparing its decision time with the boundary. Classification does not depend on delivery latency and it does not change due to CPU induced drops on that symbol. The agreed linkage, timing anchors, metrics, and formats are reused without modification.

Operational guidance follows from the same rules. When the CPU is scarce, Clause 5.2.1.6 already allows omission of some instances on that symbol. When the CPU is available, intentionally withholding reporting before the boundary is not advised. For aperiodic-reporting the base station suppresses aperiodic triggers before the boundary and resumes them after the boundary. For semi-persistent or periodic-reporting the base station uses RRC to release the existing configuration before the boundary and to activate a new configuration that starts at the boundary slot. In all other cases the UE continues to report and the boundary separates the windows. This is the most stable operating point for PDV.

Detection latency can be reduced without changing metrics or formats. If the inference measurement resource is periodic or semi persistent, the gNB configures the smallest supported value of KP for the UE so that the earliest among the most recent KP occasions defines a strict no later than deadline for the monitoring trigger. If the inference measurement resource is aperiodic, the gNB selects a small value of K and schedules timely aperiodic triggers. If the inference measurement resource is aperiodic, the controller uses a small K and timely aperiodic triggers. These policies shrink the anchor window and reduce boundary mixing while keeping the reporting obligations unchanged. When only the model behaviour changes the mapping remains and the boundary alone gates the post update window. This preserves comparability across deployments while keeping the RAN1 framework in Clauses 5.2.1.4.2, 5.2.1.4.6, 5.2.1.6, and 5.4 intact.

Observation 14:   Without a boundary on the decision time axis, per‑symbol CPU limits (Clause 5.2.1.6) can drop the first post‑update instance when multiple CSI reports start on the transition symbol, mixing pre‑ and post‑change evidence and making PDV statistics depend on load and scheduler phase.

RAN1 #122bis Agreement Following TP is endorsed as editorial corrections for TS 38.214 for Clauses 5.2.1.4.2, 5.2.1.4.6, 5.2.1.6 and 5.4. TS38.214 << Unchanged parts are omitted >> ##### 5.2.1.6 CSI processing criteria The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC or simultaneousCSI-SubReportsPerCC-r18 in a component carrier, and simultaneousCSI-ReportsAllCC or simultaneousCSI-SubReportsAllCC-r18 across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in a component carrier, UE shall use parameter simultaneousCSI-SubReportsPerCC-r18 in the component carrier; otherwise, UE shall use simultaneousCSI-ReportsPerCC in the component carrier. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use simultaneousCSI-SubReportsAllCC-r18; otherwise, UE shall use simultaneousCSI-ReportsAllCC. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds. For CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, or CSI reports configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19, OCPU= OCPU,1 is considered. For CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19, the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,x simultaneous CSI calculations it is said to have NCPU,x CSI processing units for processing CSI reports. If Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,x-Lx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,x-Lx CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU,x(n), the UE is not required to update the N-Mx requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤Mx≤N is the largest value such that n=0Mx-1OCPU,x(n)≤NCPU,x-Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2, 3 where the CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’ corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 if a CSI report is not considered within any of M and Mx, the values for OCPU,1 and OCPU,x are considered to be 0, for the procedure previously described in this clause and the UE is not required to update the CSI report. A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings. Processing of a CSI report occupies a number of CPUs for a number of symbols as follows: << Unchanged parts are omitted >> - OCPU,1=X1 and OCPU,x=Xx, for a CSI report configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19, where the values of X1 and Xx are reported by UE capability. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to ‘none’, or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to ‘none-bm-r19’ or ‘none-csi-r19’, the CPU(s) (including OCPU,1and/or OCPU,x, for CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, or ‘csi-pai-r19’, or ‘rs-pai-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 are occupied for a number of OFDM symbols as follows: << Unchanged parts are omitted >> In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, (excluding the case in which the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to ‘csi-pai-r19’), starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinkingId, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. << Unchanged parts are omitted >> For aperiodic CSI-RS, when the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to ‘csi-pai-r19’, NZP CSI-RS resource is active in a duration starting from the end of the PDCCH containing the request for the report of the corresponding CSI Reporting Setting and ending at the end of the scheduled PUSCH containing the report associated with the report of the linked CSI Reporting Setting. << Unchanged parts are omitted >>.

Proposal 1: We propose that RAN4 discuss a UE-originated effective-from boundary for Post-Deployment Validation. The indication would identify the decision-time slot from which the updated UE model becomes valid. With this boundary, the network can classify each inference and monitoring report by comparing its CSI reference resource to the boundary, so that reports before the boundary are treated as pre-change and those at or after the boundary are treated as post-change. This approach keeps the existing reporting formats, SGCS quantization, and CPU occupancy rules unchanged while improving the consistency and accuracy of validation statistics.

- Conclusion

In this contribution, we provided our viewpoints on Post-Deployment Validation based on performance monitoring

Observation 1:  Because #120 anchors CSI prediction and monitoring to the decision-time axis and admits occasions by the “no later than the first symbol of the earliest K/KP occasions” rule, the lack of a defined effective-from moment for updated UE behaviour lets the first monitoring after a model change or fine-tuning fall on either side of the transition depending on scheduler phase and selection order relative to the CSI reference resource, yielding the LCM that mix pre-and post-change reports and making statistics not comparable

Observation 2: When updating the UE model, the lack of a defined effective time-from point causes monitoring reports to mix pre-change and post-change behaviors, making statistical comparisons unreliable and masking SGCS degradation, which can delay the LCM's switching or fallback action.

Observation 3: Training reuses the CSI framework with reportQuantity = none‑CSI‑r19 and periodic or semi-persistent CSI‑RS. Inference and monitoring select inputs on the decision‑time axis by the S instance rule and the no later than rule, with KP setting the trigger deadline. Without a UE declared effective from boundary, these selectors can span a model change and mix pre- and post-reports

Observation 4:  Because the framework does not expose the exact effective‑from time of UE model changes, linked pairs near an AP trigger or an SP occasion can straddle the boundary, leaving post-update monitoring windows with pre‑update evidence and the Post Deployment Validation decision dependent on scheduler phase

Observation 5:  Without the exact decision‑time effective‑from moment, the first single‑UCI report carrying SGCS1  (predicted) and SGCS2  (non‑predicted tied by the “no later than” rule) can mix pre‑ and post‑change evidence; 4‑bit quantization and only one time instance when N4 is greater than one amplify this, yielding inconsistent windows and different pass/fail outcomes in post‑deployment validation.

Observation 6:  At a model change, if the per‑symbol CPU budget is exceeded (Clause 5.2.1.6 [TS 38.214]), the UE may skip the lowest‑priority updates, so the first post‑update inference or its linked monitoring can be dropped, causing RS‑PAI/SGCS windows to mix pre‑ and post‑update evidence and making statistics depend on incidental load and scheduler phase.

Observation 7:  With SGCS quantized to 4 bits and averaged across subbands or layers, a window that overlaps a model update without a precise decision time boundary can mix reports from before and after the change, biasing the averaged SGCS toward older behaviour and delaying detection of a performance drop.

Observation 8:  With the PMI basis fixed to the Rel‑16 eType II codebook and SGCS reported per layer in a single UCI anchored to decision time, an unknown model-update boundary can make SGCS1 reflect the updated model while SGCS2 still pairs with non‑predicted wk from occasions no later than the reference resource, mixing pre‑ and post‑change evidence and biasing the averages.

Observation 9:  Despite mandatory linkage via inferenceReportConfigId‑r19, the spec does not define when the updated UE model becomes valid on the decision‑time axis, so a model change can cause a mismatched PAI–inference pair (pre‑change monitoring linked to post‑change inference), undermining the assumed one-to-one correspondence and distorting validation statistics.

Observation 10:  Without a decision‑time effective‑from boundary at a model update, S-indexed instances admitted by the “no‑later‑than the CSI reference resource” rule can mix pre‑update and post‑update evidence in the wideband SGCS.

Observation 11:  Without an explicit effective from boundary, the S‑th time instance and the “no later than the CSI reference resource” selector can pick inputs from opposite sides of a model update, mixing pre‑ and post‑change reports in LCM.

Observation 12:  Because of CPU scheduling, the first report after a model update may not reflect the new model but older behavior, since the UE can skip or delay reports when the per-symbol CPU budget is exceeded.

Observation 13:  With decision‑time anchoring (N latest occasions no later than the CSI reference resource) and minimal slot‑offset linkage (k ≤ 64), DTX toggling around a fine‑tuning/model change mixes pre‑ and post‑change occasions (drift, back‑fill, micro‑timing), yielding incomparable statistics across deployments.

Observation 14:   Without a boundary on the decision time axis, per‑symbol CPU limits (Clause 5.2.1.6) can drop the first post‑update instance when multiple CSI reports start on the transition symbol, mixing pre‑ and post‑change evidence and making PDV statistics depend on load and scheduler phase.

Proposal 1: We propose that RAN4 discuss a UE-originated effective-from boundary for Post-Deployment Validation. The indication would identify the decision-time slot from which the updated UE model becomes valid. With this boundary, the network can classify each inference and monitoring report by comparing its CSI reference resource to the boundary, so that reports before the boundary are treated as pre-change and those at or after the boundary are treated as post-change. This approach keeps the existing reporting formats, SGCS quantization, and CPU occupancy rules unchanged while improving the consistency and accuracy of validation statistics.

4. Reference

[1] 3GPP TR 38.843, V19.0.0 (2025-09), “Study on Artificial Intelligence (AI)/Machine Learning(ML) for NR air interface”.

[2] “General Aspects-Deployment post-activation functionality testing based on performance monitoring,”  R4-2510136, RAN4 #116, Bengaluru, India.

[3] “General Aspects - Post-Deployment Validation based on performance monitoring,” R4-2513689, KTL, RAN4v #116b, Prague, Czech,