# 3GPP TSG-RAN WG4 Meeting #117		  R4-2521260

Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.6.4.1

Source:		Qualcomm Incorporated

Title:			Views on UE RF requirements for HD-FDD for Ku-band

Document for:	Approval

# 1  Introduction

UE RF requirements for HD-FDD for Ku band with FR1 numerology have been discussed in RAN4#116bis. The agreements were agreed in [1]. In this paper, we provide our views on UE RF requirements for HD-FDD for Ku-band.

# 2  Discussion

### 2.2.1 RF requirements for HD-FDD

For HD-FDD, the potential enhancements on the EIRP and EIS requirements were discussed in last meeting, and the following agreements on antenna assumption, EIRP and EIS requirements were captured in [1].

#### Issue 1-1-1: Antenna type assumption for HD-FDD VSAT - WF - The working assumption for Phase 1 is that the HD-FDD VSAT antenna type is a single physical aperture with Tx/Rx switching capability. - Other implementations are not precluded. #### Issue 1-1-5: EIRP requirements for HD-FDD - WF - Consider the existing FDD VSAT EIRP requirements for HD-FDD as the starting point - FFS on whether to modify. #### Issue 1-1-6: EIS requirements for HD-FDD - WF - Consider the existing FDD VSAT EIS requirements for HD-FDD as the starting point - FFS on whether to modify.

Compared to an FD-FDD VSAT UE, which uses a full duplexer, an HD-FDD VSAT UE applies an RF filter and a switch instead. With this simplified RF architecture, the insertion loss for HD-FDD is reduced. The impact of insertion loss improvement for HD-FDD devices was analyzed during the discussion of REFSENS requirements for NTN RedCap UEs. As a result, a tightening of up to 0.5 dB for RedCap REFSENS was agreed compared to non-RedCap NTN devices in FR1. Meanwhile, the maximum output power requirements remain unchanged, aligned with the legacy maximum output power for the respective power class.

Observation 1: Up to 0.5 dB tightening for RedCap REFSENSE was agreed compared to non-RedCap NTN devices in FR1. No changes were introduced for maximum output power requirements for RedCap NTN devices.

Therefore, following a similar principle, any potential enhancements to EIRP and EIS requirements for HD-FDD are expected to be minimal.

Observation 2: The potential enhancements to EIRP and EIS requirements due to insertion loss improvement for HD-FDD are negligible.

Propsoal 1: There is no need to consider the enhancements to EIRP and EIS requirements due to negligible insertion loss improvement for HD-FDD.

It should be noted that RF requirements for Ku-band VSAT are specified based on OTA testing, which differs from NTN RedCap for FR1, where conductive testing is applied. Therefore, antenna gain differences between a single physical antenna and two separate antenna panels should be considered when specifying EIRP and EIS requirements for HD-FDD VSAT. An evaluation has been conducted to investigate the antenna performance differences between a single physical antenna and two separate panels for UL and DL.

Table 1: Assumptions for evaluation

|  | Single physical antenna panel | Two separate antenna panels |
| --- | --- | --- |
| Carrier frequency | 14GHz for UL 11GHz for DL | 14GHz for UL 11GHz for DL |
| Physical element spacing | 0.5 * lambada_14GHz for both UL and DL | 0.5 * lambada_14GHz for UL 0.5 * lambada_11GHz for DL |
| Element number | 45*45 | 45*45 |
| Broadside | 0deg | 0deg |
| Steering angle | 30deg | 30deg |

Figure 1: Antenna pattern for single panel vs two panels

The physical element spacing of the single-panel antenna is assumed to be 0.5 λ of the Ku-band uplink (14 GHz). When the same panel transmits the downlink signal, the equivalent element spacing becomes approximately 0.4 λ of the Ku-band downlink (14 GHz). As shown in Figure 1, it can be observed that the peak gain remains unchanged in both cases, while the side lobes exhibit slight variations.

Since the minimum EIRP and EIS requirements for Ku-band FD-FDD are verified within the declared minimum elevation angle supported for transmission, we propose the following:

Proposal 2: The impact of antenna gain differences between a single physical antenna and two separate antenna panels should be investigated and considered when specifying EIRP and EIS requirements for HD-FDD VSAT.

2.2.2 VSAT with small antenna

For the issues of VSAT with small antenna, the following agreemetns were reached in [1]:

#### Issue 1-2-1: New VSAT type - WF - Study potential new VSAT type(s) for small antenna with the following candidates: - VSAT that support LEO only - VSAT that support both GSO and LEO #### Issue 1-2-2: Assumptions for VSAT with small antenna - WF - Conduct implementation and feasibility studies for the small antenna considering, but not limited to, the following aspects: - Antenna size, number of antenna elements, antenna gain, Tx power per element and link budget evaluation for supporting LEO only or supporting both GSO and LEO.

In Rel-19, it was assumed that VSAT would use 45×45 antenna elements (approximately 60 cm × 60 cm). For Rel-20, under the assumption of a 20 cm × 20 cm antenna, the number of antenna elements is approximately 15×15. The EIRP gap between a 45×45 array (~60 cm × 60 cm) and a 15×15 array (~20 cm × 20 cm) can be calculated as: 20*log10((45*45)/(15*15)) = 19dB.

Observation 3: The EIRP of a VSAT with a 45×45 (~60 cm × 60 cm) antenna array is approximately 19 dB higher than that of a VSAT with a 15×15 (~20 cm × 20 cm) antenna array.

In addition, based on the Rel-19 Ku-band coexistence study, the NTN UL and DL performance is shown in Table 2 [2].

|  | LEO 600 90deg | LEO 600 35deg | GEO 90deg | GEO 35deg |
| --- | --- | --- | --- | --- |
| 45*45(~60cm*60cm) | Available | Outage | Available | Outage |
| 78*78 (~105cm*105cm) | Not applicable | Not applicable | Available | Outage |

It can be observed that even with a 45×45 (~60 cm × 60 cm) antenna array, a cell-edge UE experiences outage for GEO at 35° elevation. With the 20 cm × 20 cm antenna assumption—which results in about 19 dB lower EIRP compared to the 60 cm × 60 cm case—it is not feasible to support GEO.

Observation 4: With the 20 cm × 20 cm antenna assumption (approximately 19 dB lower EIRP than 60 cm × 60 cm), it is not feasible to support GEO.

# 3. Conclusion

In summary, we have following observations and proposals:

Observation 1: Up to 0.5 dB tightening for RedCap REFSENSE was agreed compared to non-RedCap NTN devices in FR1. No changes were introduced for maximum output power requirements for RedCap NTN devices.

Observation 2: The potential enhancements to EIRP and EIS requirements due to insertion loss improvement for HD-FDD are negligible.

Propsoal 1: There is no need to consider the enhancements to EIRP and EIS requirements due to negligible insertion loss improvement for HD-FDD.

Proposal 2: The impact of antenna gain differences between a single physical antenna and two separate antenna panels should be investigated and considered when specifying EIRP and EIS requirements for HD-FDD VSAT.

Observation 3: The EIRP of a VSAT with a 45×45 (~60 cm × 60 cm) antenna array is approximately 19 dB higher than that of a VSAT with a 15×15 (~20 cm × 20 cm) antenna array.

Observation 4: With the 20 cm × 20 cm antenna assumption (approximately 19 dB lower EIRP than 60 cm × 60 cm), it is not feasible to support GEO.

# 4  References

- R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, MediaTek, Oct., 2025

- R4-2510240, Coexistence simulation results between TN and NTN for Ku band, Qualcomm, Aug., 2025