3GPP TSG-RAN WG4 Meeting #117	                                                                      R4-2521372

Dallas, US, November 17th-21st, 2025

Title: 	Simulation results for 64 CSI-RS ports for PMI test

Source: 	Huawei, HiSilicon

Agenda item:	6.12.3

Document for:	Discussion

Background

In this contribution we provide our simulation results for 64 CSI-RS ports for PMI tests according to [1].

- UE performance requirements

Simulation assumptions

Table 2.1-1: Simulation assumptions for typeI-SinglePanel-r19, FDD

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |
| Subcarrier spacing |  |  | kHz | 15 |
| Duplex Mode |  |  |  | FDD |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | High XP 64 x 2 High XP 64 x 4 (N1,N2) = (8,4) |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | The number of CSI-RS ports |  |  | 64 |
|  | Number of aggregated NZP CSI-RS resources |  |  | 2 |
|  | portMappingMethod |  |  | ‘method1’ |
|  | Configuration for each NZP CSI-RS resource | CSI-RS resource Type |  | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) |
|  |  | First OFDM symbol in the PRB used for CSI-RS resource 1 (l0, l1) |  | Row 17, (1, 7) |
|  |  | First OFDM symbol in the PRB used for CSI-RS resource 2 (l0, l1) |  | Row 17, (5, 12) |
|  |  | CSI-RS periodicity and offset |  | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | Not configured |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 7 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  |  | typeI-SinglePanel-r19 |
|  | typeI-codebookMode-r19 |  |  | 'modeA' |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (8,4) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,4) |
|  | (X1,X2) configuration(valueOfX1-typeI-CBSR-r19, valueOfX2-typeI-CBSR-r19) |  |  | (4,4) |
|  | CBSR configuration (typeI-CBSR-r19) |  |  | 0x FFFF FFFF |
|  | X1(s),X2(s) configuration (valueOfX1-typeI-powerScaling-r19 , valueOfX2-typeI-powerScaling-r19) |  |  | (4,4) |
|  | Power scaling factor configurations (typeI-softScalingRanks1-2-r19) |  |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF |
|  | RI Restriction |  |  | 00000010 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 11 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| Measurement channel (Note 4) |  |  |  | [R.PDSCH.x-y.z] FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 2.1-2: Simulation assumptions for eTypeII-r19, FDD

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |
| Subcarrier spacing |  |  | kHz | 15 |
| Duplex Mode |  |  |  | FDD |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | XP Medium 64 x 2 XP Medium 64 x 4 (N1,N2) = (8,4) |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | The number of CSI-RS ports |  |  | 64 |
|  | Number of aggregated NZP CSI-RS resources |  |  | 2 |
|  | portMappingMethod |  |  | ‘method1’ |
|  | Configuration for each NZP CSI-RS resource | CSI-RS resource Type |  | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  |  | First OFDM symbol in the PRB used for CSI-RS resource 1 (l0, l1) |  | Row 17, (1, 7) |
|  |  | First OFDM symbol in the PRB used for CSI-RS resource 2 (l0, l1) |  | Row 17, (5, 12) |
|  |  | CSI-RS periodicity and offset |  | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) |
|  | CSI-IM timeConfig interval and offset |  | slot | Not configured |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured |
| Sub-band Size |  |  | RB | 4 |
| csi-ReportingBand |  |  |  | 1111111111111 |
| CSI-Report interval and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 5 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration |  | Codebook Type |  | eTypeII-r19 |
|  |  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  |  | R(numberOfPMI-SubbandsPerCQI-subband-r19) |  | 1 |
|  |  | N1,N2 value (n1-n2-typeII-r19) |  | (8,4) |
|  |  | (X1(c),X2(c)) configuration (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) |  | (4,2) |
|  |  | CBSR configuration (typeII-CBSR-r19) |  | 1111 |
|  |  | RI Restriction (typeII-RI-restriction-r19) |  | 0010 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 11 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| Measurement channel |  |  |  | [R.PDSCH.x-y.z] FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |

Table 2.1-3: Simulation assumptions for typeI-SinglePanel-r19, TDD

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | XP Medium 64 x 2 XP Medium 64 x 4 (N1,N2) = (8,4) |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | The number of CSI-RS ports |  |  | 64 |
|  | Number of aggregated NZP CSI-RS resources |  |  | 2 |
|  | portMappingMethod |  |  | ‘method1’ |
|  | Configuration for each NZP CSI-RS resource | CSI-RS resource Type |  | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) |
|  |  | First OFDM symbol in the PRB used for CSI-RS resource 1 (l0, l1) |  | Row 17, (1, 7) |
|  |  | First OFDM symbol in the PRB used for CSI-RS resource 2 (l0, l1) |  | Row 17, (5, 12) |
|  |  | CSI-RS periodicity and offset |  | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | Not configured |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 16 |
| csi-ReportingBand |  |  |  | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 12 |
| CSI request |  |  |  | 6 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  |  | typeI-SinglePanel-r19 |
|  | typeI-codebookMode-r19 |  |  | 'modeA' |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (8,4) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,4) |
|  | (X1,X2) configuration(valueOfX1-typeI-CBSR-r19, valueOfX2-typeI-CBSR-r19) |  |  | (4,4) |
|  | CBSR configuration (typeI-CBSR-r19) |  |  | 0x FFFF FFFF |
|  | X1(s),X2(s) configuration (valueOfX1-typeI-powerScaling-r19 , valueOfX2-typeI-powerScaling-r19) |  |  | (4,4) |
|  | Power scaling factor configurations (typeI-softScalingRanks1-2-r19) |  |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF |
|  | RI Restriction |  |  | 00000010 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 11 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| Measurement channel (Note 4) |  |  |  | [R.PDSCH.x-y.z] TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 2.1-4: Simulation assumptions for eTypeII-r19, TDD

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | XP Medium 64 x 2 XP Medium 64 x 4 (N1,N2) = (8,4) |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | The number of CSI-RS ports |  |  | 64 |
|  | Number of aggregated NZP CSI-RS resources |  |  | 2 |
|  | portMappingMethod |  |  | ‘method1’ |
|  | Configuration for each NZP CSI-RS resource | CSI-RS resource Type |  | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  |  | First OFDM symbol in the PRB used for CSI-RS resource 1 (l0, l1) |  | Row 17, (1, 7) |
|  |  | First OFDM symbol in the PRB used for CSI-RS resource 2 (l0, l1) |  | Row 17, (5, 12) |
|  |  | CSI-RS periodicity and offset |  | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) |
|  | CSI-IM timeConfig interval and offset |  | slot | Not configured |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured |
| Sub-band Size |  |  | RB | 4 |
| csi-ReportingBand |  |  |  | 1111111111111 |
| CSI-Report interval and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 12 |
| CSI request |  |  |  | 6 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration |  | Codebook Type |  | eTypeII-r19 |
|  |  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  |  | R(numberOfPMI-SubbandsPerCQI-subband-r19) |  | 1 |
|  |  | N1,N2 value (n1-n2-typeII-r19) |  | (8,4) |
|  |  | (X1(c),X2(c)) configuration (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) |  | (4,2) |
|  |  | CBSR configuration (typeII-CBSR-r19) |  | 1111 |
|  |  | RI Restriction (typeII-RI-restriction-r19) |  | 0010 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 9 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| Measurement channel |  |  |  | [R.PDSCH.x-y.z] TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |

Simulation results

Table 2-1: Summary of SNR(dB)@ 90% of maximum throughput

|  |  | Type 1 codebook | eType 2 codebook |
| --- | --- | --- | --- |
| FDD | 2Rx | 7.5 | 2.7 |
|  | 4Rx | 1.0 | 0.8 |
| TDD | 2Rx | 7.7 | 3.4 |
|  | 4Rx | 1.0 | 0.8 |

- BS performance requirements

Simulation assumptions

Table 3-1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| MIMO layers |  | 3 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0, 1, 2} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Precoder |  | 13100010001 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Simulation results

Table 3-2: Simulation results

| case | Tx | SCS | Waveform | BW | Mapping Type | Propagation condition | MCS | SNR(dB)@70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 15kHz | CP-OFDM | 5MHz | A | TDLB100-400 | 2 | -1.9 |
| 2 |  |  |  |  | B |  |  | -1.9 |
| 3 | 3 | 15kHz | CP-OFDM | 5MHz | A | TDLC300-100 | 16 | 13.2 |
| 4 |  |  |  |  | B |  |  | 13.2 |
| 5 | 3 | 15kHz | CP-OFDM | 5MHz | A | TDLA30-10 | 20 | 14.6 |
| 6 |  |  |  |  | B |  |  | 14.7 |
| 7 | 3 | 15kHz | CP-OFDM | 50MHz | A | TDLB100-400 | 2 | -1.7 |
| 8 |  |  |  |  | B |  |  | -1.8 |
| 9 | 3 | 15kHz | CP-OFDM | 50MHz | A | TDLC300-100 | 16 | 13.8 |
| 10 |  |  |  |  | B |  |  | 13.8 |
| 11 | 3 | 15kHz | CP-OFDM | 50MHz | A | TDLA30-10 | 20 | 16.6 |
| 12 |  |  |  |  | B |  |  | 16.6 |
| case | Tx | SCS | Waveform | BW | Mapping Type | Propagation condition | MCS | SNR(dB)@70% of maximum throughput |
| 1 | 3 | 30kHz | CP-OFDM | 10MHz | A | TDLB100-400 | 2 | -2.4 |
| 2 |  |  |  |  | B |  |  | -2.4 |
| 3 | 3 | 30kHz | CP-OFDM | 10MHz | A | TDLC300-100 | 16 | 13.8 |
| 4 |  |  |  |  | B |  |  | 13.8 |
| 5 | 3 | 30kHz | CP-OFDM | 10MHz | A | TDLA30-10 | 20 | 14.7 |
| 6 |  |  |  |  | B |  |  | 14.7 |
| 7 | 3 | 30kHz | CP-OFDM | 100MHz | A | TDLB100-400 | 2 | -2.2 |
| 8 |  |  |  |  | B |  |  | -2.1 |
| 9 | 3 | 30kHz | CP-OFDM | 100MHz | A | TDLC300-100 | 16 | 13.6 |
| 10 |  |  |  |  | B |  |  | 13.8 |
| 11 | 3 | 30kHz | CP-OFDM | 100MHz | A | TDLA30-10 | 20 | 16.7 |
| 12 |  |  |  |  | B |  |  | 16.7 |

Table 3-2: Simulation results

| case | Tx | SCS | Waveform | BW | Mapping Type | Propagation condition | MCS | SNR(dB)@70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 15kHz | CP-OFDM | 5MHz | A | TDLB100-400 | 2 | -5.2 |
| 2 |  |  |  |  | B |  |  | -5.1 |
| 3 | 3 | 15kHz | CP-OFDM | 5MHz | A | TDLC300-100 | 16 | 7.2 |
| 4 |  |  |  |  | B |  |  | 7.4 |
| 5 | 3 | 15kHz | CP-OFDM | 5MHz | A | TDLA30-10 | 20 | 9.3 |
| 6 |  |  |  |  | B |  |  | 9.3 |
| 7 | 3 | 15kHz | CP-OFDM | 50MHz | A | TDLB100-400 | 2 | -4.8 |
| 8 |  |  |  |  | B |  |  | -4.8 |
| 9 | 3 | 15kHz | CP-OFDM | 50MHz | A | TDLC300-100 | 16 | 7.6 |
| 10 |  |  |  |  | B |  |  | 7.7 |
| 11 | 3 | 15kHz | CP-OFDM | 50MHz | A | TDLA30-10 | 20 | 10.0 |
| 12 |  |  |  |  | B |  |  | 10.0 |
| case | Tx | SCS | Waveform | BW | Mapping Type | Propagation condition | MCS | SNR(dB)@70% of maximum throughput |
| 1 | 3 | 30kHz | CP-OFDM | 10MHz | A | TDLB100-400 | 2 | -5.6 |
| 2 |  |  |  |  | B |  |  | -5.6 |
| 3 | 3 | 30kHz | CP-OFDM | 10MHz | A | TDLC300-100 | 16 | 7.2 |
| 4 |  |  |  |  | B |  |  | 7.2 |
| 5 | 3 | 30kHz | CP-OFDM | 10MHz | A | TDLA30-10 | 20 | 9.4 |
| 6 |  |  |  |  | B |  |  | 9.4 |
| 7 | 3 | 30kHz | CP-OFDM | 100MHz | A | TDLB100-400 | 2 | -5.5 |
| 8 |  |  |  |  | B |  |  | -5.4 |
| 9 | 3 | 30kHz | CP-OFDM | 100MHz | A | TDLC300-100 | 16 | 7.4 |
| 10 |  |  |  |  | B |  |  | 7.5 |
| 11 | 3 | 30kHz | CP-OFDM | 100MHz | A | TDLA30-10 | 20 | 10.2 |
| 12 |  |  |  |  | B |  |  | 10.2 |

- Conclusion

In this contribution we provide simulation results for 64 CSI-RS ports for PMI test and PUSCH with 3 layers

- Reference

[1] R4-2512527-Way Forward for [116][327] NR_MIMO_demod