3GPP TSG-RAN WG4 Meeting #117	R4-2521373

# Dallas, US, November 17th-21st, 2025

Title: 	Discussion on performance requirements for LP-WUS

Source: 	Huawei, HiSilicon

Agenda item:	6.16.3

Document for:	Discussion

Background

A WF[1] on NR_LPWUS_Demod was agreed, where some open issues are left for discussion. In this contribution we provide our views.

- Discussion

One issue is whether to define OOK receiver for FR2:

Receiver type for FR2 - Agreement: Consider at least OFDM receiver for FR2. FFS OOK receiver.

Standardization allows OFDM receiver to be operated in FR2, hence it’s proposed to cover this in demodulation test to ensure the test coverage.

Proposal 1: RAM4 to consider OOK receiver in FR2

One issue is if UE support both RRC idle/inactive and connected models which test will be conducted:

Whether to define separate test cases for idle/inactive and connected modes - Agreement: Define one set of requirements and add test applicability rule that the test is conducted in idle/inactive and/or connected modes based on UE supported capabilities. If UE support both RRC idle/inactive and connected modes, the test will be conducted: - Option 1: in connected mode - Option 2: in idle/inactive mode - Option 3: in both RRC idle/inactive and connected modes FFS whether to capture the test applicability w.r.t. RRC state in 38.101-4 or leave it to RAN5

In our understanding, LP-WUS is more likely to be used in RRC idle/inactive mode than connected mode, the reason is that detecting LP-WUS and related PDCCH in long period will impact the continuity of data scheduling and decrease the desired throughput. Therefore, we propose that if UE support both RRC idle/inactive and connected modes, the test will be conducted in idle/inactive mode. The applicability rules shall be defined in 38.101-4 since RAN5 has never defined the test applicability rules before.

Proposal 2: If UE support both RRC idle/inactive and connected modes, the test will be conducted in idle/inactive mode. The applicability rules shall be defined in 38.101-4

One issue is consideration of one-shot and repetition transmission mode for OFDM based receiver:

Consideration of one-shot and repetition transmission mode for OFDM-based receiver - Proposals - Option 1 - RAN4 to specify demodulation requirements for both one-shot and full-repetition cases for OFDM-based receivers.

In our understanding, one-shot means UE performs detection without any combination of repetition, which should be considered as baseline receiver for the purpose of defining minimum requirements due to the higher SNR.

Proposal 3: The performance requirements for OFDM based receiver shall be defined with the assumption of no repetition combination.

One issue is whether to define single requirements for OFDM and OOK receiver:

Issue 2-1-16: Whether to define single requirement for OFDM and OOK receivers - Proposals - Option 1: Define two sets of requirements (one for each receiver type: OOK and OFDM). - Option 2: RAN4 to study the performance for both receiver types (OOK receiver and OFDM receiver) and further discuss based on simulation results if one or two sets of requirements are to be defined

The issue is similar as the issue “Whether to define single or separate set of requirements for baseline 8Rx receiver and simplified 8Rx receiver”, the solution is that defining the threshold for performance difference between two receiver types for justification. We propose to reuse this solution for LP-WUS requirements definition:

Proposal 4: Collect simulation results for both OOK receiver and OFDM receiver and calculate the average target SNR across all contributed companies, if the SNR difference between two receiver types is larger than 0.3dB, define two sets of requirements, otherwise, define one set of requirements.

One issue is whether to define single applicability rules for OFDM and OOK receiver:

Issue 2-1-17: Whether to define applicability rules for OFDM and OOK receivers - Proposals - Option 1: Don’t define any capability rules related to OFDM and OOK receivers for demodulation requirements. - Option 2: Defer the discussion of applicability rules until simulation results have been analysed for both receivers.

We think no matter single or two sets of requirements will be defined, the capability rules related to OFDM and OOK receivers shall be defined. If single set of requirements will be defined, the capability rules shall be defined to explicitly mentioning requirement is applied to UE support either OFDM or OOK receivers. If two set of requirements will be defined, the capability rules shall be defined to indicate which set of requirements is applied to which receiver type.

Proposal 5: Define the capability rules related to OFDM and OOK receivers.

  - If single set of requirements will be defined, the capability rules shall be defined to explicitly mentioning requirement is applied to UE support either OFDM or OOK receivers.

  - If two set of requirements will be defined, the capability rules shall be defined to indicate which set of requirements is applied to which receiver type.

- Conclusion

In this contribution we provide our views on open issues for LP-WUS demodulation performance, the proposals are:

Proposal 1: RAM4 to consider OOK receiver in FR2

Proposal 2: If UE support both RRC idle/inactive and connected modes, the test will be conducted in idle/inactive mode. The applicability rules shall be defined in 38.101-4

Proposal 3: The performance requirements for OFDM based receiver shall be defined with the assumption of no repetition combination.

Proposal 4: Collect simulation results for both OOK receiver and OFDM receiver and calculate the average target SNR across all contributed companies, if the SNR difference between two receiver types is larger than 0.3dB, define two sets of requirements, otherwise, define one set of requirements.

Proposal 5: No matter single or two sets of requirements will be defined, the capability rules related to OFDM and OOK receivers shall be defined. If single set of requirements will be defined, the capability rules shall be defined to explicitly mentioning requirement is applied to UE support both OFDM and OOK receivers. If two set of requirements will be defined, the capability rules shall be defined to indicate which set of requirements is applied to which receiver type.

- Reference

[1] R4-2514813 WF for [116bis][220] NR_LPWUS_Demod