3GPP TSG-RAN4 Meeting #117	R4-2521434

Dallas, USA, 17 November – 21 November 2025

Agenda item:			8.1

Source:	Feature lead (Ericsson)

Title:	Summary of key agreements for 6G BS RF and Coexistence

Document for:	Information

# Introduction

Based on the latest approved RAN working group level SID on 6G Radio [RP-252912], RAN4 study on BS RF and coexistence has the following objective provided:

- 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles

The scope of this running summary is to capture:

- The status update of the different 6G BS RF and coexistence objectives.

- The RAN4 agreements related to 6G BS RF and coexistence aspects.

- The RAN1 agreements related to 6G BS RF and coexistence.

- The RAN2 agreements related to 6G BS RF and coexistence.

| Feature Lead | Company | Email address |
| --- | --- | --- |
| Dominique Everaere | Ericsson | dominique.everaere@ericsson.com |

# Status update

The scope is still under definition.

BS RF requirements

| Topic | Description | Expected Timeline (if any) | Status |
| --- | --- | --- | --- |
| BS type 1-H enhancement | Study BS Hybrid beamforming type of architecture |  | Not started |
| Rx requirements for cmWave bands | How BS Rx requirements should be specified, using FR2 approach or not, including if blocking should be specified based on FR2 methodology |  | Not started |

Coexistence

| Topic | Description | Expected Timeline (if any) | Status |
| --- | --- | --- | --- |

# RAN4 - 6G BS RF and coexistence agreements

## RAN4 #116bis meeting (Oct-2025)

### Key tdocs

| R4-2514510 | Topic Summary for [116bis][103] BS RF and coexistence |
| --- | --- |
| R4-2514368 | WF on 6G BS RF and co-existence |

### Key agreements

#### BS RF requirements

##### Issue 1-1-1: Starting point

Agreement:

5G BS RF requirements should be considered as starting point when defining the 6G BS RF requirements.

#### Issue 1-2-1: BS type 1-H enhancement

Agreement:

BS Hybrid beamforming type of architecture shall be studied.

#### Requirements for cmWave bands

##### Issue 1-3-3: Rx Requirements

Agreement:

For above 7 GHz, further study how BS Rx requirements should be specified, using FR2 approach or not, including if blocking should be specified based on FR2 methodology.

#### MSR aspects

##### Issue 3-1-1: RATs to be considered and new capability sets

Agreement:

Consider only new capability set(s) with 4G, 5G and 6G for MSR as starting point.

Additional capability sets with 2G and/or 3G could be considered, on operators’ request.

#### SBFD

Agreement:

For SBFD in 6G SI, RAN4 will leverage the Rel-19 and Rel-20 outcomes, and additional scope can be considered if the necessity is identified later.

# RAN1 - 6G BS RF and coexistence related agreements summary

## Overview of 6GR air interface

### RAN1 #122bis (Oct-2025)

Agreement

- High-level aspects to consider for the 6GR sync signal structure include, but not limited to

  - Sync raster design

  - Spectrum allocation

  - smallest maximum supported RF and BB UE BW without spectrum aggregation

  - mobile broadband service requirements as high priority

  - Energy efficiency for both BS and UE

  - Detection/tracking performance, latency, and complexity

  - Including initial cell search

  - Coverage target

  - Common design for diverse device types

  - Consideration of the supported deployment

  - Consideration on whether the single sync signal structure is sufficient

  - Note: Aspects impacting on the periodicity is to be discussed under AI11.5

Agreement

- The aspects to consider for supporting NTN include, but not limited to

  - Initial access, including cell search and SSB periodicity

  - Coverage

  - Duplexing

  - Capacity

  - Signalling overhead

  - GNSS-less/resilient/based operation

  - Large/varying doppler and propagation delay

  - Beamforming / beam management / beam hopping

Note:

- High-level aspects to consider to enable lower CAPEX/OPEX with respect to current networks include, but not limited to

  - UE/NW implementation complexity

  - UE/NW energy efficiency

  - MRSS

  - Spectrum efficiency

### RAN1 #122 (Aug-2025)

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- Study and identify the lessons learned from NR spectrum utilization and aggregation framework

  - DC is subject to RANP decision in June 2026

  - Note: MRSS aspects are separate discussion

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

Agreement

- Study and identify the lessons learned from NR duplex modes

- On 6GR duplexing study, RAN1 considers at least following duplex types

  - FD-FDD

  - Semi-static TDD

  - gNB semi-static SBFD

  - HD-FDD on UE side

  - Dynamic TDD

- Study whether to consider following duplexing types

  - gNB dynamic SBFD

  - UE SBFD

  - gNB FD

  - Note: Other duplex modes are not precluded

Agreement

- For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

## Waveform for 6GR air interface

### RAN1 #122bis (Oct-2025)

Agreement

Final LS R1-2508069 is endorsed.

### RAN1 #122 (Aug-2025)

Agreement

CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink

- Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions

- Other OFDM based waveforms are not precluded.

Agreement

CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink

- Enhancements/modifications on CP-OFDM will be studied as potential additions

- DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink

## Frame structure for 6GR air interface

### RAN1 #122bis (Oct-2025)

Agreement

For communication, 6GR considers NR frame structure used as a starting point for the study item,

  - Resource defined by one subcarrier and one symbol is called as resource element (RE).

  - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12

  - Radio Frame length is 10ms

  - Each radio frame is split into 10 subframes, each with a duration of 1 ms

  - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design.

  - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings.

Agreement

- RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz

- Study whether and how to enable UE to support 400MHz bandwidth

### RAN1 #122 (Aug-2025)

Agreement

- 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH.

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD, 30kHz SCS for TDD

    - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

    - FFS: 7.5kHz SCS for sub1GHz (FDD)

    - Whether to discuss the FFS will be subject to RANP decision.

  - For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

    - Whether to discuss it will be subject to RANP decision

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

- FFS subcarrier spacing for PRACH and up to initial access discussion.

Conclusion

Numerologies for sensing is up to sensing agenda discussion.

Agreement

- 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR.

  - FFS potential need for other CP

## Modulation, joint channel coding and modulation

### RAN1 #122bis (Oct-2025)

Agreement

For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model).

- FFS: How to involve RAN4 early

- FFS: Shaping of higher order modulation

- System level evaluation can be done after link level evaluation.

### RAN1 #122 (Aug-2025)

Agreement

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

## Initial access

## Duplexing

## 6GR spectrum utilization and aggregation

## NTN

## Other

# RAN2 - 6G BS RF and coexistence related agreements summary

## Radio protocol architecture

### Spectrum Aggregation

#### RAN2 #131bis (Oct-2025)

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

### System Information

#### RAN2 #131bis (Oct-2025)

Agreements on System Information

- -	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

## Mobility

### Mobility framerwork and targets

#### RAN2 #131bis (Oct-2025)

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)