3GPP TSG-RAN4 Meeting #117	R4-2521437

Dallas, US, November 17th – 21st 2025

Source:			Ericsson

Title:			6G BS other aspects

Agenda item:		8.4.4

Document for:		Approval

# Introduction

RAN4 started the discussion related to 6G BS RF aspects and 6G co-existence evaluation aspects in last RAN4#116bis meeting and a Way Forward ([2]) with initial agreements  was agreed.

In this contribution, we discuss the other BS RF aspects to be considered within the scope of RAN4 6G SI.

# Discussion

## MSR aspects

As mentioned in our RAN4#116bis contribution, we think that, with 6G, some BS will support bands in both FR1 and FR2-1. To enable this, we make the following proposal:

Proposal 1: In the 6G scope, FR2-1 support should be added to multi-standard AAS specifications.

## Conformance aspects and measurement uncertainties

With the introduction of new bands above 7 GHz, RAN4 will have to evaluate the expected measurement uncertainties associated to RF requirements testing. In the work in Rel-19 related to band n104, some potential improvements related to bands in the upper region of FR1 was identified.

The evolution of the OTA test methods towards 6G introduction is mainly affected by the trend towards increasing number of elements in the antenna array (e.g. over 1024 elements and higher) intended for operation within new bands above FR1 and higher total output power. These two parameters have direct impact on the BS physical properties as size, weight and power consumption, which directly affect the measurement distance, size of the OTA chamber, output power of the interfering signals and environmental control of OTA test range.

Among the challenges we see when 6G BS will end up on the testing, we see the need for measurements performed at distances which are smaller than far-field, especially for BS with very large antenna arrays (e.g. 1024 antenna elements or more) and also very large output power. The new frequency bands for 6G might also introduce new challenges (e.g. need for new absorbers in the OTA chamber, etc.), but these aspects are expected to be manageable and not impact the 3GPP specifications.

Description of OTA measurements for total radiated power (TRP) as presented now in the Annex I in TS 38.141-2 is already under discussion in Rel-19 and we hope a number of improvements will be adopted before copy-pasting this annex into the new 6G specifications. One of the important aspects are the definition of a reference measurement method for TRP, revision of some of current methods and eventually removal of the obsolete ones. Handling the reverberation chamber, as an alternative to the anechoic chamber, is also a subject for improvement.

A few enhancements can be also added when measuring TRP. One is to use a concept that we already introduced, but without giving many details, the beam sweeping.  There is significant potential to reduce the density of the TRP measurement grid if the beams can be swept during the measurement time. A careful choice of the sweeping period, together with proper setting of the measurement device can lead to very fast measurements compared to the ones currently described. Beam sweeping would also better mimic normal operation.

Pre-scan procedure, used to further reduce the test time in case of spurious emissions, can be enhanced so it can deliver EIRP or even TRP values that can be utilized for deciding compliance, without the need for a very large margin (e.g. 30 dB as considered today). The use of reverberation chamber during pre-scan can also help, as measurements can be made faster with a smaller number of independent samples. More precise measurements can be later performed in the same setup.

Proposal 2: Study OTA test methods with the intention to improve description (e.g. beam sweeping improvement, rep-scan procedure, …) and measurement uncertainties for bands in the upper region of FR1 and above.

# Conclusion

In this contribution, we discussed the other 6G BS RF aspects to be considered in the in the scope of the 6G SI. We made the following proposals:

Proposal 1: In the 6G scope, FR2-1 support should be added to multi-standard AAS specifications.

Proposal 2: Study OTA test methods with the intention to improve description (e.g. beam sweeping improvement, rep-scan procedure, …) and measurement uncertainties for bands in the upper region of FR1 and above.

# References

- R4-2513339, BS RF and co-existence considerations for 6G, Ericsson

- R4-2514638, WF on 6G BS RF and coexistence, Feature Lead (Ericsson)