3GPP TSG-RAN WG4 Meeting #117                                                            R4-2521522

Dallas, USA, November 17th – 21th,2025

Agenda Item:	8.2.3

Source: 	CSCN

Title: 	Views on 6G system parameters

Document for:	Discussion

- Introduction

In RAN #108 meeting, a New SID for “Study on 6G Radio” was approved [1] aims to develop 6GR to support a broad range of use cases, and meet requirements as defined during the RAN requirements study. In RAN4#116-bis meeting, some agreements about the 6G system channel bandwidth are achieved as shown in following [2].

- Agreement In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths. - RF performance evaluation with proposed max CBW - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum. - Assess the implementation feasibility and complexity and power consumption - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1. - Study the feasibility to support different NW and UE max CBW. - Collaborate with RAN1 on feasibility findings - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations. - Agreement - Compare options of defining min CBW, considering pros and cons - Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands - SCS-dependent framework - Other aspects are not precluded - Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective. - Agreement - Consider FFT size, maximum Channel Bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective, especially for 8K or 16K FFT size considering the associated SCS and also the frequency ranges - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

According to above agreements, some views on the topics of system parameters for 6GR, such as SCS, channel bandwidth, spectrum utilization, are provided in this contribution, especially from the perspective of 6G NTN (Non-Terrestrial Network).

- Discussion

The vision for 6G is to bridge the digital divide in its entirety and realize seamless, integrated connectivity across all domains: aerial, spatial, terrestrial and maritime. In the context of 6G, NTN is not merely a significant use case, it constitutes a fundamental architectural component and is indispensable to the core technology framework.

Considerations should be given to aspects of 5GR that require refinement or simplification for 6G design. The differences between 6G and 5G considerations from the perspectives of system parameters are illustrated in the following.

  - 2.1  SCS

In 5G-NR, the SCS design is highly flexible with multiple SCS options in a band, not only different Bandwidth Parts (BWPs), but the Synchronization Signal Block (SSB) and the initial Downlink Bandwidth Part (initial DL BWP) also support different SCSs. However, in actual 5G-NR deployments, most of the aforementioned flexibilities are not realized. For example, 60kHz SCS for FR1 is rarely used. Therefore,6GR can eliminate these redundant and unused flexibility to reduce unnecessary implementation and testing complexity for both gNB and UE.

From the agreements achieved in RAN1#122, we observe that for FR1 (sub 6GHz) band, support 15kHz SCS for FDD and 30kHz SCS for TDD. Meanwhile, for FR2 band (24.25GHz-52.6GHz), 120kHz SCS is supported. For 6GR, we propose that the FDD mode below 6GHz needs to support 30kHz SCS. From a technical perspective, 30kHz SCS in FDD bands demonstrates several obvious benefits over 15kHz SCS especially in frequency offset resistance and low latency performance under high speed scenarios e.g. terminal mounted over train, car, airplane and ship.

Proposal 1: For 6GR bands, consider reducing the SCS options set.

Proposal 2: For sub-6GHz (FR1) and under 10G, 30kHz SCS should be supported.

  - 2.2  CHBW

In 5G-NR, maximum FFT size of 4K is assumed, which can natively support up to 50MHz/100MHz/400MHz for 15KHz/30KHz/120KHz respectively. For 6GR NTN, it is necessary to determine the FFT size for the Orthogonal Frequency Division Multiplexing (OFDM) family of waveforms. Hardware computing resources of 6G equipment are expected to achieve a significant development compared with 5G. Assuming 6GR NTN can support higher FFT complexity (e.g.,8K), a larger FFT size can support a wider system bandwidth (when SCS is fixed), thereby improve the peak rate and spectral efficiency of a single carrier. Hence, we have

Proposal 3: Under the premise that the maximum FFT size of 6GR NTN is 8K,

- For 30kHz SCS, a maximum bandwidth of 200MHz should be supported.

- For 120kHz SCS, a maximum bandwidth of 800MHz should be supported.

  - 2.3  Spectrum

The currently available spectrum resources for 5G-NTN are relatively limited. On one hand, 6GR-NTN may consider reusing the existing frequency bands of 5G-NTN, on the other hand, portions of the 5G terrestrial network (TN) spectrum could also be allocated for supplementary coverage in NTN networks, though it must be ensured that NTN operations do not cause any interference to TN networks. NTN can be deployed either as an independent network or as a comprehensive coverage solution together with TN.

Proposal 4:  6GR should study TN-NTN spectrum sharing.

- Conclusion

In this contribution, some views on the topics of system parameters for 6G NTN are provided.

Proposal 1: For 6GR bands, consider reducing the SCS options set.

Proposal 2: For sub-6GHz (FR1) and under 10G, 30kHz SCS should be supported.

Proposal 3: Under the premise that the maximum FFT size of 6GR NTN is 8K,

- For 30kHz SCS, a maximum bandwidth of 200MHz should be supported.

- For 120kHz SCS, a maximum bandwidth of 800MHz should be supported.

Proposal 4:  6GR should study TN-NTN spectrum sharing.

Reference

- RP-251881, New SID, Study on 6G Radio, NTT DoCoMo

- R4-2514618, AGREEMENT for 6GR system parameter, Huawei