3GPP TSG-RAN WG4 Meeting #117		    R4-2521569
Dallas, USA, Nov. 17-21, 2025

Title:	On 6GR device type

Source:	OPPO

Agenda item:	8.2.5

Document for:	Approval

Introduction

In the RAN plenary (RAN#108), a study item on ‘6G RAN’ was approved [1]. This contribution discusses device type as per RAN4 guideline of 6GR discussion.

Discussion

Number of TX and RX

During the last RAN4#116bis meeting, the WF [2] has been agreed and the number of TX and RX are discussed with the agreement captured as below

- Agreement

  - Study the framework of spec impact with different number of Tx/Rx

  - Evaluate number of Tx/Rx from both a performance and implementation complexity perspective.

    - E.g. device size (e.g. foldable smartphone)/form factors constraints, antenna design

  - Co-ordinate with RAN and RAN1 for number of Tx/Rx

From UE spec framework perspective, currently the TX and RX requirements are applicable for different number of TX and RX with different spec structure. For TX requirements, currently the UL MIMO and TxD are introduced with suffix D and G, respectively. However, due to many requirements are duplicated, the suffix clauses especially the suffix G has been arranged with lots of requirements missing.

The UL MIMO and TxD features has been arranged with suffix D and G respectively to capture the UE RF requirement.

Most of the 2TX requirement are duplicated except for output power, MPR/A-MPR.

For the RX requirement, currently the REFSENS requirement has considered number of RX and no suffix is added. The different requirement for 1RX/2RX/4RX has been listed in one sub-clause. This helps to reduce the duplicated workload and makes the spec clean, however it is hard to find the exact requirement with such method.

The REFSENS requirement has considered number of RX and different requirement for 1RX/2RX/4RX has been listed in one sub-clause.

Current REFSENS requirement structure makes it hard to find the exact requirement.

With that it is better to differentiate the requirements with number of TX and RX and give instruction in the general sub-clause in spec. This can be further discussed in UE RF requirement part. Further requirement difference can be discussed in UE RF part.

To differentiate RF requirements with number of TX/RX under one sub-clause and give instruction in general sub-clause.

The discussion around the number of transmit (TX) and receive (RX) antennas in 5G NR has been crucial for balancing performance, complexity, and device form factors across different use cases. Below table 1 has provided a summary for different UE types of number of TX and RX for 5G.

Table 1 NR UE baseline number of RX and TX

|  | Rel-15 NR UE | Rel-16 | Rel-17 | Rel-18 | Rel-18 XR | Rel-19 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of RX and TX | FR1 FDD: 1T2R FR1 TDD: 1T4R | V2X: 2RX UL MIMO: 2TX | Redcap: FR1 FDD: 1T1R FR1 TDD: 1T2R | eRedCap: FR1 FDD: 1T1R FR1 TDD: 1T2R | FR1 FDD/TDD: 1T2R | 6RX |
|  |  |  | FWA: 8RX | 3TX |  |  |

As can be seen in the early release the NR UE is defined, then in Rel-16 for V2X, 2RX is introduced due to the vehicle antenna deployment complexity. UL MIMO with 2TX is introduced to increase the UL throughput and for FR2 4RX is introduced to improve the coverage. Then in Rel-17 and Rel-18, Redcap and eRedCap are introduced with 1T1R for FDD bands and 1T2R for TDD bands. Larger number of RX for FWA as 8RX is introduced in Rel-17 and further 6RX is introduced in Rel-19 with also hand-held UE supported.

For hand-held UE, maximum capability is 2TX and 6RX till the end of NR evolution.

For reduced capability UE, 1T1R for FR1 FDD and 1T2R for FR1 TDD have been agreed.

For the assumption of 6GR UE, the number of TX and RX depends on the device type definition. From our understanding, current number of TX and RX for NR UE will be reused for FR1 re-farming bands since the antenna size is based on the frequency it operates at. If there is no UE form factor change, then the antenna size limitation is the same in 6GR compared to NR. Furthermore, current handheld UE with maximum 3TX in UL, the UE power consumption has been large while with large MPR, the exact output that can be reached is limited.

Antenna size is one of the limitations of number of TX and RX, especially in FR1 NR re-farming bands with relative low frequency.

Larger number of UE TX increases UE power consumption.

For FR1 refarming bands, similar number of TX and RX of NR is proposed for 6GR.

While for the around 7GHz, with the higher frequency and larger pathloss, if we still want to guarantee the UL coverage, higher output power of UE is expected. In such case, increase the number of TX from 1T to 2TX with baseline seems appropriate.

To guarantee the UL coverage for around 7GHz band, larger output power of TX is needed.

For around 7GHz band, 2TX as baseline is proposed.

With that, below table 2 is proposed for number of TX and RX for different device differentiation.

The number of TX/RX below is proposed based on different device differentiation:

Table 2 Proposed Number of TX and RX for different device differentiation

|  | Device differentiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 6G IoT (Lowest-tier) | eMBB (Handheld UE) |  |  | FWA (e.g., CPE) |
|  |  | FR1 FDD | FR1 TDD | Around 7GHz |  |
| number of Tx/Rx | Baseline: 1T1R Optional: 2R | Baseline: 1T2R Optional: 2T/3T; 4R | Baseline: 1T4R Optional: 2T/3T; 6R | Baseline: 2T4R Optional: 3T; 6R | Baseline: 4T8R Optional: 6T;12R |

Device Type

During the last RAN4#116bis meeting, the WF [2] has been agreed and the device types are discussed with the agreement captured as below. Although RAN will finally agree on the device type definition, RAN4 can focus on the UE max capability from UE RF perspective including but not limited to number of TX and RX, max UE channel bandwidth, UE power class, etc. Hence below sub-clauses give detail consideration by looking back to NR device background and try to give instruction for 6GR device types.

- Agreement

  - The following aspects could be further studied

    - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range.

      - Study on the NW impact should also be considered

    - Study how to better support variety devices from RAN4 perspective

      - E.g. differentiate devices, use cases, features

    - Other aspects are not precluded

  - Collaborate on the framework with other WGs:

    - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

NR device background

The standardization of 5G device types has evolved to address diverse use cases, form factors, and deployment scenarios, balancing performance, cost, and power efficiency. Below table 3 has captured the different device types and their evolution during the 5G timeline.

Table 3 Device types of 5G with time evolution

|  | Rel-15 | Rel-16 | Rel-17 | Rel-18 | Rel-19 |
| --- | --- | --- | --- | --- | --- |
| Smartphone | FR1: 1T2R/4R | FR1: 2TX |  |  | 6RX with 4 layer |
| Fixed wireless access | FR1: 1T2R/4R |  | 8RX |  | 6RX with 6 layer |
| Vehicular |  | 2RX |  |  |  |
| Redcap |  |  | 1Rx/1Tx relaxed BW | eRedcap 5MHz BB bandwidth. |  |
| UAV |  | aerial UE |  |  |  |
| XR |  |  | 2RX |  |  |
| NTN |  |  | FR1 | VSAT IoT-NTN PC2/PC1.5 | Ku/Ka band |

From the table 3 we can see different device types has been introduced as some of them are depend on the usage scenario such as vehicular, UAV, XR and NTN while some of them are depend on capability such as smartphone, FWA and redcap. These types of UEs are not mutually excluded, for example, a smartphone can support NTN and in some case can fall back to a Redcap UE. Also, the differentiation of these UE types has considered: number of TX and RX, supported bandwidth, power class, and so on.

The 5G device types are not mutually exclusive. Different device types have been introduced as some of them are depend on the usage scenario such as vehicular, UAV, XR and NTN while some of them are depend on capability such as smartphone, FWA and redcap.

Below we give some detail of different device types who are to some extent as individual types and its capability.

Table 4 Detail differentiation of different UE capability

|  | R15 NR UE | R17 RedCap | R18 eRedCap | R18 XR |
| --- | --- | --- | --- | --- |
| UE number of Tx/Rx | FR1 FDD：1T2R FR1 TDD: 1T4R | FR1 FDD: 1T1R FR1 TDD: 1T2R | FR1 FDD: 1T1R FR1 TDD: 1T2R | FR1 FDD/TDD：1T2R |
| maxCBW | FR1: 100MHz | FR1: 20MHz | FR1:RF 20MHz， BB 5MHz | FR1: 100MHz |
| Duplex mode | FDD, TDD | HD-FDD, FDD, TDD | HD-FDD, FDD, TDD | FDD, TDD |
| Max MIMO layer | FR1 FDD：2 layer for UL and DL FR1 TDD: 2 layer for UL and 4 layer for DL | FR1 FDD: 1 layer for DL FR1 TDD: 2 layer for DL | FR1 FDD: 1 layer for DL FR1 TDD: 2 layer for DL | FR1 FDD/TDD：2 layer for UL and DL |
| CA | Support | Not Support | Not Support | Support |

Above in table 4 we have listed some of the aspects that we believe is most important to differentiate device types from UE RF capability perspective and they are number of TX/RX, maxCBW, duplex mode, Max MIMO layers and CA support. The proposed device type differentiation is shown as below in table 5.

In NR, the different device types from UE RF capability perspective and they are number of TX/RX, maxCBW, duplex mode, Max MIMO layers and CA support.

The maximum device capability always decides the device types in NR.

In 6GR, the maximum device capability differentiates the device types.

6G device type consideration with mandatory baseline functionality set

In 6G, the scalable design for diverse device types have been discussed in RAN1#122 and RANP#109 and the agreements are captured as below:

RAN1#122 agreement

Agreement

Study a scalable 6GR design for diverse device types, considering aspects:

- What should be commonly applicable to all 6G device types

- FFS: add-on features dedicated to specific device types, if any

And in RAN#109, the device types were also discussed with the followings agreed.

Agreement

To investigate further:

  - Motivations/justifications behind the proposed diverse device types, which should be a limited set

  - Whether/how to have one or more device types for eMBB or 6G IoT

  - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc.

  - Whether/how to explicitly standardize device types

  - Ensuring forward compatibility

  - Minimizing/avoiding potential market fragmentation

Note: the terminology “device type” is subject to further discussion and possible refinement.

Agreement

In terms of diverse device types, study further:

  - Possible parameters/factors, e.g.:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

Note: some of the above parameters/factors may be related with form factor

Note: aim to have a focused/limited set of parameters/factors for a device type

  - The value(s) for the identified parameters for a device type

From the agreement in RAN1 and RANP captured above, it can be observed that 6GR is trying to define a scalable design for diverse device types and try to figure out the common parameters/factors to build up a mandatory baseline functionality set. From RAN4 perspective, we firstly to differentiate device types with UE RF considerations and then based on these to consider the mandatory baseline functionality set.

RAN1 and RAN tries to define a scalable design for diverse device types.

RAN4 differentiate device types with UE RF considerations and based on that to consider the mandatory baseline functionality set.

Based on the study of 5G NR device types capabilities, below different device types and corresponding capability are proposed. With the three device types’ maximum capability, the minimum of these capability constructs the mandatory baseline functionality set. This is proposed in below table 5.

It is proposed to agree on table 5 on device type and mandatory baseline functionality set.

Table 5: Device type and Mandatory baseline functionality set

| Mandatory Capability | Mandatory baseline functionality set | Examples of device types |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6G IoT (Lowest-tier) | eMBB (Handheld UE) | FWA (e.g., CPE) |
| Max. UE bandwidth | 5~20MHz | 5~20MHz | 200MHz | 400MHz |
| Max. number of TX/RX | 1T1R | 1T1R | 3T6R | 4T8R |
| UE power class | N.A. | 26dBm | 29dBm | 32dBm |
| Max. DL modulation order | 64QAM | 64QAM | 1024QAM | 4096QAM |

Conclusion

This paper discussed the 6GR device type, and got below observations and proposals.

Number of TX and RX

Observation 1: 	The UL MIMO and TxD features has been arranged with suffix D and G respectively to capture the UE RF requirement.

Observation 2: 	Most of the 2TX requirement are duplicated except for output power, MPR/A-MPR.

Observation 3: 	The REFSENS requirement has considered number of RX and different requirement for 1RX/2RX/4RX has been listed in one sub-clause.

Observation 4: 	Current REFSENS requirement structure makes it hard to find the exact requirement.

Proposal 1: 	To differentiate RF requirements with number of TX/RX under one sub-clause and give instruction in general sub-clause.

Observation 5: 	For hand-held UE, maximum capability is 2TX and 6RX till the end of NR evolution.

Observation 6: 	For reduced capability UE, 1T1R for FR1 FDD and 1T2R for FR1 TDD have been agreed.

Observation 7: 	Antenna size is one of the limitations of number of TX and RX, especially in FR1 NR re-farming bands with relative low frequency.

Proposal 2: 	Larger number of UE TX increases UE power consumption.

Proposal 3: 	For FR1 refarming bands, similar number of TX and RX of NR is proposed for 6GR.

Observation 8: 	To guarantee the UL coverage for around 7GHz band, larger output power of TX is needed.

Proposal 4: 	For around 7GHz band, 2TX as baseline is proposed.

Proposal 5: 	The number of TX/RX below is proposed based on different device differentiation:

Table 2 Proposed Number of TX and RX for different device differentiation

|  | Device differentiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 6G IoT (Lowest-tier) | eMBB (Handheld UE) |  |  | FWA (e.g., CPE) |
|  |  | FR1 FDD | FR1 TDD | Around 7GHz |  |
| number of Tx/Rx | Baseline: 1T1R Optional: 2R | Baseline: 1T2R Optional: 2T/3T; 4R | Baseline: 1T4R Optional: 2T/3T; 6R | Baseline: 2T4R Optional: 3T; 6R | Baseline: 4T8R Optional: 6T;12R |

Device type

Observation 9: 	The 5G device types are not mutually exclusive. Different device types have been introduced as some of them are depend on the usage scenario such as vehicular, UAV, XR and NTN while some of them are depend on capability such as smartphone, FWA and redcap.

Observation 10: 	In NR, the different device types from UE RF capability perspective and they are number of TX/RX, maxCBW, duplex mode, Max MIMO layers and CA support.

Observation 11: 	The maximum device capability always decides the device types in NR.

Proposal 6: 	In 6GR, the maximum device capability differentiates the device types.

Observation 12: 	RAN1 and RAN tries to define a scalable design for diverse device types.

Proposal 7: 	RAN4 differentiate device types with UE RF considerations and based on that to consider the mandatory baseline functionality set.

Proposal 8: 	It is proposed to agree on table 5 on device type and mandatory baseline functionality set.

Table 5: Device type and Mandatory baseline functionality set

| Mandatory Capability | Mandatory baseline functionality set | Examples of device types |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6G IoT (Lowest-tier) | eMBB (Handheld UE) | FWA (e.g., CPE) |
| Max. UE bandwidth | 5~20MHz | 5~20MHz | 200MHz | 400MHz |
| Max. number of TX/RX | 1T1R | 1T1R | 3T6R | 4T8R |
| UE power class | N.A. | 26dBm | 29dBm | 32dBm |
| Max. DL modulation order | 64QAM | 64QAM | 1024QAM | 4096QAM |

Reference

RP-251881	New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

R4-2514641	WF on [116bis][101] 6G system parameter, Huawei