3GPP TSG-RAN WG4 Meeting #117	R4-2521578

Dallas, USA, 17th – 21st November, 2025

Agenda item:		6.13.2.3

Source:	Nokia

Title:	On existing BS RF RX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#116bis [1] regarding the conformance part of evolution of NR duplex operation. There were some agreements made that have impact on the existing BS RF requirements for SBFD operation. The core part agreements are needed to be implement to the performance part of the specification. This contribution displays Nokia’s view on the existing BS RF RX requirements for SBFD operation.

# Discussion

## Work-split for CRs drafting

The following work split was agreed during RAN4#116bis regarding the CRs drafting:

| Content for both 38.141-1 and 38.141-2 | Current/ New clauses | Responsible company |
| --- | --- | --- |
| References Defintions, symbols and abbreviations Operating bands and channel arrangement | 2, 3, 5 | CATT |
| General conducted and radiated test conditions and declarations | 4 | Ericsson |
| Conducted and Radiated Transmitter characteristics - General - Base station output power - Output power dynamics - Transmit ON/ OFF power | Old clauses: 6.1 to 6.4 New potential clauses to align how it was done with Core specs: 9.1 to 9.4 | Samsung |
| Conducted and Radiated Transmitter characteristics - Transmitted signal quality - Unwanted emissions - Transmitted intermodulation | Old clauses: 6.5 to 6.7 New potential clauses to align how it was done with Core specs: 9.5 to 9.7 | Nokia |
| Conducted and Radiated Receiver characteristics - General - Reference sensitivity level - Dynamic range - In-band selectivity and blocking | Old clauses: 6.1 to 6.4 New potential clauses to align how it was done with Core specs: 9.1 to 9.4 | Huawei |
| Conducted and Radiated Receiver characteristics - Out of band blocking - Receiver spurious emissions - Receiver intermodulation - In-channel selectivity | Old clauses: 6.5 to 6.8 New potential clauses to align how it was done with Core specs: 9.5 to 9.8 | ZTE |
| Big CR 38.141-1 and 38.141-2 |  | Moderator (Ericsson) |

The list above is missing the in-channel adjacent subband blocking, which needs to be introduced to the conformance specifications aligned with the core specification. Below one can find what has been currently agreed for the core specification for conducted in-channel adjacent subband blocking:

### 12.3.8	In-channel adjacent subband blocking for SBFD

#### 12.3.8.1	General

The in-channel adjacent subband blocking requirement is a measure of the receiver’s ability to receive a wanted signal at its assigned channel frequency at the TAB connector for SBFD-capable BS type 1-H in the presence of an interfering signal from other sector(s) and/or site(s) within the same operator’s network, which is an NR signal.

#### 12.3.8.2	Minimum requirement for SBFD-capable BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal and an interfering signal coupled to the SBFD-capable BS type 1-H TAB connector using the parameters specified in table 12.3.8.2-1. The reference measurement channel for the wanted signal is identified in table 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each SBFD UL subband bandwidth and are further specified in annex A.1. The characteristic of the interfering signal is further specified in annex D.

The in-channel adjacent subband blocking requirement shall apply to SBFD UL subband bandwidth. The interfering signal(s) shall be positioned at the centre frequency of the SBFD DL subband bandwidth(s).

Minimum conducted requirement is defined at the TAB connector for BS type 1-H.

Table 12.3.8.2-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 1-H

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (NOTE 1) | Interfering signal mean power (dBm) (NOTE 5) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE 2, 3, 4） |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | PREFSENS + 6 dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 30 kHz SCS, BW = X (in the unit of RB) |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | CP-OFDM NR signal 30 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB) |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2, respectively. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. |  |  |  |  |  |

In-channel adjacent subband blocking needs to be included in receiver test specifications.

# Conclusion

In this contribution, our views on the existing BS RF RX requirements for SBFD operation are presented. There were observations and proposals as listed below.

- In-channel adjacent subband blocking needs to be included in receiver test specifications.

# References

- R4-2515063 “Way Forward for SBFD BS conformance”, RAN4#116bis, Prague, Czech, 13th – 17th October, 2025.