3GPP TSG-RAN WG4 Meeting #117	R4-2521581

Dallas, USA, 17th – 21st November, 2025

Agenda item:		4.19.2

Source:	Nokia

Title:	On SBFD-capable BS RF requirements

Document for:		Approval

# Introduction

Way Forward was agreed in RAN4#116bis [1] regarding the maintenance of evolution of NR duplex operation. The core part for NR duplex evolution was finished in RAN4#116 with final agreements made. There were still some items that needs a return to and this contribution shares Nokia’s views on those aspects.

# Discussion

## Correction on the In-channel adjacent subband blocking for SBFD-capable BS type 1-O

The following was agreed in RAN4#116bis:

<Agreement>: Offset the adopted requirement for interfering signal mean power by OTA REFSENS. - Wide Area BS: -28 - ΔOTAREFSENS - Medium Range BS: -32 - ΔOTAREFSENS - Local Area BS: -34 - ΔOTAREFSENS

This was agreed to be corrected in the core specification. It is also needed to be captured in the conformance specification TS 38.141-2.

Capture the agreement also in the conformance specification TS 38.141-2.

## In-band blocking for FR2-1 Wide Area BS

The following was agreed in RAN4#116bis:

<Way forward>: FFS on the following approaches. - Option 1: It is defined in the range of (EISREFSENS_50M + 34 + ΔFR2_REFSENS) to (EISREFSENS_50M + 42 + ΔFR2_REFSENS). - Option 2: Absolute value based approach. - Option 2-1: Adopt -23 dBm - Option 2-2: FFS on the grid shift percentage used for deriving FR2-1 in-band blocking interferer power level. - Option 3: It is defined as (EISREFSENS_50M + X + ΔFR2_REFSENS), where X comes from co-ex simulation that used for deriving FR2-1 in-band blocking interferer power level. The grid shift is FFS.

The discussion regarding the FR2-1 in-band blocking has been ongoing for a long time. There have been numerous discussions about not only the interferer power level, but also the method of deriving the interferer power level. FR1 has been agreed to use an absolute value for the requirement and FR2-1 has been discussed to use the relative method to derive the value. In RAN4#116, there was an agreement to use provisional requirement of -44dBm, which would mean that absolute value would be used also for FR2-1. It would be crucial to firstly agree on the method on how to derive the requirements.

RAN4 to agree on the method on how to derive the in-band blocking requirement for FR2-1 WA SBFD-capable BS.

Secondly, it needs to be agreed what grid shift will be used to derive the requirement. Will the requirement follow FR1 with the 100% grid shift together with the notes agreed in RAN4#116 or shall RAN4 use a lower GS percentage value and maybe not needing any notes.

RAN4 to agree on the grid shift percentage used for deriving FR2-1 in-band blocking interferer power level for FR2-1 SBFD-capable BS.

If absolute value is agreed to be used, we have provided simulation results for 10% and 100% grid shifts in [2]. For 10% grid shift, our results show an interferer value of -37.4dBm. If, however, 100% grid shift is used, the results show -49.2dBm interferer value.

If absolute value will be used, the interferer power level is -37.4dBm with 10% GS and -49.2dBm for 100% GS.

However, if there is no agreement reached in RAN4, we propose to keep the provisional value and use the absolute value of -44dBm for the interferer power.

If no agreement is reached in RAN4, keep the provisional value of -44dBm for the interferer power.

# Conclusion

In this contribution, our views on the SBFD-capable BS RF requirements are presented. The following observations and proposals were listed:

- Capture the agreement also in the conformance specification TS 38.141-2.

- If absolute value will be used, the interferer power level is -37.4dBm with 10% GS and -49.2dBm for 100% GS.

- RAN4 to agree on the method on how to derive the in-band blocking requirement for FR2-1 WA SBFD-capable BS.

RAN4 to agree on the grid shift percentage used for deriving FR2-1 in-band blocking interferer power level for FR2-1 SBFD-capable BS.

If no agreement is reached in RAN4, keep the provisional value of -44dBm for the interferer power.

# References

- R4-2515170 “Way Forward for [116bis][304] NR_duplex_evo_Maintenance”, RAN4#116bis, Prague, Czech, 13th – 17th October, 2025.

- R4-2418021 “SBFD RX in-band blocking simulation results”, RAN4#113, Orlando, USA, 18th -22nd November, 2024.