# 3GPP TSG-RAN WG4 Meeting #117		R4-2521786

Dallas, Texas, USA, 17th to 21st November 2025

Agenda Item:	7.7.3

Source:	Ericsson

Title:	On Co-existence considerations for UAV

Document for:	Discussion

# 1	Introduction

During RAN#109, a RAN4 WI [1] was agreed on Enhanced NR requirements for UAV operations to address issues related to UL throughput and UL coverage with higher transmit power. The UAVs will be served by ground based UAV BSs by adjusting the tilt angle upwards. It has been highlighted in the WID to take reference from NR ATG study [2], which supported two ATG UE antenna types – omni-directional antenna and antenna array. The objectives for core part from the WID [1], has been highlighted below –

- Scenario:

  - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km.

  - A direct radio link between BS on the ground and UAV UE.

- Specify RF requirements for UAV UE to support higher UL output power.

  - Example bands:

    - TDD with same TDD pattern, including n41, n77, n78, n79,

    - FDD: n3

  - Consider UAV UE with omni-direction antenna or with antenna array

  - Consider UAV UE with up to 33dBm max total conductive output power

  - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed.

  - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

- Specify RRM requirements, if needed

Note 1: No RAN1 impact are expected

This contribution triggers discussion points based on the tentative agreements from last meeting.

# 2	Discussion

The agreements from RAN4#116bis have been captured in WF [3].

## 2.1 	Co-existence scenarios

In the last meeting, all possible co-existence scenarios were summarized considering both FDD and TDD. Scenario 1,2,5,6 can be down-prioritized as they represent a less severe interference case. While most of the companies agreed to prioritize Uplink scenarios - 4 and 8 for co-existence analysis, there were still questions if the Downlink scenarios - 3 and 7 should studied in co-existence analysis.

The downlink scenarios look at the interference from Terrestrial BS DL transmissions into UAV network DL receivers. Here, a UAV UE receives its DL signal from a UAV BS, but at the same time is exposed to strong interfering signals from TN BSs transmitting on the same or adjacent channels. Unlike TN UEs (which are shielded by clutter, buildings and terrains), UAVs at altitudes will have LoS to multiple TN BSs simultaneously even at altitudes up to 1 Km. UAV UEs may accumulate interference from many non-serving TN BSs. It may be that the most problematic case need not be the TN BSs that are directly under the UAV UEs as the antenna is pointing downwards. However, considering LoS conditions TN BSs which are far away at the horizon from UAV point of view could contribute to accumulated interference.

In DL case when UAV UEs are victim, UAV UEs may accumulate interference due to LoS to multiple TN BSs simultaneously even at higher altitudes.

Considering LoS conditions TN BSs which are far away at the horizon from UAV point of view could contribute to accumulated interference.

In the Rel-18 ATG study [2], Ericsson provided additional simulation results to study the impact of ATG UE height distribution on the TN DL. The common assumption to derive the ATG UE ACS was with uniform ATG UE distribution between 3-10 km altitude. However, from the additional results it was observed that the ATG UE suffers higher degradation than the acceptable 5% during take-off and landing i.e. at 3 km as compared to two other cases of uniform distribution and fixed 10 km. It was not concerning as aircrafts are usually at altitudes close to 10km or higher when cruising.

It was observed that ATG UEs suffered higher degradation than acceptable 5% at fixed 3 kms height than in uniform distribution or fixed 10 kms height.

In order to make the UAV system robust, it is essential to study the impact of TN networks on the UAV networks as UAV UEs are more exposed to aggregate interference due to LoS to multiple TN BSs at relatively low altitudes as well as when TN BS are at horizon from UAV point of view.

RAN4 to consider the TN DL to UAV DL scenarios – 3 and 7, where UAV UEs are victims due to aggregated interference from multiple TN BSs in co-existence evaluations both when TN BSs are underneath UAV UE and TN BSs are far away at horizon from UAV point of view.

Some companies expressed concerns regarding the simulation workload associated with the co-existence evaluations and therefore proposed to limit the scope of the study to link budget analysis only. However, based on the tentative agreements in the last WF [2], the current assumptions already include AAS BS and UAV UE beamforming models, whose dynamic behaviours cannot be accurately captured through simplified link-budget calculations.

Another aspect is the modelling of aggregated interference from multiple TN BSs. This is critical for realistic assessment and could become a potential showstopper if not properly addressed within the work item.

RAN4 to maintain system level coexistence simulations to ensure that the aggregated interference is explicitly modelled to reflect realistic deployment conditions and that realistic antenna models are taken into consideration in the evaluation.

## 2.2 	Network layout

From a co-existence perspective, to ensure robust specifications, in our view it makes sense to consider the worst-case among collocated and non-collocated. This assumption is also dependent on whether UAV UE is capable of beamforming or not.

The TN and UAV BS are collocated or non-collocated, depends on the UAV antenna assumption. However, non-collocated would be the worst-case scenario from co-existence perspective

For the TN network, RAN4 assumes a hexagonal grid of 19 macro sites and 3 sites per sector. However, the network layout will be highly dependent on the studied scenario. Considering aggregate interference from TN network, it would make sense to have a larger TN network (more TN BSs) as compared to UAV network to fully model DL interferences. Similarly in the UL scenario explained in the below sections, it would be reasonable to have a larger UAV network (more UAV UEs) to full model the UL interferences.

Figure 2.2-1: DL case – higher number of TN BSs as compared to UAV BSs and UAV UEs are spread out far away at the horizon from TN BSs

In Figure 2.2-1, which depicts the DL case where the UAV UEs are the victim. As UAV UEs have LoS to multiple TN BSs simultaneously and would face aggregated interference, this can be a showstopper and may overwhelm the UAV UEs. In this figure, TN network ISD is 500m comprising of close to ~300 TN BSs and UAV network ISD is 2000m comprising of 19 UAV BSs. The idea is to have comparable TN network and UAV network in coverage to fully model the aggregate interference effect on UAV UEs.

Figure 2.2-2: UL case – higher number of UAV UEs as compared to TN BSs and all UAV UEs are hovering close to center TN BS

Figure 2.2-2depicts the UL case where the TN BSs are the victim. All the UAV UEs are hovering directly above or near to TN BSs. The pathloss is low because of the short distance and LoS. However, since the TN BS antennas are down tilted to serve the TN UEs, the UAV UE may be received through TN BS antenna sidelobes. With the assumption of omni-directional UAV UE antenna type and high transmit power, the received interference power at the TN BS can be significant, even though sidelobes. Now, if we assume beamforming capable UAV UEs, the impact to the nearby TN BSs may be limited if the beam is steered towards the serving UAV BS. This would imply assuming an omni-directional antenna for UAV UE antenna with a high number of active UAVs near the TN BS could be the worst case. It is important to note that the impact may be localized among the affected TN BSs. In this figure, TN network ISD is 500m comprising of close to 19 TN BSs and UAV network ISD is 2000m comprising of 19 UAV BSs.

Figure 2.2-3: UL case – higher number of UAV UEs as compared to TN BSs and UAV UEs are spread out far away at the horizon from TN BSs

Figure 2.2-3 depicts the UL case where the TN BSs are the victim. All the UAV UEs are far at the horizon from the TN BS point of view. They will have LoS conditions to many TN BSs across a wide area. Hence, the uplink interference will be spatially distributed resulting in degradation across a cluster of TN cells. The aggregated impact scales up with UAV density, as many UAVs can appear to be at the horizon at once. In this figure, TN network ISD is 500m comprising of close to 19 TN BSs and UAV network ISD is 2000m comprising of 19 UAV BSs.

Considering aggregated interference from TN network, it would make sense to have a larger TN network (more TN BSs) as compared to UAV network to fully model DL interference. Similarly in the UL scenario explained in the below sections, it would be reasonable to have a larger UAV network (more UAV UEs) to fully model the UL interference.

RAN4 to clarify on how to model the TN and UAV network – number of TN/ UAV BSs based on the DL and UL scenarios.

Another aspect to consider is how to model the position of the UAV UEs with respect to the TN network, to ensure the co-existence between the TN and UAV systems is robust and corner cases have been evaluated. Two cases can be considered – when UAVs are hovering directly or near to a victim TN BS and when the UAVs are far at horizon from the TN BS.

RAN4 should clarify assumptions on how to place the UAVs in the network - near to victim TN BS or at horizon which will play a significant role in modelling the co-existence conditions.

## 2.3	Simulation assumptions

For the simulation assumptions related to UAV deployments, the critical parameter is the UAV UE dropping in the network. The minimum height assumption from the ground is highly dependent on whether the UAVs will be served from the existing down-tilted TN BSs or have an uptilted UAV BS. The latter would lead to regulatory concerns as discussed in the consequent sections.

If the UAVs are to be served from existing TN BSs, it is not clear if the minimum height assumption restricts the UAV UE to be only served from the side-lobe. From a co-existence perspective, the worst-case would be when the UAVs are concentrated at the fixed minimum height. However, this would be an extreme case to consider for the evaluations.

RAN4 should clarify the assumption of UAV UE dropping – minimum height from the ground, UAV distribution both vertically and horizontally and minimum horizontal distance.

For the simulation assumptions related to TN deployments, the TN BS height should be clarified as referring to TR 38.901, the TN BS height for RMa deployment is 35m and for UMa deployment is 25m.

Based on TR 38.901, the TN BS height for RMa deployment is 35m and for UMa deployment is 25m.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In DL case when UAV UEs are victim, UAV UEs may accumulate interference due to LoS to multiple TN BSs simultaneously even at higher altitudes.

Observation 2	Considering LoS conditions TN BSs which are far away at the horizon from UAV point of view could contribute to accumulated interference.

Observation 3	It was observed that ATG UEs suffered higher degradation than acceptable 5% at fixed 3 kms height than in uniform distribution or fixed 10 kms height.

Observation 4	The TN and UAV BS are collocated or non-collocated, depends on the UAV antenna assumption. However, non-collocated would be the worst-case scenario from co-existence perspective

Observation 5	Considering aggregated interference from TN network, it would make sense to have a larger TN network (more TN BSs) as compared to UAV network to fully model DL interference. Similarly in the UL scenario explained in the below sections, it would be reasonable to have a larger UAV network (more UAV UEs) to fully model the UL interference.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to consider the TN DL to UAV DL scenarios – 3 and 7, where UAV UEs are victims due to aggregated interference from multiple TN BSs in co-existence evaluations both when TN BSs are underneath UAV UE and TN BSs are far away at horizon from UAV point of view.

Proposal 2	RAN4 to maintain system level coexistence simulations to ensure that the aggregated interference is explicitly modelled to reflect realistic deployment conditions and that realistic antenna models are taken into consideration in the evaluation.

Proposal 3	RAN4 to clarify on how to model the TN and UAV network – number of TN/ UAV BSs based on the DL and UL scenarios.

Proposal 4	RAN4 should clarify assumptions on how to place the UAVs in the network - near to victim TN BS or at horizon which will play a significant role in modelling the co-existence conditions.

Proposal 5	RAN4 should clarify the assumption of UAV UE dropping – minimum height from the ground, UAV distribution both vertically and horizontally and minimum horizontal distance.

Proposal 6	Based on TR 38.901, the TN BS height for RMa deployment is 35m and for UMa deployment is 25m.

# References

[1]	RP-252908, “Enhancement of NR RF and RRM requirements for UAV”, 3GPP RAN#109, September 2025

[2]	3GPP TR 38.876 v18.2.0, “Air-to-ground network for NR”, July 2024

[3]	R4-2515159, “WF for NR_UAV”, RAN4#116bis, Moderator

[5]	3GPP TR 26.777 v15.0.0, “Study on Enhanced LTE support for Aerial Vehicles”, December 2017