# 3GPP TSG RAN WG4 Meeting #117 														            R4-2521867

# Dallas, USA, November 17th – 21st, 2025

Agenda Item:	6.13.3

Source: 	Ericsson

Title:	Simulation Results for Rel-19 SBFD CSI-RS based L1-RSRP

Document for:	Discussion

# Introduction

The agreements on SBFD operation during RAN4#116 bis meeting are captured in the approved WF [1].

# Discussion

## 2.1 Simulation for CSI-RS based L1-RSRP

Simulation Assumptions: Table 1: Simulation assumptions for L1-RSRP[1] Parameters; Value. SCS; 15 kHz, 30 kHz, 120 kHz. Side condition (SNR); -3 dB. Density (D); 3. Number of samples (M); 1 (single sample measurement). Number of PRB; 24, 48. Propagation condition; AWGN.

Table 2: Simulation results for L1-RSRP error (based on NZP CSI-RS)

| SCS (kHz) | Number of PRBs | Delta-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5% | 95% | Max(5%,95%) |
| 15 | 24 | -0.81 | 1.85 | 1.85 |
|  | 48 | -0.51 | 1.38 | 1.38 |
| 30 | 24 | -0.75 | 1.89 | 1.89 |
|  | 48 | -0.48 | 1.41 | 1.41 |
| 120 | 24 | -0.38 | 2.13 | 2.13 |
|  | 48 | -0.13 | 1.66 | 1.66 |

Table 3: L1-RSRP accuracy degradation of 24 PRBs compared to 48 PRBs

based on Max(5%,95%)

| SCS (kHz) | Ericsson | Huawei [2] | OPPO [3] |
| --- | --- | --- | --- |
| 15 | 0.47 dB | 0.77 dB | 0.58 dB |
| 30 | 0.48 dB | 0.47 dB | 0.70 dB |
| 120 | 0.47 dB | 0.46 dB | 0.72 dB |

- Observation 1: For case 3 , less than 0.5dB degradation are observed for all simulated SCS.

# Summary

The following have been observed in the current contribution:

Observation 1: For case 3 , less than 0.5dB degradation are observed for all SCS have been simulated.

# References

- R4-2508274“Simulation assumptions for CSI-RS based L1 measurement”, Huawei, August,2025

- R4-2513104 “On accuracy of CSI-RS based L1 measurement for SBFD operation”, OPPO, October 2025

- R4-2514166 “Simulation results for L1-RSRP and L1-SINR accuracy”, Huawei, HiSilicon, October 2025