3GPP TSG RAN WG4 Meeting #117	 																R4-2521916

Dallas, USA, 17 Nov. 2025 – 21 Nov. 2025

Agenda Item:	6.12.2.1

Source: 	Ericsson

Title:	Discussion on event triggered reporting test configurations for MIMO Ph 5

Document for:	Discussion

# Introduction

In this contribution we discuss the event triggered reporting test configurations for MIMO Ph 5.

# Discussion

## UE-initiated/event-driven beam management

Test configuration:

For Event-2, we have the following definitions for Scheme-1 and Scheme-2:

- Scheme-1: RS for the current beam is the QCL RS in the indicated TCI state.

- Scheme-2: RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

Generally, the QCL RS in the indicated TCI state could be TRS, and the quality of the TRS may not be fair to compare with the quality of the new beam (which could be SSB). Hence, we believe that at least Scheme-2 should be considered for SSB based event triggered reporting testing. Once the scheme for Event-2 is agreed upon, the scheme for Event-7 can follow the same principle. This is further supported by RAN1 agreement shown below.

- Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - Note: In such case, the RS type comprises SSB and CSI-RS configured in a CSI-RS resource set configured with repetition.

- For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state).

- For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state).

Another important test configuration is the PUCCH configuration for the new UCI. One relevant agreement regarding the first PUCCH from RAN1 is listed below. Among the first PUCCH configurations, we believe that at least periodicityAndOffset should be specified in the test configuration. This ensures that the UE can send the indication to the NW node when the event-triggered condition is met during the first PUCCH occasion.

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline FFS: overlapping with SR/LRR or PUSCH

- RAN4 to specify the periodic first PUCCH resource in the test configuration as follows.

| Parameter |  | Value |
| --- | --- | --- |
| BWP ID |  | 0 |
| PUCCH-Cell |  | SpCell |
| periodicityAndOffset |  | Sl5 |
| PUCCH resource ID |  | 0 |
| PUCCH resource | Starting PRB | To be determined by RAN5 |
|  | intraSlotFrequencyHopping | disabled |
|  | Format | format 2 |
|  | nrofPRBs | 2 |
|  | nrofSymbols | 1 |
|  | startingSymbolIndex | 0 |

# Summary

The following have been proposed in this contribution.

- For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state).

- For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state).

- RAN4 to specify the periodic first PUCCH resource in the test configuration as follows.

| Parameter |  | Value |
| --- | --- | --- |
| BWP ID |  | 0 |
| PUCCH-Cell |  | SpCell |
| periodicityAndOffset |  | Sl5 |
| PUCCH resource ID |  | 0 |
| PUCCH resource | Starting PRB | To be determined by RAN5 |
|  | intraSlotFrequencyHopping | disabled |
|  | Format | format 2 |
|  | nrofPRBs | 2 |
|  | nrofSymbols | 1 |
|  | startingSymbolIndex | 0 |

# References

[1] 		R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

[2]		R4-2507014, On RRM performance aspects and test cases for MIMO Ph 5, Ericsson

[3]		R4-250xxxx, Discussion on CJTC reporting and asymmetric UL TRP requirements, Ericsson

[4]		R4-2317371