3GPP TSG-RAN4 Meeting #117											R4-2521943

Dallas, USA, November 17th – 21st 2025

Agenda item:		4.30.1

Source: 		Qualcomm Incorporated

Title:	 	LS to RAN2 - Rel-19 NTN features and release independence

Document for:		Approval

## 1	Introduction

The core part for the work item for NR NTN Phase 3 was completed in RAN4#116 meeting. While new requirements were included, changes to release independence were forgotten. This contribution discusses the release independence for NTN (e)RedCap

## 2	Discussion

In previous meeting WF on release independence of NR NTN including RedCap was agreed in [2]. The WF concludes that

    - 3 MHz CBW for NR NTN is release independent from release 18

    - FD-FDD RedCap for NR NTN is release independent from release 17

    - FD-FDD eRedCap for NR NTN is release independent from release 18

    - HD-FDD RedCap and eRedCap are release independent from release 19

but also recognizes that an LS to RAN2 is needed to avoid ambiguity (e)RedCap release independence.

The feature list related to NR NTN PH3 is as follows [1]:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61. NR_NTN_Ph3 | 61-1 | Tx output power enhancement for NR-NTN UE | 1. Support of NTN (e) RedCap UE power boosting when applicable as defined in 6.2 of TS 38.101-5. |  | Yes | N/A | NTN Legacy output power limitation still apply | Per band | No | FR1 only | N/A | The feature can be supported in below scenarios: NTN (e) RedCap UE in FR1 single band with single uplink CC configured in the band where power boosting capability is indicated in this band. Additional RRC IE is used to enable/disable power boosting. | Optional with capability signalling |
|  | 61-2 | Support of (e)RedCap UE with FR1-NTN | Indicates whether the UE supports (e)RedCap UE with FR1-NTN. | Layer-1 FG: 28-1, 48-1 Layer-2 and Layer-3 FG: 34-1 | Yes | N/A | The requirements defined for (e)RedCap UE with FR1-NTN in Rel-19 may not apply. | Per UE | No | FR1 only | N/A | If the UE indicates this capability, the UE shall support (e)RedCap UE with FR1-NTN. | Optional with capability signalling |

For the support of (e)RedCap UE with FR1-NTN (FG 61-2), it would be highly beneficial to make the signalling early implementable to remove ambiguity on what the UE supports.

Therefore, we should send an LS to RAN2 for asking early implementation for the capability signalling.

Proposal 1: Send LS to RAN2 asking to make signalling of support of (e)RedCap UE with FR1-NTN early implementable, if possible.

Proposal 2: Consider draftLS in the appendix as baseline for the LS.

## 3	Conclusion

In this contribution release independence of NTN (e)RedCap. Following observations and proposals were made.

Proposal 1: Send LS to RAN2 asking to make signalling of support of (e)RedCap UE with FR1-NTN early implementable, if possible.

Proposal 2: Consider draftLS in the appendix as baseline for the LS.

## 4	References

[1]	R4-2511883, R19 RAN4 UE feature list for NR, CMCC

[2]      R4-2515127, Way Forward for [116bis][302] NR_IoT_NTN_Maintenance - Release independence, Qualcomm Incorporated

# Appendix

Title:	[draft] LS on UE release independence of NTN RedCap and related capability signalling

Response to:	-

Release:	Rel-19

Work Item:	TEI19 [NTN_Release_Independence], NR_NTN_PH3-Core

Source:	RAN4

To:	RAN2

Cc:	-

Contact Person:

Name:		Toni Lähteensuo

E-mail Address:	tlaehtee (at) qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 has discussed the release independence for NTN RedCap and eRedCap for which RF requirements were introduced in release 19. RAN4 concluded that NTN (e)RedCap is release independent as follows:

- FD-FDD RedCap from rel-17

- FD-FDD eRedCap from rel-18

- HD-FDD RedCap and eRedCap from rel-19

At the same time, RAN4 recognizes that capability signalling to indicate support for (e)RedCap was agreed in Rel-19 in FG 61-2, as sent to RAN2 in LS R4-2511884. RAN4 respectfully asks if it is possible to make this capability early implementable so that also earlier release UEs are enabled to indicate support.

# 2	Actions

To RAN2 group:

ACTION:  RAN4 respectfully requests RAN2 to take the provided information into account and introduce necessary updates to UE capabilities, if possible.

# 3	Dates of Next RAN WG4 Meetings

TSG-RAN4 Meeting #118	Feb 09 - 13, 2026	Gothenburg, Sweden

TSG-RAN4 Meeting #118bis	Apr 13 - 17, 2025	Malta, MT