3GPP TSG-RAN4 Meeting #117	R4-2522152

# Dallas, TX, USA Nov 17th – Nov 21st 2025

Source: 	Qualcomm Incorporated

Title: 	RRM requirements of Rel-19 low band carrier aggregation via switching

Agenda item:	4.8.2

Document for:	Discussion

# Introduction

3GPP introduced a new work item on low band carrier aggregation via switching in RAN #106 [1]. We discuss some relevant RRM requirements for this topic.

# Discussion

According to [2], low band spectrum carries significant traffic volume in urban indoor and rural areas. However, SDL remains under-utilized since they cannot be paired with UL. That’s why, [2] proposed to introduce a network configured switching pattern in low band carrier aggregation via switching

Figure 1: RRC configured switching pattern in low band carrier aggregation via switching [2]

Figure 1 shows how network could configure this switching pattern to enable this low band carrier aggregation. This switching pattern would instruct UE to alternate between FDD DL+UL band and SDL band in a periodic manner. We discuss different RRM requirements of SCells and PCells in this scenario.

## Timer triggered SCell Deactivation

RAN1 made the following agreement during the last meeting:

Issue 1-3-5: Activation of Switching Pattern Agreement: - TLBCA =1ms for MAC CE based, 0 ms for RRC based. Agreement: - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation. - Following RAN1 logic, the carrier switching, if applicable, shall be placed at the end of a slot. - FFS RAN4 to capture this sub-bullet in the CR. - Further discuss for timer triggered SCell deactivation Agreement: - The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation

Timer triggered SCell deactivation kicks in when sCellDeactivationTimer expires. sCellDeactivationTimer starts when SCell gets activated and the timer restarts every time DL or UL is scheduled in SCell. sCellDeactivationTimer is controlled by MAC-CE. After the timer expires, non-zero amount of time is required to instruct the lower layer to deactivate the switching pattern. Any DL/UL configuration prior to the expiry of the timer will extend the timer and keep the switching pattern active. Hence, UE can only start to prepare deactivation of SCell and switching only after the expiry of the timer.

After SCell and corresponding switching pattern get deactivated, the UL TA of PCell may require UE to start switching from SCell from the beginning of a slot. Hence, UE may not get sufficient time to instruct the lower to deactivate the switching pattern if TLBCA = 0 ms.

Observation 1: After the timer expires, non-zero amount of time is required to instruct the lower layer to deactivate the switching pattern.

Observation 2: Any DL/UL configuration prior to the expiry of the timer extends the timer and keep the switching pattern active. Hence, UE can start to prepare deactivation of SCell and switching only after the expiry of the timer.

Observation 3: After SCell and corresponding switching pattern get deactivated, the UL TA of PCell may require UE to start switching from SCell from the beginning of a slot.

Proposal 1: TLBCA = 1ms for timer triggered SCell deactivation.

## UE Feature regarding RRC based Direct SCell Addition in NR LB-CA

RAN4 made the following agreements during the past few meetings [4] [3]

Online agreement on Tuesday: - PUCCH SCell activation, Rel-19 fast SCell activation through EMR, and activation of multiple SCells is not in the scope of the WI. Agreement on Thursday: - The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature. - R16 Direct SCell activation with HO is not considered for R19 LB CA. - R17 PUCCH SCell activation is not considered for R19 LB CA. - R18 SSB-less SCell activation for NES is not considered for R19 LB CA. - R17 Fast SCell activation is not considered for R19 LB CA. - Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation. - Known/unknown conditions are reused from existing SCell activation requirements

- For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms

These agreements suggest that RRC based direct SCell addition feature is allowed in Rel-19 NR LB-CA. RRC based direct SCell addition feature was introduced as a UE capability in Rel-16. This can be seen through the following text of 38.306:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| directSCG-SCellActivation-r16, directSCG-SCellActivation-r17 Indicates whether the UE supports direct NR SCG SCell activation, as specified in TS 38.321 [8], upon SCell addition and upon reconfiguration with sync of the SCG, both performed via an RRCReconfiguration message received via SRB3 or contained in an RRC(Connection)Reconfiguration message received via SRB1, as specified in TS 38.331 [9] and TS 36.331 [17]. A UE indicating support of directSCG-SCellActivation-r16 shall indicate support of EN-DC or support of NGEN-DC as specified in TS 36.331 [17] or support of NR-DC as specified in TS 38.331 [9]. | UE | No | No | Yes (Incl FR2-2 DIFF) |

RAN4 has not, so far, introduced any separate UE capability for RRC based direct SCell addition in Rel-19 NR LB-CA. That means, according to current status, UE is expected to support RRC based direct SCell addition feature if it signals the capability to support RRC based direct SCell addition and Rel-19 NR LB-CA feature.

It is safer for UE to signal support for a feature only after it completes IODT with infra vendor regarding that feature. Since Rel-19 NR LB-CA is a new feature, infra vendors are expected to first bring up this feature with MAC-CE based SCell activation. Infra vendors may bring up Rel-19 NR LB-CA with direct SCell activation later. With the current status of UE features, UE vendors may have to pass conformance test of NR LB-CA with direct SCell activation without going through IODT. This is not desirable. This issue can be resolved by introducing a UE capability regarding RRC based direct SCell addition in NR LB-CA.

Observation 4: RRC based direct SCell addition feature was introduced as an optional UE capability in Rel-16.

Observation 5: RRC based direct SCell addition feature is allowed in Rel-19 NR LB-CA.

Observation 6: RAN4 has not introduced any separate UE capability for RRC based direct SCell addition in Rel-19 NR LB-CA. UE is expected to support RRC based direct SCell addition feature if it signals the capability to support RRC based direct SCell addition and Rel-19 NR LB-CA features.

Observation 7: It is safer for UE to signal support for a feature only after it completes IODT with infra vendor regarding that feature.

Observation 8: Infra vendors are expected to first bring up this feature with MAC-CE based SCell activation. Infra vendors may bring up Rel-19 NR LB-CA with direct SCell activation later.

Observation 9: According to current status, UE vendors may have to pass conformance test of NR LB-CA with direct SCell activation without going through IODT.

Proposal 2: RAN4 introduces a new capability that allows UE to signal whether it supports RRC based direct SCell activation in Rel-19 NR LB-CA.

- Potential details of the capability are shown in Table 1

Table 1: Details of UE capability for RRC based Direct SCell Activation in Rel-19 NR LB-CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 55. NR_LBCA_Sw | [55-x] | RRC based Direct SCell Activation in Rel-19 NR LB-CA | [UE supports RRC based direct SCell activation in Rel-19 NR LB-CA] | 55-1 | [UE will not support RRC based SCell activation in Rel-19 NR LB-CA] | [Optional with capability signaling] |

# Conclusions

Observation 1: After the timer expires, non-zero amount of time is required to instruct the lower layer to deactivate the switching pattern.

Observation 2: Any DL/UL configuration prior to the expiry of the timer extends the timer and keep the switching pattern active. Hence, UE can start to prepare deactivation of SCell and switching only after the expiry of the timer.

Observation 3: After SCell and corresponding switching pattern get deactivated, the UL TA of PCell may require UE to start switching from SCell from the beginning of a slot.

Observation 4: RRC based direct SCell addition feature was introduced as an optional UE capability in Rel-16.

Observation 5: RRC based direct SCell addition feature is allowed in Rel-19 NR LB-CA.

Observation 6: RAN4 has not introduced any separate UE capability for RRC based direct SCell addition in Rel-19 NR LB-CA. UE is expected to support RRC based direct SCell addition feature if it signals the capability to support RRC based direct SCell addition and Rel-19 NR LB-CA features.

Observation 7: It is safer for UE to signal support for a feature only after it completes IODT with infra vendor regarding that feature.

Observation 8: Infra vendors are expected to first bring up this feature with MAC-CE based SCell activation. Infra vendors may bring up Rel-19 NR LB-CA with direct SCell activation later.

Observation 9: According to current status, UE vendors may have to pass conformance test of NR LB-CA with direct SCell activation without going through IODT.

Proposal 1: TLBCA = 1ms for timer triggered SCell deactivation.

Proposal 2: RAN4 introduces a new capability that allows UE to signal whether it supports RRC based direct SCell activation in Rel-19 NR LB-CA.

- Potential details of the capability are shown in Table 1

Table 1: Details of UE capability for RRC based Direct SCell Activation in Rel-19 NR LB-CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 55. NR_LBCA_Sw | [55-x] | RRC based Direct SCell Activation in Rel-19 NR LB-CA | [UE supports RRC based direct SCell activation in Rel-19 NR LB-CA] | 55-1 | [UE will not support RRC based SCell activation in Rel-19 NR LB-CA] | [Optional with capability signaling] |

# References

[1] RP-243317, New WID on low band carrier aggregation via switching, 3GPP TSG RAN #106.

[2] RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19, 3GPP TSG RAN #106.

[3] R4-2512119, WF on RRM requirements for NR_LBCA_Sw, RAN4 115.

[4] R4-2508259, “WF on RRM requirements for NR_LBCA_Sw”, RAN4 #115

[5] 38.306, V 17.1.0, UE Radio Access Capabilities.