3GPP TSG-RAN WG4 Meeting #117						         	       		R4-2522254

Dallas, USA, Nov. 17-21, 2025

Title: 	On band n28 and n67 co-banding for CA_n28-n67 LBLB switching

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.8.3	UE RF requirements [NR_LBCA_Sw_enh-Core]

Document for:	Approval

# 1	Background

In RAN#116bis, a way forward [1] captured several aspects to be further discussed in this meeting and notably on feasibility of a combined n67+n28 filter that we brought up as a potential challenge in our contribution [2]. In his contribution, although we have not been able in the time available between the two meeting to perform a filter feasibility, we further discuss the implications of the results of such feasibility.

# Discussion

## 2.1 Way forward input

In RAN4#116bis, a way forward [1] captured several aspects to be further discussed in this meeting and notably on feasibility of combined n67+n28 filter. The relevant part is copied here in italics

Open issues related to the UE reference architecture

-	Further discuss the feasibility of a combined band n28+n67 DL filter, considering the supported bandwidth and the full band n28 implementation

-	Whether a restriction of n28 operation to the Region 1 range 703-733/758-788 MHz is relevant for the purpose of agreeing a reference architecture when deriving minimum requirements for the feature in the CA_n28A-n67A band combination

-	Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of the above

-	Further discuss front end filter architecture for the case of other LB-LB CA combinations where two DL bands are separated

## 2.2 Possible consequences of a combined n28+n67 filter feasibility result

The time between the October and November meeting did not allow us to perform a detailed filter study and as such we are not ready to conclude on its feasibility or not. However, we can already discuss a few implications depending on the feasibility result.

First, we can already assume that a full band n28 is unlikely to be supported as it overlaps with band n28B (718-748MHz) UL range thus n28B range DL is not worth supporting, limiting the n67+n28B bandwidth to 50MHz and range to 738-788MHz

It is still a challenging filter bandwidth of 50MHz or 6.6% making it more challenging than the n105 diversity Rx filter.

With only a 5MHz gap to the n28A UL range which does not allow enough isolation, it is not feasible to reuse this filter in the diversity branch for standalone FDD operation in band n28. Thus, an additional band 28 duplexer and diversity filter are needed using either full or split duplexer.

In the same region where CA_n28-n67 is supported, CA_n28-n20 is also supported and requires and n28A UL+ n20+n28A DL+ n20UL triplexer, here again a full band n28 support is very challenging to support and the DL overlaps with other bands in Asia that can also be combined with band 28 as shown in Figure 1.

Figure 1: World-wide environment for band n28

As a result, a word-wide skew would have to implement band n28 with 3 different filter schemes in both the main and diversity path:

- A band 28A/B or full duplexer and DL filter for amin and diversity path for stand alone

- A band n28A+n67 DL filter in main and diversity path to support switched CA_n28-n67

- A band n20/n28A triplexer in main path and a n28A+n20 filter in diversity path to support CA_n20-n28

- Also note that band n28B may be combined with n26 for China or Japan

In the case where a n28A+n67 filter is not feasible, too costly to add or too low in performance, the only alternative to support switched CA_n28A-n67 would require 4 low-band antennas and will still need redundant hardware to support the other band n28 related combinations.

Observation:

- Even if a combined n28A+n67 filter is proven feasible, the support of switched CA_n28A-n67 with require redundant hardware to support other band n28 ranges and combinations impacting the solution size and cost.

- In the case where such a filter proves not feasible, too poor performance or too costly, the only other alternative is a solution with 4 low-band antennas.

Proposal:

- To finalize the study of CA_n28A-n67 support with release 20 operation, the combined n28A+n67 filter feasibility should be finalized

- Given the hardware cost and challenge of a filter or a four low-band antenna solution, R20 based switching support of CA_n28A-n67 performance gains should be evaluated against supporting it with the R19 switching solution.

# Conclusion

In this contribution, we discuss the potential outcome of a combined band n28 and n67 filter feasibility and its implication on the cost performance trade-off in comparing a support with a R19 switching solution versus a R20 switching solution.

Proposal:

- To finalize the study of CA_n28A-n67 support with release 20 operation, the combined n28A+n67 filter feasibility should be finalized

- Given the hardware cost and challenge of a filter or a four low-band antenna solution, R20 based switching support of CA_n28A-n67 performance gains should be evaluated against supporting it with the R19 switching solution.

# References

[1] R4-2515156 Way Forward for [116bis][320] NR_low_band_CA_switching, Apple, RAN4#116b

[2] R4-2513810 Feasibility of LB switching FDD+SDL example combinations, Skyworks Solutions Inc., RAN4#116b