3GPP RAN WG4 Meeting #118		R4-2600146

Gothenburg, Sweden, February 9th – 13th, 2026

Agenda item:	8.3.1

Source:	Apple

Title:	Further views on 6G UE RF output power requirements framework

WI/SI:	FS_6G_Radio

Release:	Rel-20

Document for:	Approval

# 1	Introduction

The Rel-20 6G working group study item has been approved in June 2025 RAN #108 meeting [1]. Last RAN4 meeting (#117) was the second meeting to discuss the RAN4 aspects of the 6G study item where each topic was again concluded with an approved WF summarized by the Feature Lead.  For general RF and UE RF topic, the WF [2] has captured a more consolidated view for each of the following prioritized subtopics from previous RAN4 meeting (116bis) [3] and the guidance for future contributions.

- Single carrier output power requirements

- Power class framework for CA

- Tx requirements Remove general notes in 2-band and 3-band configuration tables

- Rx requirements

- Frequency range between FR1 and FR2-1

- Spectrum aggregation

- Joint UE and BS RF

- Others

Following the WF, in this contribution, we share our further views on the 6G UE RF output power requirements framework, aiming to improve UE RF performance over 5G as well as to address the 5G UE RF requirements insufficiency.

# Discussion

In last RAN4 meeting, we have shared our initial views on the fundamental UE RF requirements framework for 6G on each respective subtopic for Tx, Rx, and CA requirements with the goals and proposed approach aiming to improve UE RF performance over 5G as well as to address the 5G UE RF requirements insufficiency [4]. In this contribution, along the same lines of our last contribution, we would like to share our further views and more detailed reasoning to support our proposals on the 6G UE RF output power requirements framework.

## 2.1	Single carrier output power requirements

According to the WF agreements [2,3], the main consideration of the 6G UE maximum output power framework is to enable more flexibility for different UE RF front-end implementations and maximize the UE UL transmit power capability for both single carrier and CA where the aspects for single carrier output power study include:

- Study step size between power classes

- Study power class for multi-Tx case (TxD, UL-MIMO)

- Study how to enable fully utilizing PA capabilities

- Study need for default power limit and default power class values

For 5G, the step size between power classes has been specified as 3dB except between PC1.5 and PC1. For each power class, a +2/-2dB tolerance is allowed which is meant to accommodate production variation, but not a leverage for UE to position its mean maximum output power at higher or lower power than the nominal power class power. If a UE would intend to target its mean maximum output power at higher or lower power than the nominal power class power due to its better or worse capability, it may suffer from higher yield loss as shown in Figure 2.1-1, or the production variation would have to be tightened such as via factory calibration to avoid the yield loss, as illustrated in Figure 2.1-2.

Figure 2.1-1 5G UE power class definition

Figure 2.1-2 Production variation tightening for UE targeting at higher mean maximum output power

As a result, a UE capable of supporting 24 or 25dBm mean maximum output power would either intentionally downgrade its capability to 23 dBm or its true capability could not be precisely indicated due to the 5G power class definition.

In 6G, if UE maximum output power capability is specified with a finer granularity, such as with 1dB step as shown in Figure 2.1-3, the aforementioned issue in 5G can be mitigated.

Figure 2.1-3 UE maximum output power capability with 1dB granularity

It is worth noting that having 1dB step maximum output power granularity does not fundamentally change the 5G power class framework but adds flexibility for UE to more precisely indicate its mean maximum output power based on its true output power capability. If a UE prefers to go with the 5G way, it can still choose to just indicate 23dBm (PC3), 26dBm (PC2), or 29dBm (PC1.5). On the other hand, for UE choosing to indicate maximum output power capability other than PC3, PC2, or PC1.5, the +/-2dB production variation can be maintained which means no or simpler factory calibration. In addition, the 1dB step maximum output power granularity could potentially enable much simpler MPR specifications as further elaborated below.

In 5G, every UE power class has its own set of MPR requirements which had not only created substantial RAN4 workload but also introduced more specifications complexity. However, if we take a closer look of the MPR difference between PC2 and PC3 as shown in Table 2.1-1 and between PC1.5 and PC3 as shown in Table 2.1-2,

| Modulation |  | PC2 MPR – PC3 MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 3 | 0 | 0 |
|  | QPSK | 2.5 | 0 | 0 |
|  | 16 QAM | 1.5 | 0 | 0 |
|  | 64 QAM | 1.0 | 0 | 0 |
|  | 256 QAM | 0 | 0 | 0 |
| CP-OFDM | QPSK | 0.5 | 0 | 0 |
|  | 16 QAM | 0.5 | 0 | 0 |
|  | 64 QAM | 0 | 0 | 0 |
|  | 256 QAM | 0 | 0 | 0 |

Table 2.1-1 MPR difference between PC2 (1Tx) and PC3

| Modulation |  | PC1.5 MPR – PC3 MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 5.5 | 1.0 | 0 |
|  | QPSK | 5.5 | 1.0 | 0 |
|  | 16 QAM | 4.5 | 1.0 | 0 |
|  | 64 QAM | 4.0 | 1.0 | 0.5 |
|  | 256 QAM | 2.0 | 1.0 | 1.0 |
| CP-OFDM | QPSK | 3.5 | 1.0 | 0 |
|  | 16 QAM | 3.5 | 1.0 | 0 |
|  | 64 QAM | 3.0 | 1.0 | 0.5 |
|  | 256 QAM | 0 | 1.0 | 1.0 |

Table 2.1-2 MPR difference between PC1.5 (2Tx with 20dB antenna isolation) and PC3

It can be seen that for inner/outer allocations, there is no MPR difference between PC2 and PC3, while there is at most 1dB difference between PC1.5 and PC3. If UE maximum output power capability would be specified at full allocation (outer) such as for calibration waveform, PC2 would perform as expected while PC1.5 would be 1dB short as advertised, that is, 28dBm instead of 29dBm.

Based on the above observation, if UE is allowed to indicate 28dBm as its maximum output power capability, then only one set of MPR requirements would be sufficient for inner/outer allocations for all maximum output power capability Pn (n = 23, 24, 25, …) as shown below which also implies simpler specifications and less RAN4 workload.

MPRInner/Outer_Pn = MPRInner/Outer_P23

For edge allocations, we can also identify a formula for MPR to cover all maximum output power capability levels as below,

MPREdge_Pn = Max (Pn – 22.5dBm, MPREdge_P23)

Table 2.1-3 summarizes how edge MPR values may be specified for maximum output power levels at 23 dBm, 24 dBm, 25 dBm, 26 dBm, and 29 dBm as examples.

| Modulation |  | MPREdge_P23 | MPREdge_P24 | MPREdge_P25 | MPREdge_P26 | MPREdge_P29 |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 0.5 | 1.5 | 2.5 | 3.5 | 6.5 |
|  | QPSK | 1.0 | 1.5 | 2.5 | 3.5 | 6.5 |
|  | 16 QAM | 2.0 | 2.0 | 2.5 | 3.5 | 6.5 |
|  | 64 QAM | 2.5 | 2.5 | 2.5 | 3.5 | 6.5 |
|  | 256 QAM | 4.5 | 4.5 | 4.5 | 4.5 | 6.5 |
| CP-OFDM | QPSK | 3.0 | 3.0 | 3.0 | 3.5 | 6.5 |
|  | 16 QAM | 3.0 | 3.0 | 3.0 | 3.5 | 6.5 |
|  | 64 QAM | 3.5 | 3.5 | 3.5 | 3.5 | 6.5 |
|  | 256 QAM | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 |

Table 2.1-3 Edge MPR values for example 6G maximum output power levels based on 1dB granularity

Another benefit with 1dB maximum output power granularity is that ΔTIB can then be merged into UE maximum output power capability such that the true UE capability accounting the RF front-end component insertion loss for supporting CA can be more precisely indicated.

Based on the above assessment, our views on the goals and the proposed approach for 6G single carrier maximum output power framework for FR1 range are summarized below.

Goals:

- Uplink coverage improvement over 5G

- Maximize UE’s hardware capability.

- Minimize UE capability signaling such as transparent power boosting.

- Merge ΔTIB into the UE maximum output power capability to reduce specifications complexity.

- Address UL MIMO or Tx diversity versus 1Tx maximum output power capability.

Proposal 1: RAN4 to consider the following approach for 6G UE maximum output power framework for single carrier:

- UE maximum output power capability is specified at the 0dB MPR waveform which can be the 6G MPR calibration waveform.

- Define default maximum output power requirements for handheld UE under 1Tx.

    - [23 – 25] dBm for FDD bands

    - [23 – 26] dBm for TDD bands

- Allow UE to indicate its maximum output power capability higher than the default value with 1dB increment up to [29] dBm.

- +2/-2dB tolerance

- Define only one set of MPR requirements for inner/outer allocations which applies for all UE indicated maximum output power capability.

- Edge MPR are specified as MPREdge_Pn = Max (Pn – 22.5dBm, MPREdge_P23) where n is the maximum output power in dBm.

- ACLR = 31 dB from default maximum output power up to 29 dBm, FFS on ACLR requirement for output power higher than 29 dBm.

- For UL allocations and waveforms other than the 0dB MPR waveform, allow [3]dB more output power to accommodate power boosting.

- For UL band supporting more than 1Tx (such as nTx), the total maximum output power from the sum of all antenna connectors can be up to 10log(n)dB higher than 1Tx maximum output power.

## 2.2	Power class framework for CA

CA scenarios include DL CA with single carrier UL, intra-band UL CA, and inter-band UL CA. For DL CA with single carrier UL, the UL maximum output power capability shall not be degraded as compared to single carrier alone when one or more DL carriers are aggregated.

For intra-band contiguous UL CA, the RF characteristics in general would be similar to single carrier of the same bandwidth except the PAPR (Peak to Average Power Ratio) for DFT-s-OFDM could be slightly higher than single carrier. Therefore, if the aggregated bandwidth can be covered by a single Tx chain, in principle, the maximum output power capability should be the same as single carrier. However, when considering the substantially higher MPR for contiguous UL CA as compared to single carrier, the CA maximum output power capability would look to be much degraded. With that being said, 6G should strive to avoid intra-band contiguous UL CA operation if the same bandwidth can be configured with single carrier, such as CA bandwidth class B. For intra-band non-contiguous UL CA, owing to its poor UL efficiency caused by relatively high MPR, we propose only single switched UL is supported if the configuration would be specified.

For inter-band UL CA, in our view, the power class for total maximum output power is not so meaningful and may complicate the UL power control as well as constrain the UE per-band maximum output power capability. This latter issue has been recognized in Rel-17, and as a consequence, the higherPowerLimit-r17 capability was introduced for the mitigation. On the other hand, the coexistence between the per band power class and the UL CA power class also created lots of ambiguities on the UE UL power capability indication for which RAN4 spent near two releases of meeting cycles for the discussions and still could not reach the consensus. Therefore, we propose the maximum output power capability for inter-band UL CA is per-band based and there is no need to specify the total maximum output power capability from all constituent bands. One thing to note is that the dropping of inter-band UL CA power class definition may have the implication on RAN1 power control design. It is advised RAN4 to inform RAN1 early enough on the potential change for inter-band UL CA power class definition in 6G such that RAN1 can be well prepared for 6G power control design.

Based on the above assessment, our views on the goals and the proposed approach for 6G CA maximum output power framework for FR1 range are summarized below.

Goals:

- Uplink coverage improvement over 5G

- Maximize UE’s hardware capability.

- Avoid the ambiguity on UE UL power capability indication resulting from the coexistence between the per band power class and the inter-band UL CA power class.

Proposal 2: RAN4 to consider the following approach for 6G UE maximum output power framework for CA:

- For DL CA with single carrier UL

  - The maximum output power capability is the same as single carrier.

- For UL CA

  - Strive to avoid intra-band contiguous UL CA operation if the same bandwidth can be configured with single carrier, such as for CA bandwidth class B.

  - For inter-band UL CA, the maximum output power capability is per-band based. There is no need to specify the total maximum output power capability from all constituent bands. The per-band capability under inter-band UL CA configuration can be different from single carrier capability for the same band.

  - RAN4 to inform RAN1 early enough on the potential change for inter-band UL CA power class definition in 6G such that RAN1 can be well prepared for 6G power control design.

  - For intra-band non-contiguous UL CA, only single switched UL is supported if specified. The per-carrier maximum output power capability is the same as single carrier.

# 3	Conclusion

In this contribution, we share our further views on the 6G UE RF output power requirements framework, aiming to improve UE RF performance over 5G as well as to address the 5G UE RF requirements insufficiency.

Proposal 1: RAN4 to consider the following approach for 6G UE maximum output power framework for single carrier:

- UE maximum output power capability is specified at the 0dB MPR waveform which can be the 6G MPR calibration waveform.

- Define default maximum output power requirements for handheld UE under 1Tx.

    - [23 – 25] dBm for FDD bands

    - [23 – 26] dBm for TDD bands

- Allow UE to indicate its maximum output power capability higher than the default value with 1dB increment up to [29] dBm.

- +2/-2dB tolerance

- Define only one set of MPR requirements for inner/outer allocations which applies for all UE indicated maximum output power capability.

- Edge MPR are specified as MPREdge_Pn = Max (Pn – 22.5dBm, MPREdge_P23) where n is the maximum output power in dBm.

- ACLR = 31 dB from default maximum output power up to 29 dBm, FFS on ACLR requirement for output power higher than 29 dBm.

- For UL allocations and waveforms other than the 0dB MPR waveform, allow [3]dB more output power to accommodate power boosting.

- For UL band supporting more than 1Tx (such as nTx), the total maximum output power from the sum of all antenna connectors can be up to 10log(n)dB higher than 1Tx maximum output power.

Proposal 2: RAN4 to consider the following approach for 6G UE maximum output power framework for CA:

- For DL CA with single carrier UL

  - The maximum output power capability is the same as single carrier.

- For UL CA

  - Strive to avoid intra-band contiguous UL CA operation if the same bandwidth can be configured with single carrier, such as for CA bandwidth class B.

  - For inter-band UL CA, the maximum output power capability is per-band based. There is no need to specify the total maximum output power capability from all constituent bands. The per-band capability under inter-band UL CA configuration can be different from single carrier capability for the same band.

  - RAN4 to inform RAN1 early enough on the potential change for inter-band UL CA power class definition in 6G such that RAN1 can be well prepared for 6G power control design.

  - For intra-band non-contiguous UL CA, only single switched UL is supported if specified. The per-carrier maximum output power capability is the same as single carrier.

# 4	References

RP-251881 “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9th -13th, 2025

R4-2522451 “WF for [117][102] 6G general RF and UE RF”, Feature Lead (Qualcomm), 3GPP RAN WG4 Meeting #117, Dallas, USA, November 17th – 21st, 2025

R4-2514642 “WF for [116bis][102] 6G general RF and UE RF”, Qualcomm, 3GPP RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13th – 17th, 2025

R4-2520601 “Views on 6G UE RF requirements framework”, Apple, 3GPP RAN WG4 Meeting #117, Dallas, USA, November 17th – 21st, 2025