3GPP TSG-RAN WG4 Meeting #118		   				 		   R4-2600167

Göteborg, SE, 9th – 13th Feb., 2026

Agenda Item:	8.6.3

Source:	MediaTek Inc.

Title:	Views on other 6G RRM topics

Document for:	Discussion

# Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion starts from this RAN4 meeting. In this paper, we provide our views on the 6G RRM requirements with the focus on the following aspects:

- Mobility and HO interruption reduction

- Spectrum aggregation and SCell activation

- Other PHY signal/channel/procedure related RRM

  - UE Tx timing

  - MRTD

# Mobility and HO interruption reduction

Usually, RAN2 leads the discussion on mobility. RAN4 starts the discussion on mobility after RAN1/2 have sufficient progress. It may be not easy to carry out the discussion on enhancements with RAN1/2 dependency. But RAN4 can start from the solutions with less RAN1/2-dependency.

Proposal 1: In 6G SI, RAN4 to start from mobility solutions with less RAN1/2-dependency.

In traditional HO solution, UE needs to do DL synchronization on target cell, which contribute to a large part of HO delay and interruption. The longer SSB period is, the longer time it takes for DL synchronization. So do the HO interruption. As far as we know, there are many proposals in RAN1 to extend 6G SSB periodicity, which will potentially lead to longer HO interruption. For LTM, the delay and interruption can be reduced compared to traditional HO through early DL synchronization. But the gain can only be obtained under some conditions, e.g., measurement period is no larger than 160ms or the time gap between TCI state activated and cell switch command is less than 160ms. Further check the conditions, it can be observed that the conditions are even more difficult to meet with longer SSB periodicity. So in 6G SI, we need to study the solutions that can mitigate the problem of longer SSB periodicity.

Proposal 2: A solution that can mitigate the problem of longer SSB periodicity needs to be considered in 6G study.

Before discussing 6G HO solutions, we would like to recall the enhanced handover features in NR, i.e., CHO, DAPS, LTM and C-LTM. We summarized the pros and cons of DAPS, LTM and CHO as in Table 1.

Table 1 Enhanced handover features in 5G

|  | Pros | Cons |
| --- | --- | --- |
| DAPS | Interruption is the shortest. - Source cell will keep the link until UE sets up the connection with target cell | limited applicable scenarios or high UE complexity |
| LTM | Delay and interruption can be reduced through early RRC decoding, early DL/UL synchronization. | - Complex conditions to get the gain of early DL synchronization - measurement period is no larger than 160ms or the time gap between TCI state activated and cell switch command is less than 160ms - May be difficult to ensure measurement period within 160ms with even longer SSB periodicity in 6G. - Extra complexity to support early T/F tracking - Extra memory and conditions for early ASN.1 decoding - Additional interruption due to early RACH |
| CHO | Avoid the latency introduced by interaction between UE and NW (UE reports measurement results and then NW sends HO command) - HO initiated by UE under the guidance of NW | - Early preparation needed at Network - Potential resource waste due to network is not aware of when interruption starts. |
| Soft satellite switch | Interruption can be reduced | - Only supported in NTN |

It can be observed that the interruption length of DAPS is the shortest at the cost of higher UE complexity, i.e., extra hardware or spare CA resource. Due to this, DAPS is applicable for limited scenarios. As mentioned above, the gain of early DL synchronization can only be obtained with some conditions in LTM. In 6G Day-1, the designed HO solution should be applicable to all the scenarios and work well without conditions. Therefore, in 6G we don’t think it is enough to reuse any one of the legacy enhanced HO features.

Proposal 3: 6G Day-1 HO solution should be applicable to all the scenarios and work well without conditions.

To achieve the above goal as in Proposal 3, the 6G HO solution should get the advantages of individual enhanced HO features while resolving their problem as much as possible. Usually, NW stops the link “source cell ↔ UE” after sending HO/cell switch command, e.g., in legacy L3 HO and LTM. So HO interruption is almost the same as HO delay. And most of the time, reducing HO delay is similar as reducing HO interruption. There may be some other ways to counteract the impact of long delay, e.g., trigger HO earlier. But there is no way to counteract the impact of long interruption. So it is more important to reduce HO interruption.

Proposal 4: For 6G mobility, decouple the KPIs of HO delay and HO interruption with the priority on reducing interruption.

Even in NR DAPS, the KPIs of HO delay and HO interruption have been decoupled. The main concept of DAPS is that source cell will keep the link until UE sets up the connection with target cell. In this way, the interruption length can be much less than the delay to achieve nearly “0” interruption. But to keep dual active links, i.e., source cell ↔ UE and target cell ↔ UE, it requires UE to be able to receive/transmit with both source and target cell simultaneously, and leads to high UE complexity.

Observation 1: Based on the experience of DAPS, HO interruption can be much shorter than HO delay.

Observation 2: Based on the experience of DAPS, one method to reduce HO delay is to keep the link between source cell and UE when UE gets to set up the connection with target cell.

Checking the current HO procedure, upon receiving the HO command, UE will right away decode the RRC (TRRC) and apply all the new configurations of the target cell (Tprocessing). After Tprocessing, UE cannot be scheduled by the source cell. As a result, the interruption starts, as shown in Figure 1.

However, in our view, UE can choose when to apply the new configurations. The later UE applies the new configurations, the longer UE can maintain the link with the source cell. For example, UE can perform RRC decoding, DL synchronization as well as the time waiting for the RACH occasion, without interruption on serving cell(s). But after Tprocessing, UE usually breaks the link with serving cell except in DAPS. Inspired by DAPS and LTM, we propose a HO procedure as shown in Figure 2. In the new HO procedure, Tprocessing is performed just before RACH occasion. In this way, even for blind HO, HO interruption can be reduced to as short as “ Tprocessing”. To get the gain, it requires source cell keeps the link until UE starts to break the link with source cell, i.e., before Tprocessing. In Figure 2, we use unknown HO which includes the most procedures during HO as an example. For other scenarios, the difference is that some components are not needed, e.g., Tsearch is no needed for known scenarios. No matter which scenario it is, the proposed solution is also applicable.

Figure 1 Timeline of legacy handover procedure

Figure 2 Proposed HO procedure

Actually, similar solution has been adopted for NTN as captured in 38.133 clause 6.1C.3.2.3. As shown below, in soft satellite switch, UE is supposed to perform Tprocessing after finishing T/F tracking and t-Service. Source satellite can keep serving UE before Tprocessing. In 6G, it is preferred to have unified solution for both TN and NTN if possible.

Figure 3 Timeline of Soft satellite switch

Proposal 5: HO interruption can be reduced by postponing Tprocessing to the last, e.g., just before RACH occasion for RACH-based HO.

Proposal 6: To further reduce HO interruption, source cell should keep scheduling until UE breaks the link with source cell.

Proposal 7: Better to have unified handover solution for both TN and NTN if possible.

Based on offline discussions, it seems there are concerns on that the location of interruption may be unknown to NW. For brainstorming, we list several potential methods to solve this issue:

- Option 1: Determine the location of interruption based on the defined minimum requirements in RAN4.

A potential issue of this option is whether UE is allowed to handover to target cell faster than the minimum requirements. If interruption time starts earlier and NW keeps scheduling during the interruption time, retransmission after UE camps on target cell is needed and some resources are wasted. So UE can be allowed to handover faster but at the cost of wasting some NW radio resources.

- Option 2: Introduce new signalings to have common understanding between network and UE on when interruption starts.

- Option 3: Define the earliest location of interruption in RAN4.

It is equivalent to define a lower bound of HO delay requirement.

We believe there can be other methods to solve the ambiguity issue. We are open for more discussions.

Proposal 8: Discuss whether and how to let NW know when interruption starts. The following options can be used as a starting point.

- Option 1: Determine the location of interruption based on the defined minimum requirements in RAN4.

- Option 2: Introduce new signalings to have common understanding on when interruption starts.

- Option 3: Define the earliest location of interruption in RAN4.

- Option 4: Others.

To further reduce HO delay, early preparation before HO/cell switch command, e.g., early RRC decoding, early DL/UL synchronization, is helpful. As mentioned in Table 1, supporting early T/F tracking requires some additional UE complexity. So the number of active TCI states that UE can support is limited. So for early RRC decoding. In 5G, early T/F tracking and early RRC decoding is totally under NW control. But the fact is that UE is always the first to know which cell is getting better. There will be always some latency introduced by reporting the measurement results. Inspired by CHO, we propose to support UE initiated early RRC decoding and early T/F tracking in 6G for better use of limited UE capability.

Proposal 9: Support UE initiated early RRC decoding and early T/F tracking in 6G.

# Spectrum aggregation and SCell activation

Cell with multiple carrier

In 5G, the carrier-aggregation (CA) framework is critical for spectrum aggregation, but it limits physical layer and protocol efficiencies because by artificially enforcing a one-to-one coupling of each physical carrier component (CC) and a logical cell, resulting in inefficiency as noted in proposed observation 9.1b with regards to spectrum utilization and aggregation at RAN1#122bis.

- Slow and complex activation of additional carrier

- Features (such as HARQ) defined per carrier leads to sub-optimal performance

- Signalling/configuration overhead and UE processing complexity of PHY channels due to per CC constraint

- limited applicable scenario of SSB adaptation for Scell

- ….

Specifically, within a contiguous spectrum, the current CA framework artificially makes a multi-carrier operation appears more cumbersome and less efficient than a single wider bandwidth from a system perspective.

For 6G, we recommend decoupling this one-to-one mapping and aggregating multiple carriers with similar propagation channel properties to a single multi-carrier cell. As illustrated in Figure 4, intra-band and nearby inter-band carriers can be aggregated into a single cell when they meet specific conditions, e.g., time and frequency alignment, frequency span, and receive power difference, thereby enabling fast carrier activation and reduce the control overhead. Based on 5G experiences, for example, practical thresholds of conditions may include Maximum Receive Timing Difference (MRTD) < 3us (or CP), frequency error < 0.1ppm, and similar total received power for an intra-band-like UE receiver, where a common time/frequency synchronisation reference and AGC can be used at a UE. In addition, it is a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where these conditions are typically met.

To increase the bandwidth of a multi-carrier cell, the interruption time associated with adding an additional carriers was discussed during an offline discussion. In our view, with the simplified 6G BWP currently under discussion in RAN1, the delay can be further reduced compared with the current BWP switch; moreover, because 5G already supports a 3 ms delay for SSB-less intra-band contiguous SCell activation, the interruption time for adding a carrier to a multi-carrier cell can be similar or lower when the corresponding conditions satisfied, since 6G allows delay and interruption to be decoupled, as proposed in our mobility contribution.

Observation 3: The interruption time to add a carrier in a multi-carrier cell can be kept below the current BWP switch delay and further reduced by decoupling delay from interruption.

Note that carrier aggregation (as in 5G) remains necessary when the synchronization and power conditions described above are not met across carriers, such as when aggregating two widely separated carriers.

While RAN1 will study multi-carrier cell support and procedures, RAN4 RF will need to study RF-switch time (to switch on a new carrier within a cell) requirements, acceptable Transmit timing alignment (TAE), BS frequency error, and total power differences at UE between carriers; RAN4 RRM can then assess aggregation delay, accounting for AGC resettling and fine time tracking.

Figure 4 Illustration of multi-carrier cell

Observation 4: Aggregate multiple carriers into a single cell can reduce the siganling overhead and UE processing complexity of PHY channels, enabling more efficient spectrum aggregation and scalable spectrum utilization.

Observation 5: Multi-carrier cell can be a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where conditions (e.g. e.g. time and frequency alignment, frequency span, and received power differences) are typically met.

Proposal 10: RAN4 RF should study the conditions and requirements for aggregating multiple carriers to a single cell for 6G. For example, RF switch-time requirements, acceptable transmit timing alignment error (TAE), base-station (BS) frequency error, and total received power difference limits at the UE. And RAN4 RRM should study the corresponding interruption and delay requirements.

Carrier switch enhancements for UL and DL

Due to the limited number of Tx chains at the UE, the number of bands that the UE can be configured for is constrained. For instance, a UE with two Tx chains can either be configured with one band using UL-MIMO or with two bands without UL-MIMO (one Tx chain per band). UL Tx switching allows the UE to dynamically switch the Tx chain(s) among bands, enabling configuration with more bands and supporting UL-MIMO transmissions without requiring additional Tx chains. UL Tx switching was introduced incrementally from R16 to R19. Each release targets a different scenarios, which adds unnecessary limitation to 5G networks. For example, if more than 2 UL bands are needed, it can only be supported from Release 18. For 3Tx UE, it can only be supported in Rel-19, but it is limited in 2 UL bands. Enabling flexible carrier switching in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity by ensuring the uplink resources for the UE can be allocated where resources are available. Therefore, improvements for UL Tx switching should be investigated in 6G to simplify and streamline its operation.

Beyond UL, enabling more flexible DL carrier switching without requiring carrier aggregation could deliver benefits comparable to those achieved with UL carrier switching; in 5G it was considered only in Rel-19 with limited scope, while carrier switching is especially valuable under hardware constraints (e.g., few RF chains or synthesizers) and in the presence of DL/UL interference and intermodulation distortion, so the corresponding switching-delay will need to be studied in RAN4 RF and the interruption requirements can be further studied in RAN4 RRM.

For 6G, we expect a more flexible mechanism for DL/UL switching with forward compatibility on aspects on number of bands, number of Tx/Rx, … etc. In this 6G SI, it would be good to take this as one design target and study its corresponding impact in 3GPP specs.

Observation 6: Enabling flexible carrier switching between UL and between DL with forward compatibility to number of band/Tx/Rx in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity.

Proposal 11: RAN4 should study support of carrier switching and simplify configurations with forward compatibility; specifically, RAN4 RF should study applicable scenarios and RF switch-time requirements, and RAN4 RRM should study the corresponding interruption and delay requirements.

SCell activation, increase of spectrum aggregation

Carrier aggregation (as in 5G) remains necessary when the synchronization and power conditions described above are not met across carriers, such as when aggregating two widely separated carriers.

Since Rel-15, 5G NR has continuously refined Scell activation to address a variety of conditions (with or without measurement reports, different synchronization states, power differences), diverse RS configurations (SSB-less, A-TRS, P/SP/AP CSI-RS), and different types of UE RF architecture (intra-/inter-band, co-/non-collocated), yet only limited scenario has seen widespread adoption in commercial 5G.

For 6G, the carrier aggregation activation should ensure timely CSI so the network can schedule sufficient radio resources for busty traffic, and the Scell activation should be specified only for critical, high-likely deployment cases to simply the requirement design.

Based on 5G experience, the SCell/carrier activation at least involves RF switch, AGC settling, cell search, fine sync tracking, and CSI report. RAN4 RF will need to study RF switch-time requirements, and RAN4 RRM will assess SCell activation interruption and the overall delay, taking other factors into account.

Observation 7: 5G NR Scell activation requirements are too complicated and only limited scenarios have seen widespread adoption in commercial.

Proposal 12: RAN4 should study critical, high-likely SCell activation scenarios and ensure timely CSI acquisition; RAN4 RF should study RF switch-time requirements, and RAN4 RRM should study SCell activation interruption and overall delay.

# Other PHY signal/channel/procedure related RRM

## UE Tx timing

Initial Transmission Timing

UL performance depends on the UE’s TX timing accuracy, with requirements tied to the DL reference signal SCS (for DL timing detection) and the UL data SCS to avoid the interference between UEs beyond the CP length. High SCS can make compliance difficult, e.g. 60 kHz in NTN, especially in the present of additional impairment such as GNSS/UE positioning inaccuracies and satellite position errors in NTN scenarios. However, those errors can be corrected after initial access.

Currently RACH shares the same timing requirement as UL channels such as PUCCH/PUSCH, but PRACH’s CP can be much longer than theirs, so its tolerance to timing error can also be much higher. Unnecessarily tight PRACH timing requirements will limit the use of high SCS, because after initial access the UE’s initial timing error can be corrected via the Timing Advance command and subsequent PUCCH/PUSCH with higher SCS will have chance to reduce the timing error within CP. Given PRACH’s distinct CP length and role in initial access, defining PRACH-specific timing requirements could better enable higher SCS.

Observation 8: Relaxing PRACH timing requirements would accommodate initial timing errors within CP, allow timing correction after initial access, and enable higher SCS. Especially in NTN scenarios, where impairments such as GNSS/UE positioning inaccuracies and satellite position errors can be corrected after initial access.

Proposal 13: Study PRACH (if introduced) specific timing accuracy requirement.

Timing Adjustment

In 5G, especially for advanced use cases such as Non-Terrestrial Networks (NTN) and high-speed trains (HST), the distance between the user equipment (UE) and the base station (BS) can change rapidly. This affects signal propagation time, and the UE transmit (Tx) timing needs to be adjusted so that uplink transmissions from different UEs arrive at the BS without collision and so that the BS can track each UE’s uplink (UL) timing. UE timing may change for the following scenarios:

  - UE movement. As shown in Figure 5 (a). The received timing changes due to UE motion or a frequency offset between the BS and UE, and the UE must maintain the timing difference between the receive and transmit timing with respect to the system timing.

  - Path change. As shown in Figure 5 (b). The dominant receive path may be attenuated or disappear due to channel variations, and the UE needs to re-synchronize its transmit timing to another receive path.

  - NTN Satellite movement. With the introduction of Non-Terrestrial Networks (NTN) in 3GPP Release 17, UEs implement timing/frequency pre-compensation, based on the provided Ephemeris and UE location, to project and adjust UE’s transmission timing.

  - Residual oscillator drift. Even without movement, small mismatches in clock frequencies between UE and BS or between UE TX and UE RX can accumulate, requiring periodic timing corrections.

  - UE-based TA for LTM cell switch. UE-based TA for LTM cell switch. When the UE switches cells, the distance to the new cell’s base station may be different, so the timing advance (TA) must be updated to maintain uplink synchronization.

(a) UE movement					(b) path change

Figure 5 Scenarios for UE timing adjustment.

In 5G, two opposite-direction methods are used to manage UE timing adjustment, as illustrated in Figure 6:

Method 1: UE Gradual timing adjustment

- This legacy mechanism has present since 3G and was retained through 4G and 5G (Rel-15), where the UE monitors DL timing changes, which reflect the propagation delay changes, and shifts the UL transmit timing in the same direction of DL timing change; thus, if the DL timing is delayed, the UE further delays its UL, and the BS observes roughly double timing changes; to realign, Timing Advance Commands (TAC) must advance the UL timing (i.e., in the opposite direction of the DL change), which adds control overhead, and the adjustment rate is limited to 0.18–0.9 μs/s per TS 38.133 to prevent UL reception failures at the BS, making the mechanism potentially insufficient in fast timing-change scenarios such as NTN or HST.

Method 2: UE pre-compensation

- This mechanism whereby the UE proactively adjusts its UL transmit timing in the opposite direction of DL timing change. For example, if the UE detects the DL timing is delayed, it advances its UL timing by the same amount, preventing misalignment before the BS detects it.

- UE pre-compensation has been adopted in R17 NTN, R18 FR2 HST/POS and is typically applied when timing changes are large (e.g., > CP/4) or when TCI changes occur; in NTN, the adjustment is based on satellite/BS position to project the correct timing at satellite, though the UE can also derive it from the detected DL timing changes for TN network.

- Observation 9: In 5G, two opposite direction methods are used to adjust UE UL timing: gradual adjustment (Method 1), which doubles the timing changes observed at BS and requires frequent TAC, and UE pre-compensation, where the UE corrects timing before the BS detects misalignment.

(a) UE movement				(b) path change

Figure 6 Methods for UE timing adjustment.

In 5G systems, gradual timing adjustment is the baseline, with mandatory test cases in place since Release 15. Specifically, as defined in TS 38.133 A.6.4.1 (shown below), step 3 imposes a DL timing change of Δtp, as shown in Figure 6, and step 4 expects the UE to adjust its UL toward (2Δtp±Te), but as shown in Figure 6, if the UE performs timing pre-compensation using Method 2, the arrived timing at the NB will remains transparent to the NB yet differ from what test system expects, causing the UE to fail.

Observation 10: With propagation delay changes, UE pre-compensation (Method 2) leaves NB observed UL timing unchanged, yet it differs from what test system expects in the 5G R15 mandatory test defined in TS 38.133 A.6.4.1, so the UE fails the test.

TS 38.133. A.6.4.1.1.2	Test requirements

..

3)	The test system shall adjust the timing of the DL path by values given in table A.6.4.1.1.2-1

Table A.6.4.1.1.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first path (in time) of DL SSB used by the UE to determine downlink timing is received from the reference cell at the UE antenna.  Skip this step for test 2 with DRX configured.

In 6G, we propose studying the feasibility of adopting UE pre-compensation as the baseline approach, specifically replacing gradual timing adjustment for handling UE mobility and path changes, while retaining the Timing Advance command to correct the UE’s initial UL timing.

Proposal 14: Study the feasibility of replacing gradual timing adjustment with UE pre-compensation, while retaining the Timing Advance command.

## MRTD

In legacy systems, the maximum receive timing difference (MRTD) is defined by 3GPP as the maximum relative timing difference between serving carriers that a UE can handle. According to 3GPP, MRTD consists of two components: time alignment error (TAE) and RF propagation delay difference (∆Tprop), such that MRTD = TAE + ∆Tprop. The requirements for MRTD and TAE vary depending on the configuration. For intra-band operation, MRTD is set to 3 µs and TAE to 65 ns; for inter-band operation, MRTD is 33 µs and TAE is 260 ns. It should be noted that higher MRTD values can lead to increased complexity in UE implementation.

Given the higher performance expectations for 6GR systems, it is important to optimize TAE so that UE complexity does not increase compared to 5G NR.

Observation 11: Higher MRTD values can lead to increased complexity in UE implementation.

Proposal 15: RAN4 to seek optimisation of TAE for MRTD to a level maintaining the UE complexity lower than that for 5G systems.

# Summary

Our observations and proposals on the 6G RRM requirements are summarized as below:

// Mobility and HO interruption reduction

Proposal 1: In 6G SI, RAN4 to start from mobility solutions with less RAN1/2-dependency.

Proposal 2: A solution that can mitigate the problem of longer SSB periodicity needs to be considered in 6G study.

Proposal 3: 6G Day-1 HO solution should be applicable to all the scenarios and work well without conditions.

Proposal 4: For 6G mobility, decouple the KPIs of HO delay and HO interruption with the priority on reducing interruption.

Observation 1: Based on the experience of DAPS, HO interruption can be much shorter than HO delay.

Observation 2: Based on the experience of DAPS, one method to reduce HO delay is to keep the link between source cell and UE when UE gets to set up the connection with target cell.

Proposal 5: HO interruption can be reduced by postponing Tprocessing to the last, e.g., just before RACH occasion for RACH-based HO.

Proposal 6: To further reduce HO interruption, source cell should keep scheduling until UE breaks the link with source cell.

Proposal 7: Better to have unified handover solution for both TN and NTN if possible.

Proposal 8: Discuss whether and how to let NW know when interruption starts. The following options can be used as a starting point.

- Option 1: Determine the location of interruption based on the defined minimum requirements in RAN4.

- Option 2: Introduce new signalings to have common understanding on when interruption starts.

- Option 3: Define the earliest location of interruption in RAN4.

- Option 4: Others.

Proposal 9: Support UE initiated early RRC decoding and early T/F tracking in 6G.

// Scell activation, and spectrum aggregation

Cell with multiple carrier

Observation 3: The interruption time to add a carrier in a multi-carrier cell can be kept below the current BWP switch delay and further reduced by decoupling delay from interruption.

Observation 4: Aggregate multiple carriers into a single cell can reduce the siganling overhead and UE processing complexity of PHY channels, enabling more efficient spectrum aggregation and scalable spectrum utilization.

Observation 5: Multi-carrier cell can be a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where conditions (e.g. e.g. time and frequency alignment, frequency span, and received power differences) are typically met.

Proposal 10: RAN4 RF should study the conditions and requirements for aggregating multiple carriers to a single cell for 6G. For example, RF switch-time requirements, acceptable transmit timing alignment error (TAE), base-station (BS) frequency error, and total received power difference limits at the UE. And RAN4 RRM should study the corresponding interruption and delay requirements.

Carrier switch enhancements for UL and DL

Observation 6: Enabling flexible carrier switching between UL and between DL with forward compatibility to number of band/Tx/Rx in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity.

Proposal 11: RAN4 should study support of carrier switching and simplify configurations with forward compatibility; specifically, RAN4 RF should study applicable scenarios and RF switch-time requirements, and RAN4 RRM should study the corresponding interruption and delay requirements.

SCell activation, increase of spectrum aggregation

Observation 7: 5G NR Scell activation requirements are too complicated and only limited scenarios have seen widespread adoption in commercial.

Proposal 12: RAN4 should study critical, high-likely SCell activation scenarios and ensure timely CSI acquisition; RAN4 RF should study RF switch-time requirements, and RAN4 RRM should study SCell activation interruption and overall delay.

// Other PHY signal/channel/procedure related RRM

UE Tx timing

Observation 8: Relaxing PRACH timing requirements would accommodate initial timing errors within CP, allow timing correction after initial access, and enable higher SCS. Especially in NTN scenarios, where impairments such as GNSS/UE positioning inaccuracies and satellite position errors can be corrected after initial access.

Proposal 13: Study PRACH (if introduced) specific timing accuracy requirement.

Observation 9: In 5G, two opposite direction methods are used to adjust UE UL timing: gradual adjustment (Method 1), which doubles the timing changes observed at BS and requires frequent TAC, and UE pre-compensation, where the UE corrects timing before the BS detects misalignment.

Observation 10: With propagation delay changes, UE pre-compensation (Method 2) leaves NB observed UL timing unchanged, yet it differs from what test system expects in the 5G R15 mandatory test defined in TS 38.133 A.6.4.1, so the UE fails the test.

Proposal 14: Study the feasibility of replacing gradual timing adjustment with UE pre-compensation, while retaining the Timing Advance command.

MRTD

Observation 11: Higher MRTD values can lead to increased complexity in UE implementation.

Proposal 15: RAN4 to seek optimisation of TAE for MRTD to a level maintaining the UE complexity lower than that for 5G systems.

# References

- RP-251881, “Study on 6G Radio”, NTT DOCOMO (Moderator)

- R4-2514644, “WF on 6G RRM”, Apple, Prague, Czech Republic, October, 2025.