3GPP TSG-RAN WG4 Meeting #118				                                　 R4-2600468

Gothenburg, Sweden, 9th – 13th Feb 2026

Source: 	Huawei, HiSilicon

Title: 	Co-existence study for HPUE with 32dBm MOP

Agenda Item:	7.1.3

Document for:	Approval

- Introduction

In last meeting, simulation scenarios and assumptions are stable as in the agreed WF [1].

This document provides our simulation results based on the WF. Simulation scenarios are cited as below.

Table 1: Co-existence study scenarios

| Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 4GHz, 7GHz | Urban macro | 1st priority |
|  |  |  |  | 2.6GHz | Rural macro | 2nd priority |
|  |  |  |  | 7GHz | Suburban macro | 2nd priority |

- Simulation results

2.1 Urban macro

Based on the assumptions outlined in the WF [1], we set the baseline ACS values for the BS at 42 dB (7 GHz) and 45 dB (below 6 GHz), respectively, with a 30 dB ACLR for the UE. This baseline corresponds to a UL ACIR of approximately 29.7 dB (7 GHz) and 29.9 dB (below 6 GHz). In simulating the scenario, various delta-ACIR X values were considered to determine the appropriate requirements.

For urban macro simulations, values of inter-site distance and BS antenna height for different bands are listed in the table below:

Table 2.1-1: Network layout parameters

| Parameters | 2.6GHz | 4GHz | 7GHz |
| --- | --- | --- | --- |
| Inter-site distance | 750m | 450m/500m | 450m/500m |
| BS antenna height | 45m/25m | 20m/25m | 20m/25m |

The urban macro simulation results for the uplink throughput loss of the victim BS, with different delta-ACIR values, are presented in Table 2.1-2 for a 2.6 GHz center frequency. Under the assumption of a maximum 32 dBm output power for an urban macro scenario, the required ACIR for 2.6 GHz is 30 dB.

Table 2.1-2: Urban macro simulation results of the UL throughput loss at 2.6 GHz

| ACIR offset =X [dB] | 0 | -1 |
| --- | --- | --- |
| Average throughput loss (2.6 GHz) | 0.47 % | 0.40% |
| 5%-tile throughput loss (2.6 GHz) | 4.94 % | 5.94% |

The urban macro simulation results for the uplink throughput loss of the victim BS with different delta-ACIR values are presented in Table 2.1-3 at a center frequency of 3.5 GHz. The required ACIR for 3.5 GHz is 28 dB, assuming a maximum output power of 32 dBm in the urban macro scenario.

Table 2.1-3: Urban macro simulation results of the UL throughput loss at 3.5 GHz

| ACIR offset =X [dB] | 0 | -1 | -2 | -3 |
| --- | --- | --- | --- | --- |
| Average throughput loss (3.5 GHz) | 0.61% | 0.64% | 0.76% | 0.89% |
| 5%-tile throughput loss (3.5 GHz) | 3.83% | 3.87% | 5.09% | 7.29% |

The urban macro simulation results for the uplink throughput loss of the victim BS with different delta-ACIR values are presented in Table 2.1-4 at a center frequency of 7 GHz. The required ACIR for 7 GHz is 24 dB, assuming a maximum output power of 32 dBm in the urban macro scenario.

Table 2.1-4: Urban macro simulation results of the UL throughput loss at 7 GHz

| ACIR offset =X [dB] | 0 | -1 | -2 | -3 | -4 | -5 | -6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7 GHz) | 0.09% | 0.10% | 0.11% | 0.13% | 0.13% | 0.16% | 0.17% |
| 5%-tile throughput loss (7 GHz) | 2.25% | 2.70% | 2.79% | 3.14% | 3.19% | 3.42% | 5.53% |

Observation 1: Under the assumption of a max 32 dBm output power for urban macro scenario, the required ACIR for 2.6 GHz, 3.5 GHz and 7 GHz are as follows.

Table 2.1-5: Urban macro simulation results of ACIR for 2.6GHz, 3.5GHz and 7 GHz

|  | 2.6 GHz | 3.5 GHz | 7 GHz |
| --- | --- | --- | --- |
| ACIR | 30dB | 28 dB | 24 dB |

Observation 2: The required ACLR for 2.6 GHz, 3.5 GHz and 7 GHz are much less than the value specified for PC1 UE in the specification.

2.2 Sub-urban macro

For the sub-urban macro scenario, the simulation is conducted at 7 GHz, with an inter-site distance and BS antenna height of 900 m and 25 m, respectively.

The sub-urban macro simulation results for the uplink throughput loss of the victim BS with different delta-ACIR values are presented in Table 2.2-1 at 7 GHz. The required ACIR for 7 GHz is 25 dB.

Table 2.2-1: Sub-urban macro simulation results of the UL throughput loss at 7 GHz

| ACIR offset =X [dB] | 0 | -1 | -2 | -3 | -4 | -5 | -6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7 GHz) | 0.35% | 0.55% | 0.66% | 1.01% | 1.17% | 1.53% | 1.81% |
| 5%-tile throughput loss (7 GHz) | 2.57% | 3.19% | 3.42% | 4.23% | 4.76% | 5.36% | 6.52% |

2.3 Rural macro

For the rural macro scenario, the simulation is conducted at 2.6 GHz, with an inter-site distance and BS antenna height of 6000 m and 45 m, respectively.

The rural macro simulation results for the uplink throughput loss of the victim BS with different delta-ACIR values are presented in Table 2.3-1 at 2.6 GHz. The required ACIR for 2.6 GHz is 28 dB.

Table 2.3-1: Sub-urban macro simulation results of the UL throughput loss at 7 GHz

| ACIR offset =X [dB] | 0 | -1 | -2 | -3 | -4 |
| --- | --- | --- | --- | --- | --- |
| Average throughput loss (7 GHz) | 1.27% | 1.58% | 2.06% | 2.57% | 2.78% |
| 5%-tile throughput loss (7 GHz) | 3.33% | 4.01% | 5.74% | 6.01% | 7.27% |

It is observed that during the IMT parameter co-existence study for U6G, the agreed ACLR for PC3 is 26 dB. However, simulation results for various scenarios at ~7 GHz show that the ACLR can be lower than 26 dB, even for a 32 dBm MOP.

Observation 3: The agreed ACLR for U6G with PC3 is 26dB according to previous IMT parameter co-existence study.

Proposal 1: Based on simulation results, the proposed ACIR values for the 2.6 GHz, 3.5 GHz, and 7 GHz bands are 30 dB, 28 dB, and 25 dB, respectively.

Proposal 2: To maintain consistency and align with other power classes, the proposed ACLR for TDD bands above 2.5 GHz and below 6 GHz is 31 dB, with a maximum output power (MOP) of up to 32 dBm.

Proposal 3: For the U6G band, the proposed ACLR is 26 dB across all HPUE power classes, including PC2, PC1.5, and MOP up to 32 dBm.

- Conclusion

In this contribution, we provide simulation results of coexistence study for evaluating the ACLR requirements, based on the assumption of a 4x26 dBm PA configuration.

The observations and proposals made in this contribution are summarized below:

Observation 1: Under the assumption of a max 32 dBm output power for urban macro scenario, the required ACIR for 2.6 GHz, 3.5 GHz and 7 GHz are as follows.

Table 2.1-5: Urban macro simulation results of ACIR for 2.6GHz, 3.5GHz and 7 GHz

|  | 2.6 GHz | 3.5 GHz | 7 GHz |
| --- | --- | --- | --- |
| ACIR | 30dB | 28 dB | 24 dB |

Observation 2: The required ACLR for 2.6 GHz, 3.5 GHz and 7 GHz are much less than the value specified for PC1 UE in the specification.

Observation 3: The agreed ACLR for U6G with PC3 is 26dB according to previous IMT parameter co-existence study.

Proposal 1: Based on simulation results, the proposed ACIR values for the 2.6 GHz, 3.5 GHz, and 7 GHz bands are 30 dB, 28 dB, and 25 dB, respectively.

Proposal 2: To maintain consistency and align with other power classes, the proposed ACLR for TDD bands above 2.5 GHz and below 6 GHz is 31 dB, with a maximum output power (MOP) of up to 32 dBm.

Proposal 3: For the U6G band, the proposed ACLR is 26 dB across all HPUE power classes, including PC2, PC1.5, and MOP up to 32 dBm.

- References

- R4-2522974, Way Forward for [117][329] NR_UE_RF_Ph5_Part2: HPUE coexistence, Huawei, HiSilicon, MediaTek Inc., Qualcomm, CATT, Nokia

- TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz

- TR 38.876, Air-to-ground network for NR

- TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz

- TR 38.803, Radio Frequency (RF) and co-existence aspects.

- ITU WP 5D/792. Working document on characteristics of terrestrial component of IMT sharing and compatibility studies in preparation for WRC-27(Annex 4.32 to Working Party 5D Chair’s Report)