3GPP TSG-RAN WG4 Meeting # 118 	R4-2600678

Gothenburg, Sweden, Feb. 09-13, 2026

Source: 	vivo

Title: 	Discussion on 6G TN co-existence

Agenda Item:	8.4.2

Document for:	Approval

- Introduction

In this contribution, we provide our views on the 6G TN co-existence study.

- Discussion

Prioritized scenario: ~7 GHz including n104, Urban Macro, PC2 UE (FFS if PC3 and/or PC1.5 or PC1 will be considered).

It is for further discussion on the necessity of the new 6G coexistence study.

The companies who propose not to introduce the new 6G coexistence study are encouraged to provide the justification, including how the conclusions of the existing study can be extended to 6G scenarios and how it will impact to extend the existing 5G requirements to 6G.

Interested companies who propose to introduce the new 6G coexistence study are encouraged to

  - Identify the key assumptions differences (comparing to the assumptions and works done in TR 38.921 for 6.425-7.125GHz and TR 38.922 for 7.125 – 8.4 GHz).

  - Identify the potential impacts on the conclusions of previous coexistence studies if any (e.g. how much ACLR/ACS difference is expected).

  - Identify the next steps (e.g. update existing requirements? New requirements/new type? Regulation impacts?).

  - Identify specific band/ frequence range (e.g., n104 and/or 7.125 – 8.4 GHz frequency range)

Other scenarios (e.g. Urban micro, Indoor hotspot, Dense urban, …) are not precluded but the interested companies are encouraged to justify why the proposed scenario(s) would be more stringent comparing to Urban Macro, impacting the coexistence study results.

In [1], we agreed that:

After review latest TR38.901, TR 38.921, TR38.922, possible parameter update and analysis are listed below:

| Parameters | Update | Note |
| --- | --- | --- |
| Channel bandwidth | Add 200MHz | Impact on noise floor and UL power control |
| ISD | Update from 450m to 500m | Align the ISD for 3.5GHz and 7 GHz. Additional pathloss for cell edge UE. |
| Power class | Add PC2, two conditions can be considered: Condition 1: PC2 aggressor, PC3 victim Condition 2: PC2 aggressor, PC2 victim | In NR HPUE co-existence, only the power of aggressor is increased. If 6G decide PC2 is mandatory, the power of victim also need to be changed |
| Noise figure | Update from 9 dB to 12 dB | 9 dB is baseline assumption in TR38.921, while the agreement in TR 38.922 is 12 dB |
| BS antenna model | Update to the parameter in TR38.922 | The Parameter in TR38.922 is different from the simulation assumption in TR38.921 |
| UE antenna model | Add the UE antenna model in TR 38.901, with the assumption of 2Tx/4Rx | New UE antenna model is added in TR 38.901 |

Based on the analysis above, the simulation assumption can be considered to be updated.

Proposal 1: For 7GHz Uma, the parameter in following table can be considered to update:

| Parameters | Update | Note |
| --- | --- | --- |
| Channel bandwidth | Add 200MHz | Impact on noise floor and UL power control |
| ISD | Update from 450m to 500m | Align the ISD for 3.5GHz and 7 GHz. Additional pathloss for cell edge UE. |
| Power class | Add PC2, two conditions can be considered: Condition 1: PC2 aggressor, PC3 victim Condition 2: PC2 aggressor, PC2 victim | In NR HPUE co-existence, only the power of aggressor is increased. If 6G decide PC2 is mandatory (which is pending on UE RF decision), the power of victim also need to be changed |
| Noise figure | Update from 9 dB to 12 dB | 9 dB is baseline assumption in TR38.921, while the agreement in TR 38.922 is 12 dB |
| BS antenna model | Update to the parameter in TR38.922 | The Parameter in TR38.922 is different from the simulation assumption in TR38.921 |
| UE antenna model | Add the UE antenna model in TR 38.901, with the assumption of 2Tx/4Rx | New UE antenna model is added in TR 38.901 |

Another issue we want to discuss here is that we assume all agreement of simulation assumption is only for co-existence purpose, which means the final decision should pending on other dedicate agenda. To avoid forked discussion and unnecessary debate between companies.

Proposal 2: All agreement of this topic during SI is only for co-existence evaluation purpose, whether it can be considered in 6G design is pending on other dedicated topic, e.g., UE RF, BS RF, etc.

# Conclusion

In this contribution, we provide our views on 6G TN co-existence.

Proposal 1: For 7GHz Uma, the parameter in following table can be considered to update:

| Parameters | Update | Note |
| --- | --- | --- |
| Channel bandwidth | Add 200MHz | Impact on noise floor and UL power control |
| ISD | Update from 450m to 500m | Align the ISD for 3.5GHz and 7 GHz. Additional pathloss for cell edge UE. |
| Power class | Add PC2, two conditions can be considered: Condition 1: PC2 aggressor, PC3 victim Condition 2: PC2 aggressor, PC2 victim | In NR HPUE co-existence, only the power of aggressor is increased. If 6G decide PC2 is mandatory (which is pending on UE RF decision), the power of victim also need to be changed |
| Noise figure | Update from 9 dB to 12 dB | 9 dB is baseline assumption in TR38.921, while the agreement in TR 38.922 is 12 dB |
| BS antenna model | Update to the parameter in TR38.922 | The Parameter in TR38.922 is different from the simulation assumption in TR38.921 |
| UE antenna model | Add the UE antenna model in TR 38.901, with the assumption of 2Tx/4Rx | New UE antenna model is added in TR 38.901 |

Proposal 2: All agreement of this topic during SI is only for co-existence evaluation purpose, whether it can be considered in 6G design is pending on other dedicated topic, e.g., UE RF, BS RF, etc.

- References

- R4-2522454 WF on [117][103] 6G BS RF and co-existence, Feature lead (Ericsson)