# 3GPP TSG-RAN WG4 Meeting #118	R4-2600884

# Gothenburg, Sweden, 9th – 13th Feb 2026

Agenda item:			8.1

Source:	Feature lead (Huawei, OPPO)

Title:	Running summary of 6G system parameters

Document for:	Information

Introduction

Based on the latest approved RAN level and working group level SID on 6G Radio, RAN4 study on 6G system parameters with the detailed objectives as below:

RAN level 6G SI This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework WG level 6G SI The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing improvements [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

In this running summary, it is intended to include the following aspects relevant to the RAN4 study on 6G system parameters:

- Status update for 6G system parameters, which is used to track the status of different parameters

- RAN4 key Tdocs and key agreements achieved before this meeting.

- RAN and RAN1 summary for relevant and essential agreements related to RAN4 6G system parameters

The information for feature lead is provided:

| Feature Lead | Company | Email address |
| --- | --- | --- |
| Ye Liu (Leo) | Huawei | leo.liuye@huawei.com |
| Jinqiang Xing | OPPO | xinjinqiang@oppo.com |

Status update

The following status is provided based on RAN4 discussion till the end of RAN#116bis:

RAN4 study on system parameters:

| Parameters | Agreements (essential progress) | Status | Note |
| --- | --- | --- | --- |
| Wave form |  | Open |  |
| PA model |  | Open | - R1-2508069, LS received from RAN1 requesting PA model for waveform evaluation |
| Modulation |  | Open |  |
| Max channel bandwidth |  | Open | - LS R1-2509578 |
| Min channel bandwidth |  | Open |  |
| FFT size |  | Open |  |
| Numerology |  | Open |  |
| Spectrum utilization |  | Open |  |
| Asymmetric channel bandwidth |  | Open |  |
| Channel raster |  | Open |  |
| Sync raster |  | Open |  |
| Channel spacing |  | Open |  |
| Irregular channel bandwidth |  | Open |  |
| Number of Tx/Rx |  | Open |  |
| Device types |  | Open |  |

RAN4 - 6G system parameter agreements

RAN4 #117 meeting (Nov, 2025)

Key Tdocs

| R4-2522266 | Topic Summary for [117][101] 6G system parameter |
| --- | --- |
| R4-2522450 | WF on 6GR system parameter |

Key agreements

Waveform

Agreement:

Regarding the Net Gain for UL agreed in RAN1, RAN4 will focus on Tx power gain relative to the reference, where RAN4 metrics including existing and potential new RAN4 requirements, e.g. emission mask, should be taken into consideration.

- By the end of RAN4#117, the following aspects will be specified

  - The details of the evaluation methodology and assumptions

  - the considered RAN4 metrics. Existing RAN4 requirements will be taken as the baseline.

  - The details of the reference. The Tx impairment should be considered.

Regarding UL PA models which are used for waveform evaluations (it is FFS how the PA models used for waveform evaluation can be extended to other purpose)

  - RAN4 will continue the discussion to define a unified set of PA models in RAN4#118.

    - If no unified set of PA model can be agreed, the interested companies can use their own models

    - RAN4 can still provide RAN1 on RAN4’s considerations including the calibration methodology of PA models.

  - By the end of RAN4#117, RAN4 will target to specify the PA calibration methodology and conditions.

Agreement:

- Target bands:

  - ~7GHz with high priority for response of RAN1 LS

    - For RAN4 evaluation, PA model with applicable RF requirements should be further considered

  - Any other bands are not precluded

- Evaluation framework

  - Adopt the Net Gain metric for UL low PAPR waveform: Same metric as that agreed by RAN1

  - Evaluation of UL low PAPR waveforms with fixed NR SU for existing CBW

- Waveform evaluation assumptions

Table 1: Waveform evaluation assumptions for RAN1/RAN4

| Parameter/Requirements |  | Assumptions/Value | Note |
| --- | --- | --- | --- |
| PA model |  | TBD | Memory effect should be considered for ~7GHz with larger channel bandwidth |
| Band under evaluation |  | around 7GHz, other bands are not precluded | n104 could be assumed for ~7GHz |
| Channel Bandwidth (CBW) |  | At least 100MHz, 200MHz Other CBW based on inputs for PA models | Same SU assumed for 200MHz as 100MHz |
| Power class |  | PC2 (26dBm), PC3 (23dBm) |  |
| Complied requirements | SEM | TS 38.101-1 §6.5.2.2 | Subject to further adjustment pending on progress of UE RF, co-existence study |
|  | ACLR | TS 38.101-1 §6.5.2.4 |  |
|  | EVM | TS 38.101-1 §6.4.2.1 | Considered for high modulation order/inner RB allocation, pending on RAN1 discussion |
|  | IBE | TS 38.101-1 §6.4.2.3 |  |
| Tx impairments | Carrier Leakage | -28dBc | Subject to further adjustment pending on progress of UE RF study |
|  | IQ image | -28dBc |  |
|  | CIM3 | -60dB |  |
| PA calibration conditions | CBW | 100MHz full RB allocation | Other options are not precluded, pending on the further study in RAN4 |
|  | SCS | 30kHz |  |
|  | Waveform | DFT-s-OFDM |  |
|  | Modulation | QPSK |  |
|  | Power class | PC2/PC3 |  |
|  | Power backoff to meet ACLR | 1dB |  |

Note: The table is considered as baseline for PA calibration.

PA model

Agreement

- Staged development of the PA model used for waveform evaluation.

  - For RAN1: Provide a model for timely waveform comparison.

    - Targeted for RAN4#118 meeting

  - For RAN4: Continue internal development of PA models, if needed, with more realistic considerations for RF requirements evaluation.

- Prioritize PA model(s) for 7 GHz, PC2/PC3 for waveform evaluation.

  - Consider models like the Generalized Memory Polynomial (GMP)

  - Other options not precluded

- Develop PA models covering different frequency ranges, power classes if single PA model is not accurate enough for all evaluation scenarios.

- Agree on calibration conditions and applicable requirements for the PA model (e.g., achieved ACLR for a reference waveform at a specific MPR) to ensure fair comparisons.

Max channel bandwidth

Agreement on DL max CBW:

- Discuss the feasibility and necessity to support 400MHz either as single CC or CA for UE from RAN4 perspective

- Regarding 400MHz support, RAN4 will study both single CC with 400MHz max CBW+30kHz SCS+16k FFT (2x8k FFT for single CC is not precluded) and CA with maximum CBW of 200MHz+30kHz SCS+8k FFT (200MHz+200MHz) from the following perspectives:

  - Implementation considerations, including, but not limited to, the RF/BB architecture, feasibility (e.g. the feasibility to support 400MHz as single CC), complexity, power consumption, etc.

  - Pro and Con between single CC and CA, including, but not limited to, the system efficiency, system performance, overhead and other constraints

  - The spectrum availability. The target spectrum for this study includes ~7GHz, (any others?)

  - Support of symmetric/asymmetric DL and UL max CBW, including, but not limited to, system efficiency.

Agreement on UL max CBW for ~7GHz:

- Operators, some of infra vendors and some of UE vendors have strong interests to continue the discussion on supporting max 400MHz UL spectrum with 30kHz SCS during the 6G study, subject to the interim milestone timeline.

- From UE implementation perspective based on the inputs from UE vendors, max CBW 400MHz with 30kHz SCS for UL is considered as very challenge and the corresponding feasibility to support it is broadly questioned.

  - Some UE vendor thinks it is feasible to support max CBW 400MHz with 30kHz SCS for UL.

  - Based on the existing state-of-the-art solution, max CBW 200MHz with 30kHz SCS is considered as feasible

    - The feasibility can be further confirmed.

- Max CBW for UL will be further discussed in RAN4#118 with both operators’ requests and the feasibility issues raised by the UE vendors taken into consideration.

- Interested companies are encouraged to bring their technical analysis on the following aspects in RAN4#118

  - Availability of RF components and their performance to support max CBW

  - The feasibility, pro and con to support max CBW 400MHz+30kHz vs. max CBW 200MHz+30kHz vs. 200MHz+200MHz CA with max CBW 200MHz+30kHz.

  - UL coverage

  - Other aspects are not precluded.

Min channel bandwidth

Agreement

- Define minimum CBW based on SCS (e.g., 5 MHz for 15 kHz, 10 MHz for 30 kHz, 50 MHz for 120 kHz).

- Support 3 MHz with 15kHz SCS for particular bands subject to operator requests, commercial need and spectrum regulations,

- Work with RAN1 for the chosen minimum CBW in initial access design to accommodate majority spectrum scenarios.

Numerology

Agreement

- Evaluate the following proposals regarding numerology from RAN4 perspective

  - "Single numerology" proposal

    - Single SCS for one band or frequency sub-range

    - Same SCS for SSB and Data in one band

  - Frequency sub-range/Band specific SCS values

    - Co-ordinate and align with RAN1 discussion and agreements

  - Numerology for specific scenarios, like NTN and ISAC

    - RAN1 progress should be taken into account

RAN4 #116bis meeting (Oct, 2025)

Key Tdocs

| R4-2514508 | Topic Summary for [116bis][101] 6G system parameter |
| --- | --- |
| R4-2514650 | WF on 6GR system parameter |
| R4-2514643 | LS on system parameters |

Key agreements

Waveform

Agreement

- The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1

- To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects

  - Tx assumption including PA model

  - Related RF requirements which should be taken into consideration

  - Both UL and DL are considered

  - Take the candidate waveforms identified/discussed in RAN1 as the baseline

  - Identify the evaluation metric, e.g. net gain

  - Implementation constraints

- Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs

  - Evaluation cases could be selected step-by-step, based on RAN1 progress

  - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement

PA model

Agreement

- Consider PA modelling at least for the following aspects for RAN4 discussion

  - Memory effects for UE supporting larger CBW

  - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3)

  - Calibration conditions

  - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc.

  - PA models for different device types, handheld UE is prioritized

  - Applicable requirements

    - 5G-A requirements as starting point

  - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

Modulation

Agreement

- RAN4 evaluation work focus on the feasibility study and RF requirements impact

- To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1

  - Identify the main affected requirements for modulation evaluations

    - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions.

  - Align on the evaluation assumptions.

  - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation.

  - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed.

  - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1.

- Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations

  - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies.

  - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

Max channel bandwidth

Agreement

In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths.

  - RF performance evaluation with proposed max CBW

    - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation

    - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements

    - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers

    - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum

    - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum.

    - Assess the implementation feasibility and complexity and power consumption

    - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1.

    - Study the feasibility to support different NW and UE max CBW.

  - Collaborate with RAN1 on feasibility findings

    - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations

Min channel bandwidth

Agreement

- Compare options of defining min CBW, considering pros and cons

- Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress

  - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands

  - SCS-dependent framework

  - Other aspects are not precluded

- Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective.

FFT size

Agreement

- Consider FFT size, maximum Channel Bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective, especially for 8K or 16K FFT size considering the associated SCS and also the frequency ranges

- Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

Numerology

Agreement

- Evaluate the following proposals regarding numerology from RAN4 perspective

  - "Single numerology" proposal

  - Frequency sub-range/Band specific SCS values proposal

    - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band

    - Study numerology for SSB of initial cell search from RAN4 perspective

  - Other proposals not presented in this meeting are not precluded

- Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

Spectrum utilization

Agreement

- Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM).

  - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G

- Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled

  - Considering trade-offs between SU, RF performance, and UE/BS complexity

  - Channel bandwidth and SCS with smaller SU should be prioritized

  - SU for larger channel bandwidth shall be evaluated based on progress on CBW

  - Closely coordinate with RAN1 on different waveform candidates and SCS configurations.

Asymmetric channel bandwidth

Agreement

- Study the need and feasibility to enable asymmetric CBW

Channel raster

Agreement

- Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands

  - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster

- Investigate the interaction between the channel raster and the synchronization raster

- Investigate the necessity of channel raster or alternative ways for the channel configuration

  - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster

  - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported

- Study the listed main proposals especially for the migration and co-existence approaches

  - Other options not presented in this meeting are not precluded

- Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

Sync raster

Agreement

- Evaluation on sync raster from RAN4 perspective:

  - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4

  - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving

- Collaboration and planning:

  - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset.

Channel spacing

Agreement

- Further study in RAN4 regarding channel spacing

Irregular channel bandwidth

Agreement

- 5G concepts from TR 38.844 can be considered as starting point to study a more generic solution for 6G

  - The listed main proposals and identified issues should be taken into account for the following study of irregular/flexible/scalable channel bandwidths

  - Other options not presented in this meeting are not precluded

Number of Tx and Rx

Agreement

- Study the framework of spec impact with different number of Tx/Rx

- Evaluate number of Tx/Rx from both a performance and implementation complexity perspective.

  - E.g. device size (e.g. foldable smartphone)/form factors constraints, antenna design

- Co-ordinate with RAN and RAN1 for number of Tx/Rx

Device types

Agreement

- The following aspects could be further studied

  - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range.

    - Study on the NW impact should also be considered

  - Study how to better support variety devices from RAN4 perspective

    - E.g. differentiate devices, use cases, features

  - Other aspects are not precluded

- Collaborate on the framework with other WGs:

  - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

RAN related agreements summary

Min Channel bandwidth

RAN #109 (Sep, 2025)

Agreement

In RP-252873, proposal 1 is endorsed.

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110.

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

FFS: whether RF and BB UE BW are same or different

Numerology

RAN #110 (Dec, 2025)

Agreement

- conclusion: agreed: SCS of 30kHz for mid-band (1-2.xGHz) FDD is not supported in 6G

RAN #109 (Sep, 2025)

Agreement

- Regarding the RAN1 agreement below, the following is agreed:

- Keep using “around 7GHz” instead of using detailed frequency range definition in RAN1’s and RAN4’s study.

Agreement

- 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH.

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD, 30kHz SCS for TDD

    - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

    - FFS: 7.5kHz SCS for sub1GHz (FDD)

    - Whether to discuss the FFS will be subject to RANP decision.

  - For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

    - Whether to discuss it will be subject to RANP decision

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

- FFS subcarrier spacing for PRACH and up to initial access discussion.

Spectrum utilization

RAN #109 (Sep, 2025)

Agreement

In RP-252950, proposal 1 is agreed

- Proposal 1: 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR.

Device types

RAN #110 (Dec, 2025)

Smallest Maximum UE Bandwidth vs. Minimum Spectrum Allocation for 6GR vs. Bandwidth for Initial Access

Proposal 1:

- 6GR supports the operation (but not required to be optimized for performance) in a minimum spectrum allocation of 3MHz with a 15kHz SCS

Note: the following agreement made in RAN1#123 still holds, with the clarification that the bandwidth in Opt 1 below is assumed to be at least 5MHz with a 15kHz SCS.

Agreement

If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR,

- Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable

- Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations

- Regarding the smallest maximum UE bandwidth as discussed in the following RAN1 agreement, Opt 1 is excluded. Aim to conclude by RAN plenary no later than RAN#112 (June 2026).

  - RAN1 and RAN4 is tasked to continue providing more analysis accordingly.

  - Companies are encouraged to provide more analysis at RAN plenary particularly regarding the use cases, requirements, economy of scale, etc.

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

Device Types and UE Capabilities

Proposal 2:

- Diverse device types as in 6G SIDs are expected to include supporting at least MBB (as highest priority) and 6G IoT services

- For discussion related to device types and UE capabilities particularly related to scalability, companies are encouraged to further contribute at RAN plenary regarding the analysis of necessary aspects to help progress RAN WG technical design and evaluations.

- The detailed discussion for device types (how many, parameters, whether or not to define device types, whether or not to introduce minimum mandatory set of features, etc.) and UE capabilities (mandatory/optional, simplification, grouping/conditional or not, etc.) is on hold and to resume from RAN#113 (September 2026).

Operational requirements

Proposal 3: Capture the following aspects related to diverse device types into the TR:

- Scalable and forward compatible design

- Limited set of device types (based on the agreement in RAN#109)

Detailed wording is to be drafted in RAN#111, along with possibly other aspects (e.g., scalability, differentiated services, etc.).

For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas (RP-253802)

- Up to 4

- Up to 8

- Up to 16 (for RX only and intended only for FWA)

Note: Above does not impact future discussions on UE capability

RAN #109 (Sep, 2025)

Agreement

In RP-252873, proposal 3 & 4 are endorsed for RAN only (no WG discussion)

Proposal 3: To investigate further:

- Motivations/justifications behind the proposed diverse device types, which should be a limited set

- Whether/how to have one or more device types for eMBB or 6G IoT

- Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc.

- Whether/how to explicitly standardize device types

- Ensuring forward compatibility

- Minimizing/avoiding potential market fragmentation

Note: the terminology “device type” is subject to further discussion and possible refinement.

Proposal 4: In terms of diverse device types, study further:

- Possible parameters/factors, e.g.:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

Note: some of the above parameters/factors may be related with form factor

Note: aim to have a focused/limited set of parameters/factors for a device type

The value(s) for the identified parameters for a device type

RAN1 related agreements summary

Overview of 6GR air interface

RAN1 #122bis (Oct, 2025)

Agreement

- RAN1 provides methodology and corresponding initial analysis of potentially achievable coverage to RAN#110 to determine the coverage target(s)

Agreement

- High-level aspects to consider for the 6GR sync signal structure include, but not limited to

  - Sync raster design

  - Spectrum allocation

  - smallest maximum supported RF and BB UE BW without spectrum aggregation

  - mobile broadband service requirements as high priority

  - Energy efficiency for both BS and UE

  - Detection/tracking performance, latency, and complexity

  - Including initial cell search

  - Coverage target

  - Common design for diverse device types

  - Consideration of the supported deployment

  - Consideration on whether the single sync signal structure is sufficient

  - Note: Aspects impacting on the periodicity is to be discussed under AI11.5

Agreement

- The aspects to consider for supporting NTN include, but not limited to

  - Initial access, including cell search and SSB periodicity

  - Coverage

  - Duplexing

  - Capacity

  - Signalling overhead

  - GNSS-less/resilient/based operation

  - Large/varying doppler and propagation delay

  - Beamforming / beam management / beam hopping

Note:

- High-level aspects to consider to enable lower CAPEX/OPEX with respect to current networks include, but not limited to

  - UE/NW implementation complexity

  - UE/NW energy efficiency

  - MRSS

  - Spectrum efficiency

Agreement

For communication, 6GR considers NR frame structure used as a starting point for the study item,

  - Resource defined by one subcarrier and one symbol is called as resource element (RE).

  - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12

  - Radio Frame length is 10ms

  - Each radio frame is split into 10 subframes, each with a duration of 1 ms

  - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design.

  - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings.

Waveform

RAN1 #123 (Nov, 2025)

Agreement

Add the following metrics for UL PAPR reduction to the existing agreement (made in RAN1#122b)

- Net gain@10% BLER assuming similar spectral efficiency and same occupied bandwidth for each compared method

- ACLR, EVM, IBE

Agreement

General evaluation assumptions for UL low-PAPR proposals

|  | 3GPP 6GR |
| --- | --- |
| Carrier frequency and scenario | 4GHz |
| Channel BW | At least 100MHz for Urban (4GHz) |
| Occupied BW | To be discussed with detailed simulation assumptions |
| SCS | 30 kHz for 4GHz |
| Channel model | TDL-C 300ns for 4GHz |
| UE speed | 3km/h |
| Number of Tx antennas for TDL channel | 1 |
| Number of Rx antennas for TDL channel | 1 and 4 for 4GHz |
| Number of DMRS symbols/slot (location as defined in NR) | 2 |
| Number of PUSCH data symbols/slot | 12 |
| HARQ configuration | No retransmissions |
| Frequency hopping | Disabled |

Agreement

For single user evaluation assumption for MCS and subcarriers UL low-PAPR proposals with spectrum extension

| No Spectrum Extension |  | With Spectrum Extension |  |  |
| --- | --- | --- | --- | --- |
| MCS | #subcarriers | #SCs before extension (A) | Occupied BW: #SCs after extension (B) | Spectrum extension Extension: α=B-AB |
| NR MCS | B | (1-α)B | B | α∈16, 14,27,13,38,25,716,12 |

For single user evaluation assumption for MCS and subcarriers UL low-PAPR proposals with spectrum truncation

| No Spectrum Truncation |  | With Spectrum Truncation |  |  |
| --- | --- | --- | --- | --- |
| MCS | #subcarriers | #SCs before truncation (A) | Occupied BW: #SCs after truncation (B) | Spectrum truncation factor Truncation: α=A-BA |
| NR MCS | B | A | 𝐵 | α∈110, 210,310,410 |

Note: other values for extension or truncation are not precluded.

Agreement

For both CP-OFDM and DFT-S-OFDM, for single user evaluation assumption for MCS and subcarriers UL low-PAPR proposals with tone reservation.

| MCS | Occupied BW: #subcarriers | #reserved tones (inband/sideband) | Ratio of reserve |
| --- | --- | --- | --- |
| NR MCS | B | A | A/𝐵 |

Agreement

Table is endorsed to characterize each proposal as a potential RAN1 observation.

Characterization of each waveform proposal

|  | Description |
| --- | --- |
| Name of the proposal |  |
| Motivation of the proposal | E.g. TN, NTN, ISAC, etc… |
| Applicable link direction | DL/UL/both |
| Enhancement to CP-OFDM? | No/Yes |
| Enhancement to DFT-s-OFDM? | No/Yes |
| Additional OFDM-compatible waveform? | No/Yes |
| Target channel(s)/signal(s) | PDCCH/PDSCH/PUCCH/PUSCH/xxx |
| Target modulation |  |
| Motivation / use case | Improved spectral efficiency, … |
| Key Metric / KPI | Spectral efficiency, … |
| Key spec impact foreseen |  |
| MRSS compatibility | Please explain |
| Multiplexing/coexistence with other waveforms | Please explain |
| Multi-user multiplexing | Please explain |
| MIMO compatibility | Please explain |

Agreement

For UL PAPR reduction, values for occupied BW B:

- {2, 4, 8, 16, 24, 30, 32, 64, 128, 240, 256} PRBs.

- Other PRB allocations are not precluded.

- Edge, outer and inner PRB allocations as defined in TS 38.101 should be considered.

Agreement

- Performance benefit to be evaluated using both link level and system level simulation.

- FFS: metrics

- Link level configuration for multi-layer UL waveforms study.

| Parameters | Values |
| --- | --- |
| Carrier frequency​​ | 4 GHz |
| Subcarrier spacing​​ | 30 kHz​​ |
| UE antenna ports | 2, 4 |
| BS antenna ports | 64, port reduction is not precluded |
| FDRA | 4, 8, 16, 32, 64​​ |
| Waveform​​s | CP-OFDM DFT-s-OFDM​​ |
| MCS/modulation​​ | NR UL MCS table with 256 QAM |
| Number of layers​ | 2 layers​​, 4 layers |
| Channel model​​ | CDL-A30​​, TDL-C300, CDL-C300, |
| UE speed​​ | 3km/h​​, 30 km/h |
| Channel est.​​ | Practical​ |
| SRS periodicity | To be reported by the company |
| Receiver | LMMSE |
| HARQ retransmission​ | Disabled |
| DMRS configuration | Configuration type 1 2 DMRS symbols per slot |
| Number of PUSCH data | 12 symbols |
| Waveform and MIMO configuration | 5G codebook, to be reported by the company |
| BLER target | 10% |
| Frequency hopping | Disabled |
| Power class and power mode | Option 1: PC2, total power limited to 26 dBm. - MaxRank 2: Each PA is limited to 23 dBm - MaxRank 4: Each PA is limited to 20 dBm Option2: PC3, total power limited to 23 dBm. - MaxRank 2: Each PA is limited to 20 dBm - MaxRank 4: Each PA is limited to 17 dBm |

Agreement

Table: System level configuration for multi-layer UL CP-OFDM/DFT-s-OFDM study:

| Parameters | Values |
| --- | --- |
| System configuration | UMa, 21 cells, 10, 30 or 50 UEs/cell - Detailed parameters not listed in this table according to UMa as defined in AI 11.2 |
| Traffic model | FTP model 3, - 0.5 Mbyte packet sizes - Network load up to companies to report Full buffer (optional, for calibration) |
| Carrier frequency | 4 GHz |
| Channel bandwidth/Subcarrier spacing | At least 100 MHz, 30KHz |
| Frame structure | TDD (DDDSU), S-slot D:G:U as 10:2:2 |
| UE antenna ports | Same as link level |
| BS antenna ports | Outdoor Combination 2 for UMa as agreed in 11.2 |
| UE antenna model | The UE handheld model as agreed in AI 11.2, companies to report the used model. |
| ISD | 500 m |
| Maximum Number of layers (maxRank) per UE | Adaptive rank, maxRank 2 or 4 SU-MIMO |
| Receiver | Same as link level, port reduction mapping to be reported |
| Waveform and MIMO configuration | Same as link level |
| Power class and power mode | Same as link level |
| UL Maximum power | MPR based Pcmax according to modulation/FDRA/waveform/coherency/etc. - Option 1: 38.101-1 section 6.2D definition - Option 2: Based on a realistic PA model |
| UL Power Control Settings | P0 and Alpha to be reported by the company |
| SRS periodicity | To be reported by the company |

Table: Link level multi-user evaluation assumption for UL low-PAPR proposals

| Parameters | Values |
| --- | --- |
| User # | 2 users |
| RB allocation | X RBs per UE, with Y RB overlap. I.e. 2X-Y RBs as the total occupied BW |
| Per user configuration | As in the single user sim assumptions agreed earlier. |
| Multi-user overlap RBs | Reported by companies |
| Baseline scheme | No spectrum extension/truncation. (2X-Y)/2 RB as the occupied BW for each simulated UE. |
| Per UE MCS | Chosen to align with the spectral efficiency |
| Receiver type | The receiver type/algorithm to be reported by the proponent |

RAN1 #122bis (Oct, 2025)

Agreement

Final LS R1-2508069 is endorsed.

Agreement

- For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference

  - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

    - A realistic PA model should be used

    - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account.

    - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged.

    - FFS: Other evaluation metrics

    - Note: Companies to report how to calculate the Tx power gain, modulation and coding

Agreement

- Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

RAN1 #122 (Aug, 2025)

Agreement (first agreement for 6G!!)

CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink

- Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions

- Other OFDM based waveforms are not precluded.

Agreement

CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink

- Enhancements/modifications on CP-OFDM will be studied as potential additions

- DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink

Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

Note:

Proponents are encouraged to provide more detailed information on their proposals for the next meeting, e.g.:

Proponents to characterize the main motivation for modification/additional waveform proposals:

- Targeted link direction, i.e. DL, UL or both

- Targeted use case (e.g. NTN, specific frequency range, etc.), if any

- Potential motivations metrics used, and quantified gains for a proposal, e.g.

  - Coverage

  - Network energy efficiency

  - UE energy efficiency

  - Spectral efficiency

  - High speed tolerance

  - Scheduling flexibility

  - Integration with ISAC

Proponents provide information on the following aspects, if applicable

- MRSS compatibility

- Target channels/signals, e.g. all channels, PxSCH only, etc.

- MIMO (SU and MU-MIMO) compatibility

- Target modulations, and impact to other modulations, if applicable

- Multi-user multiplexing/scheduling flexibility

- Multiplexing/coexistence with baseline waveforms

- Impact on synchronization and initial access

- Expected specification impact

- Transmitter/receiver complexity and impact to power consumption.

Modulation

RAN1 #123 (Nov, 2025)

Agreement

For PS/GS fixed MCS performance reporting for 10% BLER (other target x% BLER can also be reported), adopt the following format for simulation as a starting point for result reporting.

| NR reference |  | Scheme A (e.g, PS, 1D-NUC, 2D-NUC etc) |  |  |
| --- | --- | --- | --- | --- |
| SE | (Mod order, coding rate) | SE point specific parameters | Baseline (uniform QAM) SNR to reach target BLER | Gain/loss in dB wrt NR baseline at target BLER x% |
| SE x | (modOrder, coding rate) | … |  |  |
| SE y | (modOrder, coding rate) | … |  |  |
| SE z | (modOrder, coding rate) | … |  |  |
| SE point independent assumptions |  | Common assumptions for the scheme simulated, including channel type (AWGN, SISO, SIMO, MIMO) and antenna configuration, number of spatial layers, number of RB allocated, TB size, shaping algorithm used (including block length), freq domain interleaver applied or not, receiver assumption, precoding assumption, realistic channel estimation, etc |  |  |

Note: For NR MCS reference, since NR has multiple MCS tables, it is not enough to provide the MCS index. Instead, need to provide the (modulation order, coding rate) pair for the simulated SE

Note: For SE point specific parameters:

- For GS, this can be a pointer to the constellation used for this SE point

- For PS, this can be a constellation size, coding rate and shaping parameter used for this SE point

Note: Other metrics (at least complexity) will be merged into the same table with other columns, if details of the metrics are agreeable.

Note: For AMC study, if possible, we can use the same table format

Agreement

On how to evaluate complexity, storage requirement, delay and parallelism/serialism for PS and GS compared with uniform QAM.

- For PS

  - The demapper complexity is compared with uniform QAM demapper complexity

    - Can report the demapper complexity with PS and the demapper complexity of NR MCS with the same spectrum efficiency, and the ratio of the complexities

      - Also report the number of spatial layers, dm-algorithm used and the receiver type (e.g., LMMSE or rML), and fixed point assumed or floating point assumed.

  - The Distribution Matcher (DM)/Distribution De-Matcher (DDM) complexity and/or storage requirement as a function of the DM algorithm used (ESS, CCDM, etc), precision of fixed point implementation, block length, and the number of bit levels shaped per symbol

    - For complexity, can report the complexity normalized by the number of information bits

      - As a reference, can also report the computation complexity of LDPC decoding with 10 iterations.

    - For storage requirement, can report the overall storage needed for DM/DDM for supporting all MCS in the MCS table and all shaping related parameters

    - DM and DDM complexity and storage will be counted separately

  - The DM/DDM processing delay, parallelism/serialism, as a function of DM design and block length, and their impact to throughput

- For GS,

  - The demapper complexity is compared with uniform QAM demapper complexity

    - Can report the ratio of GS demapper complexity over the uniform QAM demapper complexity

      - Also report if 1D-NUC or 2D-NUC is used, # of spatial layers, and the receiver type (e.g., LMMSE or rML)

    - Also need to report the assumption on complexity counting, e.g, fixed point assumed or floating point assumed

  - The storage requirement for storing all the constellations corresponding to the MCS indices in the MCS table, as a function of precision of constellation point storage

  - Processing delay and parallelism/serialism, if applicable, and their impact to throughput

Note: the complexity is represented by the numbers of comparison, addition/subtraction, and multiplication/division operations, normalized by the number of information bits.

Note: For complexity as a function of SE point, will add a column in the already agreed performance reporting table.

Note: For complexity/storage not as a function of SE point, will add a row in the already agreed performance reporting table.

Note: Spec impact will be separately evaluated, include BICM, scrambling, etc

Note:

For 4K uniform QAM DL and 1K uniform QAM UL link level performance study, the following format can be used for performance reporting.

| (modOrder, coding rate*1024, SE) | Assumed TX/RX EVM | Channel 1 | Channel 2 | Channel 3 |
| --- | --- | --- | --- | --- |
|  |  | SNR to achieve target BLER | SNR to achieve target BLER | SNR to achieve target BLER |
| (10, 900.5, 8.7939) for DL evaluation (8, 916.5, 7.1602) for UL evaluation | Legacy EVM for 1K QAM for DL and 256QAM for UL respectively |  |  |  |
| (10, 948, 9.2578) for DL evaluation (8, 948, 7.4063) for UL evaluation | Legacy EVM for 1K QAM for DL and 256 QAM for UL respectively |  |  |  |
| SE1 |  |  |  |  |
| SE2 |  |  |  |  |
| SE3 |  |  |  |  |
| Other parameters |  |  |  |  |

- For assumed TX/RX EVM, before we receive any concrete numbers from RAN4, companies can provide their assumptions. One example can be 6dB tighter than the EVM of 1K QAM for DL and 256QAM for UL.

- Other parameters include: Channel estimation assumption (genie or realistic), channel configurations (AWGN, SISO, SIMO, MIMO and TX/RX antenna configurations, channel types, number of spatial layers,), assumed residual freq offset range, number of allocated RBs, etc

- Two highest MCS points in DL 1K QAM and UL 256QAM in NR added in the table for comparison.

- This is preliminary result and not intended for TR

Note: For high order uniform QAM for DL 4K QAM and UL 1K QAM, to provide the UPT with the high order QAM (DL 4K QAM and UL 1K QAM) over the UPT without the high order QAM under the assumed deployment scenario.

Agreement

For PS/GS fixed MCS performance reporting for 10% BLER (other target x% BLER can also be reported), adopt the following format for simulation as a starting point for result reporting.

| NR reference |  | Scheme A (e.g, PS, 1D-NUC, 2D-NUC etc) |  |  |
| --- | --- | --- | --- | --- |
| SE | (Mod order, coding rate) | SE point specific parameters | Baseline (uniform QAM) SNR to reach target BLER | Gain/loss in dB wrt NR baseline at target BLER x% |
| SE x | (modOrder, coding rate) | … |  |  |
| SE y | (modOrder, coding rate) | … |  |  |
| SE z | (modOrder, coding rate) | … |  |  |
| SE point independent assumptions |  | Common assumptions for the scheme simulated, including channel type (AWGN, SISO, SIMO, MIMO) and antenna configuration, number of spatial layers, number of RB allocated, TB size, shaping algorithm used (including block length), freq domain interleaver applied or not, receiver assumption, precoding assumption, realistic channel estimation, etc |  |  |

Note: For NR MCS reference, since NR has multiple MCS tables, it is not enough to provide the MCS index. Instead, need to provide the (modulation order, coding rate) pair for the simulated SE

Note: For SE point specific parameters:

- For GS, this can be a pointer to the constellation used for this SE point

- For PS, this can be a constellation size, coding rate and shaping parameter used for this SE point

Note: Other metrics (at least complexity) will be merged into the same table with other columns, if details of the metrics are agreeable.

Note: For AMC study, if possible, we can use the same table format

Agreement

To evaluate the proposal to allow a single spectrum efficiency point to be supported by multiple MCS entries (with different modulation order and coding rate combinations with uniform QAM or with different shaping parameters, coding rate, and constellation size combinations for PS and different coding rate and constellation combinations for GS).

- When providing results, to provide the following information

  - Details on the design of MCS table with overlapping MCS entries and expected size of MCS table, including performance comparison of designs with the same expected size of MCS table

  - Performance benefit under different channel and rank assumptions

    - As baseline, provide performance with legacy MCS table up to 256 QAM

      - Can additionally provide performance with legacy MCS table up to 1K QAM

    - For PS/GS, provide performance allowing each SE point to be mapped to one or more MCS entries

      - For PS/GS, provide performance allowing each SE point to be mapped to only one MCS entry (from the set of one or more MCS entries)

    - For uniform QAM, provide performance allowing each SE point to be mapped to one or more MCS entries

  - MCS selection mechanism across multiple MCS corresponding to the same spectrum efficiency.

    - If UE feedback is needed for gNB to select between multiple MCS entries corresponding to the same SE, provide details on what is to be fed back

- FFS: How different MPR for different modulation order is captured in the simulation for uplink

- FFS: How different EVM for different modulation order is captured in the simulation

- For the purpose of this study, the same set of SE points as in legacy uniform QAM table will be used as starting point.

- When reporting performance, also report other assumptions, including channel type (AWGN, SISO, SIMO, MIMO) and antenna configuration, number of spatial layers, number of RB allocated, TB size, shaping algorithm used (including block length), freq domain interleaver applied or not, receiver assumption, precoding assumption, realistic channel estimation, etc

- To propose how to align shaping parameters or how to align coding rate for facilitating comparison.

Agreement

For DFT-s-OFDM, further study how/whether Net Gain over uniform QAM can be achieved by PS/GS.

RAN1 #122bis (Oct, 2025)

Agreement

For 6GR constellation shaping evaluation for CP-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. The evaluation and comparison should consider at least the following:

- BLER performance under AWGN channel (at least for performance calibration)

  - 1st transmission (baseline) and with HARQ re-transmission

- BLER performance under fading channel with fixed MCS

  - 1st transmission (baseline) and with HARQ re-transmission

- Throughput performance with link adaptation (adaptive MCS and rank) under fading channel

  - Needs to provide assumptions on rate adaptation (e.g., target BLER for 1st transmission, maximum # of retransmissions)

- Transmitter and receiver complexity (e.g., shaping/deshaping, demapper), latency, parallelism implementation, and storage requirements,

- Other KPI not excluded, such as PAPR, EVM, MPR/A-MPR

- Expected spec impact

- FFS detailed assumption of constellation shaping and improved MCS table

- System level evaluation can be done after link level evaluation.

Agreement

For 6GR constellation shaping study, proponent is encouraged to provide details for the PS/GS schemes considered for evaluation and comparison, including at least the following

- Probabilistic shaping for CP-OFDM and DFT-s-OFDM

  - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide constellation (including normalization), coding rate and target probabilistic distribution for each SE

    - If multiple coding rate and target probabilistic distribution pairs are provided for each SE, how to switch between them

  - Relationship between shaping and FEC, coded bits to modulation symbol mapping, and other modules (such as scrambling, interleaving), in transmit and receive chains. How to handle HARQ retransmission

  - PS algorithm details (for example, source coding based, channel coding based, etc) and parameters (such as block length, rate loss)

- Geometric shaping for CP-OFDM and DFT-s-OFDM

  - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide target constellation shape (including normalization) (1D-NUC, 2D-NUC, QAM-CS, etc) for each SE

    - If multiple constellation shapes are provided for each SE, how to switch between them

  - GS mapping details, such as bit to constellation point mapping and shape

  - Relationship with other blocks (such as scrambling, interleaving). How to handle HARQ retransmission

Agreement

For link level simulation for modulation evaluation, companies are encouraged to evaluate with the following assumptions and should report the exact scheme evaluated.

- channel configuration, including Channel profiles,Tx/RX antenna settings

- For MIMO scenario: SU-MIMO or MU-MIMO, follow agenda item 11.2 for MIMO when available.

- Precoder assumption

  - Close loop MIMO (reciprocal beamforming (e.g., SVD, SLR/RZF, etc.), codebook based)

    - Realistic CSI/SRS/AP-SRS periodicity and delay, and SRS chanEst assumptions,

    - or genie beamforming

  - Open loop MIMO

- Receiver assumption (for MIMO): LMMSE (baseline) for UL, rML or LMMSE for DL

- LLR demapper: Max-log (baseline) or Log-MAP

- Channel estimation: Realistic (baseline) or ideal

- Other assumptions: Channel coding NR-LDPC (baseline), PxSCH bandwidth, SCS, FD interleaver used or not, 5GNR BICM interleaver usage

- Note: For MIMO, SIMO, MISO and SISO are included when possible

Agreement

For 6GR constellation shaping evaluation for DFT-s-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. In addition to what has been agreed for CP-OFDM in earlier agreement, the evaluation and comparison should further consider at least the following:

- PAPR/CM of the resulting waveform

- EVM, MPR/A-MPR

Agreement

For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model).

- FFS: How to involve RAN4 early

- FFS: Shaping of higher order modulation

- System level evaluation can be done after link level evaluation.

RAN1 #122 (Aug, 2025)

Agreement

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

Max Channel bandwidth

RAN1 #123 (Nov, 2025)

Agreement

- RAN1 assumes maximum channel bandwidth 800MHz or 400MHz at network side for FR2-1

- 800MHz or 400MHz, to be down-selected in the future

- FFS: 800MHz or 400MHz at UE side.

Agreement

For how to enable UE to support 400MHz bandwidth when a network supports 400 MHz Channel Bandwidth (CBW), the following options 1/2/3/4/5 are considered from RAN1 understanding for studying

  - Option 5: Variance of Option 3 by assuming single FFT and 2 RF chain.

  - FFS which aspects of the BB processor in option 3 and 4 should be separated/parallelled.

  - Note: DL and UL design options may be considered independently.

  - To provide potential specification impact of each option.

  - To provide investigations on performance/energy efficiency/cost/complexity for the above options.

  - Inform RAN4 about the above information.

Agreement

Final LS R1-2509578 is endorsed.

RAN1 #122bis (Oct, 2025)

Agreement

RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz

- Study whether and how to enable UE to support 400MHz bandwidth

RAN1 #122 (Aug, 2025)

Agreement

Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

- Opt1: 3MHz

- Opt2: 5MHz

- Opt3: 10MHz

- Opt4: 20MHz

- FFS: the UL bandwidth may be different to the DL bandwidth.

- FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

- FFS: whether RF and BB UE BW are same or different.

- Note: RAN4 involvement is necessary.

Min Channel bandwidth

RAN1 #123 (Nov, 2025)

Agreement

If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR,

- Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable

- Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations

RAN1 #122bis (Oct, 2025)

No further agreements.

RAN1 #122 (Aug, 2025)

Agreement

For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

- Note: RAN4 involvement is necessary.

Numerology

RAN1 #122bis (Oct, 2025)

Agreement

6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band.

- FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1.

- Note: ISAC is separately discussed in ISAC session.

RAN1 #122 (Aug, 2025)

Agreement

- 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH.

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD, 30kHz SCS for TDD

    - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

    - FFS: 7.5kHz SCS for sub1GHz (FDD)

    - Whether to discuss the FFS will be subject to RANP decision.

  - For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

    - Whether to discuss it will be subject to RANP decision

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

- FFS subcarrier spacing for PRACH and up to initial access discussion.

Conclusion

Numerologies for sensing is up to sensing agenda discussion.

Spectrum utilization

No contributions before RAN1#124

RAN1 #122bis (Oct, 2025)

No Further agreements.

RAN1 #122 (Aug, 2025)

Agreement

Study and identify the lessons learned from NR spectrum utilization and aggregation framework.

- DC is subject to RANP decision in June 2026.

- Note: MRSS aspects are separate discussion.

Sync raster

No contributions before RAN1#124. Including synchronization signal and raster, broadcast signals/channel and physical random access channel, etc.

RAN1 #122bis (Oct, 2025)

No further agreements.

RAN1 #122 (Aug, 2025)

Agreement

Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

Device types

RAN1 #123 (Nov, 2025)

RAN1#123 (November’2025) made the following agreements:

Agreement

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - Overall device complexity

  - Overall system performance impact

  - Energy efficiency for both BS and UE

  - Aim at a single common signals/channels design in idle mode and initial access for diverse device types, as well as meeting mobile broadband service requirements as high priority

Agreement

If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR,

- Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable

- Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations

Agreement

- For scalable 6GR design for diverse device types, RAN1 can at least consider the following, targeting applicable to all 6G device types,

  - Basic initial access procedures from RAN1 perspective

  - Other PHY features after initial access procedure, e.g., Other DL/UL control, scheduling/HARQ

  - Coverage features to meet the identified coverage target

  - Energy saving both at BS and UE sides

  - MRSS

  - Note: whether these features are supported, mandatory or optional is separate discussion

Agreement

- For the RAN1 study of “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”,

  - The link budget template candidates 1 and 2 are used to calculate the metric(s) as starting point to compare existing 5G mid-band and 6G deployments in at least around 7 GHz, with potential future update.

  - During initial access/random access

    - Coverage target is referring the bottleneck channel (i.e. Rel-15 NR Msg3) during initial access/random access for existing 5G mid-band

  - FFS target value(s) of data rate for data channels relative to 5G mid-band

  - Following deployment scenarios are considered

    - Urban macro (both O2I and outdoor)

    - Sub-urban macro (both O2I and outdoor)

  - Following carrier frequencies are considered to calculate the metric(s)

    - [4 GHz] as the existing 5G mid-band

    - 7 GHz as 6G deployment

  - Template in R1-2509615 is to be used for collecting inputs on the values from companies.

RAN1 #122bis (Oct, 2025)

No further agreements.

RAN1 #122 (Aug, 2025)

Agreement

Study a scalable 6GR design for diverse device types, considering aspects:

- What should be commonly applicable to all 6G device types

- FFS: add-on features dedicated to specific device types, if any

Agreement

Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN.