3GPP TSG-RAN WG4 Meeting #118	R4- 2601092

# Gothenburg, Sweden, Feb. 09-13, 2026

Source:	LG Electronics

Title:	Discussion of RAN4 impacts for measurement event prediction

Agenda item:	7.12.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s moderator summary[2] and WF [1].

Discussion

- Sub-topic 3-1: Core requirements for indirect measurement event prediction

### Issue 3-1-1: Core requirements for indirect measurement event prediction

Way forward: - Further discuss the following issues based on the proposals in [1]: - Issue 1: Whether the core requirements defined for RRM measurement prediction can be reused for indirect measurement event prediction. - Issue 2: The specific core requirements defined for indirect measurement event prediction. - Issue 3: The impact of indirect event prediction on other RRM requirements (e.g., HO/CHO).

RRM measurement prediction function is used for indirect measurement event prediction operation. When we decide the core requiements for RRM measurement prediction, we can consider the use cases of the RRM measurement prediction including indirect measurement event prediction case. So, we think the core requirements for RRM measurement prediction can be reused for indirect measurement event prediction.

Proposal 1: The core requirements defined for RRM measurement prediction can be reused for indirect measurement event prediction.

Regarding the issue 2, we think specific core requirements for indirect measurement event prediction are necessary. We have similar view with Huawei’s proposal. We would like to further discuss about event prediction delay which is time distance between measurement prediction reporting time and predicted measurement event time.

Proposal 2: Specific core requirements are necessary for indirect measurement event prediction. RAN4 to discuss about event prediction delay which is time distance between measurement prediction reporting time and predicted measurement event time.

With respect to Issue 3, excluding the CHO case, when handover is performed by receiving a HO command from the network, the starting point of the handover delay is considered to be the time at which the HO command is received. Based on the discussions in RAN2 so far, although there are multiple possible options for when the HO command is received, they do not appear to affect the time between from receiving the HO command to complete the HO. In addition, even in the CHO case, while there may be a Tmeasure component that includes measurement and some prediction time, it is expected to represent the same overall timing. Therefore, we believe that the indirect event prediction does not impact to the RRM core requirements.

Proposal 3: There is no impact of indirect event prediction on the handover delay requirements.

- Sub-topic 3-2: Performance requirements for indirect measurement event prediction

### Issue 3-2-1: Performance metrics for indirect measurement event prediction

Further discuss based on the open issues in [1] and the following way forward: - Discuss whether the following performance metrics need to be defined and align the understanding on the definitions: - Option 1: Required window length under certain precision/accuracy - Option 1-1: define ETD of event prediction as a requirement, under the conditions of certain fixed precision and recall or F1 score. - Option 1-2: define ETD of event prediction as a requirement, under the condition of certain event prediction accuracy (e.g., true event prediction or false event prediction). - Option 2: Required precision/accuracy under certain window length - Option 2-1: define precision and recall or F1 score of event prediction as a requirement, under the condition of certain fixed ETD. - Option 2-2: define event prediction accuracy (e.g., true event prediction or false event prediction) of event prediction as a requirement, under the condition of certain fixed ETD. - Option 2-3: Windowed precision ≥ P_min - Precision is the ratio of correctly predicted A3 events to all predicted events, counting only matches within the allowed lead-time window. - Option 3: Lead-time of event prediction - Option 3-1: Lead-time window [L_min, L_max] - Each accepted prediction must arrive at least L_min but no more than L_max before the actual A3 trigger. - Option 3-2: Event prediction latency (i.e., how far ahead of the actual ground truth event occurrence time the event is predicted): - It is defined as the time gap between the report time of the predicted event and the actual occurrence time of the ground truth event.

Regarding the performance metric, L3 RSRP accuracy requirement and F1 score can be baseline. F1 score is defined at TR38.744 as following:

F1 score=2×Precision ×Recall Precision+Recall

Where:

Precision= n3n1+n3

Recall= n3n2+n3

For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following:

- Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa;

- Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD];

- Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD].

The ETD, i.e. timing difference between ground-truth event and predicted event, is illustrated in Figure 5.3.1-1:

We believe it is more reasonable to evaluate accuracy metrics within a limited window length. If accuracy is fixed first, determining the window length required to meet that condition could be very complex and make testing overly complicated. Therefore, it seems more practical to first set a fixed window length and then measure accuracy based on the F1 score definition for performance evaluation. So, we support option 2-1.

Proposal 4: Our preference is to first set a fixed window length and then measure accuracy based on the F1 score definition for performance evaluation. So, we support option 2-1.

### Issue 3-2-2: Predicted RSRP accuracy requirements

- Proposal 1 (Nokia): - RAN4 to consider following performance metrics for indirect measurement event prediction. - Event Prediction Reporting RSRP Accuracy: Refers to the accuracy of the reported relative /absolute predicted RSRP values associated with the event, compared to ground truth value. - Proposal 1a (OPPO, CMCC, ZTE, Samsung, LGE): - The absolute and/or relative accuracy requirements defined for measurement prediction can be also reused for the predicted L3-RSRP in indirect measurement event prediction. - Proposal 1b (Apple, Nokia): - RAN4 shall assume similar RSRP accuracy requirements as those applied to predicted measurement RSRPs. The applicable accuracy levels may be further refined considering key factors such as observation-window length, prediction-window duration, and other configuration parameters. - Proposal 2-1 (MTK): - Prefer not to define a set of requirements for different TTT values. Further discuss which TTT value to define requirements for. - For measurement event prediction, for the purpose of defining requirements, the assumed UE behaviour is that UE starts to predict when enter condition of the event is met. - Proposal 2-2: (Xiaomi) - RAN4 to define requirement for RRM results in predicted event triggered reporting. - The TTT configuration needs to be explicitly considered in defining requirements for event prediction. - Proposal 2-3 (LGE): - the TTT configuration does not need to be considered as the side condition. - Proposal 3: (QC) - The performance metrics for AI/ML-based measurement event prediction include, but are not necessarily limited to, the following: - L3-RSRP accuracy: - RAN4 to further clarify “In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP” agreed upon at RAN4#114 as follows: - For the absolute and/or relative accuracy of the predicted L3-RSRP, if the predicted event report includes predicted L3-RSRPs, the corresponding ground truth should be taken from the predicted event occurrence time, not the actual ground truth event occurrence time. - Recommended WF - Agree to define the accuracy requirements for predicted L3-RSRP for indirect measurement event prediction. - Further discuss whether the Predicted L3-RSRP accuracy requirements defined for RRM measurement prediction can be reused. - Further discuss the clarification on the time point of ground truth. - Discuss whether TTT configuration should be the side condition of the Predicted L3-RSRP accuracy requirements for indirect measurement event prediction.

We believe that the Predicted L3-RSRP accuracy requirements defined for RRM measurement prediction can be reused for indirect measurement event prediction. However, we consider that TTT configuration is largely unrelated to the Predicted L3-RSRP accuracy requirement. So, the TTT configuration does not need to be considered as the side condition of the Predicted L3-RSRP accuracy requirements for indirect measurement event prediction.

Proposal 5: The Predicted L3-RSRP accuracy requirements defined for RRM measurement prediction can be reused for indirect measurement event prediction. However, the TTT configuration does not need to be considered as the side condition.

- Conclusion

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s moderator summary [2] and WF [1], and we propose

Proposal 1: The core requirements defined for RRM measurement prediction can be reused for indirect measurement event prediction.

Proposal 2: Specific core requirements are necessary for indirect measurement event prediction. RAN4 to discuss about event prediction delay which is time distance between measurement prediction reporting time and predicted measurement event time.

Proposal 3: There is no impact of indirect event prediction on the handover delay requirements.

Proposal 4: Our preference is to first set a fixed window length and then measure accuracy based on the F1 score definition for performance evaluation. So, we support option 2-1.

Proposal 5: The Predicted L3-RSRP accuracy requirements defined for RRM measurement prediction can be reused for indirect measurement event prediction. However, the TTT configuration does not need to be considered as the side condition.

Reference

[1] R4-2522668, “WF on NR_AIML_Mob_RRM”, Moderator(OPPO)

[2] R4-2520846, “Topic summary for [117][221] NR_AIML_Mob_RRM”, Moderator(OPPO)