3GPP TSG-RAN WG4 Meeting #118				                           R4-2601131

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda Item:	8.3.1

Source: 	LG Electronics

Title: 	Discussion on 6G Output power requirements

Document for:	Discussion

Introduction

In the RAN4#117, the WF on “[117][102] 6G general RF and UE RF” was approved. In this discussion, we share our point of view for 6G general RF and UE RF based on the approved WF [1].

Discussion

Single carrier output power requirements

Topic #1: Single carrier output power requirements - Agreements for power class - Further study step size between power classes. Encourage further inputs considering e.g. - Benefits/drawbacks of potential finer granularity between power classes - Granularity for MPR/A-MPR requirement definition - Output power tolerance - REFSENS requirement impact - Potential regulatory restrictions - Flexibility for implementation - Further study power class for multi-TX case (TxD, UL-MIMO) - Consider benefits and drawbacks for - Defining power class to be agnostic of number of Tx chains and/or PAs - Considering that each chain and/or PA should be able to reach maximum output power - Consider impact of transparent vs. non-transparent TxD - Further study how to enable fully utilizing PA capabilities considering e.g. - Relaxation or removal of upper tolerance without sacrificing output power accuracy - Power boosting - Maximum power capability reporting - Removing the nominal power limit in the PCMAX upper bound - Further study need for default power class and default power class values considering e.g. - Network UL coverage / UL link budget at different frequencies/ System performance - Impact on REFSENS definition for FDD bands - Maximum channel bandwidth - Cell planning from the operators - Potential regulatory output power restrictions - Agreements for configured maximum output power - Further study SAR solution considering - P-MPR as baseline below 6 GHz - Whether/how SAR or MPE applies at higher than 6GHz and applicable solutions - Achieving more clarity on UE output power - Further study potential UL duty cycle requirement to optimize component size/cost considering - Use cases and deployment scenarios as well as network needs - Further review the 5G configured Tx power equation parameters considering at least the following - ∆TC,c : can it be removed, considering also new wider CBW and FFS on whether and where to accommodate this relaxation in requirements. - ∆TRxSRS: should it be separated from general Pcmax equation as it only addresses SRS transmission and FFS on how it can be accommodated. - ∆Tib: can the requirement be removed and accommodated to per band power capability and FSS on whether and where to accommodate this relaxation in requirements. - ΔPPowerClass: Is it still needed in some cases even if not associated with SAR

Power class framework

The current framework defines a limited set of distinct power tiers (e.g, PC3, PC2, PC1.5 ..). These classes have historically been designed with a step size of approximately 3 dB, corresponding to nominal output power such as 23 dBm, 26 dBm, and 29 dBm, respectively.

In the 6 G UE RF discussions, a key issue has been identified regarding the granularity of UE power classes. Specifically, it is debated whether to maintain the legacy interval of approximately 3 dB or to introduce a finer granularity, such as 1 dB steps.

With the anticipated diversification of 6G UE PA and RF front-end implementations, it is observed that the actual maximum output power of a UE may fall between the existing power class boundaries (e.g, between 23 dBm and 26 dBm). Under the legacy mechanism, such UEs are unable to declare the higher power class. Consequently, concerns are raised that the available PA headroom is effectively lost or unutilized from the perspective of the standard and network optimization.

Observation 1: Due to diverse RF implementations in 6G, a UE's actual capability may lie between defined classes, leading to unutilized PA headroom if the UE is forced to declare the lower class.

Traditional ~3 dB Step Framework

The legacy framework, characterized by power classes separated by approximately 3 dB, presents the following technical merits and drawbacks:

Pros:

- Simplicity and Proven Operability: The 3 dB step structure consists of a limited number of power tiers. This simplicity facilitates the definition of RF requirements (e.g., MPR, A-MPR, ACLR) and conformance testing procedures, thereby minimizing the fragmentation of requirements and test cases

- Clear Class Separation: A step size of approximately 3 dB ensures distinct separation between classes. This structural margin mitigates the issue of overlapping grades, particularly when measurement tolerances are taken into account.

- Alignment with Legacy Ecosystem: The 3 dB step is not merely a numerical interval but represents a historical and industrial framework for classifying UE Tx performance. Established testing and verification procedures (including test setups and tier-specific requirements) are built around this structure. Drastic changes in 6G could necessitate significant re-alignment of the testing, certification, and verification ecosystem, extending beyond simple specification updates.

Cons:

- Suboptimal PA utilization: If a UE is capable of stable transmission at an intermediate power level (e.g., 24- 25 dBm) but falls short of the next class threshold (e.g., 26 dBm), it is forced to declare the lower class (3.g., 23 dBm). From the network perspective, this results in the UE being recognized as a lower-tier device, limiting potential improvements in UL coverage and performance.

Observation 2: Traditional power class framework is beneficial for maintaining simplicity in specifications and conformance testing, avoiding the fragmentation of requirements.

Finer Step Framework (e.g., 1 dB)

The introduction of a finer granularity, such as 1 dB steps, is discussed with h the following considerations:

Pros:

- Maximization of PA capability: A finer step size allows the UE to report its Maximum Output Power (MOP) with greater precision. This reduces the “rounding down “effect caused by rigid class boundaries and enable the network to utilize the UE’s power capability more accurately. This aligns with the objective of enabling accurate reporting of UE power capabilities.

Cons:

- Increased Complexity and Fragmentation: If 1 dB steps are treated as distinct power classes, there is a concern regarding the explosion of requirements. Specific requirement such as MPR and A-MPR may need to be defined for each step, leading to increased specification complexity and a larger volume of conformance test cases.

- Overlap Issues with Tolerance: As the step size decreases (e.g., to 1 dB), the interval becomes comparable to the measurement tolerance. This can lead to situations where adjacent steps overlap within the tolerance range, blurring the distinction between classes and complicating standard operation.

Observation 3: Fine granularity power class framework offers the potential to maximize UE PA utilization but may introduce additional complexity in requirement organization and conformance testing, which requires careful assessment.

Summary on power class framework

At this stage, it may be premature to mandate detailed technical solutions that re-define the power class structure. Instead, we propose a pragmatic direction: maintain the legacy ~3 dB-step power class framework while strengthening UE reporting procedures. Retaining the ~3 dB-step framework enables maximum reuse of well-established 4G/5G RF requirements and conformance test structures, thereby minimizing specification fragmentation and ecosystem disruption. Meanwhile, the previously observed issue of under-utilization of UE transmit capability (e.g.., conservative classification near class boundaries) can be addressed more effectively by enhanced reporting, allowing the network to recognize and exploit UE output capability more accurately without introducing new power classes.

More accurate visibility of UE effective maximum output power may improve UL link budget estimation, scheduling margin setting, and cell-edge performance optimization without modifying the fundamental power class structure.

Proposal 1: For single Tx case, keep the legacy ~3 dB-step power classes and address UE power under-utilization through strengthened reporting, avoiding any new finer-granularity classes.

Multi-Tx Cases: TxD and UL-MIMO

While Section 2.1.1 discussed the power class framework mainly from a 1Tx baseline perspective, 6G UEs are expected to increasingly adopt multi-Tx architecture (multiple Tx chains and/or PAs), including TxD and UL-MIMO. In such cases, the relationship boundaries become more complex, especially when Tx chains are asymmetric.

A key consideration is whether the existing power class framework can remain unchanged (to maximize RF requirement and test reuse), while enabling enhanced reporting to express the effective maximum output power more accurately for multi-Tx implementations.

Transparent TxD

For TxD, the maximum output power is naturally characterized by the sum of the maximum output power from multiple UE Tx paths/connectors. This “summation” concept is already reflected in existing RF requirement framing for TxD, where the maximum output power is defined as the sum across UE antenna connectors.

Therefore, for transparent TxD, the UE’s effective maximum output power can be expressed primarily from a total/sum-power perspective, independent of the number of Tx chains and/or Pas.

Observation 4: TxD is well aligned with a sum-power definition of maximum output power, and thus the key issue is not redefining power classes, but enabling reporting that represents effective total Pmax more accurately when it falls between legacy class boundaries.

UL-MIMO

Unlike transparent TxD, UL-MIMO is non-transparent to the network in the sense that multiple antenna ports and transmission modes can be explicitly configured and scheduled. In practice, the UE’s ability to realize full or near-full power in UL-MIMO may depend on UE optional modes/capabilities such as ULFPTx and the active transmission configuration.

At this stage, in line with Proposal 1, it is recommended to avoid redefining the power class structure. Instead, the same reporting overlay (ΔPmax, sum‑based) can be introduced as a baseline capability indication, while further study is needed on whether optional dynamic supplementation (e.g., MAC CE/PHR-related enhancements) is beneficial for UL‑MIMO cases with mode-dependent effective Pmax.

Observation 5: While a sum-based effective Pmax representation is still usefull for UL-MIMO, additional mode-dependent constraints may exist. Therefore, a single static metric may not fully capture all operational constraints for UL-MIMO across configurations.

From a system perspective, such configuration-dependent effective Pmax may influence scheduling strategies and link adaptation assumptions for multi-port UL operation.

Transparent vs Non‑transparent TxD

Transparent TxD:

- From an RF requirements point of view, the summed maximum output power concept provides a straightforward basis. Reporting enhancements can primarily focus on representing effective total Pmax near class boundaries.

Non-Transparent multi-port operation:

- While the total/sum effective Pmax remains relevant, mode-dependent constraints (e.g., ULFPTx behavior) may require additional considerations. However, such mechanisms should be handled as study topics to avoid immediate specification fragmentation.

Summary for multi-Tx cases

Consistent with the conclusion of Section 2.1.1, it remains premature to redefine the power class structure (3.g., 1 dB classes) given the resk of requirement/test fragmentation. Instead, a pragmatic direction is to retain legacy ~3 dB power classes and introduce a reporting overlay for effective maximum output power in multi-Tx UEs.

Proposal 2: For TxD cases, maintain the legacy ~3 dB power class framework and introduce a sum-based effective maximum output power offset Pmax (0–3 dB) as a UE capability indication. For UL-MIMO cases where mode-dependent constraints exist, study whether optional dynamic reporting supplementation is beneficial.

Further study on enabling full utilization of PA capabilities

This section discusses potential directions to enable better utilization of UE PA capability, in the context of configured transmitted power bounds in 38.101-1 clause 6.2.4. The intention is to identify approaches that improve practical UL performance while minimizing disruption to the legacy power class and RF conformance framework.

Relaxation of “upper tolerance” in PCMAX bounds

Observation 6: “Tolerance” is fundamentally an accuracy concept (i.e., how closely the UE can set/track the intended transmit power), not a mechanism to increase allowable maximum output power. Therefore, using tolerance relaxation as a lever to “boost” maximum power is conceptually misaligned and risks conflating two different intents: accuracy vs. maximum power limits.

Power boosting

Observation 7: Power boosting is already defined with conditional applicability in legacy system (LTE/NR), and it is tied to specific conditions such as modulation/waveform and other operating constraints. As a result, the benefit is scenario-dependent and may be limited in terms of general applicability for “fully utilizing PA capability.”.

Maximum power capability reporting

Observation 8: As discussed in Section 2.1.1 and 2.1.2, retaining the legacy ~3 dB power class framework is beneficial for specification/test reuse, while under-utilization near class boundaries can be mitigated by strengthened reporting. Therefore, explicit maximum power capability reporting (e.g., sum-based effective PCMAX indication) is a pragmatic and controlled approach to enable better utilization without redefining power class granularity.

Removing the nominal power limit in PCMAX upper bound

In the configured transmitted power framework (38.101-1 clause 6.2.4), the configured maximum output power PCMAX,f,c is bounded by PCMAX_L,f,c and PCMAX_H,f,c. The upper bound PCMAX_H,f,c is defined as the minimum of a network-related limit (PEMAX,c) and a UE nominal power-class-related limit (PPowerClass term, with applicable offsets such as ΔPpowerClass and potential powerBoost terms). This structure provides a stable linkage between the declared UE power class and the maximum transmit power used by power control.

Modern UE RF architectures often reuse a common PA core across multiple bands, while each band (and even different channel locations within a band) can experience different front-end/filter insertion losses. In such cases, for a fixed PA output operating point, the antenna-connector power could vary across bands/channels. This motivates a discussion on whether the nominal power-class-based cap in PCMAX_H could be relaxed so that the UE can exploit lower insertion loss conditions to radiate higher power on some bands/channels.

Observation 9: Power class is intended to provide a consistent nominal reference for maximum output power in RF requirements and within the PCMAX bounding framework. If higher transmit power is allowed on selected bands or channel locations without corresponding capability visibility, this could introduce implicit intermediate operating points under the same declared power class. Such situations may warrant further study regarding how the integrity and interpretability of the power-class concept can be maintained.

Observation 10: Allowing higher transmit power under the same declared power class may introduce additional operating conditions (e.g., band- or channel-dependent higher power levels) that could require further assessment of RF requirement assumptions, such as emissions, modulation quality, and related power reduction mechanisms. The impact on requirement organization and conformance evaluation would benefit from careful study.

Observation 11: If the UE-side nominal power-class-based cap is relaxed and greater reliance is placed on network-configured limits (e.g., PEMAX/p-Max), this may shift more responsibility to network configuration policies for maintaining predictability and interference management. The implications for operational consistency and for assumptions based on stable PCMAX behavior could be further studied.

Summary on enabling full utilization of PA capabilities

Based on the discussion in Section 2.1.3.1-2.1.3.4, it is observed that approaches directly relaxing the PCMAX bounds (e.g., by interpreting “tolerance” as additional usable headroom) are conceptually misaligned with the intent of accuracy requirements, and may inadvertently blur the distinction between output power accuracy and maximum allowable output power. In addition, while power boosting mechanisms exist, their applicability is conditional and therefore limited as a general-purpose solution for fully utilizing PA capability.

In contrast, capability-based approaches that preserve the legacy power class and configured transmitted power framework (including the PCMAX bounding mechanism in 38.101-1) provide a more controlled way to improve practical UL utilization without increasing specification fragmentation. In particular, strengthened reporting of effective maximum power capability can address under-utilization near class boundaries, while maintaining a stable and predictable power-class-based maximum output power framework.

Proposal 3: Keep the legacy power class framework and PCMAX bounding structure, and address PA capability under-utilization through strengthened reporting of effective maximum output power. Structural changes such as introducing finer power classes or modifying PCMAX bounds should be studied carefully considering their impact on RF requirement organization and conformance testing.

Conclusion

Observation 1: Due to diverse RF implementations in 6G, a UE's actual capability may lie between defined classes, leading to unutilized PA headroom if the UE is forced to declare the lower class.

Observation 2: Traditional power class framework is beneficial for maintaining simplicity in specifications and conformance testing, avoiding the fragmentation of requirements.

Observation 3: Fine granularity power class framework offers the potential to maximize UE PA utilization but introduces significant challengers regarding requirement complexity (MPR/A-MPR definition) and class distinctness due to tolerance overlap.

Observation 4: TxD is well aligned with a sum-power definition of maximum output power, and thus the key issue is not redefining power classes, but enabling reporting that represents effective total Pmax more accurately when it falls between legacy class boundaries.

Observation 5: While a sum-based effective Pmax representation is still usefull for UL-MIMO, additional mode-dependent constraints may exist. Therefore, a single static metric may not fully capture all operational constraints for UL-MIMO across configurations.

Observation 6: “Tolerance” is fundamentally an accuracy concept (i.e., how closely the UE can set/track the intended transmit power), not a mechanism to increase allowable maximum output power. Therefore, using tolerance relaxation as a lever to “boost” maximum power is conceptually misaligned and risks conflating two different intents: accuracy vs. maximum power limits.

Observation 7: Power boosting is already defined with conditional applicability in legacy system (LTE/NR), and it is tied to specific conditions such as modulation/waveform and other operating constraints. As a result, the benefit is scenario-dependent and may be limited in terms of general applicability for “fully utilizing PA capability.”.

Observation 8: As discussed in Section 2.1.1 and 2.1.2, retaining the legacy ~3 dB power class framework is beneficial for specification/test reuse, while under-utilization near class boundaries can be mitigated by strengthened reporting. Therefore, explicit maximum power capability reporting (e.g., sum-based effective PCMAX indication) is a pragmatic and controlled approach to enable better utilization without redefining power class granularity.

Observation 9: Power class is intended to provide a consistent and predictable nominal maximum output power reference for RF requirements and for the PCMAX bounding mechanism. If the UE can exceed the nominal class limit opportunistically on selected bands/channels without explicit capability declaration, this can be interpreted as implicit intermediate power tiers under the same declared power class, potentially undermining the integrity of the power-class concept.

Observation 10: Allowing higher transmit power under the same declared power class introduces additional higher-power operating points (band-/channel-dependent) that may require careful assessment of RF requirements and conformance assumptions (e.g., emissions, modulation quality, and related power reduction mechanisms). These risks increasing specification complexity and test burden, which conflicts with the objective of maximizing reuse of the legacy power class and conformance framework.

Observation 11: Even if a network-configured limit (e.g., via PEMAX/p-Max) exists, removing the UE-side nominal class cap shifts more responsibility to network configuration policy to maintain predictability and interference management. This can increase operational dependency and variability across deployments, potentially affecting network-side assumptions that rely on stable PCMAX behavior.

Proposal 1: For single Tx case, keep the legacy ~3 dB-step power classes and address UE power under-utilization through strengthened reporting, avoiding any new finer-granularity classes.

Proposal 2: For TxD cases, maintain the legacy ~3 dB power class framework and introduce a sum-based effective maximum output power offset Pmax (0–3 dB) as a UE capability indication. For UL-MIMO cases where mode-dependent constraints exist, study whether optional dynamic reporting supplementation is beneficial.

Proposal 3: Maintain the legacy power class framework and address PA under-utilization via strengthened reporting of effective maximum power capability, rather than introducing new finer-granularity power classes or relaxing PCMAX bounds.

- References

# [1] R4-2522451, WF on [117][102] 6G general RF and UE RF (QualComm)