3GPP TSG RAN WG4 Meeting #118	R4-2601364

Gothenburg, Sweden, Feb. 09-13, 2026

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements AI beam management

Agenda item:	6.11.2

Document for:	Approval

Introduction

RRM core requirements for AI beam management are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. In this paper we will provide our initial views on RRM performance requirements on AI beam management.

Discussion

In previous meetings, we have spent lots of time on discussing core requirements on AI beam management. To our understanding, the core requirements are quite stable so far and the performance requirements can be discussed in this meeting.

During core part discussion, RAN4 mainly discussed three parts as following:

- Beam ID prediction accuracy;

- RSRP prediction accuracy;

- Prediction reporting delay.

RAN4 shall consider related accuracy requirements and test cases regarding aforementioned three parts.

Regarding RSRP prediction accuracy, it contains absolute RSRP accuracy and relative RSRP accuracy. For absolute RSRP accuracy, the existing absolute RSRP accuracy requirements for FR2 shall be the benchmark or baseline. For relative RSRP accuracy, the existing relative RSRP accuracy requirements for FR2 shall be the baseline. However, the reason for studying AI beam management is the overhead reduction such as measurement RS reduction with similar accuracy requirements as legacy, or has some performance gain such as accuracy compared to legacy accuracy requirements. Consequently, we shall consider the final accuracy requirements comprehensively. That means the legacy accuracy requirements are reused with RS overhead reduction (Set B < Set A). However, the benchmark shall be legacy accuracy requirements with Set B equals to Set A beams. Consequently, two kinds of accuracy shall be considered for RSRP prediction accuracy:

- Benchmark: Set B equals to Set A, legacy accuracy requirements shall be at least maintained.

- Verification: Set B beam is smaller than Set A beams, legacy accuracy requirements shall be at least maintained.

- Proposal 1: For verification of RSRP prediction accuracy, the following shall be considered:

- Benchmark: Set B equals to Set A, legacy accuracy requirements shall be at least maintained.

- Verification: Set B beam is smaller than Set A beams, legacy accuracy requirements shall be at least maintained.

- Regarding beam ID prediction accuracy, RAN4 does not have accuracy requirements to verify the successful rate. This kind of accuracy requirements based on the values of K, N and x. We shall wait for the discussion in core part and then discuss how to define this kind of accuracy requirements.

- Observation 1: For beam ID prediction accuracy requirements, it highly depends on the values of K, N and x.

- Proposal 2: RAN4 shall wait for the discussion in core part and then discuss how to define the accuracy requirements for beam ID prediction accuracy.

- Regarding prediction reporting delay, it includes measurement delay, inference delay and reporting delay, where RAN1 agreed that the inference delay shall be contained in reporting delay. RAN4 shall verify whether UE could predict the Tx beam and report results to network within defined prediction reporting delay.

- Proposal 3: RAN4 shall verify whether UE could predict the Tx beam and report results to network within defined prediction reporting delay.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR NTN phase 3:

- Proposal 1: For verification of RSRP prediction accuracy, the following shall be considered:

- Benchmark: Set B equals to Set A, legacy accuracy requirements shall be at least maintained.

- Verification: Set B beam is smaller than Set A beams, legacy accuracy requirements shall be at least maintained.

- Observation 1: For beam ID prediction accuracy requirements, it highly depends on the values of K, N and x.

- Proposal 2: RAN4 shall wait for the discussion in core part and then discuss how to define the accuracy requirements for beam ID prediction accuracy.

- Proposal 3: RAN4 shall verify whether UE could predict the Tx beam and report results to network within defined prediction reporting delay.

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133 Clause A.14.