3GPP TSG-RAN WG4 Meeting #118		   				 		   						    R4-2601841

Göteborg, Sweeden, 9th – 13th of February, 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0119 | rev | 1 | Current version: | 18.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance CR on frequency error requirement for TS 36.102 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nordic Semiconductor |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIoT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Reason for change are clarified in R4-2600053 Observation-1: Text “When a repetition period is configured on the uplink for which repetition period (R) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]” is hard to understand. - “repetition period is configured” is unclear, in NB-IoT repetition period is dynamically indicated. - “ongoing repetition period” is unclear, is it period of all four repetitions or periodic of each of four repetitions. - “change Doppler pre-compensation” is unclear, does it mean UE is not allowed to adjust UL frequency at all, or UE may adjust UL frequency if it maintains UL phase continuity. - above restriction is lacking any logic - when NRep=R=1 and 10 resource units are scheduled, NPUSCH is 80ms long, UE may change Doppler pre-compensation during an ongoing repetition period. - When NRep=R=4 and 2 resource units are scheduled, NPUSCH is 64ms long, UE may NOT change Doppler pre-compensation during an ongoing repetition period. Proposal-1: RAN4 to clarify meaning of the terms in Observation 1 Obsevation-2: If rate of frequency change in UL is small, e.g. less than 1 or 2 Hz, the phase continuity during segment is preserved. Proposal-2: UE should be allowed to perform up to [2] Hz continuous adjustments with rate of [2] ms or lower during a segment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove an unclear part of requirement. Allow UE to change Doppler precompensation during one segment, if UE performs updates with rate of 2ms or lower. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how UE shall perform Doppler precomensations. Sub-optimal performance in the field. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------<<< Start Change 1 >>>----------------------------------

## 6.4B	Transmit signal quality for category NB1 and NB2

### 6.4B.1	Frequency error for UE category NB1 and NB2

For UE category NB1 and NB2, the UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331 [6]), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 [8] clause 23.21.2.2).

The UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4B.1-1, observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the ideally pre-compensated reference uplink carrier frequency.

When a repetition period is configured on the uplink for which repetition period (R ) >1,The UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or if UE changes Doppler pre-compensation with rate smaller or equal to once a 2ms. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift corresponding to the estimated Doppler frequency at the beginning of the transmission.]

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

------------------------------<<< End Change 1 >>>-------------------------