3GPP TSG-RAN WG4 Meeting #118	R4-2502007

# Gothenburg, Sweden, 9th – 13th February 2026

Agenda item:		10.2.3

Source: 		Qualcomm Inc.

Title:	 	PC1 A-MPR inconsistencies

Document for:		Approval

## 1	Introduction

During Rel-19 and earlier releases PC1 power class has been introduced to multiple operating bands. Some of these bands have additional emission requirements, and A-MPR is allowed to enable UEs to meet those requirements. In review of those requirements, some inconsistencies needing correction were found. This document discusses those inconsistencies and invites companies to provide analysis on the A-MPR.

## 2	Discussion

PC1 requirements for n7 and n41 were introduced in release 18. Both of these bands have additional emissions requirements for which A-MPR is specified.

n7 NS_46

NS_46 A-MPR for PC1 has inconsistencies at least for A-MPR values between DFT-s-OFDM and CP-OFDM, as well as in some cases allowed A-MPR is too low and e.g. clearly reduced compared to PC2. As an example, 50 MHz CBW is examined.

Table 1: NS_46 PC2 A-MPR regions for 50 MHz CBW

| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥ 0, < 9 | > 0 | A4 |
| --- | --- | --- | --- | --- |
|  |  | ≥ 9, < 21.6 | > max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥ 21.6, < 31.5 | > 18 | A6 |
|  |  | ≥ 31.5, < 39.6 | > 16.2 | A7 |
|  |  | ≥ 39.6 | > 0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 16.2 | A10 |

Table 2: A-MPR for NS_46 (power class 2)

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 7 | 11 | 6.5 | 3.0 | 5.0 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 7 | 11 | 6.5 | 3.0 | 5.0 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 7 | 11 | 6.5 | 3.5 | 5.0 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 7 | 11 | 6.5 | 3.5 | 5.0 |
|  | 256 QAM | 6.5 |  |  | 7 | 11 | 6.5 |  | 5.0 |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 8 | 12 | 8 | 4.5 | 6.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 8 | 12 | 8 | 4.5 | 6.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 8 | 12 | 8 | 4.5 | 6.5 |
|  | 256 QAM | 6.5 |  |  | 8 | 12 | 8 |  |  |

Table 3: NS_46 PC1 A-MPR regions for 50 MHz CBW

| 50 MHz | Fc > 2535 | ≤ 19 | ≥ 1.17*12*SCS*RBend – 10 | A7 |
| --- | --- | --- | --- | --- |
|  |  | > 19, ≤ 29.5 | ≥ 1.17*12*SCS*RBend – 10 | A11 |
|  |  | > 29.5 | > 12.8, ≥ -1.15*12*SCS*RBend + 60 | A9 |
|  |  | > 41 | > 7.2, ≤ 12.8 | A12 |
|  |  | ≥ 33 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 33 | ≤ 2.2 | A9 |
|  | 2525 < Fc ≤ 2535 | ≤ 14 | ≥ 1.17*12*SCS*RBend – 6 | A7 |
|  |  | > 14, ≤ 26 | ≥ 1.17*12*SCS*RBend – 6 | A11 |
|  |  | ≥ 34 | > 22.5, ≥ -1.25*12*SCS*RBend + 77.5 | A8 |
|  |  | ≥ 43 | > 7.2, ≤ 22.5 | A12 |
|  |  | ≥ 35 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 35 | ≤ 2.2 | A9 |
|  | Fc = 2525 | ≥ 47 | ≥ -1*12*SCS*RBend + 94 | A13 |
|  |  | ≤ 9 | ≥ 1.14*12*SCS*RBend – 2 | A7 |
|  |  | ≥ 37.4 | > 2.2, ≤ 7.6 | A8 |
|  |  | ≥ 37.4 | ≤ 2.2 | A9 |

Table 4: NS_46 PC1 A-MPR

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 | A12 | A13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 2 |  | 2 | 1 | 1 | 3 | 9.5 | 5 | 6 | 4 |  | 4 | 2.5 |
|  | QPSK | 3 | 4 | 3 | 2 | 2 | 6 | 11 | 6 | 6 | 5 |  | 4 | 4 |
|  | 16 QAM | 3 | 6.5 | 3 |  |  | 6 | 12 | 6 | 6.5 | 5 |  | 4 | 4 |
|  | 64 QAM | 3 | 7 | 3 |  |  | 6 | 12.5 | 6 | 7 | 5 |  | 4 | 4 |
|  | 256 QAM |  | 7 |  |  |  | 6 | 12.5 | 6 | 7 | 5 |  |  |  |
| CP-OFDM | QPSK | 4 | 12.5 | 6 | 4 |  | 6.5 | 15 | 9 | 12.5 | 5.5 | 10 | 5 | 4 |
|  | 16 QAM | 4 | 12.5 | 6 | 4 |  | 6.5 | 15 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 64 QAM | 4 | 13 | 6 |  |  | 6.5 | 15 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 256 QAM |  | 13 |  |  |  |  | 15 | 9 | 13 |  | 10 |  |  |

Observation 1: NS_46 PC1 A-MPR includes regions where there is 10 dB difference between DFT-s-OFDM and CP-OFDM (yellow highlight in Table 4).

Observation 2: For narrow allocations near top side of the channel, A-MPR is up to 5 dB smaller for PC1 than for PC2, meaning that PC1 is required to transmit 10 dB higher output power (blue highlight in Tables 1 to 4).

A-MPR difference of 10 dB between DFT-s-OFDM and CP-OFDM has not been observed before, and the reason behind this is unknown. It would be useful to re-evaluate the A-MPR and confirm the validity of the numbers.

Even though PC1 requires better ACLR performance, the improved linearity is not sufficient to close the 10 dB difference in output power. Narrow allocations will cause intermodulation which requires

Proposal 1: Re-evaluate A-MPR for NS_46 PC1.

Proposal 2: Consider A7 A-MPR for regions highlighted in blue in Table 3.

n41 NS_04

NS_04 for PC1 has similar issue to NS_46 for A-MPR being excessively lower than PC2/PC1 A-MPR. Specifically, A-MPR for IMD3 is defined

Which for 1 RB allocation yields 3 dB A-MPR.

For PC2 and PC1.5 the A-MPRs are significantly higher e,g. for DFT-s-OFDM 64QAM for PC1.5 A-MPR is 8 dB, 5 dB more than for PC1. Columns PC2_A4 and PC1.5_A6 are relevant for narrowband IMD. This again results in PC1 having both higher nominal power class and significantly reduced A-MPR, which makes it extremely challenging to support PC1.

Observation 3: For narrow allocations A-MPR for PC1 NS_04 is significantly higher for PC2 and PC1.5 than for PC1, and results in cases where PC1 is required to sustain e.g. 7 dB higher output power than PC1.5 (2 dB higher nominal power, 5 dB less A-MPR). Better linearity of PC1 is not sufficient to bridge the gap.

Proposal 3: Re-evaluate A-MPR for NS_04 PC1, at least for narrow allocations.

## 3	Conclusion

In this contribution A-MPR for PC1 for NS_46 and NS_04 was discussed. Following observations and proposals were made.

Observation 1: NS_46 PC1 A-MPR includes regions where there is 10 dB difference between DFT-s-OFDM and CP-OFDM (yellow highlight in Table 4).

Observation 2: For narrow allocations near top side of the channel, A-MPR is up to 5 dB smaller for PC1 than for PC2, meaning that PC1 is required to transmit 10 dB higher output power (blue highlight in Tables 1 to 4).

Proposal 1: Re-evaluate A-MPR for NS_46 PC1.

Proposal 2: Consider A7 A-MPR for regions highlighted in blue in Table 3.

Observation 3: For narrow allocations A-MPR for PC1 NS_04 is significantly higher for PC2 and PC1.5 than for PC1, and results in cases where PC1 is required to sustain e.g. 7 dB higher output power than PC1.5 (2 dB higher nominal power, 5 dB less A-MPR). Better linearity of PC1 is not sufficient to bridge the gap.

Proposal 3: Re-evaluate A-MPR for NS_04 PC1, at least for narrow allocations.