3GPP TSG-RAN WG4 Meeting #118	R4-2602011

# Gothenburg, Sweden, 09 – 13 February 2026

Agenda item:		10.2.2

Source: 		Qualcomm Incorporated

Title:	 	PC2 FDD reference measurement channel

Document for:		Approval

## 1	Introduction

PC2 FDD was defined in Rel-17. In the resulting specification default 50% duty cycle is applied for PC2 FDD, and when duty cycle exceeds 50% delta-powerclass parameter is set to 3 dB resulting in power reduction. Testing with limited duty cycle is not enabled as reference measurement channel with less than 50% duty cycle is not specified. Therefore, PC2 FDD specification needs a correction.

## 2	Discussion

Power reduction when duty cycle exceeds 50% and no additional signaling is present is specified twice in TS 38.101-1: in UE maximum output power (6.2.1) as well as in configured maximum output power (6.2.4)

UE maximum output power:

Configured maximum output power:

Neither of these specification sections differentiates between TDD and FDD. Additionally, the signaling parameter maxUplinkDutyCycle-PC2-FR1 is specified with no differentiation for TDD and FDD in TS 38.306

Observation 1: Default duty cycle of 50% has been specified for all PC2 UEs, and applies equally for TDD and FDD

In earlier meetings there have been views that specification is not aligned with Rel-17 TR 38.861. The conclusions of the TR have been copied below, and include statements supporting claims from both camps. The conclusions say that duty cycle is considered as standardized solution, and that duty cycle used as UE implementation method is not concluded.

Of course, study item cannot draw normative conclusions and final specifications were handled in work item phase and implementations are done based on the specifications, not technical reports.

The work item outcome is aligned with the SI conclusion: duty cycle based power reduction where evaluation period is up to UE but minimum of one radio frame is fully aligned covers the duty cycle as standardized solution as well as includes the UE implementation method with the evaluation period being up to the UE.

Observation 2: Work item defines normative specifications, not study item. Implementations are based on normative specifications.

Observation 3: Work item outcome is aligned with study item conclusion, and it is recognized that captured SI conclusions leave room for to apply both duty cycle and P-MPR for SAR management for FDD UEs.

Reference measurement channel used in conformance testing are specified in Annex A. Currently, all UL RMCs for FDD use 100% duty cycle. This means that in conformance testing scenario PC2 FDD may reduce its output power to PC3 power level, and PC2 requirements are not properly verified at PC2 power level.

Observation 4: Missing PC2 FDD RMC with up to 50% duty cycle leads in unintended UE behavior in conformance testing, and PC2 requirements may not be properly verified.

Clearly, PC2 FDD requirements are slightly broken in this area, and RAN4 needs to take an action to complete the specification. The simplest approach is the add a new RMC which includes an RMC with up to 50% duty cycle. Up to 50% duty cycle ensures that UE operates at PC2 power level, as power reduction is allowed only when duty cycle exceeds 50%.

This results in slightly different behavior in testing and in the field, as in field operation UE would keep transmitting but use lower power. However, for conformance testing it is purposeful to prioritize stable power level at PC2 level and align the specified default behavior.

Observation 5: Adding a new RMC for PC2 FDD enables testing to be done aligned with PC2 FDD requirements.

Proposal 1: Specify a new RMC for PC2 FDD with up to 50% duty cycle to align with the specified default behavior.

If necessary, further updates can be done to more accurately accommodate the duty cycle capability indicated by the UE. Keeping the default duty cycle at 50% and enabling testing with higher duty cycles would enable test requirements to accurately reflect RAN4 core specification and also allow RAN5 to do testing following the same procedure for all releases.

Proposal 2: If necessary, RMCs can be defined for all duty cycle capability values and testing performed aligned with the UE capability indication.

Furthermore, it is even possible to mandate UE to indicate 100% duty cycle. However, given the existing implementations such indication should be made mandatory only in an open release.

Proposal 3: If necessary, UE can be mandated to indicate 100% duty cycle in open release.

A companion CR is provided in [1]

A draft LS to RAN5 is included in the Appendix to inform them about the update.

Proposal 4: Send an LS to RAN5 to inform them about the updates.

## 3	Conclusion

In this contribution PC2 FDD requirements and duty cycle was discussed and following observations and proposals were made:

Observation 1: Default duty cycle of 50% has been specified for all PC2 UEs, and applies equally for TDD and FDD

Observation 2: Work item defines normative specifications, not study item. Implementations are based on normative specifications.

Observation 3: Work item outcome is aligned with study item conclusion, and it is recognized that captured SI conclusions leave room for to apply both duty cycle and P-MPR for SAR management for FDD UEs.

Observation 4: Missing PC2 FDD RMC with up to 50% duty cycle leads in unintended UE behavior in conformance testing, and PC2 requirements may not be properly verified.

Observation 5: Adding a new RMC for PC2 FDD enables testing to be done aligned with PC2 FDD requirements.

Proposal 1: Specify a new RMC for PC2 FDD with up to 50% duty cycle to align with the specified default behavior.

Proposal 2: If necessary, RMCs can be defined for all duty cycle capability values and testing performed aligned with the UE capability indication.

Proposal 3: If necessary, UE can be mandated to indicate 100% duty cycle in open release.

Proposal 4: Send an LS to RAN5 to inform them about the updates.

## 4	References

[1]	R4-26xxxxx, (NR_PC2_UE_FDD-Core) CR to TS 38.101-1: Addition of missing RMC, Qualcomm Incorporated, Nokia, Huawei, Ericsson, Skyworks

# Appendix

3GPP TSG-RAN WG4 Meeting #118

Gothenburg, Sweden, February 9th – 13th, 2026

Title:	draft LS on PC2 FDD conformance testing

Response to:	-

Release:	Rel-17

Work Item:	NR_PC2_UE_FDD-Core

Source:	RAN4

To:	RAN5

Cc:

Contact person:	Toni Lähteensuo

tlaehtee_at_qti.qualcomm.com

# 1	Background

When RAN4 introduced the requirements for PC2 FDD in Release 17 the addition of RMC was missed, leaving the specification incomplete. RAN4 has now agreed to append RMCs with information on active UL slots during testing, and as such ensure that UE maintains the transmission power at PC2 power level aligned with core requirements. Active UL slots are dependent on IE maxUplinkDutyCycle-PC2-FR1.

RAN4 would kindly request RAN5 to take these new RMCs into account in testing throughout the transmitter the receiver RF requirements.

# 2	Actions

ACTION: 	RAN4 respectfully asks RAN5 to update the conformance testing for PC2 FDD throughout transmitter and receiver requirements to align with RAN4 specifications.

# 3	Dates of next TSG RAN WG4 meetings

3GPP RAN4#118-Bis       		        April 2026				 Malta, MT

3GPP RAN4#119       		        May 2026                           China, CN