3GPP TSG-RAN4 Meeting #117	R4-2520170

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.191 | CR | 0002 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.191, Correction on A-IoT Device Output RF spectrum emissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To clarify the spectrum emission mask requirements apply to all D2R CBW for A-IoT Device |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify the spectrum emission mask requirements apply to all D2R CBW |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spectrum emission mask requirements for A-IoT Device will be unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2520170 |  |  |  |  |  |  |  |  |

==============First change==============

6.2.1.2	Spectrum emission mask

The spectrum emission mask of the device applies to frequencies (ΔfOOB) starting from the centre of the assigned D2R channel bandwidth. For frequencies offset greater than ΔfOOB as specified in Table 6.2.1.2-1 the spurious requirements in clause 6.2.2 are applicable.

The emission within ΔfOOB of a device shall be lower than the level specified in Table 6.2.1.2-1 compared to the D2R backscatter power under same incident CW power level. The spectrum emission limits in Table 6.2.1.2-1 apply for all D2R channel bandwidths. The requirement is verified with the test metric of EIRP and the incident CW power level at the device set to -5 dBm. The test direction is the same as the backscatter output power as specified in clause 6.1.

Table 6.2.1.2-1 Spectrum emission mask for device

| ΔfOOB (MHz) | Spectrum emission limit (dBc) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.5*D2R CBW - FOOB | 10 | 0.5* Nominal D2R transmission Bandwidth without SFO |
| NOTE 1: FOOB is the OOB boundary frequency specified in Table 6.2.1.2-1. |  |  |

==============End of change==============