3GPP TSG-RAN WG4 Meeting #118	R4-2601898

Gothenburg, Sweden, 09 – 13 February, 2026

Source:	Huawei, Hisilicon

Title:	draft TP to TR 37.720-11-11 to add DC_40_n28

Agenda item:			5.3.2

Document for:	Approval

# 1. Introduction

The related Rel-20 basket WID was approved in [1]. This contribution provides a text proposal for TR 37.720-11-11 to include DC_40C_n28A.

2. Reference

[1] RP-253842 New WID for DC_xBLTE_yBNR_R20

[2] TR 37.719-11-11 v1.0.0

# 3. Text Proposal

<Start of Text Proposal>

### 6.1.x	DC_40_n28

#### 6.1.x.1	Configuration for DC

Table 6.1.x.1-1: Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_40C_n28A | DC_40A_n28A DC_40C_n28A | No |

#### 6.1.x.2	Maximum output power for DC

Table 6.1.x.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_40C_n28A | 23 | +2/-3 |

#### 6.1.x.3	Spurious emission band UE co-existence for DC

For DC_40-n28, spurious emission requirements have already been specified in TS 38.101-3.

#### 6.1.x.4	Co-existence analysis for DC

The analysis for harmonics/harmonic mixing, cross-band isolation and inter-band UL IMD products refer to TR 37.719-11-11 v1.0.0 clause 6.1.16.4. The analysis for intra-band UL CA and 3CC UL case can be found as below.

Table 6.1.x.4-4: Band n28 and CA 40C intra-band UL CA IMD overlap with the other DL band analysis

| (All in MHz) | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| 40 fUL | 2300 | 2400 | Order | fmin | fmax |
| n28 fDL | 758 | 803 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 40 |  | 0 | 40 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | fmin | fmax |  | 0 | 80 |
| IMD3 (2-1) | fUL_low-Max2CCBW | fUL_high+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2260 | 2440 |  | 0 | 120 |
| IMD5 (3-2) | fUL_low-2*Max2CCLBW | fUL_high+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2220 | 2480 | Order | fmin | fmax |
| IMD7 (4-3) | fUL_low-3*Max2CCBW | fUL_high+3*Max2CCBW | IMD4 (3-1) | 2*fUL_low-Max2CCBW | 2*fUL_high+Max2CCBW |
|  | 2180 | 2520 |  | 4560 | 4840 |
| IMD9 (5-4) | fUL_low-4*Max2CCBW | fUL_high+4*Max2CCBW | IMD6 (4-2) | 2*fUL_low-2*Max2CCBW | 2*fUL_high+2*Max2CCBW |
|  | 2140 | 2560 |  | 4520 | 4880 |
| IMD11 (6-5) | fUL_low-5*Max2CCBW | fUL_high+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 2100 | 2600 | Order | fmin | fmax |
| IMD13 (7-6) | fUL_low-6*Max2CCBW | fUL_high+6*Max2CCBW | IMD5 (4-1) | 3*fUL_low-Max2CCBW | 3*fUL_high+Max2CCBW |
|  | 2060 | 2640 |  | 6860 | 7240 |
| Analysis | There is no IMD issue for n28 DL caused by CA_40C. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band UL CA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous UL CA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fUL_high-fUL_low) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or Sim Rx/Tx TDD bands <1GHz - IMD6 is considered for FDD or Sim Rx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band. |  |  |  |  |  |

Table 6.1.x.4-5: Band 40 and Band n28 for 3CC UL IMD products

| Band / CA1 | n28 |  | CA_40C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 703-748 |  | 2300-2400 |  |
| fDL | 758-803 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 - 40 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 663-703 |  | 748-788 |  |
| Analysis | There is an issue of IMD, but no requirements, since two bands are not in the same or adjacent band groups. |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

#### 6.1.x.5	∆TIB,c and ∆RIB,c values

For DC_40-n28, the TIB,c and RIB,c values have already been specified in TS 38.101-3.

#### 6.1.x.6	Analysis of MSD requirements

For DC_40-n28, the MSD requirements have already been specified in TS 38.101-3.

<End of Text Proposal>