3GPP TSG-RAN4 Meeting #117	R4-2522139

Dallas, USA, 17th – 21th , November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.191 | CR | 0006 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for device RF requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CMCC, VIVO, ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test configuration and FRC for D2R measurement are missing. There’re errors in the R2D FRC as well as the requirements for D2R channel bandwdith. “The general part” for clause 6 is missing. The requirement applicablity for bandwidth is not clearly stated in the spec, e.g., backscatter loss, reference sensitivity |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes have been endorsed in draft CR R4-2515142: Add test configuration and FRC for D2R. Correct the error for R2D FRC and D2R channel bandwidth requirements. Adding clause 6.0 the general part. Align the terminology of R2D channel bandwdith Clarify the backscatter loss is applied to all devcie D2R channel bandwidth Clarify the Reference sensitivity and EIS partial sphere coverage are applied to all R2D channel bandwdith. Avoid the unused reference; Remove the BS related description in the device specification in clause 5.3 Add more clarification on Annex B; Improve the wording on Annex E; Additionally changes in this CR include: Annex A: add test configuration for D2R power measurement Annex E: remove D2R CBW for BS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors remain in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1, 5.3.2, 6.0, 6.2, 7.2, Annex A, Annex B, Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Add new sub-clause 6.0 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2522139. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void

[3]	3GPP TS 38.291: "NR; Ambient IoT Physical layer".

[4]	ITU-R Recommendation SM.329, "Unwanted emissions in the spurious domain".

[5]	3GPP TS 38.870: “Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)”.

[6]	3GPP TS 38.391: "NR; Ambient IoT Medium Access Control (MAC) protocol".

<<< NEXT CHANGES >>>

#### 5.3.1.2	R2D Transmission bandwidth

The transmission bandwidth NRB for each R2D channel bandwidth and subcarrier spacing is specified in table 5.3.1.2.-1.

Table 5.3.1.2-1: R2D Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 200 kHz | 400 kHz | 600 kHz | 800 kHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 15 | 1 | 2 | 3 | 4 |

NOTE:	Device Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.1.2-1.

#### 5.3.1.3	Minimum guardband and R2D transmission bandwidth configuration

The minimum guardband for each R2D channel bandwidth and SCS is specified in table 5.3.1.3-1.

Table 5.3.1.3-1: Minimum guardband (kHz) (FR1)

| R2D CBW | 200kHz | 400kHz | 600kHz | 800kHz |
| --- | --- | --- | --- | --- |
| Minimum guardband(kHz) | 2.5 | 12.5 | 22.5 | 32.5 |

The number of RBs configured in any R2D channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.1.3-1: reader PRB utilization

#### 5.3.1.4	RB alignment

For each R2D channel bandwidth, BS transmission bandwidth configuration must fulfil the minimum guardband requirement specified in clause 5.3.1.3.

#### 5.3.1.5	R2D channel bandwidth per operating band

The requirements in this specification apply to the combination of R2D channel bandwidths, SCS and operating bands shown in table 5.3.1.5-1 for FR1. The transmission bandwidth configuration in table 5.3.1.2-1 shall be supported for each of the R2D channel bandwidths within the BS capability. The R2D channel bandwidths are specified for the Tx path.

Table 5.3.1.5-1: R2D channel bandwidths and SCS per operating band

| NR Band | SCS (kHz) | Reader channel bandwidth (kHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 200 | 400 | 600 | 800 |
| n8 | 15 | 200 | 400 | 600 | 800 |

<<< NEXT CHANGES >>>

### 5.3.2	D2R Channel bandwidth

#### 5.3.2.1	General

The D2R channel bandwidth supports a single AIoT RF carrier in the D2R link at the device. From a BS perspective, different device channel bandwidths may be supported within the same spectrum for backscattering from devices connected to the BS.

From a device perspective, the device is allocated with its own device channel bandwidth. The device does not need to be aware of the BS channel bandwidth.

#### 5.3.2.2	D2R Transmission bandwidth

The transmission bandwidth for each D2R channel bandwidth is specified in table 5.3.2.3-1.

#### 5.3.2.3	D2R channel bandwidth per operating band

The requirements  specified in table 5.3.2.3-1 for device apply to all the operating bands in Table 5.2-1.

Table 5.3.2.3-1: Device D2R channel bandwidth

| Device D2R channel bandwidth (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal D2R transmission Bandwidth without SFO (kHz) | Nominal Small frequency shift without SFO (kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 17 | 25 | 42 | 75 | 141 | 273 | 534 | 1065 |  |
| 30 |  | 33 | 50 | 83 | 149 | 281 | 545 | 1073 |  |
| 60 |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  |
| 120 |  |  |  | 132 | 198 | 330 | 594 | 1122 |  |
| 240 |  |  |  |  | 264 | 396 | 660 | 1188 |  |
| 480 |  |  |  |  |  | 528 | 792 | 1320 |  |
| 960 |  |  |  |  |  |  | 1056 | 1584 |  |
| 2880 |  |  |  |  |  |  |  |  | 3168 |

<<< NEXT CHANGES >>>

## 6.0	General

Unless otherwise stated, the transmitter characteristics are specified over the air (OTA).

<<< NEXT CHANGES >>>

## 6.1	Backscatter power

### 6.1.1	Device backscatter power

The backscatter power is defined as mean filtered power measured over the duration of the D2R signal, excluding the power at the carrier frequency. The backscatter loss is defined as the difference between the input CW power at the device antenna in dB scale to the backscatter power at the device antenna in dB scale.

The backscatter power is measured with the test metric of EIRP as specified in clause 8.2.1. The minimum requirement on backscatter loss in Table 6.1.1.1-1 shall apply to all device D2R channel bandwidth defined in Table 5.3.2.3-1 and shall be met with the test parameters defined in Annex A, at the peak antenna gain direction declared by the device vendor.

Table 6.1.1.1-1: Maximum allowable backscatter loss

| Operating band | Maximum backscatter loss (dB) |  | CW power at the device antenna (dBm) |
| --- | --- | --- | --- |
| n8 | OOK | 10 | -27 |
|  | BPSK | 6 |  |
|  | OOK | 15 | -10 |
|  | BPSK | 11 |  |

<<< NEXT CHANGES >>>

## 7.2	Reference sensitivity power level

### 7.2.1	General

The reference sensitivity power level REFSENS is defined as the EIS level at the centre of the quiet zone in the RX gain peak direction, at which the successful detection rate shall meet or exceed the requirements for the specified reference measurement channel.

### 7.2.2	Reference sensitivity power level

The successful detection rate shall be ≥ 90% of the reference measurement channels as specified in Annexes B.1, B.2 and B.3 with peak reference sensitivity specified in Table 7.2.2-1. The requirement is applied to all R2D channel bandwidth defined in Table 5.3.1.5-1 and verified with the test metric of EIS at the peak antenna gain direction as specified in clause 8.2.1.

Table 7.2.2-1: Reference sensitivity

| Operating band | Sensitivity Level | REFSENS (dBm) |
| --- | --- | --- |
| According to subclause 5.2 | L1 | -34 |

NOTE:	The peak reference sensitivity is measured at the low, middle and high frequency of the supported

band(s), and the average value is verified against the requirement.

### 7.2.3	EIS partial sphere coverage

The reference measurement channels and detection criterion shall be as specified in clause 7.2.2

The maximum EIS measured over the partial sphere around the device is defined as the partial sphere coverage requirement and is found in Table 7.2.3-1 below. The requirement is applied to all R2D channel bandwidth defined in Table 5.3.1.5-1 and verified with the test metric of EIS as specified in clause 8.2.1.

Table 7.2.3-1: EIS partial sphere coverage

| Operating band | EIS (dBm) | Angular width |
| --- | --- | --- |
| According to subclause 5.2 | -28.5 | ±45 degrees |

NOTE:	The EIS partial sphere coverage requirement is verified at the middle frequency of the supported

band(s).

<<< NEXT CHANGES >>>

## B.2	R2D reference measurement channels

### B.2.1	Fixed Reference Channels for reference sensitivity level (OOK)

Table B.2.1-1: Fixed Reference Channels for reference sensitivity level (OOK)

| Component | Parameter | Unit | Value |
| --- | --- | --- | --- |
| General | PRB | RBs | 1 |
|  | SCS | kHz | 15 |
| SIP | Bit length | Bits | 8 |
|  | Mapping to OFDM | Chips/Symbol | 4 |
| CAP | Bit length | Bits | 4 |
|  | M | Chips/Symbol | 6 |
| PRDCH | TBS | Bits | Depending on the size of the MAC PDU of A-IoT CFA paging message |
|  | CRC | Bits | 16 |
|  | M | Chips/Symbol | 6 |
| Postamble | Bit length | Bits | 4 |
|  | M | Chips/Symbol | 6 |
| Padding | Padding for last OFDM symbol | Chips | Depending on the TBS |

### B.2.2	Fixed Reference Channels for maximum input levvel

Table B.2.2-1: Fixed Reference Channels for maximum input level

| Component | Parameter | Unit | Value |
| --- | --- | --- | --- |
| General | PRB | RBs | 3 |
|  | SCS | kHz | 15 |
| SIP | Bit length | Bits | 8 |
|  | Mapping to OFDM | Chips/Symbol | 4 |
| CAP | Bit length | Bits | 4 |
|  | M | Chips/Symbol | 24 |
| PRDCH | TBS | Bits | Depending on the size of the MAC PDU of A-IoT CFA paging message |
|  | CRC | Bits | 16 |
|  | M | Chips/Symbol | 24 |
| Postamble | Bit length | Bits | 4 |
|  | M | Chips/Symbol | 24 |
| Padding | Padding per OFDM symbol excluding SIP and Postamble | Chips | 2 |
| Padding | Padding for last OFDM symbol | Chips | Depending on the TBS |

## B.3	D2R reference measurement channels

### B.3.1	Fixed Reference Channels for transmitter characteristics

Table B.3.1-1: Fixed Reference Channels for backscatter power, SEM and spurious emissions

| Parameter | Unit | Value |
| --- | --- | --- |
| Transmission BW | kHz | 15 |
| TBS | Bits | Depending on the length of AIoT device ID |
| CRC | Bits | 16 |
| FEC code rate |  | 1/3 |
| Block repetition number |  | 1 |
| Preamble length | Bits | 31 |
| Midamble length | Bits | 31 |
| Interval for midamble insersion | Bits | 48 |
| Additional midamble insertion |  | No |
| Small frequency shift | kHz | 480 |
| Modulation |  | BPSK/OOK (NOTE1) |
| NOTE 1: The modulation scheme used is up to device implementation. |  |  |

<<< NEXT CHANGES >>>

Annex E (informative):
D2R channel bandwidth

The following describes the equation to derive device D2R channel bandwidth.

D2R CBW for device (kHz)

= ceiling (2SB Transmission BW without SFO× (1/2) +2× Small frequency shift without SFO)

= ceiling ((2+2R)/Tb × (1+∣SFO∣))

= ceiling ((1+R)/ (Tc ×R) × (1+∣SFO∣))

<<< END OF CHANGES >>>