# 3GPP TSG-RAN WG4 Meeting #116	R4-2512135

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:			7.23.1

Source:	Moderator (Ericsson)

Title:	Ad-hoc minutes for Netw_Energy_NR_enh

Document for:	Approval

# Introduction

This document is the Ad-hoc minutes for Netw_Energy_NR_enh with the following topics covered.

Note 1: The following terminologies are used.

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

  - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification) when the center frequency locations of always-on SSB and on-demand SSB are same.

  - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported when the center frequency locations of always-on SSB and on-demand SSB are same.

  - Alt Scenario 3: When the center frequency locations of always-on SSB and on-demand SSB are different and AO-SSB and OD-SSB are located in the same BWP.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #2A refers to “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

- Scenario #3A refers to “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

- Scenario #3B refers to “When SCell activation is completed and SCell is activated” or “After SCell activation is completed and SCell is activated”

Note 2: The following abbreviations are used.

- AO-SSB: Always-on SSB

- OD-SSB: On-demand SSB

- FMW: Fast Measurement Window

- EMP: Effective Measurement Periodicity

# Topic #1: SSB adaptation (AI 7.27.3.3)

### Issue 1-2-2: L1 requirements – Measurement restriction impact

- Proposals

  - Option 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. (Huawei)

  - Option 2: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply. (CATT, Nokia, Ericsson, Samsung, CTC, vivo)

- Recommended WF:

  - Agree on option2.

- Tentative agreement

  - When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply.

Discussion:

HW: Support option 2

Agreement:

  - When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply.

### Issue 2-1-1: RRM requirements impacts for OD-SIB1

RAN4 agreements in R4-2508441 Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption may be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

- Proposals

  - Proposal 1: 	RAN4 to consider following change to paging interruption during cell reselection to an OD-SIB1 cell: Add the phrase “At intra-frequency and inter-frequency cell re-selection, the interruption time may be longer if the target cell is a NES cell supporting on-demand SIB1 transmission.” in clause 4.2.2.6 of TS 38.133. (Nokia)

  - Proposal 2: 	RAN4 to consider following change to cell reselection delay in idle mode, when UE performs cell reselection to an OD-SIB1 cell: Add the phrase “At intra-frequency cell re-selection, Tdetect,NR_Intra , Tmeasure,NR_Intra and Tevaluate,NR_Intra may be longer if the target Cell is a NES Cell supporting on-demand SIB1 transmission.” for intra-frequency NR cells in clause 4.2.2.3 of TS 38.133 and the same phrase in clause 5.1.2.3 for inactive mode. Add similar statements in clauses 4.2.2.4 and 5.1.2.4 for inter-frequency NR cells and in clauses 4.2.2.5 and 5.1.2.5 for inter-RAT E-UTRAN cells. (Nokia)

  - Proposal 3: 	RAN4 to consider the impact on RRM core requirements for one or more UL-WUS Msg1 retransmission attempts in the NES cell, if configured. (Nokia)

- Tentative agreement

- Discuss in the CR directly.

Discussion:

Moderator: Nokia check with other companies and if it is not agreeable, postpone to next meeting

Nokia: This is different to OPPO CR

HW: Should not read SIB1 during cell reselection, why it is considered?

Nokia: Not only to test case, there is impact to core requirements (i.e., cell reselection delay) also.

OPPO: We can further consider issue proposed by Nokia

HW: Is it for paging or cell reselection?

Nokia: There is no framework

Moderator: Keep current agreement and if companies think additional requirements are needed discuss in maintenance stage.

Agreement:

Keep current agreement and if companies think additional requirements are needed discuss in maintenance stage.

# Topic #2: On-demand SSB(OD-SSB) (AI 7.27.3.1)

### Issue 1-1-1: OD-SSB in FMW collided with MG

- Proposals

  - Option 1: Qualcomm, CATT, Apple, OPPO, China Telecom, CMCC, vivo, ZTE, LG, Xiaomi, Ericsson, Samsung, Nokia, Huawei, ZTE

    - UE is allowed to skip the OD-SSB measurement if OD-SSB in FMW collides with MG occasion.

      - ‘Kp’ should be introduced to extend the FMW length.

  - Option 2: ZTE

    - UE to prioritize the OD-SSB carrier/layer within FWM

- Tentative agreement

  - UE is allowed to skip the OD-SSB measurement if OD-SSB in FMW collides with MG occasion.

    - ‘Kp’ should be introduced to extend the FMW length.

Discussion

ZTE: option 1

Agreement;

    - UE is allowed to skip the OD-SSB measurement if OD-SSB in FMW collides with MG occasion.

      - ‘Kp’ should be introduced to extend the FMW length.

### Issue 1-1-3: FMW clarification

- Background

RAN4 #114 Sub-issue 1-1 – The length of FMW Agreement: - The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period

- Proposals

  - Option 1: Qualcomm

    - RAN4 should confirm that the UE can stop doing fast measurements after time Tidentify even if no measurement report has been sent to the network.

- Tentative agreement

  - The actual FMW measurement duration equals min(Treport , Tidentify) from the end of uncertain time.

    - Treport is the first OD-SSB based L3 measurement reporting

    - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

    - Note: The UE can stop doing fast measurements after time Tidentify even if no measurement report has been sent to the network.

Discussion

OPPO: Agree in principle. How do we capture it in CR.

- Moderator: Please check CR

Nokia: We already have Tidentify. Are we defining new Tidentify for OD-SSB? Or legacy one?

HW: Do we need to capture something in CR

QC: No need to capture.

ZTE: Agreement in 114 already included or covered this. Why we need new agreement?

- QC: this is just a clarification. To make sure we are on the same page.

Agreement:

  - The actual FMW measurement duration equals min(Treport , Tidentify) from the end of uncertain time.

    - Treport is the first OD-SSB based L3 measurement reporting

    - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

    - Note: The UE can stop doing fast measurements after time Tidentify even if no measurement report has been sent to the network.

### Issue 1-3-2: Multiple OD-SSBs based deactivated SCells measurement

In RAN4 #115 meeting, the following agreement has been made.

Issue 1-4-1: OD-SSB based multiple SCells activation Agreement - RAN4 to define the requirements in the case of multiple OD-SSB SCells. - It doesn’t impact the WI’s completion. - The scope can be limited as follows: In deactivated SCell stage, the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping. [For multiple Scells activation, multiple Scells are purely OD-SSB’s Scells. OB-SSB is assumed to be activated on all the multiple Scells to be activated before Scell activation command.]

- Information

Overlapping case

Tentative agreement

  - There is no deactivated SCell measurement requirement for OD-SSB when UE receives the OD-SSB indication for another SCell during the OD-SSB transmission period in a deactivated SCell.

  - Otherwise, the requirement of deactivated SCells measurement with multiple OD-SSBs shall be the same as the deactivated SCells measurement requirement for single OD-SSB.

Discussion

Nokia: In principle we agree. However, we need to work on the wording. Based on the last meeting agreement, first case should not occur.

Nokia: what is intention of the this scenario?

OPPO:

Agreement:

- In deactivated SCell stage, when the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping.

  - the requirement of deactivated SCells measurement with multiple OD-SSBs SCell shall be the same as the deactivated SCells measurement requirement for single OD-SSB.

- Otherwise, NO requirements

### Issue 1-3-3: Multiple OD-SSBs based SCells activation

- Background

  - In RAN4 #115 meeting, the following agreement has been made.

Issue 1-4-1: OD-SSB based multiple SCells activation Agreement - RAN4 to define the requirements in the case of multiple OD-SSB SCells. - It doesn’t impact the WI’s completion. - The scope can be limited as follows: - In deactivated SCell stage, the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping. - [For multiple Scells activation, multiple Scells are purely OD-SSB’s Scells. - OB-SSB is assumed to be activated on all the multiple Scells to be activated before Scell activation command.]

- Proposal

  - Option 1: Ericsson

    - RAN4 to define OD-SSB based multiple SCells activation delay requirements.

      - All Scells being activated are OD-SSB based Scells.

      - Both known and unknown SCell activation in FR1 and FR2 should be supported.

  - Option 2: Nokia

    - To assume only one OD-SSB is activated when activating the SCell hence do not consider multiple SCell activation.

- [Information]Possible scenarios

Two OD-SSB Scenario 2A SCells

OD-SSB Scenario 2A + Scenario 2

Two OD-SSB Scenario 2 SCells with non-overlapping of OD-SSB based deactivated SCell measurement

- Tentative agreement

  - RAN4 to define the multiple OD-SSB based SCells activation delay requirements.

    - Reuse the principle of legacy multiple SCells activation requirements

    - At least one of the OD-SSB activation command comes together with SCell activation command

    - No requirements if two or more SCells’ OD-SSBs transmission are overlapped in the deactivated SCell stage

Discussion

Apple: is it possible that NW activates OD-SSB before the SCell activation for one of the cell? NW can activate both the OD-SSBs before

Agreement:

  - RAN4 to define the multiple OD-SSB based SCells activation delay requirements.

    - Reuse the principle of legacy multiple SCells activation requirements

    - For case1, at least one of the OD-SSB activation command comes together with SCell activation command

    - No requirements if two or more SCells’ OD-SSBs transmission are overlapped in the deactivated SCell stage

### Issue 1-1-4: Requirement for RAN1 Alt Time-C2

- Proposals

  - Option 1: CATT, China Telecom, CMCC, Ericsson, Nokia, ZTE, Huawei, Samsung

    - RAN4 to define the requirement based on the OD-SSB periodicity.

  - Option 1a: CATT, Nokia, Ericsson

    - No scheduling restriction is expected for AO-SSB when OD-SSB is activated.

  - Option 1b: Nokia

    - L1 measurement requirement is based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

  - Option 1c (Qualcomm)

    - RAN4 to define the delay requirement only based on the OD-SSB periodicity only if the periodicity of AO-SSB and OD-SSB are same

  - Option 2: LG

    - RAN4 to define the L3 and L1 measurement requirement only based on the OD-SSB periodicity only if the periodicity of AO-SSB and OD-SSB are same.

      - For L3 measurement requirement, RAN4 to take into account only OD-SSB

      - For L1 measurement requirement, RAN4 to take into account union of AO-SSB and OD-SSB

  - Option 3: OPPO

    - RAN4 to define the requirement following legacy AO-SSB within SMTC.

- Recommended WF

  - Moderator suggests the group to check whether the following proposal can be agreed.

    - RAN4 to define the requirement for Alt Time-C2 only based on the OD-SSB periodicity.

      - It applies to both deactivated SCell L3 measurement and L1 measurement.

      - No scheduling restriction is expected for AO-SSB in Alt Time-C2.

Discussion

LGE: in general we are fine with this proposal for L3 measurement. FFS for L1 measurement

OPPO: AO-SSB based neighbour cell measurement should be considered.

Nokia: Which L1 measurement are we talking about here?

Agreement:

- RAN4 to define the requirement for Alt Time-C2 only based on the OD-SSB periodicity.

  - It applies to deactivated SCell L3 measurement

  - It applies to L1 measurement after the SCell activation [command].

  - No scheduling restriction is expected for AO-SSB outside the SMTC in Alt Time-C2.

Note: QC to check and confirm.

### Issue 1-4-7: Cell identification

The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios.

- Background

TS 38.133 9.2.4.3 … If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra provided the timing to that cell has not changed more than 3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used

- Proposals

  - Option 1: Ericsson

    - UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within 5s and then the subsequent OD-SSB is transmitted with the same spatial reception parameter.

  - Option 2: Xiaomi

    - Case 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band.

    - Case 2: Always-on SSB on the cell

- Recommended WF

  - If UE has already measured OD-SSB for Tidentify_intra_without_index_ODSSB or Tidentify_intra_with_index_ODSSB and the OD-SSB deactivated period is within 5s and then the OD-SSB is activated again with the same spatial reception parameter, UE shall skip the OD-SSB identification component provided the timing to that cell has not changed more than  3200/2µ Tc.

Discussion

Apple: we need to add side condition that cell should be detectable.

Nokia: what does the meaning of target cell remains detectable. Which target cell? OD-SSB can be ON and OFF within the 5 sec.

Agreement:

- If UE has already measured OD-SSB for Tidentify_intra_without_index_ODSSB or Tidentify_intra_with_index_ODSSB and the OD-SSB deactivated period is within 5s and the SCell is detectable when OD-SSB is transmitted, then the OD-SSB is activated again and the SCell remain detectable with the same spatial reception parameter and same frequency, UE shall skip the OD-SSB identification component provided the timing to that cell has not changed more than  3200/2µ Tc.

- Note: Option 2 can be discussed in maintenance stage.

### Issue 1-4-8: L1 measurement – collision with CSI-RS

- Proposals

  - Option 1: vivo

    - The OD-SSB is measured when OD-SSB collides with CSI-RS.

  - Option 2: Huawei

    - RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB.

- Tentative agreement

  - When OD-SSB colliding with CSI-RS, existing L1 measurement restriction can apply.

Discussion

Agreement:

  - When OD-SSB colliding with CSI-RS, existing L1 measurement restriction can apply.

### Issue 1-4-5: OD-SSB interruption spec. impact

- Background

  - In RAN4 #114b meeting, RAN4 agreed to introduce the interruption requirements as follows.

Issue 1-4-9: Interruption Agreement - New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation. - New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command. - The interruption can be applied in both Case 1 and Case 2. - The detail condition, duration and position of the interruption is FFS. - The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation. - No new interruption requirement is defined. - The interruption requirement follows the same principle of legacy SCell activation.

- Proposals

  - Option 1: Xiaomi, Ericsson

    - RAN4 to define new OD-SSB related interruption requirements as follows.

      - Interruptions at SCell addition/release

      - Interruptions at SCell activation/deactivation

      - Interruptions during measurement on deactivated SCC

  - Option 2: Nokia

    - No interruption is allowed during the RRC reconfiguration procedure for SCell addition/release before OD-SSB is indicated in Case 1.

    - No interruption is allowed during measurements on deactivated SCC before OD-SSB is indicated in Case 1.

  - Option 3: CMCC

    - For Case 1 Scenario 2 and Scenario 3(OD-SSB and AO-SSB are within different frequencies), new interruption is allowed due to OD-SSB activation/deactivation.

    - For Case 2 Scenario 2, new interruption is not allowed due to OD-SSB activation/deactivation.

    - For both Scenario 2 and Scenario 3, if UE does not receive SCell activation command at the end of FMW, the same interruption of deactivated SCell measurement interruption can be applied, provided that the measCycleSCell equal or longer than 640ms.

- Recommended WF

  - From moderator’s understanding, none of the proposals contradict each other, and all can be agreed.

  - The details can be checked in CR directly.

agreement

- Discuss in the CR directly.

Discussion

# Topic #3: NES enhancement performance requirements (AI 7.27.4)

### Sub-topic 3-1: OD-SSB performance part

Sub-topic description: This sub-topic covers OD-SSB based performance part.

### Issue 3-1-4: Test case list

RAN4 to discuss whether the following OD-SSB test cases can be agreed.

- SCell activation in Case 1

  - Support: Qualcomm, CATT, Apple, LG, Xiaomi, CMCC, China Telecom, Huawei, ZTE, Samsung, Ericsson

  - Note: The detail scenario is FFS(known/unknown; scenario 2/2a)

- SCell activation in Case 2 Alt Time C-1

  - Support: Qualcomm, CATT, Apple, LG, Xiaomi, CMCC, China Telecom, Huawei, ZTE, Samsung, Ericsson

  - Note: The detail scenario is FFS(known/unknown; scenario 2/2a)

- SCell activation in Case 2 Alt Time C-2

  - Support: CMCC

- OD-SSB based multiple SCell activation

  - Support: Qualcomm, CATT, LG, Ericsson

- OD-SSB based direct SCell Activation at SCell addition in Case 1

  - Support: CATT, LG, CMCC, China Telecom, Samsung, Ericsson

  - lower priority: Qualcomm,

- OD-SSB based direct SCell Activation at SCell addition in Case 2 Alt Time C-1

  - Support: CATT, China Telecom, Huawei, Samsung

  - lower priority: Qualcomm,

- OD-SSB based direct SCell Activation at SCell addition in Case 2 Alt Time C-2

  - Support: CMCC, Huawei

  - Not support: Qualcomm,

- OD-SSB based PUCCH SCell Activation in Case 1

  - Support: CATT, LG, China Telecom, Huawei, Samsung

  - lower priority: Qualcomm,

- OD-SSB based PUCCH SCell Activation in Case 2

  - Support: CATT, China Telecom, Huawei, Samsung, Ericsson

  - lower priority: Qualcomm,

- Fast SCell Activation with OD-SSB;

  - Support: LG, Samsung

  - Not support: Qualcomm, Ericsson

- OD-SSB based SCell Activation with the L3 reporting during activation in Case 1

  - Support: CATT, Apple, LG, Huawei, Samsung, Ericsson

  - lower priority: Qualcomm,

- OD-SSB based SCell Activation with the L3 reporting during activation in Case 2

  - Support: Huawei, Samsung

  - lower priority: Qualcomm,

- Intra-frequency measurement(deactivated SCell) in Case 1

  - Support: CATT, LG, Xiaomi, CMCC, China Telecom, Huawei, ZTE, Ericsson

  - Not support:

- Intra-frequency measurement(deactivated SCell) in Case 2

  - Support: CATT, LG, CMCC, China Telecom, Huawei, ZTE, Ericsson

  - Not support:

- Intra-frequency measurement in Case 1

  - Support: CATT, LG, Samsung, Ericsson

  - Not support:

- Intra-frequency measurement in Case 2

  - Support: CATT, LG, Samsung, Ericsson

  - Not support:

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

  - Support: Apple, LG, China Telecom, ZTE, Samsung, Ericsson

  - Not support: OPPO

- Interruption at OD-SSB activation/deactivation

  - Support: CATT, Apple, LG

  - Merge: Ericsson

- Interruptions during measurements on deactivated NR SCC

  - Support: CATT, Apple, LG, Samsung

  - Merge: Ericsson

- Multiple OD-SSB carriers in Case 1

  - Support: Huawei

- Multiple OD-SSB carriers in Case 2

  - Support: Huawei

- Recommended WF

  - Moderator suggests companies checking whether at least the following test cases can be defined.

  - SCell activation part

    - SCell activation in Case 1

    - SCell activation in Case 2 Alt Time C-1

    - Note: The detail scenario is FFS, such as known/unknown; scenario 2/2a etc.

  - Deactivated SCell L3 measurement part

    - Intra-frequency measurement(deactivated SCell) in Case 1

    - Intra-frequency measurement(deactivated SCell) in Case 2

    - Note: The detail scenario is FFS

  - Active SCell L3 measurement part

    - Intra-frequency measurement in Case 1

    - Intra-frequency measurement in Case 2

    - Note: The detail scenario is FFS, such as OD-SMTC, SSB parameter update; Alt-TimeC1/Alt-TimeC2/scenario 3 etc

  - Active SCell L1 measurement part

    - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

    - Note: The detail scenario is FFS,

Discussion

### Sub-topic 3-3: SSB adaptation performance part

Sub-topic description: This sub-topic covers SSB adaptation based performance part.

### Issue 3-3-1: Test case list

- RAN4 to discuss whether the following SSB adaptation test case list can be agreed.

  - L1-RSRP

    - Support: Qualcomm, CATT, Apple, LG, Xiaomi, CMCC, China Telecom, ZTE, Samsung, Nokia

    - Not support: OPPO

  - L1-SINR

    - Support: Qualcomm, CATT, LG, Xiaomi, CMCC, China Telecom, ZTE, Nokia

    - Not support: OPPO, Samsung

  - CBD

    - Support: Qualcomm, CATT, LG, Xiaomi, CMCC, China Telecom, ZTE, Nokia

    - Not support: OPPO, Samsung

  - L3 intra-frequency measurement

    - Support: Qualcomm, CATT, Apple, LG, Xiaomi, CMCC, China Telecom, ZTE, Samsung, Nokia

    - Optional: OPPO

  - Deactivated SCell measurement, Nokia

    - Support: LG

    - Not support: Qualcomm, Xiaomi

    - Postpone: Huawei

  - Normal SCell activation

    - Support: Apple, LG, Xiaomi, CMCC, China Telecom, Nokia

    - Optional: OPPO

    - Not support: Qualcomm, Xiaomi

    - Postpone: Huawei

  - Direct SCell activation

    - Support: LG, China Telecom, Nokia

    - Not support: Qualcomm, Xiaomi

    - Postpone: Huawei

  - PUCCH SCell activation

    - Support: LG, China Telecom, Nokia

    - Not support: Qualcomm, Xiaomi

    - Postpone: Huawei

- Recommended WF

  - Moderator suggests the group to check the initial test case list for SSB adaptation as follows.

    - L1-RSRP

    - L3 intra-frequency measurement

    - FFS other possible test case

Discussion

# Topic #4: draft CR status

| T-doc number | Title | Company | Recommendation |
| --- | --- | --- | --- |
| R4-2509221 | DraftCR on introduction and measurement restriction for L1-SINR measurement | MediaTek inc. | Revised |
| R4-2509281 | Draft CR to TS 38.133 on carrier-specific scaling factor for R19 NES | CATT | Revised |
| R4-2509282 | Draft CR to TS 38.133 on intra-frequency measurements without measurement gaps for R19 NES | CATT | Revised |
| R4-2509451 | DraftCR of interruptions for OD-SSB transmission | Apple | Revised |
| R4-2509624 | Draft CR on OD-SSB based SCell activation delay with L3 reporting | Nokia, Nokia Shanghai Bell | Revised |
| R4-2509630 | Draft CR for OD-SSB based SCell Deactivation Delay Requirement for Activated SCell | LG Electronics Inc. | Revised |
| R4-2509694 | Draft CR on OD-SSB related RRM requirements | OPPO | Revised |
| R4-2509797 | draftCR on Rel19 NES (8.2.2.2.1 Interruptions at SCell addition/release) | Xiaomi | Revised |
| R4-2509798 | draftCR on Rel19 NES (8.2.2.2.2 Interruptions at SCell activation/deactivation) | Xiaomi | Revised |
| R4-2509799 | draftCR on Rel19 NES (8.2.2.2.3 interruption during measurements on deactivated SCC) | Xiaomi | Revised |
| R4-2509926 | DraftCR: Scheduling availability during UE intra-frequency measurements based on OD-SSB | Intel Corporation | Agree |
| R4-2510027 | (Netw_Energy_NR_enh-Core) draftCR on Introduce OD-SSB based direct SCell Activation requirement at SCell addition | CMCC | Revised |
| R4-2510090 | Draft CR on OD-SSB based L1-RSRP measurements for Reporting | China Telecom | Revised |
| R4-2510196 | Draft CR for Scheduling for L1-SINR measurements for Reporting and minimum requirements at transitions | vivo | Revised |
| R4-2510597 | DraftCR on OD-SSB based SCell Activation Delay Requirement for Deactivated PUCCH SCell | Huawei, HiSilicon | Revised |
| R4-2510721 | Draft CR on scheduling restriction of L1-RSRP measurement of R19 NES | ZTE Corporation, Sanechips | Agree |
| R4-2510722 | Draft CR on transition requirements of L1-RSRP measurement for R19 NES | ZTE Corporation, Sanechips | Revised |
| R4-2510917 | draftCR on OD-SSB based NR intra-frequency measurements | Ericsson | Revised |
| R4-2511084 | Draft CR on OD-SSB based SCell Activation Delay Requirement for Deactivated SCell | Samsung | Revised |