# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513510

# Prague, Czech, October 13th – 17th, 2025

Agenda item:			6.10.1

Source:	Moderator (Ericsson)

Title:	Topic summary for [116bis][204] NR_RRM_Ph5_Part1

Document for:	Information

# Introduction

This topic summary includes topic summary for core-part of NR_RRM_Ph5 (4.15) in maintenance phase and RRM performance part of FR2-1 SSB based L3 measurement delay by optimizing CSSF outside gap in CA/DC (6.10.2.2).

List of candidate target of discussions for this topic.

- Mainly discuss on

  - Core part: issue 1-1-2, 1-1-3, 1-2-1,1-3-1,1-1-1,1-1-4,1-2-2.

  - Performance part: 2-1-4, 2-1-1, 2-1-3, 2-1-5 (rely on 1-2-1), 2-1-2.

# Topic #1: Maintenance of core requirements of NR_RRM_Ph5 (4.15)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513089 | OPPO | FBS: Proposal 1: Do not define T2 for re-evaluation after UE quits from FBS mode. Proposal 2: Do not define the requirements when multiple bands are involved and UE reports different BSF values for different bands. Proposal 3: When the target and serving carrier frequency are different, the requirements apply to measurement with gap including per-FR gap and Per-UE gap. SCell activation: Proposal 4: The feature group for R19 SCell activation based on EMR is defined as below. Features; Index; Feature group; Components; Prerequisite feature groups; Consequence if the feature is not supported by the UE. 49. NR_RRM_Ph5; 49-x; Fast SCell activation based on early measurement report; Supports fast SCell activation specified in TS38.133 clause 8.3.2A; 39-8 or 39-9 in R18 RAN4 feature list or 18-7 (idleInactiveNR-MeasReport-r16); UE cannot meet fast SCell activation requirements as specified in TS38.133 clause 8.3.2A. |
| R4-2513212 | CATT | FBS: Proposal 1: It is up to UE implementation when to re-enter the FBS after quitting the last FBS with RAN4 confirming that UE needs to evaluate the triggering conditions and re-enter the FBS mode immediately if the thresholds are satisfied in the following two cases: - Case 1: Triggering conditions are reconfigured by network and they are satisfied; - Case 2: For unchanged entering thresholds, when entering thresholds that are unsatisfied become satisfied, UE needs to re-enter the FBS mode immediately. Observation 1: This issue only exists for inter-frequency measurements and when serving carrier and target carrier are in different bands. Proposal 2: For inter-frequency measurements and when serving carrier and target carrier are in different bands, the BSF value reported on the band to which the target carrier belongs should be used in the requirements. |
| R4-2513467 | Huawei, HiSilicon | FBS: Proposal 1: The scenario that multiple bands are involved and UE reports different BSF values for different bands is out of the Rel-19 BSF scope. Since the scenario of Rel-19 BSF is one serving cell and one target MO. CSSF: Observation 1: There is no requirement or UE capability that would allow the UE to manage the serving cell connection alongside one intra-frequency and two inter-frequency CC measurements. Proposal 2: To avoid contradicting the gap enhancement requirement, the 3-searcher solution should not be applied to inter-frequency MOs without a measurement gap directly. Instead, a relaxation for inter-frequency MOs without a measurement gap is needed in the 3-searcher solution. SCell activation: Observation 2: There is no need to add the R16 feature in the Prerequisite feature group part. Proposal 3: The details of UE capability for Rel-19 fast SCell activation based on EMR are shown in Annex Table 5.1. |
| R4-2513610 | Apple | FBS: Proposal 2: It is proposed to resolve the open issues with the following agreements - When the target and serving carrier frequency are different, FBS applies to target carrier only. - In the requirements, the applicable BSF is determined based on the band of the target carrier to be measured. - Since UE is not configured with CA or DC, per-FR measurement gap can be handled in the same way as for per-UE measurement gap. In other words, no special consideration is needed. CSSF: Proposal 1: RAN4 to not apply the enhanced CSSFoutside_gap,i scaling factor for the inter-frequency MOs without MG that are being measured outside of MG. |
| R4-2513881 | ZTE Corporation, Sanechips | FBS: Proposal 1: Considering only the RSRP/RSRQ of serving cell in FR2 is allowed to evaluate the activation/deactivation of FBS, the situation of per-FR gap configured is similar as per-UE gap configured. No need to further optimize for the case of per-FR gap configured. Proposal 2: Specify the UE re-evaluation of FBS after quitting, with a solution of both ensurance the UE implementation flexibility and addressing NW concern as below: - Introduce a predefined time duration T2 - UE decides when to re-evaluate the FBS as long as no later than T2 from the time point of quitting FBS |
| R4-2514065 | Ericsson | FBS: Observation 1: Having not a timer to re-evaluate FBM is not a rational design from either a signaling load or network configuration perspective to design a signaling that UE only mandatorily follows once. Proposal 3: After the UE exits FBS, it shall measure the serving cell quality and evaluate the FBS activation condition: - If the FBS activation criteria is met, it is up to the UE implementation to decide when to re-enter FBS. - If the FBS activation criteria is not met, the UE shall re-evaluate the activation condition and may re-enter FBS mode after a timer T2 expires. Proposal 4: Regarding the issue concerning different bands, RAN4 should check the applicable scenarios of FBS to include the cases with multiple MOs configured. Proposal 5: We’re open to speeding up the serving carrier by FBS in the case that the target and serving carrier frequency applying FBS are on different. CSSF: Proposal 1: Supportive to applying enhanced CSSFoutside_gap,i scaling factor for the inter-frequency MOs without MG that are being measured outside of MG. Proposal 2: RAN4 to confirm CSSF Solution 1 can be applied to an inter-frequency neighbor cell (without measurement gap) that is intra-band but not intra-frequency with the serving cells. |
| R4-2514257 | Nokia | FBS: Proposal 1: After timer expiration, if the FBS condition stops being met and is met again, the UE shall re-enter FBS. This is already covered and has no specification impact. Proposal 2: After timer expiration, if the FBS condition continues to be met, we prefer not to define a re-entry condition for FBS, but if a rule for re-entry to FBS is needed, a second signal level based threshold would be more useful than defining a second timer. Observation 1: Existing measurement delay requirements consider different measurement delay for each measurement object. Proposal 3: When multiple bands are involved, the measurement requirement is calculated as in legacy requirements, i.e. per measurement object. Proposal 4: FBS applies for inter-frequency measurement with per-FR measurement gap. SCell activation: Proposal 5: For Fast Scell with EMR capability, add both measValidationReportEMR-r18 and measValidationReportReselectionMeasurements-r18 as pre-requisite. Proposal 6: For Fast Scell with EMR capability, add idleInactiveNR-MeasBeamReport-r16 as pre-requisite for the capability for FR1 and FR2. |
| R4-2514322 | Qualcomm ncorporated | SCell activation: Proposal 1: UE capability for R19 SCell activation based on EMR is defined as below: - The UE shall support measValidationReportEMR-r18 (per band) and idleInactiveNR-MeasReport-r16 (per UE). - For FR2, it shall additionally support idleInactiveNR-MeasBeamReport-r16 (per UE) - For EMR-based fast direct SCell activation, the UE shall support directMCG-SCellActivation (per UE with FR1-FR2 DIFF). |
| R4-2514401 | MediaTek inc. | FBS: Proposal 2: FBS enhancement can apply to inter-frequency measurement with per-FR MG and there is no spec impact. CSSF: Proposal 1: Discuss whether to exclude the 3-searcher enhancement for inter-frequency measurement without MG scenario. |
| R4-2513501 | vivo | Proposal 1: When multiple bands are involved and UE reports different BSF values for different bands, the requirement can be based on the largest BSF value reported over all the bands where an MO is configured. Proposal 2: For which CC to apply the FBS when the target and serving carrier frequency are on different, introducing UE capability which indicate UE can support FBS on both carrier frequency could be considered. Proposal 3: No need further discussion on T2 at the maintenance phase. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Issue 1-1: issues related with FBS

Issue 1-1-1: Condition of re-entering FSB after existing

- Option 1 (OPPO, CATT, Nokia, vivo): Do not define T2 for re-evaluation after UE exits FBS mode.

  - Option 1A (CATT): It is up to UE implementation when to re-enter the FBS after quitting the last FBS. UE needs to evaluate the triggering conditions and re-enter the FBS mode immediately if the thresholds are satisfied in the following two cases:

    - Case 1: Triggering conditions are reconfigured by network and they are satisfied;

    - Case 2: For unchanged entering thresholds, when entering thresholds that are unsatisfied become satisfied, UE needs to re-enter the FBS mode immediately.

  - Option 1B (Nokia):  UE evaluates FBS activation criteria after T1 expires.

    - If the FBS condition stops being met and is met again, the UE shall re-enter FBS. This is already covered and has no specification impact.

    - If the FBS condition continues to be met, no re-entry condition for FBS.

      - but if a rule for re-entry to FBS is needed, a second signal level based threshold would be more useful than defining a second timer.

- Option 2 (ZTE): Define T2 for re-evaluation after UE exits FBS mode.

  - UE decides when to re-evaluate the FBS as long as no later than T2 from the time point of quitting FBS.

- Option 3 (Ericsson):  Define conditional T2 for re-evaluation after UE exits FBS mode.

  - UE evaluates FBS activation criteria after UE quits from FBS mode.

    - If the FBS activation criteria is met, it is up to the UE implementation to decide when to re-enter FBS.

    - If the FBS activation criteria is not met, the UE shall re-evaluate the activation condition and may re-enter FBS mode after a timer T2 expires.

- Recommended WF

  - [Moderator]: Check if it can be agreed NOT to define T2 for re-evaluation after the UE exits FBS mode.

    - If it’s agreed, discuss re-evaluation solutions for the below cases (after UE exists FBS mode):

      - Case 1: The FBS condition stops being met and is met again.

      - Case 2: The FBS condition remains met.

      - Case 3: The FBS condition is met and then becomes not met.

      - Case 4: The FBS condition remains not met.

  - Does not discuss reconfiguration by the network with the same or different triggering conditions, as it does not impact the RAN2/4 specifications.

Issue 1-1-2: Multi-band supported by FBS

- Option 1 (OPPO, Huawei, Ericsson): Do not define the requirements when multiple bands are involved and UE reports different FBS values for different bands.

- Option 2 (CATT, Nokia, Apple): For inter-frequency measurements and when serving carrier and target carrier are in different bands, use the FBS value reported for the band of the target carriers in the requirements.

- Recommended WF

  - [Moderator]: Suggest to agree on Option 1, since multi-band support is out of the scope of the agreed applicability.

    - Note: Requirement applicability is duplicated as follows:

The requirements related to the support of Rel-19 L3 fast Rx beam sweeping are applicable when - For intra-frequency measurements, the carrier configured for L3 SSB measurement in the FR2-1 band is the serving carrier, and - For inter-frequency measurements, in addition to the serving carrier, at most one carrier in the FR2-1 band is configured for L3 SSB measurement, and - UE is not configured with CA or DC, and - L3 fast Rx beam sweeping operation is activated.

Issue 1-1-3: Measurement gap types supported for FBS

- Option 1 (Apple, OPPO, ZTE, Ericsson): FBS applies for inter-frequency measurement with both per-FR measurement gap and per-UE measurement gap.

- Option 2 (Nokia, MTK): FBS applies for inter-frequency measurement with per-FR measurement gap.

- Recommended WF

  - [Moderator]: Check if Option 1 is agreeable.

Issue 1-1-4: Applicability of FBS when the target and serving carrier frequency are on different

- Option 1 (Apple): When the target and serving carrier frequency are different, FBS applies to target carrier only.

- Option 2 (Ericsson): We’re open to speeding up the serving carrier by FBS in the case that the target and serving carrier frequency applying FBS are on different.

- Option 3 (vivo): Consider introducing UE capability which indicate UE can support FBS on both target and serving carrier frequency.

- Recommended WF

  - [Moderator]: Check if Option 1 is agreeable.

### Issue 1-2: issues related with CSSF enhancement

Issue 1-2-1: Inter-frequency MOs without a measurement gap supported by CSSF solution 3

- Option 1 (Huawei, Apple): Not apply CSSF solution 3 for the case configuring inter-frequency MOs without a measurement gap.

  - Option 1a (MTK): Discuss whether to exclude the 3-searcher enhancement for inter-frequency measurement without MG scenario.

- Option 2 (Ericsson): Apply CSSF solution 3 for the case configuring inter-frequency MOs without a measurement gap.

- Recommended WF

  - [Moderator]: Check if Option 1 is agreeable.

Issue 1-2-2: CSSF solution 1 extension to inter-frequency neighbour cell

- Proposal 1 (Ericsson): RAN4 to confirm CSSF Solution 1 can be applied to an inter-frequency neighbor cell (without measurement gap) that is intra-band but not intra-frequency with the serving cells.

- Recommended WF

  - [Moderator]: Discuss Proposal 1.

### Issue 1-3: issues related with SCell activation

Issue 1-3-1: Detailed UE capability for SCell activation

- Proposal 1 (OPPO): The feature group for R19 SCell activation based on EMR is defined as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-x | Fast SCell activation based on early measurement report | Supports fast SCell activation specified in TS38.133 clause 8.3.2A | 39-8 or 39-9 in R18 RAN4 feature list or 18-7 (idleInactiveNR-MeasReport-r16) | UE cannot meet fast SCell activation requirements as specified in TS38.133 clause 8.3.2A |

- Proposal 2 (Huawei): The feature group for R19 SCell activation based on EMR is defined as below.

Table 5.1 UE capability for Rel-19 fast SCell activation based on EMR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-x | Fast SCell activation based on EMR measurement | Supports fast SCell activation based on EMR measurement as specified in TS38.133 clause 8.3.2A | 39-8 or 39-9 (RAN4 R18 feature list) | Yes | N/A | UE does not support fast SCell activation requirements as specified in TS38.133 clause 8.3.2A | Per-UE | No | No | N/A |  | Optional without capability signaling |

- Proposal 3 (Nokia): Add idleInactiveNR-MeasBeamReport-r16 as pre-requisite for the capability for FR1 and FR2. Add both measValidationReportEMR-r18 and measValidationReportReselectionMeasurements-r18 as pre-requisite.

- Proposal 4 (Qualcomm):  UE capability for R19 SCell activation based on EMR is defined as below:

  - The UE shall support measValidationReportEMR-r18 (per band) and idleInactiveNR-MeasReport-r16 (per UE).

    - For FR2, it shall additionally support idleInactiveNR-MeasBeamReport-r16 (per UE)

  - For EMR-based fast direct SCell activation, the UE shall support directMCG-SCellActivation (per UE with FR1-FR2 DIFF).

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-4 | Fast SCell activation based on early measurement report | Supports fast SCell activation based on early measurement reports as specified in TS38.133 clause 8.3.2A | measValidationReportEMR-r18 and idleInactiveNR-MeasReport-r16 For FR2, it shall additionally support idleInactiveNR-MeasBeamReport-r16 If Idle/Inactive mode inter-frequency measurement-based fast direct SCell activation, directMCG-SCellActivation | Yes | N/A | NW assumes that the SCell activation timeline follows the behavior defined prior to Release 19. | Per UE | N/A | No | N/A |  | Optional with capability signaling |

- Recommended WF

  - [Moderator]: Moderator suggests splitting the issue to two parts:

    - The texts EXECEPT FOR the column of prerequisite feature groups.  Moderator plan to use the below table (from Qualcomm) as reference for amendment in an off-line email discussion during the meeting week.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-4 | Fast SCell activation based on early measurement report | Supports fast SCell activation based on early measurement reports as specified in TS38.133 clause 8.3.2A |  | Yes | N/A | NW assumes that the SCell activation timeline follows the behavior defined prior to Release 19. | Per UE | N/A | No | N/A |  | Optional with capability signaling |

    - Prerequisite feature groups. Discuss the options as follow:

      - Option 1: The UE shall support 39-8 or 39-9 in R18 RAN4 feature list or 18-7 (idleInactiveNR-MeasReport-r16)

      - Option 2: The UE shall support 39-8 or 39-9 (RAN4 R18 feature list).

      - Option 3: The UE shall support idleInactiveNR-MeasBeamReport-r16 for FR1 and FR2, and support both measValidationReportEMR-r18 and measValidationReportReselectionMeasurements-r18

      - Option 4:  The UE shall support measValidationReportEMR-r18 (per band) and idleInactiveNR-MeasReport-r16 (per UE).

        - For FR2, it shall additionally support idleInactiveNR-MeasBeamReport-r16 (per UE)

        - For EMR-based fast direct SCell activation, the UE shall support directMCG-SCellActivation (per UE with FR1-FR2 DIFF).

### Draft CRs in this meeting

- Recommended WF

  - [Moderator]:  Check and provide comments directly during meeting.

| R4-2513478 | Huawei, HiSilicon | Draft CR on CSSF solution 3 |
| --- | --- | --- |
| R4-2513712 | Nokia, Nokia Shanghai Bell | Draft CR on CSSF solution 1 |
| R4-2514037 | ZTE Corporation, Sanechips | Draft CR on CSSF solution 1 |
| R4-2514038 | ZTE Corporation, Sanechips | Draft CR on FBS |
| R4-2514071 | Ericsson | Draft CR on FBS |

# Topic #2: RRM performance requirements of FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC (6.10.2.2)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513469 | Huawei, HiSilicon | Proposal 1: To unify Solution 3 test cases, the test set of Solution is able to refer to four cells: one PCell, one neighbour Cell of PCC or one inter-RAT E-UTRAN Cell, and two deactivated SCells. |
| R4-2513713 | Nokia | Observation 1: In legacy test cases, the measurement delay is verified by configuring event triggered measurement reporting on a neighbour cell and no CSSF is applied. Proposal 1: A scenario with at least two carrier frequencies in one FR2 band is required to verify the measurement delay with enhanced CSSF. Proposal 2: The test cases for enhanced CSSF are designed by configuring event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA). Proposal 3: To define sub-test to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per FR2 band. |
| R4-2514066 | Ericsson | Proposal 1: RAN4 to define a test to verify the measurement delay with enhanced CSSF when the network indicates the CC to be measured in the FR2 band. • In the test, the UE is able to measure the CC indicated by the network, among multiple CC(s) within the same band. |
| R4-2514403 | MediaTek inc. | Proposal 1: Not need to introduce additional sub-test to verify measurement delay with enhanced CSSF based on network indication. Proposal 2: Remove TC#3 for solution 3 since RAN4 agreed that 3 searcher solution for CSSF enhancement does not apply to Inter-RAT measurement without MG. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Issue 2-1: Test case design for FR2-1 L3 measurement delay reduction by optimizing CSSF

Recall the previous agreements on TC list for CSSF optimization as follow:

TC list for CSSF optimization solution 1 - Agreement TC index; Scenario; CA/DC Operation mode; Configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; Nokia. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; Ericsson. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; ZTE. Note: network indication is up to the other RAN2/4 core part discussion. In case network indication for a specific SCC measurement is replaced by other new solution (e.g., SCC selected by UE implementation), use the new solution for CA mode test.

TC list for CSSF optimization solution 3 Agreement on Monday: TC index; Aspects; CA/DC mode; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them.; Without DRX and without SSB index reporting; Ericsson. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them.; Without DRX and without SSB index reporting; OPPO. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them.; Without DRX and without SSB index reporting; Huawei.

Issue 2-1-1: Cell configurations in TCs for CSSF solution 1

- Proposal 1 (Nokia): Test set for CSSF solution 1 with the cell configurations including:

  - At least two carrier frequencies in one FR2 band

  - Event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA).

- Recommended WF

  - [Moderator]:  Check if Proposal 1 is agreeable.

    - Note: Moderator understands Proposal 1 doesn’t revoke the previous agreements as duplicated in the below:

      - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC

      - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC

Issue 2-1-2: Network indication in TCs for CSSF solution 1

- Option1 (Nokia, Ericsson): Define sub-test to verify measurement delay with enhanced CSSF based on network indication.

- Option 2 (MTK): Not need to introduce additional sub-test to verify measurement delay with enhanced CSSF based on network indication.

- Recommended WF

  - [Moderator]:  Discuss Option 1 and Option 2.

Issue 2-1-3: Cell configurations in TCs for CSSF solution 3

- Proposal 1 (Huawei): Test set for CSSF solution 3 with the cell configurations including four cells: one PCell, one neighbour Cell of PCC or one inter-RAT E-UTRAN Cell, and two deactivated SCells.

- Recommended WF

  - [Moderator]:  Check if Proposal 1 is agreeable.

    - Note: Moderator suggests proponents of Proposal 1 to check whether the previous agreements, which is duplicated in the blow, is impacted by Proposal 1:

      - (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands;

      - (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC;

      - (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC.

Issue 2-1-4: Cell configurations in TCs for CSSF solution 3

- Proposal 1 (MTK): Remove TC#3 for solution 3 since RAN4 agreed that 3 searcher solution for CSSF enhancement does not apply to Inter-RAT measurement without MG.

- Recommended WF

  - [Moderator]:  Suggest to agree on Proposal 1.

Issue 2-1-5: TC list for CSSF solution 3

- TC list in previous agreements:

| TC index | Aspects | CA/DC mode | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them. | Without DRX and without SSB index reporting | Ericsson |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them. | Without DRX and without SSB index reporting | OPPO |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them. | Without DRX and without SSB index reporting | Huawei |

- Recommended WF

  - [Moderator]:  Check that the TC list for CSSF Solution 3 contains TC1 and removes TC2 and TC3.

    - Note: Based on the agreement in the last meeting, 3 searcher solution for CSSF enhancement does not apply to Inter-RAT measurement without MG. And TC2 depends on the outcome of Issue 1-2-1.

### Draft CRs in this meeting

- Recommended WF

  - [Moderator]:  Check and provide comments directly during meeting.

| T-doc number | Company | Proposals / Observations | Moderator Comments on TC validity |
| --- | --- | --- | --- |
| R4-2513477 | Huawei, HiSilicon | Draft CR on Inter-RAT SSB measurement without MG test of 3-searcher solution for Rel-19 | The CR isn’t needed since CSSF solution 3 isn’t applicable for the case of inter-RAT measurement without MG. |
| R4-2513714 | Nokia, Nokia Shanghai Bell | Draft CR on test case for intra-frequency measurement with CSSF solution 1 |  |
| R4-2514043 | ZTE | Draft CR on Test case of Inter-RAT measurement without MG of R19 CSSF solution 1 |  |
| R4-2514068 | Ericsson | Draft CR on SSB based Inter-frequency measurement without MG with CSSF optimization solution 1 |  |
| R4-2514069 | Ericsson | Draft CR on SSB based Intra-frequency measurement without MG with CSSF solution 3 |  |
| R4-2513103 | OPPO | Draft CR on TC of inter-frequency measurement without MG with CSSF solution 3 | Depends on the outcome of Issue 1-2-1. |

# Annex (agreements from previous meetings)

## RAN4#110bis, April 2024

R4-2404841 Topic summary for [110bis][230]

R4-2406392 WF for [110bis][230]

Issue 1-1: Work plan for NR_RRM_Ph5 WI

- Work plan for whole WI in R4-2404368.

Agreement: The content of the work plan is agreeable.

Issue 2-2-1: Clarification on the bullets in WID for this CSSF optimization

Agreement:

Rel-19 discussion on CSSF optimization starts for the case UE is not capable of Rel-18 multi-Rx simulaeous reception, further discuss whether/how it can be applied to the case UE is capable of Rel-18 multi-Rx simulaeous reception but work in single-Rx currently.

Issue 2-2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Agreement:

  - Only consider CSSF outside MG case.

Proposal for further discussion:

  - Scenarios to use L3 measurement delay reduction by optimizing CSSF:

    - SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

    - SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter and TSSB_measurement_period_inter

## RAN4#111, May 2024

R4-2408025	Topic summary for [111][228]

R4-2410260	Way Forward for [111][228] NR_RRM_Ph5

Issue 2-1-1: Applicability requirement of L3 measurement delay reduction by optimizing Rx BSF

Applicability requirement:

Agreement:

Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that:

- the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and

- UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC.

Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

Issue 2-1-2: Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

  - FFS: UE is in RRC CONNECTED

- Agreement: Only support multi-Rx L3 measurement for CONNECTED UE

Issue 2-1-5: measurement performance requirement when apply L3 measurement delay reduction by optimizing Rx BSF

- Agreement:

  - RAN4 is not to change existing measurement performance requirement when consider optimization of Rx BSF in L3 measurement delay reduction.

Issue 2-2-6: measurement performance requirement when apply L3 measurement delay reduction by optimizing CSSF

- Agreement:

  - RAN4 is not to change existing measurement performance requirement when consider optimization of CSSF in L3 measurement delay reduction.

## RAN4#112, Aug 2024

| R4-2413876 | Ad-hoc minutes for NR_RRM_Ph5 WI | Apple |
| --- | --- | --- |
| R4-2414026 | WF for NR_RRM_Ph5_Part1 | Apple |

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor (8.15.2.1)

Applicability requirement:

Updated Agreement in online session:

Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that:

- the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and

- UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC.

Note: Target and serving carrier frequency can be the same or different.

Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

UE Power class:

Agreement in online session:

Baseline: RAN4 to consider UE supporting FR2-1 power class 3 as first priority.

Note: whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3. These extra FFS parts will NOT delay the WI completion.

Other clarification on WID:

Agreement in online session:

Remove {FFS: “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”}.

Issue 1-2-1: FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature

Agreement in online session:

    - The conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode. And it does not assume that the condition of in active mode is same as that for Rel-18 multi-Rx simultaneous reception.

Issue 1-2-2: FFS: UE’s mobility status, e.g., whether HST is precluded or not

Agreement in online session:

      - RAN4 to consider UE in non-HST case as first priority.

      - Note: whether or how HST case could use the outcome of the WI discussion can be FFS after concluding on non-HST case. These extra FFS parts will NOT delay the WI completion.

Issue 1-2-6: FFS: UE has prior knowledge on the cell to be measured

Agreement:

On top of the UE capability of supporting Multi-Rx, no additional conditions of prior knowledge for target cell is needed

Issue 1-2-7: FFS: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment

Agreement:

Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment

Issue 1-2-10: FFS: Simultaneous operation between L3 and L1 measurements

Agreement:

- Do not consider simultaneous Multi-Rx operation for both L1 and L3 measurement on the same symbols in this WI.

Issue 1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF

Which scenarios are considered to use L3 measurement delay reduction by optimizing Rx BSF:

  - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

  - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

  - Scenario 5: Handover

  - Scenario 6: PSCell addition

  - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

  - Scenario 8: SCell activation

  - Scenario 9: SCG activation

  - Scenario 10: CGI identification

  - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

Agreement:

  - Scenario 1/2/3/4/5/7 can be considered as first priority. After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the baseline scenarios 1/2/3/4/5/7, RAN4 can discuss whether and how the solutions(s) can be extended to the other scenarios. The discussion on other scenarios will NOT delay the WI completion.

Issue 1-6: scheduling/measurement restriction relaxation

Agreement:

- RAN4 to not consider simultaneous operation between L3 measurement and data reception on the same OFDM symbols.

Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC (8.15.2.2)

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Agreement in online session:

  - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting:

    - Aspect 1: SSB based Intra-frequency measurement without MG, including:

      - TPSS/SSS_sync_intra and TSSB_measurement_period_intra

      - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15

    - Aspect 2: SSB based Inter-frequency measurement without MG, including:

      - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

      - CSSFinter for inter-frequency measurement without gap.

    - Aspect 3: Inter-RAT SSB measurement without MG, including:

      - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement.

    - MG related features to be considered in aspect 1/2/3 including:

      - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP

      - R17 NCSG measurement with ‘nogap-noncsg’

      - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

Agreement:

    - RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i

    - Final applicability of the solutions can be FFS

      - EN-DC

        - EN-DC with FR2 only intra band CA

        - EN-DC with FR2 only inter band CA

        - EN-DC with FR1 +FR2 CA (FR1 PSCell)

        - EN-DC with FR1 +FR2 CA (FR2 PSCell)

      - NE-DC

        - NE-DC with FR2 only intra band CA

        - NE-DC with FR2 only inter band CA

        - NE-DC with FR1 +FR2 CA (FR1 PCell)

      - SA

        - FR1+FR2 CA (FR1 PCell)

        - FR1+FR2 CA (FR2 PCell)

        - FR2 only intra-band CA

        - FR2 only inter-band CA

      - NR-DC

        - FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell)

## RAN4#112bis, Oct 2024

| R4-2416853 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| --- | --- | --- |

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor (6.15.2.1)

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF:

Agreement in online session:

For scenario 1/2/3/4:

  - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions.

    - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions.

  - The detailed criteria/condition(s) is FFS.

  - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details.

For Scenario 5/7:

  - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch).

  - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” :

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 1-1-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 1-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF

Agreement:

- RAN4 is to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19.

  - The detailed definition can be postponed to the end of the core part discussion.

Issue 1-4-1: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC (6.15.2.2)

Issue 2-1: Clarification on the bullets in WID for this CSSF optimization

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF

Agreement in online session:

On top of the previous agreement on the baseline assumption of 2 searchers, for 3 searchers:

- Discuss the applicable scenario(s) and/or solution(s) based on 3 searchers. Decide whether or not to define the requirement later.

Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Candidate options for further discussion:

- Option 1: UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CCs to be measurement, further discuss option 1A and option 1B

  - Option 1A: Reuse the legacy principle for neighboring cell measurement.

  - Option 1B: Based on network indication

    - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band.

- Option 2: UE can change the searcher occupancy ratio of measurement among the measured CCs

  - Option 2A: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions

For 3 searchers: In addition to option 1 and 2

- Option 3: consider to use 3 searchers for CSSF reduction.

  - Depending on issue 2-4.

Issue 2-6: feature capability of L3 measurement delay reduction by optimizing CSSF

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 2-7: Other WID scope discussion

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

## RAN4#113, Nov 2024

| R4-2420093 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| --- | --- | --- |

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions):

Agreement:

- RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement.

Agreement:

- Not support UE mobility status based triggering condition in Rel-19.

Issue 1-1-3: searcher number assumption for RRM measurement with two panels activated on single carrier

Agreement:

Directly discuss the SSB processing time.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC

Agreement:

- For scenario 1/2/3/4

  - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement.

    - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms

    - where TSSB_processing = 2 ms

Agreement:

- For scenario 7

  - to consider an additional processing delay (2ms) after the last SMTC periodicity

Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX

Agreement:

L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case.

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Agreement:

For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability:

- FFS: details of the capability

Issue 1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF

Agreement:

- Discuss this capability detail by end of the core part

Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Issue 2-1: Clarification on the bullets in WID for this CSSF optimization

Agreement:

•	RAN4 to follow the revised WID (WID RP-242380) in RANP#105 meeting, and no need to have further discussion on this issue.

Issue 2-4-1: Tentative solution 1: measure one serving CC per band

[Wayforwad]:

FFS the following solution:

  - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured,

    - Reuse the legacy principle for neighbour cell measurement

      - Measure PCC if PCC in band or

      - Measure PSCC if PSCC in band or

      - Measure SCC with neighbor cell MO or

      - UE implementation

    - FFS whether indication is per-UE or per-band

      - Other options are not precluded.

  - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

[Wayforward]:

FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1:

  - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement.

    - FFS whether network indication is needed

      - Details is FFS

    - FFS on whether and how PCC and PSCC resource sharing is supported

Issue 2-4-3: Tentative solution 3: 3 searchers for CSSF

Agreement:

RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is:

    - FFS: FR1 scenario (CA and NR-DC) only.

    - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only.

    - FFS: All scenarios.

Issue 2-5: feature capability of L3 measurement delay reduction by optimizing CSSF for tentative solution 1 and 2

Agreement:

    - RAN4 to introduce new individual capability(s) for CSSF reduction in R19.

      - delay the capability detail discussion to the end of the core part.

Issue 2-7: Other WID scope discussion

Agreement:

-	after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable

## RAN4#114, Feb 2025

The following tdocs have been approved/endorsed in RAN4#114 meeting for core part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2502576 | Ad-hoc minutes for NR_RRM_Ph5 WI | Apple |
| R4-2502592 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2502662 | LS on CSSF optimization for NR RRM Phase 5 | Apple |

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Sub-topic 1-1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF

Agreement in ad-hoc session:

- Discuss if additional conditions are needed to activate FBS-based L3 measurements.

  - Do not consider additional conditions beside the agreements from RAN4#113 meeting.

Agreement in RRM session (Monday):

- Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold.

  - Further discuss whether to limit the serving cell to FR2 serving cell.

  - Further discuss whether to add the hysteresis and timeToTrigger parameters.

  - Note: the scenario related to single carrier or CA/DC is discussed in another issue.

- The signaling design is up to RAN2.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Agreement in ad-hoc session:

- UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4.

Further discuss this issue.

Issue 1-1-3: SSB processing delay/time for processing multiple beams received in a SMTC

Agreement in RRM session (Monday):

- No need to add additional processing time in the delay requirement for scenario 5.

Sub-topic 1-2 Requirement of L3 measurement delay reduction by optimizing Rx BSF

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Further discuss this issue.

Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

Further discuss this issue.

Issue 1-2-3: Requirement at transition between the normal L3 measurement and FBS L3 measurement

Agreement:

- Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

Sub-topic 1-3 CR split for FBS L3 measurement

| CR No. | Scenario | Clauses impacted | Volunteering company |
| --- | --- | --- | --- |
| 1 | 1. SSB based Intra-frequency measurement without MG | 9.2.5.1, 9.2.5.2 | CATT |
| 2 | 2. SSB based Intra-frequency measurement with MG | 9.2.6.2 | Apple |
| 3 | 2. SSB based Intra-frequency measurement with MG | 9.2.6.3 | MTK |
| 4 | 3. SSB based Inter-frequency measurement without MG | 9.3.9.1, 9.3.9.2 | Huawei |
| 5 | 4. SSB based Inter-frequency measurement with MG | 9.3.4, 9.3.5 | CMCC |
| 6 | 5. Handover | 6.1.1.4.2, 6.1.1.5.2 | Nokia |
| 7 | 7. RRC Re-establishment | 6.2.1.2.1 | ZTE |
| 8 | 7. RRC Connection Release with Redirection | 6.2.3.2.1 | OPPO |

Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Issue 1-1: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

- Option 1 (Apple, CMCC, HW, Ericsson, vivo): The enhanced CSSF apply to all the 3 aspects and do not define the priority.

- Option 2 (Nokia, ZTE, CATT, CTC, vivo): To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2) to simplify the CSSF optimization discussion.

Agreement:

  - Option 1 is agreed

Issue 1-3: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Issue 1-3-1: Tentative solution 1: measure one serving CC per band

Agreement:

- If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings,

  - Network indication of enable/disable this CSSF enhancement is per-UE.

  - Follow the following principle for serving and neighboring cell measurement

    - Measure PCC if PCC in band

    - Measure PSCC if PSCC in band

    - Measure SCC on which UE is configured to report SSB based measurements, if

      - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

- Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

- Note: Following the WID, the above agreement applies for the measurement without gap.

Issue 1-3-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF

Agreement:

Support Solution 3 in Rel-19:

- FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only.

  - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC).

  - For FR1+FR2 scenario, PCell is FR1

  - FFS whether NR-DC is included or not

Design for solution 3 (agreed in ad-hoc session)

- Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future.

  - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule,

    - the 3rd searcher resource sharing is same as the second searcher sharing.

Not support solution 2 in Rel-19.

Issue 1-3-4: Others

Proposals (Nokia):

- RAN4 to define optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

- The SCell activation delay requirement shall be improved considering the optimized measurement for CSSF reduction.

Agreement:

  - This issue is closed

## RAN4#114bis, Apr 2025

The following tdocs have been approved/endorsed in RAN4#114bis meeting for core part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2503530 | Test case (performance part) plan for R19 RRM Phase 5 WI | Apple |
| R4-2504916 | Ad-hoc minutes for NR_RRM_Ph5 | Apple |
| R4-2504899 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2504900 | WF on RRM requirements for NR_RRM_Ph5_Part2 | CATT |

Issue 0: Work plan of test case (performance part) for NR_RRM_Ph5

Agreement in online session: Approve the work plan in R4-2503530.

Issue 1-1: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

Agreement:

- the CSSF optimization can be applied to the CSSF tables for the related CA/DC scenarios.

  - For solution 1: CSSF tables for CA, NR-DC, EN-DC and NE-DC

  - For solution 3: CSSF tables for CA, NR-DC, EN-DC with FR1 NR only

- Details of whether/how the CSSF factor is modified can be left to CR discussion.

Issue 1-2-1: revision on solution 1 (measure one serving CC per band)

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503618.

Issue 1-2-2: Expand solution 1 to FR1 case

[Way forward]:

- FFS:

  - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band.

    - Option 1a (Nokia):

      - Expand the solution 1 to at least FR1 intra-band contiguous CA.

      - Details:

        - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately.

        - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA.

  - Option 2 (Huawei): Not to expand solution 1 to FR1 band.

  - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

Issue 1-3-1: whether solution 3 includes NR-DC scenario or not?

Agreement in online session:

- Solution 3 includes NR-DC of FR1 and FR1 + FR2

Issue 1-3-2: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC)

[Way forward]:

- FFS:

  - Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

    - Note: 3-searchers will be used by UE if UE report this capability.

- FFS: Other restriction (triggered by Huawei):

  - Option 1: NW can configure per-FR measurement gaps to UE when UE supports per-FR measurement gaps and new 3-searcehr capability simultaneously. But the SMTC occasions of outside gap measurement object in one FR should not be overlapped by per-FR measurement gaps in another FR. (HW, MTK, VIVO, Apple)

  - Option 2: When UE supports per-FR measurement gaps and 3-searcher capability simultaneously, no new requirements. (MTK, ZTE, Apple)

  - Option 3: UE is able to support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. (HW, MTK, Apple, VIVO, Ericssion, ZTE, CATT)

    - Option 3a: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. (HW, OPPO)

Issue 1-3-3: detailed CSSF table design for solution 3

Agreement in online session:

- This requirement applies regardless of whether measurement gap is configured or not.

Issue 1-4-1: UE capability for CSSF enhancement

Agreement:

- UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR.

- UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

CR splitting

- FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

CR; Company. CR for solution 1 (measure one carrier per-band); Apple. CR for solution 3 (3 searchers); Huawei.

Issue 2-1-1: TC list for Rx BSF optimization

Agreement in online session: Further discuss based on the following table:

| Index | Scenario | Need or not | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | CMCC |
| 3 | Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | CATT |
| 2 | Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | Huawei |
| 4 | Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | Samsung |
| 5 | Scenario 5: Handover | 1 or 2 TCs | TBA(e.g., Intra-frequency or inter-frequency, unknown) | OPPO |
| 6 | Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection | 1 or 2 TCs | TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown) | Intel |

Issue 2-2-1: TC list for CSSF optimization

- TC for solution 1:

Agreement in online session:

- Define the test cases for solution 1 of CSSF enhancement following the table below:

| TC index | Scenario | Need or not | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | Nokia |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | Ericsson |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | ZTE |

- TC for solution 3:

[Way forward]:

- FFS:

  - Define the test cases for solution 3 of CSSF enhancement following the table below:

| TC index | Aspects | CA/DC mode | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA | TBA |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA | TBA |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA | TBA |

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Sub-topic 1-1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF

Agreement in RRM session (Monday):

    - For the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell.

    - Leave the decision to RAN2. Further discuss the details for the LS to RAN2, and the LS can only be approved if the content is agreeable.

      - Details of the LS can refer to R4-2504962.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Further discuss this issue.

Issue 1-1-3: Status report of fast beam sweeping

Further discuss this issue.

Sub-topic 1-2 Requirement of L3 measurement delay reduction by optimizing Rx BSF

Issue 1-2-1: Applicability of L3 measurement delay reduction by optimizing Rx BSF

Further discuss this issue.

Issue 1-2-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Agreement in the ad-hoc:

- Introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability in issue 1-2-4.

Issue 1-2-3: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

Agreement in the ad-hoc:

  - For each scenario for PC3 UE, the described factors in the table should be replaced with capability value.

| Scenarios | Factors |
| --- | --- |
| 1 | Mpss/sss_sync_w/o_gaps = [reduced N]*3 Mmeas_period_w/o_gaps = [reduced N]*3 |
| 2 | Mpss/sss_sync_with_gaps = [reduced N]*3 Mmeas_period_with_gaps = [reduced N]*3 |
| 3 | Mpss/sss_sync_inter = [reduced N]*3 Mmeas_period_inter = [reduced N]*3 MSSB_index_inter = [reduced N]*3 |
| 4 | Mpss/sss_sync_inter = [reduced N]*5 Mmeas_period_inter = [reduced N]*5 MSSB_index_inter = [reduced N]*3 |
| 5 | If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then N in the HO requirement will be replaced by [reduced N] when the target cell is in FR2-1 and the PC3 UE supports [Rel-19 Rx FBS]. |
| 7 | RRC Re-establishment/redirection Can be discussed directly in the draft CR. |

Issue 1-2-4: Other aspects of UE capability design

Further discuss this issue.

Topic #2: Fast SCell activation for UE supporting Rel-18 EMR

Sub-topic 2-1 Core requirements of Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-1-1: Whether to define new UE capability

Further discuss this issue.

Issue 2-1-2: Update requirements of unknown PUCCH SCell activation based on EMR

Directly treat the CR (R4-2503864) online. No agreement is needed.

Sub-topic 2-2 Performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-2-1: Scenarios of test cases

Further discuss this issue.

Issue 2-2-2: Details of test case design

Further discuss this issue.

## RAN4#115 meeting, May 2025

The following tdocs have been approved/endorsed in RAN4#114bis meeting for core part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2508265 | Ad-hoc minutes for NR_RRM_Ph5 WI | Apple |
| R4-2508264 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2508266 | WF on RRM requirements for NR_RRM_Ph5_Part2 | CATT |

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Agreement:

For the quitting condition of FBS:

- Introduce a new UAI indicating “overheating” issue:

  - RAN4 will not discuss the UE behaviour after reporting UAI

- Introduce a timer-based solution:

  - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition

    - Further discuss the candidate numbers for T1.

Issue 1-1-2: Status report of fast beam sweeping

Agreement:

  - No need to define UE status report of fast beam sweeping.

Issue 1-1-3: UE capability for Rx BSF optimization

Agreement:

For FBS capability design:

  - UE capability is per-band and optional

  - TDD only

  - FR2-1 only

  - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

Issue 2-1-1: TC list for Rx BSF optimization (update based on last meeting agreement)

- Agreement:

| Index | Scenario | Number of TC | Configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 | - Non-DRX - In TC1/2/3/4, UE is only required to pass TC1+TC4 or TC2+TC3. | CMCC |
| 3 | Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter | 1 |  | CATT |
| 2 | Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 | - With DRX, DRX.1 for sub-test1 and DRX.7 for sub-test2 (same as in table A.7.6.1.2.1-2) - In TC1/2/3/4, UE is only required to pass TC1+TC4, or TC2+TC3. | Huawei |
| 4 | Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter | 1 |  | Samsung |
| 5 | Scenario 5: Handover | 1 | - Intra-frequency handover from FR2 to FR2 - Unknown target cell | OPPO |
|  |  | 1 | - Inter-frequency handover from FR2 to FR2 - Unknown target cell |  |
| 6 | Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection | 1 | - Inter-frequency RRC Re-establishment; - Unknown target cell - FR2 to FR2 | Intel |
|  |  | 1 | - RRC Connection release with redirection - FR2 to FR2 |  |

Issue 2-1-2: baseline setup for BSF test case

- Agreement:

  - For BSF optimization, for scenario 1/2/3/4, the existing test configurations in A.7.6.1 and A.7.6.2 can be used as baseline.

  - For BSF optimization, for scenario 5/7, the existing test configurations in A.7.3.1 and A.7.3.2 can be reused except that the test requirements need to be updated based on the reduced beam sweeping factor reported by UE.

Issue 2-1-3: whether or not to test FBS quitting

Agreement on Monday:

No need to test UE not using FBS for scenario 1~4 if the FBS condition is not met.

Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Issue 1-2-1: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC)

Agreement on Monday:

  - For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing.

Issue 1-2-2: applicability limitation for solution 3

Agreement in AdHoc:

  - For UE supporting both per-FR measurement gap and new 3-searcher capability,

    - If network configures per-FR measurement gap, UE will use legacy requirement (i.e., 2-searcher requirement) when at least one SMTC occasion of outside gap measurement objects in one FR are overlapped by per-FR measurement GAP in another FR. Otherwise, the 3-searcher requirement applies.

  - For UE supporting new 3-searcher capability,

    - If network configures per-UE measurement gap and does not configure per-FR measurement gap, the 3-searcher requirement always applies.

Issue 1-3-1: UE capability for CSSF enhancement option 1 (measure one carrier per band)

Agreement in AdHoc:

  - For CSSF enhancement solution 1 in Rel-19, a new optional per-UE capability, apply for FR2-1 only.

Note: the UE capability can be revisited if this feature is expanded to FR1 case.

Issue 1-3-2: detailed UE capability for CSSF enhancement

Please refer to the R4-2508264.

- General for solution 1 and 3

Issue 2-2-1: General assumption for TCs of solution 1 and 3

Agreement on Monday:

Not to define additional test cases for MG related features (i.e., NCSG with nogap-noncsg, Rel-18 Needforgaps with no-gap-no-interruption or no-gap-with-interruption) with solution 1 or solution 3 of CSSF optimization.

- TC for solution 1 (issue 2-2-2/3/4):

Issue 2-2-2: TC list for CSSF optimization solution 1

- Agreement

| TC index | Scenario | CA/DC Operation mode | Configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them | - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC | Nokia |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them | - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC | Ericsson |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them | - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC | ZTE |

Note: network indication is up to the other RAN2/4 core part discussion. In case network indication for a specific SCC measurement is replaced by other new solution(e.g., SCC selected by UE implementation), use the new solution for CA mode test.

Issue 2-2-3: baseline TC setup for CSSF optimization solution 1

- Agreement

  - For solution 1 of CSSF optimization, the legacy test configurations in A.7.6.1 and A.7.6.2 can be used as baseline.

Issue 2-2-4: other detailed TC design for CSSF optimization solution 1

- discuss this issue directly in draft CRs.

- TC for solution 3 (issue 2-2-5/6/7):

Issue 2-2-5: TC list for CSSF optimization solution 3

Agreement on Monday:

| TC index | Aspects | CA/DC mode | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them. | Without DRX and without SSB index reporting | Ericsson |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them. | Without DRX and without SSB index reporting | OPPO |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them. | Without DRX and without SSB index reporting | Huawei |

Issue 2-2-6: baseline TC setup for CSSF optimization solution 3

Agreement on Monday:

  - For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, for solution 3, existing L3 intra-frequency/inter-frequency measurement test cases for SA and EN-DC modes, e.g., A.5.6.1 and A.5.6.2, A.7.6.1 and A.7.6.2, can be used as the baseline for FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC.

Topic #3: Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-2-1: Test case list

Agreement:

  - Scenario:

    - Normal SCell activation for FR1 and FR2

    - Direct SCell activation at SCell addition for FR1

    - PUCCH SCell activation for FR1 and FR2

    - FFS:

      - Direct SCell activation at SCell addition for FR2

  - UE capabilities and network configurations:

    - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

    - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

    - FFS:

      - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

## RAN4#116 meeting, May 2025

The following tdocs have been approved/endorsed in RAN4#114bis meeting for core part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2512123 | Ad-hoc minutes for NR_RRM_Ph5 | Apple |
| R4-2512124 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2512125 | WF on RRM requirements for NR_RRM_Ph5_Part2 | CATT |

Topic #1: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC (7.14.3.2)

Issue 1-1: issues related with solution 1 (measure one serving CC per band)

Issue 1-1-1: LS reply to RAN2 (R2-2504728) on solution 1

Agreement:

Reply LS R4-2512161 has been agreed.

RAN4 would like to thank RAN2 for the LS on the CSSF optimization for NR RRM Phase 5. RAN4 had discussion on the alternatives from RAN2, and following conclusions have been made: - About the network indication of a specific SCC per-band for measurement, RAN4 agreed to adopt an alternative 3 as following: - If UE support R19 CSSF solution 1 (measurement one CC per FR2-1 band) and network enables this feature, - Follow the new RRC network indication (if present) to measure the indicated SCC in an FR2-1 band, if - Neither PCC nor PSCC is in the same FR2-1 band, and - The indicated SCC to be measured shall be one of the SCCs on which UE is configured to report SSB based measurements in the same FR2-1 band. - For case that only MO(s) are configured on SCC(s) in an FR2-1 band, when neither PCC nor PSCC is in the same FR2-1 band, and there is no SCCs on which UE is configured to report SSB based measurements in this FR2-1 band, UE will select and perform serving and intra-frequency neighbour cell measurements on one SCC in that FR2-1 band. SCC selection in this FR2-1 band to measure is up to UE implementation. RAN4 kindly requests RAN2 to implement RRC signalling enabling network to indicate which SCC the UE shall measure in an FR2-1 band.

Issue 1-1-2: revision on solution 1 according to issue 1-1-1

Discuss in draft CR

Issue 1-1-3: Expand solution 1 to FR1 case

Agreement:

Not extend the solution 1 to FR1.

Issue 1-2: issues related with solution 3 (3 searchers)

Issue 1-2-1: applicability limitation for solution 3

Agreement:

For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG (for the scenario of NR SA) that are being measured outside of MG.

FFS: For UE supporting [new 3-searcher capability], whether or not the Enhanced CSSFoutside_gap,i scaling factor applies for the inter-frequency MOs without MG that are being measured outside of MG. This part can be discussed in maintenance stage.

Issue 1-3: issues related UE capability for CSSF enhancement

Issue 1-3-1: detailed UE capability for CSSF enhancement

Agreement:

Solution 1 (measure one CC per band):

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-3 | L3 serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap | Support of serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured in a same band. |  | Yes |  | UE does not support serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured in a same band. | Per-UE | No | FR2-1 only |  | UE indicating this capability shall meet the corresponding enhanced requirements in TS38.133 Clause 9.2.3.2, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.3, and 9.1.5.1.4 | Optional with capability signaling |

Solution 3 (3 searchers):

This capability has been agreed in last meeting, and following is just to remove the square bracket

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-1 | Simultaneous L3 measurement on three carriers for measurements without measurement gap under CA/DC operation. | Support of measuring serving cell and neighbor cells measurement on three carriers simultaneously for measurements without measurement gap. Support of separate indications for different cases: {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA PCell is FR1 only}, and {FR1+FR2 NR-DC PCell is FR1 only}. |  | Yes |  | UE does not support measuring serving cell and neighbor cells on three carriers simultaneously for measurements without measurement gap. | Per-UE | No | No |  | UE indicating this capability shall meet the corresponding enhanced requirements defined in TS38.133 Clause 9.1.5.1.1, 9.1.5.1.2, and 9.1.5.1.3 if the feature is supported. | Optional with capability signaling |

Topic #2: RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode (7.14.5)

Issue 2-1: Test case design for FR2-1 L3 measurement delay reduction by optimizing Rx BSF

Issue 2-1-1: baseline setup for BSF test case

- Proposal 1(Samsung):

  - For the AoA setup of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, RAN4 to discuss whether AoA Setup #5 can be considered.

  - To define the new test case of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, the test requirement should be defined by using serving cell’s quality based triggering condition

    - How to define the test parameters involving hysteresis and the TTT/evaluation period of the timer can wait for further solid conclusions.

- [Way forward]

  - Continue discussion in RAN4#116bis.

Issue 2-1-2: whether or not to test FBS quitting

- Option 1(CMCC): it is proposed to discuss whether to test FBS quiting when FBS conditions are met, e.g. whether to test that UE fallback to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition.

- [Way forward]

- Continue discussion in RAN4#116bis.

Issue 2-2: Test case design for FR2-1 L3 measurement delay reduction by optimizing CSSF

TC for solution 1:

Issue 2-2-1: baseline TC setup for CSSF optimization solution 1

- Proposal 1(Nokia): A scenario with at least two carrier frequencies in one FR2 band is required to verify the measurement delay with enhanced CSSF.

- Proposal 2 (Nokia): The test cases for enhanced CSSF are designed by configuring event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA).

- [Way forward]

  - Continue discussion in RAN4#116bis.

Issue 2-2-2: sub-TC for CSSF optimization solution 1 with network indication

- Option 1(Apple): for CSSF enhancement solution 1, if the network indication for a specific SCC measurement is replaced by “SCC selection by UE implementation”, remove the TCs related with network indication in last meeting.

- Option 2 (Nokia): To define sub-test to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per FR2 band.

- [Way forward]

  - Continue discussion in RAN4#116bis.

Issue 2-2-3: TC for CSSF optimization solution 1 in FR1

- Proposal 1(Nokia, CMCC):

  - To define the test cases to verify the measurement delay by measuring one SCC per FR1 band, if this would be agreed in core requirement discussion.

- Agreement:

  - This issue is closed since issue 1-1-3 agreed to not extend the solution 1 to FR1

TC for solution 3:

Issue 2-2-4: baseline TC setup for CSSF optimization solution 3

- Proposal 1(Huawei):

  - To unify the test set of Solution 3 test cases, the test set of Solution is able to refer to four cells: one PCell, one neighbour Cell of PCC or one inter-RAT E-UTRAN Cell, and two deactivated SCells.

- [Way forward]

  - Continue discussion in RAN4#116bis.

Topic #1: RRM core requirements for FR2-1 L3 measurement delay by optimizing Rx BSF

Sub-topic 1-1 RRM core requirements for FR2-1 L3 measurement delay by optimizing Rx BSF

Issue 1-1-1: Value for T1

Agreement:

A fixed value of 90 seconds.

Issue 1-1-2: Re-evaluation mechanism

Agreement:

FFS following options in the maintenance stage:

- Option 1: Introduce a predefined time duration T2, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. Value of T2 can be:

  - Alt 1: Use the values for T1 as a starting point.

  - Alt 2: 5 s, which is predefined in RAN4 spec.

- Option 2: It is up to UE implementation when to re-enter the FBS after quitting the last FBS.

- Option 3: It is up to UE implementation when to re-evaluation of the activation condition for FBS after quitting the last FBS.

Agreement in Friday online:

- No consensus to introduce a predefined time duration T2 in the WI core part in RAN4.

Issue 1-1-3: UE capability for Rx BSF optimization

Agreement:

The following UE capability is defined:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-2 | Fast Rx beam sweeping factor for FR2-1 L3 measurement delay reduction | Supports fast Rx beam sweeping factor reduction for L3 measurement for FR2-1. |  | Yes | N/A | UE does not support fast Rx beam sweeping for FR2-1 L3 measurement delay reduction. | Per band | TDD only | FR2-1 only | N/A | Candidate values: {2, 4, 6} for FR2-1. Note: It is only supported for power class 3. | Optional with capability signalling |

Issue 1-1-4: Requirements when UE reports different BSF values for different bands

Way forward:

Further discuss this issue in the maintenance stage:

- Option 1:

  - It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured.

Issue 1-1-5: Revision of the agreement on applicability requirement

Agreement in Tuesday ad-hoc:

- The FBS enhancement applies for the case when the target and serving carrier frequency are same.

- FFS in maintenance which CC to apply the FBS when the target and serving carrier frequency are on different.

Agreement in Thursday online:

- When the target and serving carrier frequency are different,

- FBS requirement does not apply to inter-frequency measurement without measurement gap.

- FFS inter-frequency measurement with per-FR measurement gap.

- Note: inter-RAT is not in the scope.

Topic #2: RRM requirements of Fast SCell activation for UE supporting Rel-18 EMR

Sub-topic 2-1 Core requirements of Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-1-1: UE capability for EMR-based fast SCell activation

Agreement in Friday online:

- Introduce one single UE capability for R19 SCell activation based on EMR, further discuss the detail.

Sub-topic 2-2 Performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-2-1: Test case list

Further discuss this issue taking the options in the summary R4-2509055 into consideration.

Issue 2-2-2: Applicability rules of test case

Further discuss this issue taking the options in the summary R4-2509055 into consideration.

Issue 2-2-3: Test case design

Further discuss this issue taking the options in the summary R4-2509055 into consideration.

Issue 2-2-4: Report mechanism of EMR results in the test cases

Further discuss this issue taking the options in the summary R4-2509055 into consideration.