3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2514637
Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:			8.1

Source:	Samsung

Title:	WF on 6G AI

Document for:	Approval

# Topic#1: General scope and timeline

##### Issue 1-1: RAN4 6G AI/ML study scope

Agreement:

- In 6G AI/ML study, the detailed RAN4 scope shall include

  - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

    - RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility. The detailed principles are captured under 3-1.

    - Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance.

  - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

    - LCM procedures: RAN4 can study relevant RAN4 requirements for LCM procedures. If the use cases are identified by other WG, RAN4 related study should be based on the agreements and/or requests by other WG. The exact LCM context will be separately discussed and decided in the responding WGs.

    - RAN4 may study the feasibility and testability of the AI/ML functionalities. The study may include different aspects. It will be further decided how to manage the discussion under different agenda items, e.g. AI or testability.

##### Issue 1-2: RAN4 6G AI/ML study timeline

Agreement:

- RAN4 timeline for Rel-20 6G AI/ML study

  - Stage 1: Q4'25 and Q1'26

    - To clarify study scope for RAN4 AI/ML framework

    - To select the top-prioritized RAN4-led AI/ML use cases, with the detailed evaluation plan concluded for Stage-2

  - Stage 2: Q2’26 to Q2’27

    - To analyse and conclude RAN4 AI/ML framework (including RAN4 aspects for LCM procedures and other RAN4 specific issues)

    - To study comprehensively the RAN4-led use case(s) selected in Stage 1.

      - Other new use case(s) are not precluded depending on the consensus and the limited RAN4 bandwidth

    - Based on the agreement and/or the request from other WG, RAN4 will study the selected other WG-driven use cases with RAN4 analysis and evaluation. RAN4 related bandwidth limitation should be taken into consideration.

# Topic#2: RAN4 AI/ML framework

##### Issue 2-2: Post-deployment enhancement

Agreement:

- RAN4 will discuss the potential post-deployment testing enhancements scope in 6G

##### Issue 2-4: Model generalization

Agreement:

- For AI/ML functionality generalization, RAN4 shall study generalization of UE and BS based AI/ML functionality for 6G and identify the potential standard impact.

  - The study will begin no early than Q2 2026.

  - Study and identify the typical parameters/aspects that are expected to be generalized by AI/ML functionality for each use case firstly, and then discuss how to test the generalization for the corresponding use case.

  - RAN4-led use cases will be prioritized.

  - The related study on other WG-led use cases will be based on the requests from the corresponding WG.

##### Issue 2-6: AI/ML UE category

Agreement:

- Regarding metrics related to AI/MI complexity supported by UE/NW, it can be discussed if a study is needed in 6G no earlier than Q2 2026.

  - The definition and the context of AI/MI complexity needs to be further clarified if the study is agreed.

# Topic#3: RAN4-driven AI/ML use case (General Issues)

##### Issue 3-1: Principle for AI/ML use cases selection

Agreement:

- Principles for AI/ML RAN4-driven use cases selection

  - RAN4 shall closely coordinate with other WG and strive not to duplicate the efforts

    - RAN4 should work with other WG to clarify the split between RAN4 driven AI/ML work and RAN1/2/3 driven work.

    - Coordinated discussions with RAN1/2/3 are required to avoid overlapping scope.

  - Typical RAN4-driven use case shall

    - ensure performance gain or gain from other aspects, by focusing on some RF/Demod/RRM topics

  - Prioritize the use case by taking into account:

    - use cases with gain over non-AI counterpart

      - gain may include performance gain or gain from other aspects, like overhead reduction.

      - trade-off between performance and complexity/overhead should also considered.

    - use cases with potential standard impact.

    - Note: the prioritization principle is the general principle and no intention to have implication on RAN4 6G AI work plan.

##### Issue 3-2: Aspects of RAN4-driven use case to be studied

Agreement:

- Aspects of RAN4-driven use case to be studied

  - By following RAN1 agreement, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following:

    - Definition of each (sub-)use case, including at least AI model input/output

    - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results

    - Assumption on training types, e.g.,

      - offline training, online training, finetuning

      - Label construction (if applicable), including whether/how to obtain label data for model training

    - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model

    - Collaboration/interaction between UE and NW, e.g.,

      - no collaboration/interaction

      - UE/Network collaboration targeting at separate or joint ML operation

    - High level potential specification impact

  - Additionally, the following RAN4 aspect(s) are encouraged to study:

    - Feasibility issues, including complexity, and other aspects related to implementation

    - Other aspects identified by companies are not precluded.

- Here is the exemplary template for company to study/report the details of one certain use case:

| Sub-use case X |  | Company-A | Example |
| --- | --- | --- | --- |
| AI model input | Input in training |  | e.g., estimated DMRS |
|  | Input in inference |  | e.g., estimated data signal |
| AI model output | Label in training (if applicable) |  | e.g., original DMRS |
|  | Output in inference |  | e.g., corrected signal from non-linearity |
| Assumption on training | Training type |  | e.g., offline training, online training, finetuning, etc. |
|  | Label construction (if applicable) |  | whether/how to obtain label data for model training |
| model location for inference |  |  | e.g., UE-sided model, NW-sided model, and two-sided model |
| Collaboration/interaction between UE and NW |  |  | e.g., no collaboration/interaction, UE/Network collaboration targeting at separate or joint ML operation |
| Evaluation methodology |  |  | e.g., system level simulation, link level simulation, with detailed description |
| Evaluation assumption |  |  | Detailed table can be provided for evaluation assumption |
| Evaluation KPI |  |  | e.g., Tput |
| Evaluation benchmark |  |  | Description of non-AI solution to be compared with |
| Preliminary evaluation results |  |  | Detailed results can be provided. |
| High level potential specification impact |  |  | e.g., impacted existing clause, potential new requirement, impacts to other WG’s specifications |
| Feasibility issues, including complexity, and other aspects related to implementation |  |  | e.g., description of model complexity, other implementation complexity etc. |

# Topic#4: AI/ML use case for RF issues (Per-use case discussion)

### Use Case#1: AI-nonlinearity compensation

##### Issue 4-1b: Sub use case definition and selection for AI-DPD/DPoD

Agreement:

- The scope of potential RAN4-led AI use case, AI-DPD/DPoD, will be further discussed in the next meeting. The candidate sub-use cases include

- UL:

  - UE: AI-DPD

  - NW: AI-DPoD

- DL:

  - NW: AI-DPD

  - UE: AI-DPoD

- To facilitate the 6G AI scope discussion, the interested companies are encouraged to timely provide more analysis, including the performance benefit, spec impact, the assumption behind their analysis, on the specific sub-use case(s).

For information:

- The following information is provided by vivo to clarify the proposed of UE AI-DPD for UL

| AI model input | Input in training | Time domain samples after PA distortion |
| --- | --- | --- |
|  | Input in inference | Time domain samples before pre-distortion |
| AI model output | Label in training (if applicable) | Time domain samples w/o distortion |
|  | Output in inference | Option1：Time domain samples after pre-distortion Option2：LUT coefficients |
| Assumption on training | Training type | Offline training；Online finetune |
|  | Label construction (if applicable) | Time domain samples w/o distortion |
| model location for inference |  | UE-sided |
| Collaboration/interaction between UE and NW |  | As for UE side model |
| Evaluation methodology |  | link level simulation |
| Evaluation assumption |  | Model PA model; Parameter. yn= k∈Kpakx(n)x(n)2k; ak; k∈[0,1,…,7]: [-0.618347-0.785905i; 2.0831-1.69506i; -14.7229+16.8335i; 61.6423-76.9171i; -145.139+184.765i; 190.61-239.371i; -130.184+158.957i; 36.0047-42.5192i]. |
| Evaluation KPI |  | BLER, EVM, MPR |
| Evaluation benchmark |  | No DPD |
| preliminary evaluation results |  | 1-3 dB |
| High level potential specification impact |  | 1. RF requirements 2. Dedicated signal (RS/data) for training or monitoring data collection. |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Inference complexity: Option1：38.5 MFlops（per slot）,1K parameters in total Option2: similar as LUT-based DPD |

For information:

- The following information is provided by Samsung to clarify the proposed of NW AI-DPoD for UL.

| AI model input | Input in training | The estimated RS signal time domain sample with 1st, 3rd terms |
| --- | --- | --- |
|  | Input in inference | The estimated data signal time domain sample with 1st, 3rd terms |
| AI model output | Label in training (if applicable) | original/known RS signal |
|  | Output in inference | data signal with non-linear distortion compensated |
| Assumption on training | Training type | Online training/finetune |
|  | Label construction (if applicable) | DMRS with 1st, 3rd terms |
| model location for inference |  | NW-sided model |
| Collaboration/interaction between UE and NW |  | N/A |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameters; Value. Carrier frequency; 3.5 GHz. # of TX layers/users; 1/2. # of RX antennas; 4/8/64. Waveform; NR based. # of PRB; 50. # of symbol per slot; 12. # of DMRS per slot; 2. # of DMRS re-used for training; 1. DMRS subcarrier position; Odd index (0 for even index). FFT size; 2048. CP length; 144. Modulation; 256 / 1024/ 4096 QAM. Coding rate; LDPC 0.9395. Transport Block size; 45120 for 1Tx/256 QAM 56392 for 1Tx/1024 QAM 67680 for Tx/4096 QAM 90200 for 2Tx/256 QAM. Channel model; TDL-A 30ns. Subcarrier spacing; 15 kHz. Evaluation metric; BLER. Benchmark; Conventional receiver (without non-linearity compensation). |
| Evaluation KPI |  | BLER, MPR, EVM, throughput |
| Evaluation benchmark |  | Legacy receiver without DPoD Receiver with non-AI DPoD |
| preliminary evaluation results |  | Detailed results can be provided. |
| High level potential specification impact |  | UE capability report on supported MPR relaxing RAN4 measurement requirement on MPR delta |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Performance benefit: AI-NC provides 1-2dB improvement at 10% BLER when using 256QAM, 1024QAM and 4096QAM for the given EVM values. |

##### Issue 4-1c: Evaluation for AI-DPoD

Agreement:

- Non-linearity model(s) of transmission signals:

  - Discussed in other 6G agenda (e.g., system parameter agenda and UE RF agenda), which is intended to discuss the common model as much as possible.

    - Using separate models for AI and non-AI DPoD is not precluded.

- Evaluation Procedures and some detailed discussion points

  - Step-1: Non-linearity model(s) of transmission signals is determined to be used in the evaluation.

  - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

    - FFS a joint session or separate session of AI and non-AI non-linearity compensation.

    - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

      - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA impairment including PA and other RF components non-linearity.

      - FFS suitable AI/ML model, type of training.

      - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs).

      - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations

      - Focusing on higher modulation orders focusing on high-SNR conditions

      - Other details are not precluded

  - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation.

    - The potential MPR reduction enabled by AI-DPoD is evaluated.

      - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

    - The impact on UL coverage and UE transmit energy efficiency is evaluated.

- Other design principle to be considered:

  - UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

##### Issue 4-1d: Evaluation for AI-DPD

Agreement:

- Non-linearity model(s) of transmission signals:

  - Discussed in other 6G agenda, which is intended to discuss the common model as much as possible.

# Topic#5: AI/ML use case for Demod (Per-use case discussion)

### Use Case#2: AI-based channel estimation

Agreement:

- Postpone RAN4 discussion on AI-based channel estimation including both sub-use cases (AI-based channel estimation with DMRS reduction and AI-based channel estimation with CSI-RS reduction)

  - If triggered by RAN1, RAN4 will start the study to provide RAN4 analysis and evaluation relevant to implementation and performance.

### Use Case#3: AI-SRS-assisted channel reconstruction

##### Issue 5-2a: Use case definition and supporting level from companies

Agreement:

- The scope of potential RAN4-led AI use case of AI-SRS assisted channel reconstruction, will be further discussed in the next meeting.

For information:

- The following information is provided by Huawei to clarify the proposed of AI-based SRS IL compensation.

| AI model input | Input in training | UL measured channel matrix from SRS with IL imbalance |
| --- | --- | --- |
|  | Input in inference | UL measured channel matrix from SRS with IL imbalance |
| AI model output | Label in training (if applicable) | UL SRS measurement without IL (assuming it is compensated by UE at certain conditions) or DL CSI-RS measurement |
|  | Output in inference | DL channel matrix with IL compensated |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | Channel model: TDL-A XPL medium Training/validation/testing Dataset construction: 100000TTI * 80% (TDD pattern 8：2) * 68 subband (4 RBs per subband) = 5.44M - For training：4.352M（5.44M * 70%） - For validation：0.544M（5.44M * 10%） - For testing：1.088M （5.44M * 20%） |
| model location for inference |  | NW-sided AI |
| Collaboration/interaction between UE and NW |  | 1. Inference: UE reporting on the range of SRS IL for model selection and model monitoring (ensuring generalization/consistency between training and inference) 2. Data collection/monitoring: UE reporting of channel matrix with DL CSI-RS measurement |
| Evaluation methodology |  | Parameters; Values. Carrier frequency/BW; 5GHz/100MHz/272RBs. CSI-RS ports; 32. Tx/Rx number @UE; 1T4R. Channel model; TDL-A XPL medium. Doppler; 1Hz. TDD ULDL pattern; DDDDDDDSUU. Power imbalance distribution; [0, -2.5, -5, -7.5]±1dB. SRS pattern; 4 frequency hopping, 40ms between two SRS hopping. SRS pattern: |
| Evaluation assumption |  | A common AI model is trained which applies to each subband. The model input/output is the channel matrix from one subband. Channel aging issue is not considered in this use case, hence the Doppler valued is set as 1Hz. |
| Evaluation KPI |  | SGCS Steps for SGCS calculation during model inference - Step-1: For each subband, after collecting SRS measurements from each SRS port (i.e., from each Rx antenna), using AI to compensate the IL loss and generate the compensated channel matrix. - Step-2: For each D slot, compare the genie-aided channel matrix at this D slot with the compensated channel matrix based on the nearest SRS measurements. |
| Evaluation benchmark |  | 1) SRS without IL imbalance; 2) non-AI based SRS IL imbalance compensation |
| preliminary evaluation results |  | • AI/ML based compensation outperforms non-AI/ML based compensation across the entire SNR range, with more obvious performance gains observed at medium to low SNRs. • AI/ML is capable of compensating without the exact values – even with random IL variation, only minor performance loss is observed for the generalized model compared with the scenario-specific model trained with exact IL values. Note: Refer to R4-2513310 for more detailed results. |
| High level potential specification impact |  | At least UE reporting on SRS IL range. Potential UE RF related testing. |
| Feasibility issues, including complexity, and other aspects related to implementation |  | CNN-based AI model |

For information:

- The following information is provided by Samsung to clarify the proposed of AI-based SRS-assisted channel reconstruction for both NW-sided model and two-sided model.

|  |  | AI-based SRS assisted channel reconstruction Network-side model | AI-based SRS assisted channel reconstruction Two-sided model |
| --- | --- | --- | --- |
| AI model input | Input in training | CSI feedback and SRS measurement | Explicit channel feedback with spatially sub-sampled CSI measurement SRS measurement for decoder input beside the compression CSI |
|  | Input in inference | CSI feedback and SRS measurement | Explicit channel feedback with spatially sub-sampled CSI measurement SRS measurement for decoder input beside the compression CSI |
| AI model output | Label in training (if applicable) | Ideal CSI with full channel dimension | Ideal CSI with full channel dimension |
|  | Output in inference | The reconstructed CSI with full channel dimension | The reconstructed CSI with full channel dimension |
| Assumption on training | Training type | Offline | Offline |
|  | Label construction (if applicable) | Ideal channel estimation | Ideal channel estimation |
| model location for inference |  | Network-sided | UE and NW sided model |
| Collaboration/interaction between UE and NW |  | Yes | Yes |
| Evaluation methodology |  | Link level simulation | Link level simulation |
| Evaluation assumption |  | Parameter; Value; Parameter; Value. M (measured CSI-RS ports); 64; Channel; UMa Dense-urban. N (NW antenna ports); 256; SRS measurement interval; 30ms. Nr (number of UE receive ports); 4; Mean power imbalance (4 antennas); [0 -3 -6 -9]dB. Carrier frequency/BW; 7 GHz/ 20MHz 13 SBs; Power imbalance distribution; Uniform [-1,1] dB centered on the mean.. |
| Evaluation KPI |  | SGCS, Tput | SGCS, Tput |
| Evaluation benchmark |  | SRS only | CSI feedback only with two sided-model |
| preliminary evaluation results |  |  | The proposed wo-sided model-based approach achieves a significant gain, e.g., close to 100% SGCS gain for layer 1, as compared to the impaired SRS measurement |
| High level potential specification impact |  | Interoperability and testability LCM procedure: including data collection, performance monitoring, and model interference pending the assumption of AI/ML receiver |  |
| Feasibility issues, including complexity, and other aspects related to implementation |  |  |  |

### Use Case#4: AI-enabled PRACH receiver

##### Issue 5-3a: Use case definition and supporting level from companies

Agreement:

- The scope of potential RAN4-led AI use case of AI-enabled PRACH will be further discussed in the next meeting.

For information:

- The following information is provided by Samsung to clarify the proposed of AI-PRACH receiver.

| AI model input | Input in training | Detected signal with PDP for each detection window |
| --- | --- | --- |
|  | Input in inference | Detected signal with PDP for each detection window |
| AI model output | Label in training (if applicable) | Known whether there is preamble transmission for each detection window |
|  | Output in inference | Whether there is preamble detected or not |
| Assumption on training | Training type | offline |
|  | Label construction (if applicable) | Known preamble transmission for each detection window |
| model location for inference |  | NW-sided model |
| Collaboration/interaction between UE and NW |  | N/A |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameter; Values. Channel type; AWGN/TDLC300-100. PRACH format; C0. Ncs; 0/23. Logical index; 0/1. Preamble ID; 0/5. Number of Preamble; 64. Number of UE; 1/2. Doppler Shift; 2334Hz for restrict set B. Number of Tx/Rx; 1/2. Timing offset between 2UE; 20samples. |
| Evaluation KPI |  | Miss detection rate and false detection rate |
| Evaluation benchmark |  | Legacy receiver with threshold-based method |
| preliminary evaluation results |  | For multi-UE scenario, when the two UE have equal power, the corresponding SNR in AWGN is -8dB and in TDLC300-100 it is -2 dB, with the probability of correct multi-user detection being 99.96% and 97.06%, respectively. When the power difference between the two user devices is 3 dB, the probability of multi-user detection reaches 95.6%. For single UE scenario, the proposed AI based solution can also achieve the outstanding performance for single UE detection under non-HST scenario, almost 2dB gain compared with conventional threshold-based approaches And also, for HST scenario, the outstanding performance can be achieved with around 2dB gain under high-speed scenario in the low SNR region |
| High level potential specification impact |  | 1. RAN 4 requirements including both demodulation requirement and RRM requirement for random access 2. LCM procedure, data collection, performance monitoring, and model interference pending on the assumption of AI/ML receiver |
| Feasibility, complexity/cost |  | Comparable with conventional threshold-based approaches |

# Topic#6: AI/ML use case for RRM (Per-use case discussion)

### Use Case#5: AI-based measurement prediction

##### Issue 6-1c: Additional principle/method for AI-RRM use case selection

Agreement:

- Additional principle/method for AI-RRM use case selection

  - Whether the proposed AI-RRM use case should be driven by RAN4:

    - Existing 5G-A use cases included in RAN2 and RAN1 WIs should be precluded in RAN4 6G study.

    - Coordination with RAN1 and RAN2 when potential AI-RRM use cases are agreed in RAN4 to avoid overlapping scope.

##### Issue 6-1a: Use case definition and supporting level from companies

Agreement:

- The scope of potential RAN4-led AI-RRM use case(s) will be further discussed in the next meeting.

For information:

  - Below table is provided to summarize AI-RRM proposals in RAN4#116bis.

  - For each use case, preference/opposition from companies will be collected in future meeting for down-selection.

| Index | use cases | Model location (Training method) | Proposed by companies | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- |
|  | L1 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Spatial domain (inter-cell) - Time and spatial domain (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c), Nokia ((d, e), as RAN1-led study), Apple (c, d, e) |  |  |  |
|  | L3 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Time and freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) - Freq. domain prediction for N carriers single cell | NW and UE sided model (Offline training) | OPPO (a, b, c, e), Apple (c,d, e), CMCC (e), ZTE (e), Samsung (e, f), Nokia (c, d, e) |  |  |  |
|  | L3 cell measurement: - Time domain (intra-cell) - Freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c) Apple (c), Nokia (c) |  |  |  |
|  | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) | Apple, vivo, Samsung |  |  |  |
| 5a (5+6 combined) | Dynamic Adaptation of Measurement Procedure | UE sided model | MediaTek, Qualcomm, Apple |  |  |  |
| 7a (7+8 combined) | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | Apple (L1 for scheduling restriction and delay reductions), vivo Qualcomm, Apple, CMCC, Xiaomi |  |  |  |
|  | AI/ML based RLM/BFD/CBD, LTM by measurement prediction | UE-sided model | OPPO, vivo (AI for RLM) |  |  |  |
|  | AI/ML based SCell related enhancement by measurement prediction | UE-sided model | OPPO |  |  |  |
|  | AI-based prediction for enhanced s-measure mechanism | UE-sided model | vivo |  |  |  |
|  | AI/ML based AGC through temporal/frequency domain prediction | UE-sided model | ZTE |  |  |  |
|  | AI/ML based DL sync through temporal/frequency domain prediction | UE-sided model | ZTE |  |  |  |
|  | AI abstraction of the wireless environment to facilitate data scheduling and network power saving | NW-sided model | CATT |  |  |  |

## Reference for RAN4#116bis (Oct. 2025, Prague)

R4-2513031		Discussion on AI/ML-Based 6G RRM Use Cases				Apple

R4-2513042		On RAN4-driven AI/ML aspects of 6GR						Qualcomm Technologies Ireland

R4-2513049		Discussion on AI framework and use cases for 6GR			Samsung

R4-2513060		RAN4 study on 6G AI/ML										Ericsson AB.

R4-2513072		6G Considerations on AI										Nokia

R4-2513079		Views on 6G AI/ML												MediaTek Inc.

R4-2513125		Discussion on AI/ML for 6GR study							CMCC

R4-2513229		Discussion on RAN4 driven AI use cases						CATT

R4-2513283		Overview for 6GR RAN4 centric AI use cases 				Xiaomi

R4-2513286		Views on AI study in 6G										ZTE Corporation, Sanechips

R4-2513293		6G RAN4 Study on AI											NTT DOCOMO, INC.

R4-2513298		Views on 6G AI study RAN4 aspects							China Unicom

R4-2513310		Consideration on RAN4 AI study for 6G						Huawei, HiSilicon

R4-2513318		RAN4 driven AI based use cases								OPPO

R4-2513324		Initial discussion on 6G AI for RAN4 driven cases			vivo