3GPP TSG-RAN WG4 Meeting # 117											R4-2520842

Dallas, USA, 17th ‒ 21st November 2025, 2025

Agenda item:			6.18.1

Source:	Moderator (Nokia)

Title:	Topic summary for [117][217] NR_XR_Ph3_RRM

Document for:	Information

# Introduction

This summary includes the proposals and issues for discussion related to Rel-19 enhancements for eXtended Reality for NR Phase 3, under thread [117][217] NR_XR_Ph3_RRM.

This summary is based on the contributions submitted to the agenda items:

- 4.24 RRM core requirements	[NR_XR_Ph3-Core]

- 6.18.2 RRM performance requirements	[NR_XR_Ph3-Perf]

The recommended priority for discussion is as follows:

- Issue 1-1-1: Formulas for LTM

- Issue 2-1-1: Scenarios for the definition of the test cases

- Issue 2-1-2: Test target – accuracy requirements

- Issue 2-1-3: Test target – processing timeline requirements

- Issue 2-1-4: Test target – scheduling restrictions

- Issue 2-1-5: List of test cases – accuracy requirements

- Issue 2-1-6: Complete list of test cases for discussion

- Issue 2-2-1: SSB time index detection

- Issue 2-3-1: Cancelation model for XR test cases

- Issue 2-3-2: Cancelation ratio for test cases

- Issue 2-4-1: Toffset between DCI and start of MG

- Issue 2-4-2: Types of DCI with cancellation indication

- Issue 2-4-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’

- Issue 2-5-1: How to capture gap cancellation model

- Issue 2-5-2: How to include configuration tables based on TC of other releases

# Topic #1: RRM core requirements maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2521250 | Nokia | CR 38.133 Big CR on core maintenance for NR_XR_Ph3_RRM Resubmission of endorsed CR in RAN4#116bis |
| R4-2521251 | Nokia | CR 38.133 Maintenance of XR core requirements Changes in clauses - 9.1.14 - 9.4.2 - 9.4.3 - 9.15.5 |
| R4-2522016 | Ericsson | draftCR 38.133 Measurement requirements with XR Changes in clauses - 9.4.2 - 9.4.3 - 9.5.4.1 - 9.5.4.2 - 9.8.4 - 9.15.5 |
| R4-2522101 | Nokia | Discussion on maintenance of XR ph3 RRM core requirements Proposal 1: Formulas for measurement delay extension of L1/L2 triggered mobility in FR2-1 are Condition; TL1-RSRP_Measurement_Period_SSB_Inter. No DRX; Max(Treport, Ceil(Kgap (M*N +Lcancel)) Max(MGRP, SSB period) CSSFinter). DRX cycle ≤ 320 ms; Max(Treport, Ceil(1.5 * Kgap (M*N +Lcancel)) Max(MGRP, SSB period, DRX cycle) CSSFinter). DRX cycle > 320 ms; Ceil(Kgap (M*N +Lcancel)) DRX cycle CSSFinter. |

## Open issues summary

### Sub-topic 1-1 – RRM core maintenance

#### Issue 1-1-1: Formulas for LTM

- Proposals

  - Proposal 1 (Nokia): Formulas for measurement delay extension of L1/L2 triggered mobility in FR2-1 are

| Condition | TL1-RSRP_Measurement_Period_SSB_Inter |
| --- | --- |
| No DRX | Max(Treport, Ceil(Kgap (M*N +Lcancel)) Max(MGRP, SSB period) CSSFinter) |
| DRX cycle ≤ 320 ms | Max(Treport, Ceil(1.5 * Kgap (M*N +Lcancel)) Max(MGRP, SSB period, DRX cycle) CSSFinter) |
| DRX cycle > 320 ms | Ceil(Kgap (M*N +Lcancel)) DRX cycle CSSFinter |

- Recommended WF

  - Discuss the proposal.

# Topic #2: RRM performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2522100 | Nokia, Qualcomm | Rapporteur Inputs <moderator> document for information |
| R4-2520949 | Huawei, HiSilicon | Discussion on performance requirements for XR Phase 3 Proposal 1: RAN4 defines the test of the UE processing timeline of receiving the DCI-based MG skipping indication. · The DCI processing time offset depends on the UE capability in the test configuration. Observation 1: The accuracy requirement needs a ratio of canaelled gap, but RAN4 does not have a consensus on this issue yet. Proposal 2: The Accuracy requirements definition is related to the gap cancellation ratio issue and needs further discussion. Proposal 3: RAN4 to design a fixed cancellation gap pattern for test cases, and further discuss the pattern. Observation 2: There is no requirement defined in the core part for the cancellation ratio. Proposal 4: It is not reasonable to define the cancellation ratio in the performance part of XR. |
| R4-2521009 | vivo | On RRM performance requirements for XR Proposal 1: SSB time index detection is not considered in the tests for XR. Proposal 2: Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 5ms Toffset. Proposal 3: DCI 1-1 is used in the test for gap cancellation indication. Proposal 4: Time offset of both 3ms and 5ms are tested in one test, where time offset for DCI triggering is based on UE capability. Proposal 5: UE bbehavior of Option 1 and Option 2 for indication of value ‘0’ are tested in one test, where test requirement on measurement delay is based on UE capability. |
| R4-2521931 | ZTE Corporation, Sanechips | Discussion on Perf part of R19 XR Proposal 1: For the verification of measurement procedure, both L3 measurement and LTM are verified in FR2-1. Some of them are for non-DRX, others are for DRX. Proposal 2: Verify the accuracy requirements for XR, at least for FR2-1. Proposal 3: For the Toffset between DCI and start of MG, define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition. Proposal 4: Define the inter-frequency measurement in FR1 with SSB time index detection, and the inter-frequency measurement in FR2 without SSB time index detection. Proposal 5: Pick either DCI format 0-1 or 1-1 in the test case. Proposal 6: Measurement gap cancellation is determined randomly by the test equipment, the 1st canceled measurement gap occasion should be right 3ms/5ms later than the DCI receiving. The cancellation ratio could be set as a unified value for both 3ms and 5ms timeline, e.g. 50%. |
| R4-2522018 | Ericsson | On RRM performance issues for XR Proposal 1: Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1. Proposal 2: The following test cases for measurement accuracy for XR can be specified: Table 1: Candidate test cases for measurement accuracy (NR-SA) under XR operation TC title; FR (FR1, FR2-1); DRX or non-DRX. Inter-frequency with gaps (SS-RSRP); A.6.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell; All cells in FR1; Non-DRX. A.7.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell; All cells in FR2-1; Non-DRX. Inter-frequency with gaps (SS-RSRQ); A.6.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell; All cells in FR1; Non-DRX. A.7.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell; All cells in FR2-1; Non-DRX. Proposal 3: In all XR test cases, the DCI indication is configured 5 ms or more before the start of the measurement gap occasion. |
| R4-2522069 | MediaTek inc. | Discussion on RRM performance requirements for XR Proposal 1: RAN4 RRM not to define test cases for accuracy requirements for NR-SA, and FR1 and FR2-1. Proposal 2: For measurement procedure, define test cases without SSB time index detection. Proposal 3: The measurement gap cancelation is determined randomly by the test equipment. Proposal 4: The ratio of gap occasions to be cancelled during the measurement period is 20% in the tests. Proposal 5: RAN4 RRM define a single test case that include time offset of 5ms. Proposal 6: RAN4 RRM define some test cases can be define with DCI 0-1 and other TCs with DCI 1-1 without duplicating the test cases. Proposal 7: RAN4 RRM not to define test cases to cover indication of value ‘0’. |
| R4-2522102 | Nokia | Discussion on XR ph3 RRM performance requirements Proposal 1: RAN4 must verify the UE processing timeline of receiving the DCI-based MG skipping indication. Proposal 2: RAN4 should verify the accuracy requirements in XR test case(s). Proposal 3: RAN4 should verify cancellation of scheduling restrictions due to L3 and L1 measurements without gaps during cancelled gaps. Proposal 4: Define at least one accuracy test case in FR1 and in FR2-1. Proposal 5: The accuracy requirements are tested in limited number of accuracy specific test, including - Inter-frequency measurements with Type 1 gaps, without DRX in FR1, - Inter-frequency measurements with Type 1 gaps, without DRX in FR2-1. Proposal 6: For measurement procedure, define test cases without SSB time index detection. Observation 1: NR-U CCA model for RRM test cases includes limitation on the maximum number of samples with non-successful CCA. Proposal 7: Measurement gap cancellation is determined randomly by the test equipment. Proposal 8: Measurement gap cancelation is modelled as a fixed pattern for the test cases, where - The pseudo random pattern is generated for at least 40 gap occasions. - For every test iteration, the TE starts at a random position of the pseudo random pattern. Proposal 9: For Type 1 MGs test cases, RAN4 should agree on Proposal 2, i.e., - Measurement gap cancellation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset. - Measurement gap cancellation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset. Proposal 10: The processing time 3ms/5ms shall be verified in the test cases. Proposal 11: Define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition. Observation 2: XR gap canceling feature includes a field for the UE to declare support of additional DCI, where formats 0_1 and 1_1 are mandatory. Proposal 12: Define RAN4 test cases using DCI 1-1 for the cancelling indication. Proposal 13: RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows: - Test Option 2 in every test case. - For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability. |
| R4-2522200 | Qualcomm Incorporated | RRM performance requirements for XR enhancements Observation 1: RAN4 has not defined any new measurement accuracy requirements for the MG cancellation feature. Observation 2: Measurement accuracy tests are conducted under AWGN conditions and cannot reflect potential degradation caused by extended measurement delays. Proposal 1: RAN4 should not introduce measurement accuracy test cases for the measurement gap cancellation feature, as they are unlikely to provide useful validation. Observation 3: A large number of gap cancellations increases test duration and complexity. Proposal 2: RAN4 should define test cases with a single or at most two MG cancellations. Proposal 3: In the test cases, the TE may randomly cancel one or two MG occasions with equal probability. Proposal 4: Define test case using 5ms offset only. Proposal 5: RAN4 to use an incremental test-case design approach to specify new test cases, where only additional parameters and configurations from the baseline test case are specified along with the new test requirements. |
| R4-2521014 | vivo | draft CR for LTM Inter-frequency L1-RSRP measurement with measurement gap cancellation in FR1 |
| R4-2521955 | ZTE Corporation, Sanechips | Draft CR on test case of event triggered reporting with MG cancellation for FR2-1_R19 XR |
| R4-2522017 | Ericsson | draftCR38133 Test case for inter-frequency requirements with XR |
| R4-2522103 | Nokia | Draft CR XR RRM performance requirements. |
| R4-2522100 | Nokia, Qualcomm | Rapporteur Inputs |

## Open issues summary

### Sub-topic 2-1 – Test case list

#### Issue 2-1-1: Scenarios for the definition of the test cases

- Proposals

  - Proposal 1 (ZTE): For the verification of measurement procedure, both L3 measurement and LTM are verified in FR2-1. Some of them are for non-DRX, others are for DRX.

- Recommended WF

  - The list of TCs in Issue 1-1-6 of R4-2514869 already include TCs for LTM and L3 in FR2-1. Please check if further discussion is needed.

#### Issue 2-1-2: Test target – accuracy requirements

- Company proposals

  - Proposal 1 (Huawei): The Accuracy requirements definition is related to the gap cancellation ratio issue and needs further discussion

  - Proposal 2 (ZTE): Verify the accuracy requirements for XR, at least for FR2-1.

  - Proposal 3 (Ericsson): Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

  - Proposal 4 (Nokia): RAN4 should verify the accuracy requirements in XR test case(s).

  - Proposal 5 (Mediatek): RAN4 RRM not to define test cases for accuracy requirements for NR-SA, and FR1 and FR2-1.

  - Proposal 6 (Qualcomm): RAN4 should not introduce measurement accuracy test cases for the measurement gap cancellation feature, as they are unlikely to provide useful validation.

- Options merged by the moderator

  - Option 1: Define test cases for accuracy requirements.

  - Option 2: Do not define test cases for accuracy requirements.

  - Option 3: Accuracy is also verified in the measurement procedure test case.

- Recommended WF:

  - Discuss the options

#### Issue 2-1-3: Test target – processing timeline requirements

- Company proposals

  - Proposal 1 (Huawei): RAN4 defines the test of the UE processing timeline of receiving the DCI-based MG skipping indication.

    - The DCI processing time offset depends on the UE capability in the test configuration.

  - Proposal 2 (ZTE): For the Toffset between DCI and start of MG, define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition.

  - Proposal 3 (Nokia): RAN4 must verify the UE processing timeline of receiving the DCI-based MG skipping indication.

  - Proposal 4 (Ericsson): In all XR test cases, the DCI indication is configured 5 ms or more before the start of the measurement gap occasion, i.e., not verified for the different capabilities.

- Recommended WF:

  - Discuss the options

#### Issue 2-1-4: Test target – scheduling restrictions

- Company proposals

  - Proposal 1 (Nokia): RAN4 should verify cancellation of scheduling restrictions due to L3 and L1 measurements without gaps during cancelled gaps.

- Recommended WF:

  - Discuss the proposal

#### Issue 2-1-5: List of test cases – accuracy requirements

- Company proposals

  - Proposal 1 (ZTE): Verify the accuracy requirements for XR, at least for FR2-1.

  - Proposal 2 (Nokia): Define at least one accuracy test case in FR1 and in FR2-1.

  - Proposal 3 (Nokia): The accuracy requirements are tested in limited number of accuracy specific test, including

    - Inter-frequency measurements with Type 1 gaps, without DRX in FR1,

    - Inter-frequency measurements with Type 1 gaps, without DRX in FR2-1.

  - Proposal 3 (Ericsson): Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

  - Proposal 4 (Ericsson based on Table 1): Define accuracy test cases for one or  both of SS-RSRP and SS-RSRQ in both FR1 and FR2-1.

  - Proposal 5 (Mediatek): RAN4 RRM not to define test cases for accuracy requirements for NR-SA, and FR1 and FR2-1.

  - Proposal 6 (Qualcomm): RAN4 should not introduce measurement accuracy test cases for the measurement gap cancellation feature, as they are unlikely to provide useful validation.

  - Proposal 7 (Qualcomm): RAN4 to use an incremental test-case design approach to specify new test cases, where only additional parameters and configurations from the baseline test case are specified along with the new test requirements.

- Options merged by the moderator

  - Option 1: Define accuracy test cases

    - Option 1a: TC at least for FR2-1

    - Option 1b: 1 TC for FR1 and 1 TC for FR2-1

    - Option 1c: Define accuracy requirements for one or both of:

      - SS-RSRP in FR1 and FR2-1

      - SS-RSRQ in FR1 and FR2-1

  - Option 2: Do not define test cases for accuracy.

  - Option 3: Accuracy is also verified in the measurement procedure test case.

- Recommended WF

  - Discuss the merged proposals.

#### Issue 2-1-6: Complete list of test cases for discussion

- Agreement from RAN4 116bis

<Agreement> Draft CR; Volunteering company. FFS: A.3.x Model for measurement gap cancellation; Nokia. A.6.6.2.x SA event triggered reporting with measurement gap cancellation for FR1under non-DRX; Ericsson. A.7.6.2.x SA event triggered reporting with measurement gap cancellation for FR2-1 under DRX; ZTE. A.6.6.x LTM Inter-frequency L1-RSRP measurement with measurement gap cancellation in FR1; Vivo. A.6.6.x LTM Inter-frequency L1-RSRP measurement with measurement gap cancellation in FR2-1; Nokia.

- Company proposals

  - Proposal 1 (Ericsson): The following test cases for measurement accuracy for XR can be specified:

    - Table 1: Candidate test cases for measurement accuracy (NR-SA) under XR operation

|  | TC title | FR (FR1, FR2-1) | DRX or non-DRX |
| --- | --- | --- | --- |
| Inter-frequency with gaps (SS-RSRP) | A.6.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX |
|  | A.7.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | All cells in FR2-1 | Non-DRX |
| Inter-frequency with gaps (SS-RSRQ) | A.6.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX |
|  | A.7.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | All cells in FR2-1 | Non-DRX |

- Recommended WF:

  - Based on the agreements of the other issues, discuss how the list of CRs should be updated.

### Sub-topic 2-2 – Test case configuration

#### Issue 2-2-1: SSB time index detection

- Company proposals

  - Option 1 (Mediatek, vivo, Nokia, Ericsson): For measurement procedure, define test cases without SSB time index detection.

  - Option 2 (ZTE): Define the inter-frequency measurement in FR1 with SSB time index detection, and the inter-frequency measurement in FR2 without SSB time index detection.

- Recommended WF:

  - Discuss the proposals.

### Sub-topic 2-3 – MG cancellation model

#### Issue 2-3-1: Cancelation model for XR test cases

- Company proposals

  - Proposal 1: (Mediatek, Nokia) The measurement gap cancelation is determined randomly by the test equipment.

  - Proposal 2 (vivo): Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 5ms Toffset.

  - Proposal 3 (ZTE): Measurement gap cancellation is determined randomly by the test equipment, the 1st canceled measurement gap occasion should be right 3ms/5ms later than the DCI receiving. The cancellation ratio could be set as a unified value for both 3ms and 5ms timeline, e.g. 50%.

  - Proposal 4 (Qualcomm): In the test cases, the TE may randomly cancel one or two MG occasions with equal probability

  - Proposal 5 (Huawei): RAN4 to design a fixed cancellation gap pattern for test cases, and further discuss the pattern.

  - Proposal 6 (Nokia): Measurement gap cancelation is modelled as a fixed pattern for the test cases, where

    - The pseudo random pattern is generated for at least 40 gap occasions.

    - For every test iteration, the TE starts at a random position of the pseudo random pattern.

  - Proposal 7 (Ericsson): During time duration T2, for each measurement gap occasion the test equipment determines, based on probability Pcancel, whether the measurement gap is to be configured for cancellation at the UE via DCI indication.

- Proposals merged by the moderator

  - Option 1: Measurement gap cancelation is determined randomly by the test equipment.

  - Option 2: Measurement gap cancelation is modelled as a fixed pattern for the test cases.

  - Option 3: Measurement gap cancelation is modelled as a fixed pattern for the test cases, where

    - The pseudo random pattern is generated for at least 40 gap occasions.

    - For every test iteration, the TE starts at a random position of the pseudo random pattern.

- Recommended WF

  - Discuss the proposals.

#### Issue 2-3-2: Cancelation ratio for test cases

- Company proposals

  - Proposal 1 (ZTE): Measurement gap cancellation is determined randomly by the test equipment, the 1st canceled measurement gap occasion should be right 3ms/5ms later than the DCI receiving. The cancellation ratio could be set as a unified value for both 3ms and 5ms timeline, e.g. 50%.

  - Proposal 2 (vivo): Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 5ms Toffset.

  - Proposal 3 (Nokia): For Type 1 MGs test cases, RAN4 should agree on Proposal 2, i.e.,

    - Measurement gap cancellation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset.

    - Measurement gap cancellation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset.

  - Proposal 4 (Mediatek, Ericsson): The ratio of gap occasions to be cancelled during the measurement period is 20% in the tests.

  - Proposal 5 (Qualcomm): RAN4 should define test cases with a single or at most two MG cancellations.

  - Proposal 6 (Huawei): It is not reasonable to define the cancellation ratio in the performance part of XR.

- Options merged by the moderator

  - Option 1: 50%

  - Option 2: 40%

  - Option 3:

    - 50% for UE supporting Toffset = 5 ms.

    - 40% for UE supporting Toffset = 3 ms.

  - Option 4: 20%

  - Option 5: 1 or 2 MG cancellations

  - Option 6: It is not reasonable to define the cancellation ratio in the performance part of XR.

- Recommended WF:

  - Discuss the proposals.

### Sub-topic 2-4 – DCI aspects

#### Issue 2-4-1: Toffset between DCI and start of MG

- Company proposals

  - Proposal 1 (vivo): Time offset of both 3ms and 5ms are tested in one test, where time offset for DCI triggering is based on UE capability.

  - Proposal 2 (ZTE, Nokia): For the Toffset between DCI and start of MG, define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition.

  - Proposal 3 (Ericsson): In all XR test cases, the DCI indication is configured 5 ms or more before the start of the measurement gap occasion.

  - Proposal 4 (MediaTek): RAN4 RRM define a single test case that include time offset of 5ms.

  - Proposal 5 (Qualcomm): Define test case using 5ms offset only.

- Options merged by the moderator

  - Option 1: The test cases include time offset of 5ms and 3ms with UE capability used as a condition.

  - Option 2: The test cases include 5ms offset only.

- Recommended WF

  - Discuss the proposals.

#### Issue 2-4-2: Types of DCI with cancellation indication

- Company proposals

  - Proposal 1 (vivo): DCI 1-1 is used in the test for gap cancellation indication.

  - Proposal 2 (Nokia): Define RAN4 test cases using DCI 1-1 for the cancelling indication.

  - Proposal 2 (ZTE): Pick either DCI format 0-1 or 1-1 in the test case.

  - Proposal 3 (Mediatek): RAN4 RRM define some test cases can be define with DCI 0-1 and other TCs with DCI 1-1 without duplicating the test cases.

- Options merged by the moderator

  - Option 1: only DCI 1-1 is used

  - Option 2: some test cases use DCI 0-1 and other test cases use DCI 1-1

- Recommended WF

  - Discuss the proposals. Discuss whether different types of DCI are to be included in the tests, and how to verify them.

#### Issue 2-4-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’

- Options

  - Proposal 1 (vivo): UE bbehavior of Option 1 and Option 2 for indication of value ‘0’ are tested in one test, where test requirement on measurement delay is based on UE capability.

  - Proposal 2 (Nokia): RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows:

    - Test Option 2 in every test case.

    - For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability.

  - Proposal 3 (Mediatek): RAN4 RRM not to define test cases to cover indication of value ‘0’.

- Options merged by the moderator

  - Option 1: indication of value ‘0’ for the UE supporting indicationField-r19 = ‘option2’ is verified

  - Option 2: indication of value ‘0’ for the UE supporting indicationField-r19 = ‘option2’ is NOT verified

- Recommended WF

  - Discuss the proposals.

### Sub-topic 2-5 – CR drafting

The issues are included here to discuss some of the style differences among CRs for the performance part.

#### Issue 2-5-1: How to capture gap cancellation model

- Options

  - Option 1 (R4-2521014, R4-2521955, R4-2522017): Repeated in every test case.

  - Option 2 (R4-2522103) : In a separate clause in A.3.

- Recommended WF

  - Discuss the options considering potential specification simplification, and workload related to revision of the CRs.

#### Issue 2-5-2: How to include configuration tables based on TC of other releases

- Options

  - Option 1 (R4-2521014, R4-2521955, R4-2522017): Repeated in every test case.

  - Option 2 (R4-2522103) : No repetition, use reference to the test cases which were used as a baseline for the test.

- Recommended WF

  - Discuss the options considering potential specification simplification, and workload related to revision of the CRs.