# 3GPP TSG-RAN WG4 Meeting #117	R4-2522213

# Dallas, USA, Nov. 17-21, 2025

Agenda item:			6.8.1

Source:	Moderator (Huawei)

Title:	Topic summary for [117][305] NR_BS_RF_Part1_SE_CLTA

Document for:	Information

# Introduction

This thread covers the following topics:

- Transmitter co-existence spurious emission requirements re-evaluation in Topic #1.

- TRP estimation methods simplification in Topic #2.

The following agenda items were covered in this summary: 6.8.3.1, 6.8.4.2.2, 6.8.4.3.

Related objectives were extracted from the WID in RP-252578 are provided for reference:

| Core | Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz. |
| --- | --- |
| Perf | OTA test enhancement - TRP simplification: Simplify BS TRP estimation methods (e.g., Annex I in TS 38.141-2 on TRP measurement procedures) for FR1 and FR2 by improving their applicability and description and not removing any of the existing TRP test methods. Transmitter co-existence spurious emission requirements - Conformance testing corresponding to co-existence spurious emission requirements, if needed. |

According to the last WF in R4-2512578, the following timeline was greed for the Topic #1:

- Simulation calibration in October meeting.

- Aim for conclusions and final CRs (if any) for November meeting.

# Topic #1: TX spurious emissions co-existence requirements re-evaluation

## Companies’ contributions summary

The following contributions were submitted to the meeting:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2521761 | ZTE Corporation, Sanechips | Observation 1: Antenna elements/antenna sub-array of 2GHz can still achieve comparable gain at the 3rd harmonic freq within U6GHz. Observation 2: The antenna gain of 2nd harmonic of 3.5GHz signal is much reduced compared with 3.5GHz due to the internal counteract of the antenna’s physical structure design. Observation 3: For different percentile of BS2BS CDF to derive the coexistence spurious emission requirement, there are around 20dB coupling loss difference between 99% to 99.99%. Proposal 1: For coexistence spurious emission requirement between aggressive BS and victim BS, propose to use the more stringent requirement between the 99.9% of coupling loss and 5% throughput loss criteria to derive the requirement. Observation 4: There are roughly around addition 19-26dB ACIR offset needed on top of baseline ACIR 42.46dB to meet the 5% throughput loss. Observation 5: Using 99.99% of coupling loss to keep the current requirement unchanged is not necessary, there are still around 13dB relaxation allowed according to the legacy coexistence criteria. Proposal 2: Propose to define -39dBm/MHz as coexistence spurious emission requirement for 2GHz and 3.5GHz BS coexisting with band n104. Observation 6: For different percentile of BS2UE CDF to derive the coexistence spurious emission requirement, there are around 40dB coupling loss difference between 99% to 99.99%. Observation 7: Compared with the legacy MCL assumption 53dB between aggressive BS and victim UE, there is a big gap between 99.9 or 99.99% of coupling loss and 53dB. Observation 8: For dense urban coexistence case, once the coexistence spurious emission -15.73dBm/MHz is met, the it should be acceptable to maintain the 5% throughput loss. Proposal 3: for coexistence spurious emission requirement between 2GHz/3.5GHz BS and band n104, propose to define the requirement based on the BS2BS spurious emission requirement. |
| R4-2521762 | ZTE Corporation | CR to TS 38.106: update on coexistence spurious limit for protection of band n104 |
| R4-2521763 | ZTE Corporation | CR to TS 38.174: update on coexistence spurious limit for protection of band n104 |
| R4-2520275 | Nokia | Simulation results for transmitter co-existence spurious emission requirements |
| R4-2520276 | Nokia | Observation 1: The 67 dB BS to BS MCL used to derive the current -49 dBm/MHz transmitter co-existence spurious emission requirements for UL protection seems to remain reasonable at 6425 MHz carrier frequency. Observation 2: The 53 dB BS to UE MCL used to derive the current -52 dBm/MHz transmitter co-existence spurious emission requirements for DL protection seems to remain reasonable at 6425 MHz carrier frequency. Proposal 1: To keep the 67 dB BS to BS MCL assumption for deviation of the transmitter co-existence spurious emission requirements for UL protection in the upper region of FR1. Proposal 2: To keep the 53 dB BS to UE MCL assumption for deviation of the transmitter co-existence spurious emission requirements for DL protection in the upper region of FR1. |
| R4-2521784 | Ericsson | Observation 1: The combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminates the possibility for the antenna array to achieve full array gain. Observation 2 : The BS-to-BS coupling with 100% grid shift and correlation factor 0.5 assumption at 99.99%tile and 99.95%tile indicates a potential correction of ~4 dB and ~8 dB respectively. Observation 3 : The BS-to-UE coupling without polarization gain and correlation factor 0.5 assumption at 99.99%tile and 99.95%tile indicates a potential correction of ~28 dB and ~31 dB respectively. Proposal 1 : For the modelling of antenna gain for unwanted emissions, RAN4 to assume correlation factor ρ = 0.5 as a tradeoff between performance and hardware limitations. Proposal 2 : For the requirements derivation, consider the coupling loss CDF %tile as 99.99%tile or 99.95%tile instead of the 100%tile minimum coupling loss value. Proposal 3 : For BS to UE, do not consider the additional 3 dB polarization gain. Proposal 4 : With BS-to-BS coupling being the dominant scenario, there should be a similar adjustment of ~4 dB/ 8 dB compared to Rel-99 values for both BS-to BS and BS-to-UE co-existence spurious emission requirements. |
| R4-2521785 | Ericsson | (FS_NR_IMT_param) (NR_BS_RF_req_evo-Core) draftCR to TR 38.921 Addition of BS Transmitter Spurious emission co-existence work |
| R4-2522179 | Huawei, HiSilicon | Observation 1: the simplification that design frequency and excitation frequency are the same is not valid in this case. Proposal 1: Use aggressor frequency as the design frequency and victim frequency as the excitation frequency. Proposal 2: Use aggressor frequency as the sub-array phase shifter design frequency and victim frequency as the excitation frequency. Proposal 3: Cube the wanted signal weighting vector to generate the harmonic excitation weighting vector. (simply multiple the phase shifts by 3 in the model if this is more practical). Proposal 4: As victim frequency is 6.425GHz use 2.14GHz as aggressor fundamental frequency. Proposal 5: Continue using the existing element model. Observation 2: As the aggressor beam is variable the total interference is not directly related to the MCL. Observation 3: the level of the wanted signal and the level of the aggressor are directly related. Proposal 6: MCL is not an appropriate metric to represent the interference of a dynamic AAS to AAS network Observation 4: If MCL is used as a metric then only beam pointing cases where the wanted signal is at a low level should be considered or a lower percentage probability should be selected. Proposal 7: Use a low percentile throughput degradation metric to find the suitable interference level requirement for the aggressor BS. Observation 5: Using even very strict throughput loss metrics at least 9dB margin can be seen over the current requirement. Observation 6: the higher frequency and frequency-based PL do not directly lead to a different requirement, however the dynamic AAS networks required to operate at higher frequencies do lead to a different requirement. Proposal 8: Based on initial throughput loss simulations the current emissions levels do appear to be over specified as we consider higher frequencies and potentially higher gain AAS systems we should develop a methodology to correctly set these limits based on throughput degradation. |
| R4-2522180 | Huawei, HiSilicon | Draft LS on the status of the work on TX spurious emission co-existence requirements limits for high frequency bands |

## Open issues summary

The following sub-topics were identified based on contributions listed in clause 1.1.

### Sub-topic 1-1: Polarization gain

Moderator: Previous WF in R4-2515171:

For BS-UE case, as well as for BS-BS:

- Consider 0 dB gain for calibration.

- For requirement derivation keep both 0 dB and 3 dB for polarization gain. Final decision to be taken during RAN4#117 meeting.

Open issues and candidate options before meeting:

- Proposal 1: For BS to UE, do not consider the additional 3 dB polarization gain. (R4-2521784, Ericsson)

Recommended WF: Confirm online. Agreement to be captured in the set of simulation assumptions (TR).

### Sub-topic 1-2: Antenna / array gain considerations

Moderator: Previous WF in R4-2515171: Antenna / array gain considerations: FFS

Observations before meeting:

- Observation 1: Antenna elements/antenna sub-array of 2 GHz can still achieve comparable gain at the 3rd	 harmonic frequency within U6GHz. (R4-2521761, ZTE)

- Observation 2: The antenna gain of 2nd harmonic of 3.5 GHz signal is much reduced compared with 3.5 GHz due to the internal counteract of the antenna’s physical structure design. (R4-2521761, ZTE)

- Observation 3: The simplification that design frequency and excitation frequency are the same is not valid in this case. (R4-2522179, Huawei)

Open issues and candidate options before meeting:

- Proposal 1: Use aggressor frequency as the design frequency and victim frequency as the excitation frequency. (R4-2522179, Huawei)

- Proposal 2: Use aggressor frequency as the sub-array phase shifter design frequency and victim frequency as the excitation frequency. (R4-2522179, Huawei)

- Proposal 3: Cube the wanted signal weighting vector to generate the harmonic excitation weighting vector. (simply multiple the phase shifts by 3 in the model if this is more practical). (R4-2522179, Huawei)

- Proposal 4: As victim frequency is 6.425 GHz use 2.14 GHz as aggressor fundamental frequency. (R4-2522179, Huawei)

- Proposal 5: Continue using the existing element model. (R4-2522179, Huawei)

Recommended WF: Discuss online how to capture harmonic model findings in the TR.

### Sub-topic 1-3: Correlation factor for OoB

Moderator: Previous WF in R4-2515171: Correlation factor for OoB: FFS

Open issues and candidate options before meeting:

- Observation 1: The combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminates the possibility for the antenna array to achieve full array gain. (R4-2521784, Ericsson)

Open issues and candidate options before meeting:

- Proposal 1: For the modelling of antenna gain for unwanted emissions, RAN4 to assume correlation factor ρ = 0.5 as a tradeoff between performance and hardware limitations. (R4-2521784, Ericsson)

Recommended WF: As the co-ex requirement relaxation is not concluded, clarify if there is any need for further discussion on the correlation factor for OoB. Agreements on the correlation factor for OoB for calibration phase were captured in previous WF.

### Sub-topic 1-5: Performance metrics

Moderator: Previous WF in R4-2515171: CDF %-tile value to be used for the requirement derivation to be agreed at RAN4#117.

Observations before meeting:

- Observation 1: For different percentile of BS2BS CDF to derive the coexistence spurious emission requirement, there are around 20dB coupling loss difference between 99% to 99.99%. (R4-2521761, ZTE)

- Observation 2: There are roughly around addition 19-26 dB ACIR offset needed on top of baseline ACIR 42.46 dB to meet the 5% throughput loss. (R4-2521761, ZTE)

- Observation 3: For different percentile of BS2UE CDF to derive the coexistence spurious emission requirement, there are around 40 dB coupling loss difference between 99% to 99.99%. (R4-2521761, ZTE)

- Observation 4: Compared with the legacy MCL assumption 53 dB between aggressive BS and victim UE, there is a big gap between 99.9 or 99.99% of coupling loss and 53 dB. (R4-2521761, ZTE)

- Observation 5: For dense urban coexistence case, once the coexistence spurious emission -15.73 dBm/MHz is met, the it should be acceptable to maintain the 5% throughput loss. (R4-2521761, ZTE)

- Observation 6: As the aggressor beam is variable the total interference is not directly related to the MCL. (R4-2522179, Huawei)

- Observation 7: The level of the wanted signal and the level of the aggressor are directly related. (R4-2522179, Huawei)

- Observation 8: If MCL is used as a metric then only beam pointing cases where the wanted signal is at a low level should be considered or a lower percentage probability should be selected. (R4-2522179, Huawei)

Open issues and candidate options before meeting:

- Proposal 1: For the requirements derivation, consider the coupling loss CDF %tile as 99.99%tile or 99.95%tile instead of the 100%tile minimum coupling loss value. (R4-2521784, Ericsson)

- Proposal 2: For coexistence spurious emission requirement between aggressive BS and victim BS, propose to use the more stringent requirement between the 99.9% of coupling loss and 5% throughput loss criteria to derive the requirement. (R4-2521761, ZTE)

- Proposal 3: MCL is not an appropriate metric to represent the interference of a dynamic AAS to AAS network. (R4-2522179, Huawei)

- Proposal 4: Use a low percentile throughput degradation metric to find the suitable interference level requirement for the aggressor BS. (R4-2522179, Huawei)

Recommended WF: Discuss how to capture MCL-based, as well as throughput degradation based metrics in the TR.

### Sub-topic 1-7: Simulation results collection

Moderator: Simulation results were provided in the following contributions:

| R4-2520275 | Nokia | Simulation results for transmitter co-existence spurious emission requirements |
| --- | --- | --- |

Recommended WF: Final simulation results collection to be updated into the Excel template based on R4-1610157.

### Sub-topic 1-8: Requirements impact

Moderator: Previous WF in R4-2515171: Final requirement to be derived at RAN4#117 to conclude technical work.

Observations before meeting:

- Observation 1: The BS-to-BS coupling with 100% grid shift and correlation factor 0.5 assumption at 99.99%tile and 99.95%tile indicates a potential correction of ~4 dB and ~8 dB respectively. (R4-2521784, Ericsson)

- Observation 2: The BS-to-UE coupling without polarization gain and correlation factor 0.5 assumption at 99.99%tile and 99.95%tile indicates a potential correction of ~28 dB and ~31 dB respectively. (R4-2521784, Ericsson)

- Observation 3: Using 99.99% of coupling loss to keep the current requirement unchanged is not necessary, there are still around 13dB relaxation allowed according to the legacy coexistence criteria. (R4-2521761, ZTE)

- Observation 4: The 67 dB BS to BS MCL used to derive the current -49 dBm/MHz transmitter co-existence spurious emission requirements for UL protection seems to remain reasonable at 6425 MHz carrier frequency. (R4-2520276, Nokia)

- Observation 5: The 53 dB BS to UE MCL used to derive the current -52 dBm/MHz transmitter co-existence spurious emission requirements for DL protection seems to remain reasonable at 6425 MHz carrier frequency. (R4-2520276, Nokia)

- Observation 6: Using even very strict throughput loss metrics at least 9dB margin can be seen over the current requirement. (R4-2522179, Huawei)

- Observation 7: the higher frequency and frequency-based PL do not directly lead to a different requirement, however the dynamic AAS networks required to operate at higher frequencies do lead to a different requirement. (R4-2522179, Huawei)

Open issues and candidate options before meeting:

- Proposal 1: With BS-to-BS coupling being the dominant scenario, there should be a similar adjustment of ~4 dB/ 8 dB compared to Rel-99 values for both BS-to BS and BS-to-UE co-existence spurious emission requirements. (R4-2521784, Ericsson)

- Proposal 2: Propose to define -39dBm/MHz as coexistence spurious emission requirement for 2GHz and 3.5GHz BS coexisting with band n104. (R4-2521761, ZTE)

- Proposal 3: for coexistence spurious emission requirement between 2GHz/3.5GHz BS and band n104, propose to define the requirement based on the BS2BS spurious emission requirement. (R4-2521761, ZTE)

- Proposal 4: To keep the 67 dB BS to BS MCL assumption for deviation of the transmitter co-existence spurious emission requirements for UL protection in the upper region of FR1. (R4-2520276, Nokia)

- Proposal 5: To keep the 53 dB BS to UE MCL assumption for deviation of the transmitter co-existence spurious emission requirements for DL protection in the upper region of FR1. (R4-2520276, Nokia)

- Proposal 8: Based on initial throughput loss simulations the current emissions levels do appear to be over specified as we consider higher frequencies and potentially higher gain AAS systems we should develop a methodology to correctly set these limits based on throughput degradation. (R4-2522179, Huawei)

Recommended WF: As the co-ex requirement relaxation is not concluded, discuss how to capture input results on the envisioned requirement relaxations (inc. assumed performance metrics) in the TR.

### Sub-topic 1-8: CRs

The following (Draft) CRs were submitted to the meeting:

| R4-2521762 | ZTE Corporation | CR to TS 38.106: update on coexistence spurious limit for protection of band n104 Moderator: Implementation of [-39] dBm for band n104 in the co-existence table, which will be removed by conflicting CR under thread [301]. |
| --- | --- | --- |
| R4-2521763 | ZTE Corporation | CR to TS 38.174: update on coexistence spurious limit for protection of band n104 Moderator: Implementation of [-39] dBm for band n104 in the co-existence table, which will be removed by conflicting CR under thread [301]. |
| R4-2521785 | Ericsson | (FS_NR_IMT_param) (NR_BS_RF_req_evo-Core) draftCR to TR 38.921 Addition of BS Transmitter Spurious emission co-existence work Moderator: Wrong TR number used. It shall be TR 38.922. |

Recommended WF:

| R4-2521762 | ZTE Corporation | To be Not pursued |
| --- | --- | --- |
| R4-2521763 | ZTE Corporation | To be Not pursued |
| R4-2521785 | Ericsson | To be Revised Use Draft CR in R4-2521785 as baseline. |

### Sub-topic 2-4: LS to ETSI MSG TFES

Open issues and candidate options before meeting:

- Proposal 1: R4-2522180	Draft LS on the status of the work on TX spurious emission co-existence requirements limits for high frequency bands (R4-2522180, Huawei)

- Proposal 2: draft LS response to ETSI MSG TFES embedded in (R4-2521784, Ericsson)

Recommended WF: Revise Draft LS in R4-2522180 and merge contents of R4-2522180 and R4-2521784 in the formal LS.

# Topic #2: TRP estimation methods simplification

## Companies’ contributions summary

The following contributions were submitted to the meeting:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2522134 | Ericsson | CR to TS 38.141-2: Improvements on Annex I - TRP measurement procedures |
| R4-2520277 | Nokia | It is proposed to adapt the notes in ERC/REC 74-01 into annex I of TS 38.141-2 as shown (adjusted wordings are highlighted): For the wanted signal, if the directivity is known, TRP can be obtained by measuring the peak EIRP and adjusting it with the directivity to obtain TRP. Care shall be taken for using this method for unwanted emissions since its directivity can be different from the wanted signal directivity, in particular at large frequency separations from the carrier. If the directivity is not known but it can be verified that the emission is from the antenna array, TRP can be obtained by measuring EIRP in the cardinal cuts and using pattern multiplication to extrapolate the EIRP values in other directions. Pattern multiplication exploits the fact that the array factor of a rectangular array can be separable in two factors along the two symmetry planes. If none of the above options are applicable, the final option is to measure the EIRP on a full-sphere grid with the reference angular step. The measured values can then be integrated to obtain the TRP. All the above options are accurate methods for TRP assessment provided that the mentioned pre-conditions are fulfilled. For spurious emissions at larger frequency separations from the carrier, it is important to first identify the frequencies with notable emissions for further measurement. This can be achieved through a pre-scan which can be performed on a very sparse grid. If no notable emission is identified at a certain frequency during the pre-scan, no further investigation is required for that frequency. For frequencies which are identified for further measurement, a number of orthogonal cuts (two or three) or a sparse spherical grid can be used. These methods are based on the assumption that spurious emissions at larger frequency separation are not fully correlated and are less directive. For spurious emissions, an upper bound assessment for TRP can be used to ensure the compliance and the exact TRP value may not be required. Therefore, a systematic correction factor can be added to the estimated TRP value in order to ensure an overestimate with a certain confidence. The systematic correction factor is dependent on the chosen grid type and angular step. Orthogonal cut measurements shall be measured with the reference angular steps. The sparse full sphere measurement can be performed on angular steps larger than the reference step and smaller than 15 degrees. A larger angular step results in a shorter measurement time while it imposes a larger systematic correction. |
| R4-2520278 | Nokia | CR to TR 37.941 on definition of anechoic chamber |
| R4-2522175 | Huawei, HiSilicon | Concluding on remaining issues on the preferred OTA test methods and TRP simplifications Proposal 1: For the TRP annex modifications, focus at least on the General part of the Annex; most of other proposed corrections belong to Maintenance agenda (Rel-15 onwards). Proposal 2: For the TRP annex modifications on the pre-scan method, first decide if those modifications shall be capture in Rel-19, or rather from Rel-15 onwards (under Maintenance agenda). Proposal 3: For the General section of the TRP annex, reuse guidance text for the the selection of proper measurement methodology from Table 6 in Annex 2 of ERC Recommendation 74-01, with no additional modifications introduced. Proposal 4: Clarified whether the “systematic error” term used in ERC Recommendation 74-01 is the same as the ΔTRP correction factor already used in the TRP annex. Proposal 5: Send LS to ETSI MSG TFES, information that Rel-19 specifications TS 38.141-2 and TS 37.145-2 will incorporate agreements on the “preferred” test method for the TRP measurements. |
| R4-2522176 | Huawei, HiSilicon | Draft CR to TS 38.141-2: Improvements on Annex I on TRP measurement procedures |
| R4-2522177 | Huawei, HiSilicon | Draft CR to TS 37.145-2: implementation of the preferred test method and TRP annex improvements |
| R4-2522178 | Huawei, HiSilicon | Draft LS on implementation of the preferred test method for TRP measurements of the NR BS and AAS BS |

## Open issues summary

The following sub-topics were identified based on contributions listed in clause 2.1.

### Sub-topic 2-1: Reuse of notes from ERC/REC 74-01

Open issues and candidate options before meeting:

- Proposal 1: Adapt the notes in ERC/REC 74-01 into annex I of TS 38.141-2 as shown in R4-2520277 (R4-2520277, Nokia)

- Proposal 2: For the General section of the TRP annex, reuse guidance text for the selection of proper measurement methodology from Table 6 in Annex 2 of ERC Recommendation 74-01, with no additional modifications introduced. (R4-2522175, Huawei)

- Proposal 3: Clarify whether the “systematic error” term used in ERC Recommendation 74-01 is the same as the ΔTRP correction factor already used in the TRP annex. (R4-2522175, Huawei)

Recommended WF: Reuse guidance text for the selection of proper measurement methodology from Table 6 in Annex 2 of ERC Recommendation 74-01, with the following:

- Modifications required to reflect 3GPP drafting rules,

- Modification to follow existing terminology, i.e., “systematic error” vs. “ΔTRP correction factor”.

### Sub-topic 2-2: Other proposals

Open issues and candidate options before meeting:

- Proposal 1: For the TRP annex modifications, focus at least on the General part of the Annex; most of other proposed corrections belong to Maintenance agenda (Rel-15 onwards). (R4-2522175, Huawei)

- Proposal 2: For the TRP annex modifications on the pre-scan method, first decide if those modifications shall be capture in Rel-19, or rather from Rel-15 onwards (under Maintenance agenda). (R4-2522175, Huawei)

Recommended WF: The above 2 proposals were related to the Draft CR from RAN4$116bis, which was simplified in R4-2522134. No discussion, nor decision needed on those two proposals.

### Sub-topic 2-3: CRs

The following (Draft) CRs were submitted to the meeting:

| R4-2520278 | Nokia | CR to TR 37.941 on definition of anechoic chamber Moderator: This CR has the same changes as the endorsed draft CR in R4-2515072. |
| --- | --- | --- |
| R4-2522134 | Ericsson | CR to TS 38.141-2: Improvements on Annex I - TRP measurement procedures |
| R4-2522176 | Huawei, HiSilicon | Draft CR to TS 38.141-2: Improvements on Annex I on TRP measurement procedures |
| R4-2522177 | Huawei, HiSilicon | Draft CR to TS 37.145-2: implementation of the preferred test method and TRP annex improvements |

Recommended WF:

Clarify whether Draft CRs of formal CRs are handled this meeting.

| R4-2520278 | Nokia | Agree |
| --- | --- | --- |
| R4-2522134 | Ericsson | Revised: implement agreements based on Sub-topic 2-1. Align with CR to TS 37.145-2. |
| R4-2522176 | Huawei, HiSilicon | Not pursued |
| R4-2522177 | Huawei, HiSilicon | Revised: implement agreements based on Sub-topic 2-1. Align with CR to TS 37.145-2. |

### Sub-topic 2-4: LS to ETSI MSG TFES

Open issues and candidate options before meeting:

- Proposal 1: Send LS to ETSI MSG TFES, information that Rel-19 specifications TS 38.141-2 and TS 37.145-2 will incorporate agreements on the “preferred” test method for the TRP measurements. (R4-2522175, Huawei)

Recommended WF:

- Check if Proposal 1 is agreeable.

- Draft LS in R4-2522178 can be used baseline for further refinements, if any.