3GPP TSG-RAN WG4 Meeting #116	R4-2511776

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi, MediaTek Inc., OPPO

Title: 	WF on Modified MPR Behavior

Agenda item:	4.2.1

Document for:	Approval

- Background

Discussions and CRs are submitted in RAN4#116 to discuss the modified MPR behavior issue for TS 38.101-1. General rules for future releases and how to improve R19 and earlier releases are discussed. The agreements are captured in this WF.

- Common understanding and general rules for future releases

The followings are agreed to guide the handling on the modifiedMPR-Behavior bits in TS 38.101-1 from release 20.

- The following wording will be used in TS 38.101-1 and 38.101-2 spec to indicate whether the bit is set or not.

  - “The bit is set to 1” and “The bit is set to 0”

    - The following are clarifications for better understanding, but they will not be included in the spec.

      - If at least one  modifiedMPR bit in the bit string for the band is set to 1 by UE, UE shall send the entire bit string including the bits with value 0.

      - If all bits are set to 0, UE may omit sending the bit string and then network interprets all the bits in the bit string are set to 0.

- For the open release in which the MPR/AMPR requirements are modified, and also for the later releases, the new MPR/AMPR requirements are applied. For releases earlier than this modification, the new MPR/AMPR requirements may be applied depending on UE indication.

  - For example, if MPR requirement is changed/updated for R19 specification before Rel-19 ASN.1 freeze, the modifiedMPR-Behaviour bit “shall” be set to 1 for R19 UE. For early releases, the modifiedMPR-Behaviour bits “may” be set to 1.

  - Some exceptions can be discussed case by case and no impact to on-going product development is expected.

- If different UE types have different requirements, separate ModifiedMPR-Behavior bit is used.

- For spec version number, the followings are some candidates for further discussion in next meeting.

  - Keep the version number and add the following note outside the table.

    - NOTE 2: Specification version in the “Definition” column in Table L.1-1 refer to the version N in which the requirements were modified. If UE supports release P which is later than release N then UE supports requirements defined in that release P.

  - Remove the version number following the approach similar with 38.101-2.

  - Remove the version number and using wording like “MPR/AMPR defined in clause xxx of present specification”.

  - Other solutions are not excluded.

- If the requirements are changed (tighten or relaxation) more than once, further discuss the following Options:

  - Option 1: Use separate ModifiedMPR-Behavior bits for the second changed MPR/AMPR

    - Note: Error correction can be made but the modifiedMPR-Behaviour bit is kept unchanged

  - Other options are not excluded

- Improvement/correction of existing requirements R15-R19

The followings are agreed to guide the improvement/correction on the modifiedMPR-Behavior bits in TS 38.101-1 for R19 and the earlier releases.

- Wording correction to indicate whether the bit is set or not follows the agreements in clause 2.

- “shall” or “may” can be discussed by case by case in the CRs for releases earlier than the MPR/AMPR changed release.

- Spec version number handling follows the agreements in clause 2.

- n41 correction: Remove MPR in bit 2 can be a candidate for further check, others follow the above agreements.

| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
| --- | --- | --- | --- |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |

- The bits related to form factor FWA: Current structure is kept.

- FFS if some general clarification is necessary to be added in spec to show that value 0 means the old requirements are supported by UE.

- References

R4-2509810	Discussion on ModifiedMPR-Behavior and spec	Xiaomi

R4-2509898	(NR_newRAT-Core) R16 correction on modifiedMPR-Behaviour	OPPO

R4-2509899	(NR_newRAT-Core) R17 correction on modifiedMPR-Behaviour	OPPO

R4-2509900	(NR_newRAT-Core) R18 correction on modifiedMPR-Behaviour	OPPO

R4-2509901	(NR_newRAT-Core) R19 correction on modifiedMPR-Behaviour	OPPO

R4-2509902	(NR_newRAT-Core) R16 correction on modifiedMPR-Behaviour (refer to latest spec version)	OPPO

R4-2509903	(NR_newRAT-Core) R17 correction on modifiedMPR-Behaviour (refer to latest spec version)	OPPO

R4-2509904	(NR_newRAT-Core) R18 correction on modifiedMPR-Behaviour (refer to latest spec version)	OPPO

R4-2509905	(NR_newRAT-Core) R19 correction on modifiedMPR-Behaviour (refer to latest spec version)	OPPO

R4-2509906	R16 discussion on Modified MPR behaviour	OPPO

R4-2510282	Discussion on Modified MPR Behavior	vivo

R4-2510283	Draft CR 38.101-1 R19 cleanup for ModifiedMPR table	vivo

R4-2510284	Draft CR 38.101-1 R18 cleanup for ModifiedMPR table	vivo

R4-2510319	Discussion on ModifiedMPR-Behavior updates	MediaTek Korea Inc.

R4-2510369	Discussion on modifiedMPR	Nokia

R4-2510820	Discussion on how to capture the modifiedMPR-behaviour for n1	NTT DOCOMO, INC., KDDI

R4-2510928	Considerations on ModifiedMPR-Behaviour	ZTE Corporation,Sanechips

R4-2510954	Discussion on requirements for ModifiedMPR-Behaviour [Modified_MPR]	Ericsson

R4-2510955	CR to 38.101-1 Rel-16. To add indication of modified MPR behavior [Indicate_NS_05_NS_05U, Modified_MPR]	Ericsson, Samsung

R4-2511423	(TEI18) Corrections for ModifiedMPR Table [Modified_MPR]	Huawei, HiSilicon

R4-2511424	(TEI19) Corrections for ModifiedMPR Table [Modified_MPR]	Huawei, HiSilicon