3GPP TSG-RAN WG4 Meeting #116		R4-2511174
Bangalore, India, 25th – 29th Aug 2025

Title:	WF for AMPR alignment between FDD PC2 and PC3

Source:	OPPO, Skyworks, [ ]

Agenda item:	4.2.1

Document for:	Approval

- Introduction

Below captures the WF for AMPR alignment between FDD PC2 and PC3 for future FDD HPUE AMPR requirement definition.

- AMPR for PC2 compare with PC3

WF:

- The total power back off (OBOtotal(1T) and OBOtotal(2T) ) shall not be higher than 3dB for PC2 than PC3.

  - OBOtotal(1T) = max (MPRPC2(1T), A-MPRPC2(1Tx)) – max (MPRPC3(1T), A-MPRPC3(1T))

  - OBOtotal(2T) = max (MPRPC2(2T), A-MPRPC2(2Tx)) – max (MPRPC3(2T), A-MPRPC3(2T))

- AMPR for 2T compare with 1T in PC2

WF:

- One AMPR table apply for both PC2 1T and 2T.

- 0.7dB is applied for all modulation orders for 2T AMPR comparing to 1T AMPR to account for the RIMD impact if the OBOtotal(2T) is no larger than 3dB, otherwise, OBOtotal(2T) = 3dB is applied.

- AMPR regions between PC3 and PC2

WF:

- In future, when providing AMPR triangles, the MPR shall be subtracted from the results to align the pure AMPR region.

- When providing proposals for A-MPR values, companies are encouraged provide the delta total backoff (OBOtotal ) between PC2 and PC3 to facilitate cross check