3GPP TSG-RAN WG4 Meeting # 116											draft R4-2511800

Bengaluru, India, 25th -29th August, 2025

Agenda item:			7.26.1

Source:	Moderator (Huawei)

Title:	WF on requirements for A-IoT BS and CW

Document for:	Approval

Introduction

This document captures the agreements about the RF requirements for A-IoT BS and CW on Rel-19 WI solutions for Ambient IoT in NR (RP-243326). The topic summaries are R4-2500687 (RAN4#114), R4-2504688 (RAN4#114bis) , R4-2507574 (RAN4#115) and R4-2509102 (RAN4#116). The previous WFs are R4-2502859 (RAN4 #114), R4-2505097 (RAN4 #114bis) and R4-2508101 (RAN4#115).

Topic #1: A-IoT BS TX

- Issue 1-1: Modulation quality

- Agreement:

  - Parameters definition:

    - An is measured peak high level for the nth chip, in units of V/m or A/m

    - Bn is measured peak low level for the nth chip, in units of V/m or A/m

    - Anavg is the measured average high level for the nth chip during 1/2 duration above 90%An, in units of V/m or A/m

    - Bnavg is the measured average low level for the nth chip during 1/2 duration below 10%An, in units of V/m or A/m

  - Modulation Depth:

    - For each chip, Modulation depth =(Anavg-Bnavg)/Anavg

    - (Anavg–Bnavg)/Anavg>=80%

  - RF Envelope Ripple

    - Ripple_high (%) = ((An − Anavg) / (Anag-Bnavg)) × 100% <=±15%

    - Ripple_low (%) = ((Bn − Bnavg) / (Anavg-Bnavg)) × 100%<=±15%

  - RF Envelope Rise/Fall Time:

    - RF Envelop Rise Time: The time from 0.1 ×(Anavg-Bnavg) +Bnavg to 0.9 ×(Anavg-Bnavg)+Bnavg 	<=0.66Tc

    - RF Envelop Fall Time: The time from 0.9 ×(Anavg-Bnavg) +Bnavg to 0.1 ×(Anavg-Bnavg)+Bnavg		 <=0.66Tc

  - RF Pulsewidth:

    - The pulse width is the time between two points on the pulse where the signal reaches 50% of (Anavg-Bnavg)+Bnavg, PW <=1.3 Tc

- Table 1: A-IoT BS RF envelope parameters

| R2D Chip duration：Tc | Parameter | Symbol | Value | Units |
| --- | --- | --- | --- | --- |
| Tc=103M*15(us) M∈ {2,6,12,24} | Modulation Depth | (A–B)/A | 80 | % |
|  | RF Envelope Ripple | Ripple_high Ripple_low | <=±15 | % |
|  | RF Envelop Rise Time | Tr,10-90 | <=0.66Tc | µs |
|  | RF Envelop Fall Time | Tf,10-90 | <=0.66Tc | µs |
|  | RF Pulsewidth | PW | Option 1:<=1.3 Tc | µs |

Note: Modulation quality requirements can be further updated in the maintenance phase up to companies check.

- Issue 1-2: ACLR

- Agreement:

  - 100kHz offset, 40/45dB

- Issue 1-3: OBUE

- Agreement:

  - For 200 kHz R2D CBW, reuses the operating band unwanted emissions requirements of standalone NB-IoT Medium Range BS for A-IoT micro BS.

  - For 400/600/800kHz R2D CBW, use the operating band unwanted emissions requirements in Table 2 and 3.

Table 2 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power 31 < Prated,c  38 dBm , for 400/600/800kHz R2D CBW

| R2D CBW | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 400kHz | 0 MHz f < 0.4 MHz | 0.015 MHz f_offset < 0.415 MHz | Prated,c -40dB - 110.4 (f_offsetMHz - 0.015) dB | 30 kHz |
|  | 0.4 MHz f < 0.8 MHz | 0.415 MHz f_offset < 0.815 MHz | Prated,c - 51dB- 50.4 (f_offsetMHz-0.415) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c - 56dB | 30 kHz |
|  | 1.6 MHz f < fmax | 1.6 MHz f_offset < f_offsetmax | -25dBm | 100kHz |
| 600kHz | 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | Prated,c – 40dB - 130.6 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.6 MHz f < 1.2 MHz | 0.615 MHz f_offset < 1.2 MHz | Prated,c – 53dB - 50.6 (f_offsetMHz-0.615) dB | 30 kHz |
|  | 1.2 MHz f < 1.8 MHz | 1.2 MHz f_offset < 1.8 MHz | Prated,c - 58dB | 30 kHz |
|  | 1.8 MHz f < fmax | 1.8 MHz f_offset < f_offsetmax | -25dBm | 100K |
| 800kHz | 0 MHz f < 0.8 MHz | 0.015 MHz f_offset < 0.815 MHz | Prated,c – 40dB- 140.8 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c – 54dB- 50.8 (f_offsetMHz-0.815) dB | 30 kHz |
|  | 1.6 MHz f < 2.4 MHz | 1.6 MHz f_offset < 2.4 MHz | Prated,c - 59dB | 30 kHz |
|  | 2.4 MHz f < fmax | 2.4 MHz f_offset < f_offsetmax | -25dBm | 100K |

Table 3 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power Prated,c  31 dBm , for 400/600/800kHz R2D CBW

| R2D CBW | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 400kHz | 0 MHz f < 0.4 MHz | 0.015 MHz f_offset < 0.415 MHz | -9dBm - 110.4 (f_offsetMHz - 0.015) dB | 30 kHz |
|  | 0.4 MHz f < 0.8 MHz | 0.415 MHz f_offset < 0.815 MHz | -20dBm- 50.4 (f_offsetMHz-0.415) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | -25dBm | 30 kHz |
|  | 1.6 MHz f < fmax | 1.6 MHz f_offset < f_offsetmax | -25dBm | 100kHz |
| 600kHz | 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | -9dBm - 130.6 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.6 MHz f < 1.2 MHz | 0.615 MHz f_offset < 1.2 MHz | -22dBm - 50.6 (f_offsetMHz-0.615) dB | 30 kHz |
|  | 1.2 MHz f < 1.8 MHz | 1.2 MHz f_offset < 1.8 MHz | -27dBm | 30 kHz |
|  | 1.8 MHz f < fmax | 1.8 MHz f_offset < f_offsetmax | -25dBm | 100K |
| 800kHz | 0 MHz f < 0.8 MHz | 0.015 MHz f_offset < 0.815 MHz | -9dBm - 140.8 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | -23dBm - 50.8 (f_offsetMHz-0.815) dB | 30 kHz |
|  | 1.6 MHz f < 2.4 MHz | 1.6 MHz f_offset < 2.4 MHz | -28dBm | 30 kHz |
|  | 2.4 MHz f < fmax | 2.4 MHz f_offset < f_offsetmax | -25dBm | 100K |

Note: OBUE requirements can be further updated in the maintenance phase up to companies check.

Topic #2: A-IoT BS RX

- Issue 2-1: Reference sensitivity level

- Agreement:

  - CW testing signal source: real CW node

  - CW input level to BS Antenna: -38dBm

  - Desens target 30dB

  - Note： The reference sensitivity power level is derived based on single RX and CW phase noise cancellation is not included.

- Issue 2-2: SNR value

- Agreement:

  - SNR for BPSK is -5dB

  - If BPSK reference sensitivity is X dBm，at the same other side condition, OOK sensitivity is  X+3dB

- Issue 2-3: Dynamic range

- Agreement:

  - No need to define the dynamic range requirement

- Issue 2-4: ACS

- Agreement:

- Table 4: Adjacent channel selectivity for A-IoT Medium Range BS

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note) | -53 | ±100 | 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | PREFSENS + 6dB (Note) | -53 | ±100 | 3 MHz DFT-s-OFDM NR signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table X |  |  |  |  |

- Issue 2-6: General intermodulation

- Agreement:

  - No need.

- Issue 2-7: Narrowband intermodulation

- Agreement:

  - Adopt the following narrow band IMD requirements.

Table 5-1: Narrowband blocking requirement for A-IoT Medium Range BS

|  | NB-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] (Note) | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| Medium Range BS | 200 | PREFSENS + 6dB | -53 | See Table 7.5.1-2a |
|  | 3520 | PREFSENS + 6 dB | -53 | 3 MHz DFT-s-OFDM NR signal |
| Note : PREFSENS depends on the sub-carrier spacing as specified in Table |  |  |  |  |

Table 5-2: Interfering signal for Narrowband blocking requirement for A-IoT Medium Range BS

| NB-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- |
| 200 | ±(240 +m*180), m=0, 1, 2, 3, 4, 9, 14 | 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 3520 | ±(240 +m*180), m=0, 1, 2, 3, 4, 9, 14 | 3 MHz DFT-s-OFDM NR signal |
| Note*: Interfering signal consisting of one resource block is positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |

Topic #3: CW

- Issue 3-1: OFF power MBW

- Agreement:

  - OFF power MBW is 1MHz

- Issue 3-2: Phase noise

- Agreement:

Table 6 Phase noise requirement for the CW node

| Frequency distance toward the CW centre | Phase noise (dBc/Hz) |
| --- | --- |
| 7.5kHz | -97 |
| 120kHz | -102 |

- Issue 3-3: Unwanted emission

- Agreement:

  - Adopt the following proposal for CW node.

- Table 7: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 200 | -18 | 30 kHz |
| 250 | -20 | 30 kHz |
| 350 | -25 | 30 kHz |
| 800 | -26 | 30 kHz |
| 1200 | -19 | 1 MHz |
| 5200~fmax | -23 | 1 MHz |

- Issue 3-4: CW channel bandwidth

- Agreement:

  - No need to define.

- Issue 3-6: CW others

- Agreement:

  - No need to specify the timing requirement for CW node in RF spec. It can be discussed under RRM.

  - Discuss how to capture “CW transmission and A-IoT BS downlink data transmission are non-concurrent”.

Topic #4: draft TP	 to TS38.914

- Agreement:

  - Endorse the revised TPs.

| No. | Section in TS38914 | Volunteer company | RAN4#116 TPs submitted | Revised to |
| --- | --- | --- | --- | --- |
| 1 | 5.3 BS channel bandwidth 5.4 Channel arrangement | CMCC | R4-2509717, CMCC（submitted in 7.22.2） |  |
| 2 | 6.1 General 6.2 Base station output power 6.3 Transmit ON/OFF power | CATT, LGE | R4-2509330，CATT R4-2510079, LGE |  |
| 3 | 6.4 Transmitted signal quality | Huawei，ZTE | R4-2509884，Huawei |  |
| 4 | 6.5 Unwanted emissions 6.6 Transmitter intermodulation | CMCC，CATT | R4-2509331，CATT R4-2509718，CMCC |  |
| 5 | 7.1 General 7.2 Reference sensitivity level 7.3 Dynamic range | ZTE, Huawei | R4-2511127，ZTE R4-2509885，Huawei |  |
| 6 | 7.4 In-band selectivity and blocking 7.5 Out-of-band blocking | Ericsson | R4-2511433, Ericsson |  |
| 7 | 7.6 Receiver spurious emissions 7.7 Receiver intermodulation 7.8 In-channel selectivity | Ericsson, ZTE | R4-2511433, Ericsson |  |
| 8 | 8.1 General 8.2 CW Output power | Huawei, OPPO | R4-2511290, Huawei |  |
| 9 | 8.3 Frequency error 8.4 Unwanted emission | Vivo，ZTE | R4-2510252, Vivo |  |