3GPP TSG-RAN4 Meeting #116	R4-251XXXX

# Bengaluru, India, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for L1-CLI-RSSI measurement period |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | L1 CLI measurement requirements are introduced in Rel-19 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement period requirement of L1-SRS-RSRP measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No measurement period requirement of L1-SRS-RSRP measurement which introded in Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

9.X.3.5	L1-CLI-RSSI measurement period

The UE capable of supportL1-CLI-RSSI-Measurement shall perform L1-CLI-RSSI measurement based on the configured L1-CLI-RSSI measurement resource. For periodic and semi-persistent SRS resource, the UE shall be capable of reporting L1-CLI-RSSI measured over measurement period of TL1-CLI_RSSI_measurement_period for FR1 and FR2.

Table 9.X.3.5-1 Measurement period TL1-CLI_RSSI_measurement_period

| Configuration | TL1-CLI_RSSI measurement_period (ms) |
| --- | --- |
| No DRX | Max(TReport, (M+L1) X TCLI-RSSI) |
| DRX cycle ≤ 320ms | Max(TReport, Ceil(1.5 X (M+L1)) X max(TCLI-RSSI, TDRX)) |
| DRX cycle > 320ms | (M+L1) X TDRX |
| NOTE 1: TCLI-RSSI is L1-CLI-RSSI measurement periodicity configured RSSI-PeriodicityAndOffset, and TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. NOTE 3: L1=0 if higher layer parameter timeRestrictionForChannelMeasurement is configured. Otherwise, when DRX is not configured L1 is the number of invalid symbol type at the UE during TL1-CLI_RSSI_measurement_period, and when DRX is configured L1 is the number of DRX cycles in which the symbol is not valid at the UE during TL1-CLI_RSSI_measurement_period. |  |

For FR2, if the configured L1-CLI-RSSI measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TL1-CLI_RSSI_measurement_period.

For FR1, if the configured L1-CLI-RSSI measurement resources are partially or fully overlapping with measurement gaps or SSB for L1/L3 measurement and configured with a different SCS, requirements are not specified for TL1-CLI_RSSI_measurement_period.

----------------------END OF CHANGE----------------------------